3GPP TSG RAN WG1 Meeting #96bis	R1-1905906

Xi’an, China,  8th April – 12th April, 2019

Title:	LS on sidelink HARQ feedback for groupcast

Response to:

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	RAN1

To:	RAN2

Contact Person:

Name:	Hanbyul Seo

E-mail Address:	hanbyul.seo@lge.com

Attachments:	None

Introduction

RAN1 discussed HARQ procedures for sidelink groupcast and would like to inform RAN2 that RAN1 agreed to support the following two options for the sidelink HARQ feedback from the receiver UEs to the transmitter UE when HARQ feedback is enabled for groupcast:

- Option 1: Receiver UE transmits HARQ-NACK on PSFCH if it fails to decode the corresponding TB after decoding the associated PSCCH. It transmits no signal on PSFCH otherwise.

- Option 2: Receiver UE transmits HARQ-ACK on PSFCH if it successfully decodes the corresponding TB. It transmits HARQ-NACK on PSFCH if it does not successfully decode the corresponding TB after decoding the associated PSCCH which targets the receiver UE.

In addition, RAN1 made the following agreement on the operation of the two options:

- In HARQ feedback for groupcast,

  - When Option 1 is used for a groupcast transmission, it is supported

    - all the receiver UEs share a PSFCH

    - FFS: a subset of the receiver UEs share a PSFCH

    - FFS: all or a subset of receiver UEs share a pool of PSFCH.

  - When Option 2 is used for a groupcast transmission, it is supported

    - each receiver UE uses a separate PSFCH for HARQ ACK/NACK.

    - FFS: all or a subset of receiver UEs share a PSFCH for ACK transmission and another PSFCH for NACK transmission

  - FFS on which entity and how to allocate PSFCH resource to the receiver UE(s)

  - FFS whether or not to additionally support a mixture of option 1 and option 2 for a groupcast transmission

- Note: Each PSFCH is mapped to a time, frequency, and code resource.

Actions to RAN2

RAN1 respectfully requests RAN2 to take the above into account in the related work.

Date of next TSG-RAN WG1 meetings:

TSG-RAN WG1 Meeting#97,			13th May to 17th May 2019, 				Reno, Nevada, USA.

TSG-RAN WG1 Meeting#98,			26th August to 30th August 2019,			Prague, Czech Republic.