3GPP TSG RAN WG1 #97	R1-1906394

Reno, USA, 13th – 17th May, 2019

Agenda Item:	7.2.4.5

Source: 	NEC

Title:	Physical layer procedures for NR sidelink

Document for:	Discussion/Decision

Introduction

The work item of 5G V2X with NR sidelink was approved in RAN#83[1], in RAN1#96bis, physical layer procedures were discussed with following agreements and working assumption [2]:

Agreements:

- Confirm the following working assumption:

  - Working assumption:

    - When HARQ feedback is enabled for groupcast, support (options as identified in RAN1#95):

      - Option 1: Receiver UE transmits only HARQ NACK

      - Option 2: Receiver UE transmits HARQ ACK/NACK

- Note: RAN1 has not concluded the respective applicability of option 1 vs. option 2 yet

- Send LS to RAN2 to inform RAN1’s agreement on HARQ feedback for groupcast – draft LS to be prepared in R1-1905790 (Hanbyul, LGE),which is approved with final LS in R1-1905906

Agreements:

- In HARQ feedback for groupcast,

  - When Option 1 is used for a groupcast transmission, it is supported

    - all the receiver UEs share a PSFCH

    - FFS: a subset of the receiver UEs share a PSFCH

    - FFS: all or a subset of receiver UEs share a pool of PSFCH.

  - When Option 2 is used for a groupcast transmission, it is supported

    - each receiver UE uses a separate PSFCH for HARQ ACK/NACK.

    - FFS: all or a subset of receiver UEs share a PSFCH for ACK transmission and another PSFCH for NACK transmission

  - FFS on which entity and how to allocate PSFCH resource to the receiver UE(s)

  - FFS whether or not to additionally support a mixture of option 1 and option 2 for a groupcast transmission

- Note: Each PSFCH is mapped to a time, frequency, and code resource.

Working assumption:

- Regarding the use of TX-RX geographical distance and/or RSRP in determining whether to send HARQ feedback for groupcast

  - Support at least the use of TX-RX geographical distance

    - FFS whether or not to additionally use L1-RSRP

- Companies are encouraged to perform additional evaulations/analysis

Agreements:

- It is supported, in a resource pool, that within the slots associated with the resource pool, PSFCH resources can be (pre)configured periodically with a period of N slot(s)

  - N is configurable, with the following values

    - 1

    - At least one more value >1

      - FFS details

  - The configuration should also include the possibility of no resource for PSFCH. In this case, HARQ feedback for all transmissions in the resource pool is disabled

- HARQ feedback for transmissions in a resource pool can only be sent on PSFCH in the same resource pool

Agreements:

- Support at least Sidelink CSI-RS for CQI/RI measurement

  - Sidelink CSI-RS is confined within the PSSCH transmission

In this contribution, we discuss and give our views on HARQ procedures, CSI acquisition for unicast and power control.

Discussion

HARQ procedures

Layer 1 IDs to be conveyed via SCI

As agreed in SI phase, for support of HARQ procedure in unicast and groupcast, information such as layer-1 destination ID, layer-1 source ID, HARQ process ID, NDI and RV are included in the SCI. However, it is still open on how to determine the layer-1 source ID and layer-1 destination ID, and respective size.

The functionality of destination ID is to identify the receiver at PHY layer, this Layer-1 ID can be derived from higher layer provided destination ID, e.g. 8 LSB of higher layer destination ID.  The size of the layer-1 destination ID should be decided properly to fit into the capacity of SCI and meanwhile ensure an acceptable level of layer-1 destination ID collision (i.e. multiple receivers using same layer-1 destination ID).

If one receiver can have more than one unicast sessions, as shown in Figure 1, or there is layer-1 destination ID collision, the receiver can receive transmissions with same destination ID from different transmitters. In this case, it is crucial for the receiver to identify which transmitter the received transmission is from. For this purpose, the Layer-1 source IDs used by all transmitters that can be received by a given UE have to be different.  Hence, the Layer-1 source ID should not be derived from the higher layer source ID as Layer-1 destination ID, otherwise the possibility of source ID collision may be not trivial unless the size of Layer-1 source ID is large enough. To address the source ID collision issue, the Layer-1 source ID should be designated by the receiver. Moreover, in case of the size of Layer-1 source ID is large, Layer-1 source ID can be omitted if the receiver has only one unicast session such as to reduce the SCI size.

Figure 1 One receiver having 2 unicast links simultaneously

Proposal 1:

- The Layer-1 source ID should be designated by the receiver UE.

HARQ feedback in groupcast

As agreed in the last meeting, both Option 1 and Option 2 are supported for HARQ feedback in groupcast.

In Option 1 a receiver UE only feedback HARQ-NACK on PSFCH if it fails to decode the corresponding TB, and transmit no signal otherwise. This option can minimize PSFCH transmissions, and it allows multiple receiver UEs in the group using same PSFCH resource, and hence it is beneficial to reduce PSFCH resource overhead on sidelink carrier. However, this option cannot identify some receivers that missed the PSCCH (which will not feedback either NACK or ACK), and this can eventually degrade reliability of PSSCH.

There can be 2 sub-options for Option 2:

- Option 2-1: Two PSFCH resources, one receiver transmits a HARQ-ACK on one resource if it successfully decodes the corresponding TB, and transmit HARQ-NACK on the other resource if it fails to decode the corresponding TB after decoding the associated PSCCH;

- Option 2-2: dedicated resource for each receiver, one receiver transmits HARQ-ACK or HARQ-NACK according to decoding result of the corresponding TB;

Option 2-1 has similar advantage and same issue (cannot identify DTX receivers) as Option 1. Option 2-2 can fully avoid the DTX issue at the cost of heavier PSFCH resources overhead. Hence, in our view, in a groupcast transmission, mixture Option1 and Option 2 (i.e. part of receiver UEs in groupcast session using Option 1 and others using Option 2-2) can be supported such that the two options can compensate each other.

Proposal 2:

- In a groupcast transmission, mixture of Option1 and Option 2 should be supported;

CSI acquisition

Following working assumption was made in RAN1#96 meeting [3]:

Working assumption: - For unicast, the following CSI reporting is supported based on non-subband-based aperiodic CSI reporting mechanism assuming no more than 4-port: - CQI - RI - PMI - CSI reporting can be enabled and disabled by configuration. - It is supported to configure a subset of the above metric for CSI reporting. - There is no standalone RS transmission dedicated to CSI reporting in Rel-16 - NR sidelink CSI strives to reuse the CSI framework for NR Uu. - Discuss details during WI phase

Considering the conclusion in the last RAN plenary, the working assumption should be confirmed for CQI/RI reporting.

Proposal 3:

- Confirm the working assumption on CSI reporting made in RAN#96 with restriction that CSI reporting only includes CQI and RI.

In the unicast session, CSI is fed back from the UE that receiving unicast data (secondary UE) to the UE that transmitting the unicast data (Primary UE). In case of CSI reporting is enabled by configuration, the secondary UE should report CSI according to the request from the primary UE, one bit in SCI can be used for this purpose. Moreover, the secondary UE should be allowed to report CSI autonomously if the channel status has been changed significantly, e.g. the rank has been changed from 2 to 1, such that the primary UE can adapt the transmission scheme accordingly.

Proposal 4:

- Use 1 bit in SCI to trigger aperiodic CSI feedback;

- Under certain conditions, UE should be allowed to feedback sidelink CSI autonomously;

In LTE sidelink, transmission resource pool and reception resource pool are separately configured. If this logic is reused in NR sidelink, the PSSCH carrying CSI will be transmitted in one of (pre-) configured transmission resource pools of the secondary UE. The transmission resource pool for CSI feedback has to be (pre-) configured as reception resource pool of the primary UE, otherwise the CSI feedback cannot be received. The transmission resource pool for CSI should be on the same carrier as the transmission resource pool for the associated unicast, from resource efficiency point of view, the transmission resource pool for CSI should be fully overlapped with the transmission resource pool for the associated unicast.

Proposal 5:

- Transmission resource pool for a unicast should be (pre-)configured as transmission resource pool for associated CSI feedback;

Open loop power control

As agreed in RAN1 Ad-Hoc Meeting 1901[2], open loop power control based on SL pathloss (pathloss between TX UE and RX UE) is supported at least for unicast. This mechanism should also be applied to groupcast, as in some groupcast use cases, e.g. platooning [4], the transmitter and receivers may be very close to each other, lower transmission power may suffice for reliable reception, while higher transmission power may cause more serious interference and impair the entire system. And the largest pathloss between transmitter and group members can be used for the transmission power calculation.

Furthermore, in addition to pathloss, open loop power control parameters (i.e. p0 and alpha) are also needed to decide the initial transmission power. For SL pathloss based open loop power control, as sidelink CSI and HARQ are not reported to gNB, and the gNB may not monitor sidelink signals also, gNB has no information to decide the value of P0 and alpha. As there are some information exchanged between transmitter and the receiver to set up unicast/groupcast session, the receiver is able to measure the received SINR based on these received signals from the transmitter. By comparing the current received SINR and the expected SINR, the receiver can decide the accurate values of the open loop power control parameters. The values can be provided to the transmitter via sidelink signaling (e.g. the signaling for SL-RSRP feedback) for the transmission power calculation.

Proposal 6:

- SL pathloss based open loop power control is supported for groupcast;

- Parameters (i.e. p0 and alpha) for SL pathloss based open loop power control should be provided by the receiver via sidelink signalling.

As sidelink transmission may be subject to open loop power control based on both Uu pathloss (pathloss between TX UE and gNB) and SL pathloss, these 2 power control processes should be carried out independently, however, to protect Uu operation, the actual transmission power should be upper bounded by transmission power calculated according to the power control based on Uu pathloss.

Proposal 7:

- Open loop power control based on Uu pathloss and open loop power control based on SL pathloss should be carried out independently, and actual transmission power should be upper bounded by the transmission power calculated according to the power control based on Uu pathloss.

Conclusion

In this contributions, we discussed the issues related to physical layer procedures, we have following proposals:

Proposal 1:

- The Layer-1 source ID should be designated by the receiver UE.

Proposal 2:

- In a groupcast transmission, mixture of Option1 and Option 2 should be supported;

Proposal 3:

- Confirm the working assumption on CSI reporting made in RAN#96 with restriction that CSI reporting only includes CQI and RI.

Proposal 4:

- Use 1 bit in SCI to trigger aperiodic CSI feedback;

- Under certain conditions, UE should be allowed to feedback sidelink CSI autonomously;

Proposal 5:

- Transmission resource pool for a unicast should be (pre-)configured as transmission resource pool for associated CSI feedback;

Proposal 6:

- SL pathloss based open loop power control is supported for groupcast;

- Parameters (i.e. p0 and alpha) for SL pathloss based open loop power control should be provided by the receiver via sidelink signalling.

Proposal 7:

- Open loop power control based on Uu pathloss and open loop power control based on SL pathloss should be carried out independently, and actual transmission power should be upper bounded by the transmission power calculated according to the power control based on Uu pathloss.

References

- RP-190766, New WID on 5G V2X with NR sidelink, LG Electronics, Huawei;

- Chairman's Notes RAN1#96bis;

- Chairman's Notes RAN1 Ad-Hoc Meeting 1901;

- Chairman's Notes RAN1#96;

- 3GPP TR 22.886 V16.0.0, “3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on enhancement of 3GPP Support for 5G V2X Services (Release 16)”;