3GPP TSG RAN WG1 Meeting #97								R1-1907869

# Reno, USA, 13th – 17th May 2019

Title:	LS on Reference Point for Timing Related Measurements

Release:	Rel-16

Work Item:	NR_POS

Source:	RAN1

To:	RAN4

Cc:	RAN3

Contact Person:

#### Name:	Ren Da

#### E-mail Address:	renda@catt.cn

Attachments:	None

1. Overall Description:

RAN1 is currently working on the definition of reference point of UE and gNB measurements for NR positioning for Rel16.

For the reference point of RSTD measurements in FR1, RAN1 has decided that the reference point of the RSTD measurement is the Rx antenna connector of the UE.

For the reference point of RSTD measurements in FR2, RAN1 has considered the following two options for the definition of the reference point of RSTD measurements:

- Option 1: the combined signal from antenna elements corresponding to a given receiver

- Option 2: the Rx antenna of the UE

In addition, it is also RAN1’s view that it would be better for RAN4 to make the final decision on the reference point of the RSTD measurement in FR2. It is RAN1’s understanding that RAN4 may also consider other options.

In addition, it is RAN1’s view that the definition of the reference point of the RSTD measurement, if decided, can be extended to other timing related UE and gNB measurements in FR2.

It is RAN1’s understanding that it is part of the work of the RAN3 WG to provide a name for the “transmission measurement function”. For simplicity of the discussion in RAN1 this function is referred to as “gNB”

2. Actions:

To RAN4: 	RAN1 kindly requests RAN4 to provide the definition of a reference point for RSTD measurements in FR2 and inform RAN1 whether it is applicable to other timing related measurements, namely, the UE Rx-Tx timing difference, gNB Rx-Tx timing difference and UL RTOA. If the provided definition for the reference point is not applicable to these other timing related measurements in FR2, RAN1 requests RAN4 to provide definitions for the reference points for these other timing related measurements in FR2.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #98 	26 – 30 Aug 2019			Prague, Czech Republic

TSG-RAN WG1 Meeting #98bis                       14 – 18 October 2019    		China