3GPP TSG RAN WG1 meeting #98						 	R1-1908013

Prague, CZ, August 26 - 30, 2019

Agenda Item: 7.2.5.1

Source: MediaTek

Title: On beam layout definition and system level performance for NR-NTN

Document for:	Discussion and decision

# Introduction

For NR NTN, potential impacts have been identified in TR 38.811 “Study on NR to support non-terrestrial networks”. Solutions are currently being investigated in RAN1 meeting and captured in TR 38.821 “Solutions on NR to support non-terrestrial networks”. For NTN, RAN1 TUs have been allocated starting from April 2019 to November 2019.

In RAN1#97 meeting, agreements on the following items are given in [1]

- Satellite parameters for system level simulations

- UE characteristics for system level simulations

- Beam layout definition

There are two sets of satellite parameters agreed in [1]. In this contribution, we provide our views on beam layout definition for satellites with Set-1 and Set-2 parameters, and show our simulation results on SINR and DL throughput distribution.

# Beam layout definition

Our beam layout is generated by the following procedure:

- Specify beam centers on UV plane

- Transform the uv beam centers to ECEF coordinates

- The contour of a beam is defined by the 3dB beam width based on the satellite antenna pattern [2]

The ECEF coordinates is illustrated in Figure 1. A point in UV plane has an 1-to-1 correspondence to a point in ECEF coordinates. Specifically, the relation between UV plane and ECEF coordinates is described below.

Figure 1: ECEF coordinates

Let [X, Y, Z] be a point in ECEF coordinates which lies on the surface of Earth. Let x, y, z=X, Y, Z/R, where R = 6371km is the radius of Earth, then the corresponding point [u, v] in the UV plane is given by

u=0.5+tan-1(y/x)2π  and  v=0.5-sin-1zπ.

Conversely, for any point [u,v] in UV plane, the corresponding point [X, Y, Z] in the ECEF coordinates is given by

X=R⋅cos0.5-v⋅π⋅cosu-0.5⋅2π

Y=R⋅cos0.5-v⋅π⋅sinu-0.5⋅2π

Z=R⋅sin0.5-v⋅π

Basically [u, v] corresponds to [ϕ, θ] as shown in Figure 1, the difference is that u and v are both shifted and scaled into the value range [0, 1].

We assume that the satellite is right above the point [6371, 0, 0] in ECEF. As such, the positions of the satellite in LEO-600 and LEO-1200 scenarios are [6371+600, 0, 0] and [6371+1200, 0, 0], respectively. Note that [0.5, 0.5] in UV plane corresponds to [6371, 0, 0] in ECEF.

## Beam layout for Set-1 satellite parameters

For LEO-600 and LEO-1200 scenarios with Set-1 satellite parameters, the beam centers in UV plane is shown in Figure 2. Note that the center beam is located at [0.5, 0.5] and the adjacent beam spacing (ABS) is a constant in each scenario. For LEO-600 and LEO-1200, the ABS is set to 0.0013 and 0.0025, respectively.

(a)                                                                                 (b)

Figure 2: beam centers on UV plane for Set-1; (a) LEO-600; (b) LEO-1200

Central beam center elevation angle is set to be 90 degrees. The corresponding beam centers and beam layouts on ECEF are shown in Figure 3.

(a)                                                                                 (b)

Figure 3: beam centers and beam layouts on ECEF for Set 1; (a) LEO-600; (b) LEO-1200

Note that we form 61 beams (4-tiers) in total, but for simulation, UE are dropped only in central 19 beams.

Proposal 1: Adopt the beam layout definition as shown in Figure 2-3 for LEO-600 and LEO-1200 scenario with Set-1 satellite parameters

We have the following proposal:

## Beam layout for Set-2 satellite parameters

For LEO-600 and LEO-1200 scenarios with Set-2 satellite parameters, the beam centers in UV plane is shown in Figure 4. Again, the center beam is located at [0.5, 0.5] and the ABS is a constant in each scenario. For LEO-600 and LEO-1200, the ABS is set to 0.0026 and 0.0052, respectively

(a)                                                                                 (b)

Figure 4: beam centers on UV plane for Set-2; (a) LEO-600; (b) LEO-1200

Central beam center elevation angle is set to be 90 degrees. The corresponding beam centers and beam layouts on ECEF are shown in Figure 5.

(a)                                                                                 (b)

Figure 5: beam centers and beam layouts on ECEF for Set 2; (a) LEO-600; (b) LEO-1200

Proposal 2: Adopt the beam layout definition as shown in Figure 4-5 for LEO-600 and LEO-1200 scenario with Set-2 satellite parameters

We have the following proposal:

# Simulation Results

Basically we follow the channel model in TR 38.811 and the settings in RAN1#97 agreements. Additional simulation settings that need to be specified are given in Appendix.

From SLS we obtain SINR distribution. For DL throughput distribution, we perform SLS + LLS simulation to obtain the results, i.e., for each UE we run LLS to simulate its DL data reception with channel information generated by SLS.

## Simulation results for Set-1 satellite parameters

For LEO-600 with Set-1 parameters, the SINR and throughput distribution are shown in the following Figure:

Figure 6: SINR and throughput distribution for LEO-600 with Set-1 parameters

For LEO-1200 with Set-1 parameters, the SINR and throughput distribution are shown in the following Figure:

Figure 7: SINR and throughput distribution for LEO-1200 with Set-1 parameters

We have the following statistics from the above simulation result:

| LEO-600 Set-1 | SINR (dB) | Throughput (Mbps) |
| --- | --- | --- |
| Frequency reuse factor 1 | 80%UE: -1.226 | Avg: 8.3 80%UE: 10.3 |
| Frequency reuse factor 3 | 80%UE: 5.884 | Avg: 6.18 80%UE: 7.6 |

Table 1: SINR and throughput statistics for LEO-600 with Set-1 parameters

| LEO-1200 Set-1 | SINR (dB) | Throughput (Mbps) |
| --- | --- | --- |
| Frequency reuse factor 1 | 80%UE: -1.723 | Avg: 7.7 80%UE: 9.7 |
| Frequency reuse factor 3 | 80%UE: 5.536 | Avg: 6.01 80%UE: 7.11 |

Table 2: SINR and throughput statistics for LEO-1200 with Set-1 parameters

Observation 1: For LEO-600 with Set-1 parameters, for 80% percentile UE, we have 7.26dB SINR gain and 2.59 Mbps throughput drop by using frequency reuse factor 3 instead of using frequency reuse factor 1 deployment. This shows a trade-off between SINR and DL throughput performance. Similar results can be observed in LEO-1200 scenario

From the simulation results, we have the following observation:

## Simulation results for Set-2 satellite parameters

For LEO-600 with Set-2 parameters, the SINR and throughput distribution are shown in the following Figure:

Figure 8: SINR and throughput distribution for LEO-600 with Set-2 parameters

For LEO-1200 with Set-2 parameters, the SINR and throughput distribution are shown in the following Figure:

Figure 9: SINR and throughput distribution for LEO-1200 with Set-2 parameters

We have the following statistics from the above simulation results:

| LEO-600 Set-2 | SINR (dB) | Throughput (Mbps) |
| --- | --- | --- |
| Frequency reuse factor 1 | 80%UE: -2.608 | Avg: 5.82 80%UE: 7.017 |
| Frequency reuse factor 3 | 80%UE: 1.709 | Avg: 3.7 80%UE: 4.362 |

Table 3: SINR and throughput statistics for LEO-600 with Set-2 parameters

| LEO-1200 Set-2 | SINR (dB) | Throughput (Mbps) |
| --- | --- | --- |
| Frequency reuse factor 1 | 80%UE: -2.641 | Avg: 5.78 80%UE: 7.017 |
| Frequency reuse factor 3 | 80%UE: 1.757 | Avg: 3.68 80%UE: 4.261 |

Table 4: SINR and throughput statistics for LEO-1200 with Set-2 parameters

Observation 3: Frequency reuse factor 3 is more robust than frequency reuse factor 1. With frequency reuse factor 1, UEs with SINR ~- 3dB are unlikely to get much throughput due to poor synchronization, AFC, and channel estimation. In addition, such UEs may suffer high latency if many HARQ re-transmissions are required

Observation 2: For LEO-600 with Set-2 parameters, for 80% percentile UE, we have 4.4dB SINR gain and 2.76 Mbps throughput drop by using frequency reuse factor 3 instead of using frequency reuse factor 1 deployment. Similar results can be observed in LEO-1200 scenario

From the simulation results, we have the following observation:

Proposal 3: Support frequency reuse factor 3 for NR-NTN performance evaluation and deployment. FFS frequency reuse factor 1

From system viewpoint, as many UEs as possible should work with an operational SINR ~4 dB  to avoid not able to get service with reasonable QoS. Thus, we make the following proposal:

# Conclusions

The observations and proposals are summarized below:

Proposal 1: Adopt the beam layout definition as shown in Figure 2-3 for LEO-600 and LEO-1200 scenario with Set-1 satellite parameters

Proposal 2: Adopt the beam layout definition as shown in Figure 4-5 for LEO-600 and LEO-1200 scenario with Set-2 satellite parameters

Observation 1: For LEO-600 with Set-1 parameters, for 80% percentile UE, we have 7.26dB SINR gain and 2.59 Mbps throughput drop by using frequency reuse factor 3 instead of using frequency reuse factor 1 deployment. This shows a trade-off between SINR and DL throughput performance. Similar results can be observed in LEO-1200 scenario

Observation 2: For LEO-600 with Set-2 parameters, for 80% percentile UE, we have 4.4dB SINR gain and 2.76 Mbps throughput drop by using frequency reuse factor 3 instead of using frequency reuse factor 1 deployment. results can be observed in LEO-1200 scenario

Observation 3: Frequency reuse factor 3 is more robust than frequency reuse factor 1. With frequency reuse factor 1, UEs with SINR ~- 3dB are unlikely to get much throughput due to  poor synchronization, AFC, and channel estimation. In addition, such UEs may suffer high latency if many HARQ re-transmissions are required

Proposal 3: Support frequency reuse factor 3 for NR-NTN performance evaluation and deployment. FFS frequency reuse factor 1

# Reference

- Chairman's Notes RAN1#97

- TR 38.811 “Study on NR to support non-terrestrial networks”

# Appendix

SLS simulation settings for LEO-600 and LEO-1200

| Carrier frequency | 2GHz |
| --- | --- |
| Subcarrier spacing | 15kHz |
| DL bandwidth for each beam | 30 MHz for freq. reuse factor = 1 10 MHz for freq. reuse factor = 3 |
| Ionospheric scintillation | Weak, S4 = 0.3 |
| Rx polarization loss | 3dB |
| Number of UEs per beam | 20 for LEO-600 Set-1 30 for LEO-600 Set-2, LEO-1200 Set-1, and LEO-1200 Set-2 |
| UE channel estimation | Realistic MMSE |