3GPP TSG-RAN WG1 Meeting #98	R1-1908022

Prague, Czech Republic, 26th – 31st August 2019

Agenda Item:	6.2.1.5

Source:	Ericsson

Title:	Quality report in Msg3 and connected mode in LTE-MTC

Document for:	Discussion and Decision

# 1	Introduction

The Rel-16 work item description (WID) on “Additional MTC enhancements for LTE” [1] includes the following objectives for improvement of DL transmission efficiency and/or UE power consumption for BL/CE UEs.

- Specify quality report in Msg3 at least for EDT [RAN1, RAN2, RAN4]

- Specify aperiodic quality report in connected mode using same quality definition as in Msg3 [RAN1, RAN2, RAN4]

Earlier RAN1 and RAN2 agreements are listed in [2] and Section 1.6 of [3], respectively. The contributions to the previous RAN1 meeting are summarized in [4] and [5]. RAN1 has sent LS to RAN2 (cc RAN4) in [6], and RAN2 has sent LS to RAN1 (cc RAN4) in [7]. In this contribution, we discuss further aspects of quality reporting for LTE-MTC.

# 2	Discussion

## 2.1	Background

Periodic and aperiodic CSI reporting is supported in connected mode in CE mode A but not in CE mode B. For periodic CSI reporting, PUCCH is used. For aperiodic CSI reporting, PUSCH is used according to section 7.2.1 of TS 36.213 [8] as follows.

A BL/CE UE shall perform aperiodic CSI reporting using the PUSCH upon decoding either:

-	an uplink DCI format, or

-	a Random Access Response Grant,

for serving cell  if the respective CSI request field is set to trigger a report and is not reserved. The subframe(s) in which the PUSCH carrying the corresponding aperiodic CSI reporting triggered by an UL DCI format is transmitted is determined according to Subclause 8.0.

The Random Access Response (RAR) Grant formats for BL/CE UE are given in Table 1 (for the non-EDT case) and Table 2 (for the EDT case), and detailed interpretation of each field is given in section 6.2 in [8]. Notice that in the (RAR) Grant, there is a 1-bit CSI request field, which can request the UE to report CSI. However, in contention based random access procedure, the CSI request field is reserved.

Table 1: Random Access Response Grant Content field size for non-EDT case

| DCI contents | CE mode A | CE mode B |
| --- | --- | --- |
| Msg3 PUSCH narrowband index |  | 2 |
| Msg3 PUSCH Resource allocation | 4 | 3 |
| Number of Repetitions for Msg3 PUSCH | 2 | 3 |
| MCS | 3 | 0 |
| TBS | 0 | 2 |
| TPC | 3 | 0 |
| CSI request | 1 | 0 |
| UL delay | 1 | 0 |
| Msg3/4 MPDCCH narrowband index | 2 | 2 |
| Zero padding | 4 - | 0 |
| Total Nr-bits | 20 | 12 |

Table 2: Random Access Response Grant Content field size for EDT case

| DCI contents | CE mode A | CE mode B |
| --- | --- | --- |
| Msg3 PUSCH narrowband index |  | 3 |
| Msg3 PUSCH Resource allocation | 5 | 3 |
| Number of Repetitions for Msg3 PUSCH | 2 | 3 |
| TPC | 3 | 0 |
| CSI request | 1 | 0 |
| UL delay | 1 | 0 |
| Msg3/4 MPDCCH narrowband index |  | 3 |
| Zero padding | 8 - | 0 |
| Total Nr-bits | 20 | 12 |

The Msg3/4 MPDCCH narrowband index indicates the narrowband used for first subframe of MPDCCH configured by Temporary C-RNTI and/or C-RNTI during random access procedure as given in Table 6.2-B (for the non-EDT case) and Table 6.2-H (for the EDT case) in TS 36.213 [8].

Figure 1 and Figure 2 show the definitions of the MAC RAR message for BL/CE UEs [9]. As can be seen from the figures, the UL grant field is 4+8+8=20 bits for CE mode A and 4+8=12 bits for CE mode B.

Figure 1: Definition of MAC RAR for CE mode A (PRACH CE level 0 & 1)

Figure 2: Definition of MAC RAR for CE mode B (PRACH CE level 2 & 3)

If EDT is not configured, Table 3 and Table 4 show the TBS tables for Msg3 in CE modes A and B, respectively. If EDT is configured, the UE can send up to 1000 bits in Msg3 in CE mode A and up to 936 bits in Msg3 in CE mode B [8].

Table 3: TBS table for Msg3 in CE mode A

| IMCS | NPRB |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 6 |
| 0 | 16 | 32 | 56 | 152 |
| 1 | 24 | 56 | 88 | 208 |
| 2 | 32 | 72 | 144 | 256 |
| 3 | 40 | 104 | 176 | 328 |
| 4 | 56 | 120 | 208 | 408 |
| 5 | 72 | 144 | 224 | 504 |
| 6 | 328 | 176 | 256 | 600 |
| 7 | 104 | 224 | 328 | 712 |

Table 4: TBS table for Msg3 in CE mode B

| ITBS | NPRB |  |
| --- | --- | --- |
|  | 1 | 2 |
| 0 | 56 | 152 |
| 1 | 88 | 208 |
| 2 | 144 | 256 |
| 3 | 176 | 328 |

## 2.2	Quality reporting in Msg3

### 2.2.1	Measurement narrowband(s)

RAN1 has agreed that “for DL quality report in Msg3 for IDLE mode UEs, the narrowband(s) for downlink quality measurement includes at least the narrowband(s) on which MPDCCH of RAR is monitored”. Notice that for Cat-M1 UEs, the RF bandwidth may be as small as 1.4 MHz. Therefore, it is not possible for such a UE to measure several different narrowbands at the same time. For UEs in extended coverage, it may also take considerable amount of time and energy to perform reliable measurements. Hence, from a UE battery consumption point of view, it is better to keep the UE measurement efforts as low as possible. Furthermore, due to the RF bandwidth of a Cat-M1 UE, it may not be possible to monitor two narrowbands at the same time. Therefore, one natural choice for the measurement narrowband(s) are MPDCCH narrowband(s) that the UE monitors for scheduling of RAR. If frequency hopping is not used, the measurement will only involve a single MPDCCH narrowband, otherwise it will involve two or four MPDCCH narrowbands.

For DL quality report in Msg3 for IDLE mode UEs, the narrowband(s) for downlink quality measurement are the narrowband(s) on which MPDCCH of RAR is monitored.

### 2.2.2	Trigger for reporting

RAN1 has agreed that the “DL quality report in Msg3 in IDLE mode is configured per PRACH CE level or per CE mode in the SI”. Upon detecting such configurations, it is preferred that the UE should always report its DL quality reporting in Msg3 if the Msg3 TBS is large enough.

If DL quality reporting is configured in SI and the scheduled Msg3 TBS is large enough, then the UE reports its DL quality in Msg3.

### 2.2.3	Whether to report aggregation level in CE mode A

RAN1 has agreed that for CE mode B, the DL channel quality reporting is based on the repetition number that the UE needs to decode hypothetical MPDCCH with BLER of 1%. RAN1 has agreed on the same definition in CE mode A in case the estimated number of repetitions is larger than 1, but for the case when the estimated number of repetitions is 1, it is FFS whether to (also) report an aggregation level.

A unified solution that can be applied to both CE mode A and CE mode B is preferred for the DL channel quality report in Msg3. This can both simplify the standardization efforts and implementations.

An important potential use for the DL channel quality report in Msg3 is to give the network information to determine a set of proper RRC parameters to setup the communications, e.g., Rmax for USS in Msg4, rather than to be used for link adaptation. Therefore, for both CE mode A and CE mode B, there is little use of reporting the aggregation levels, as eNB usually would leave certain margins when setting up the connection to take e.g. UE mobility into account.

Msg3 reporting of aggregation level will not make any practical difference when eNB configures Rmax for USS in Msg4.

In CE mode A, when 16QAM or even 64QAM is used for PDSCH, accurate channel quality information helps guide the link adaptation for PDSCH. However, for this purpose, CE mode A already supports both periodic and aperiodic CSI reporting since Rel-13, and one additional quality report in Msg3 will not make much difference for the PDSCH link adaptation.

Msg3 reporting of aggregation level will not provide any significant additional advantage over existing Rel-13 CSI reporting when eNB performs link adaptation for PDSCH.

Based on the above observations, we prefer to adopt a unified solution for both CE mode A and CE mode B, which makes the signalling design easier.

For the DL quality reporting in Msg3 in CE mode A (PRACH CE levels 0 and 1), only repetition number is reported with a pre-defined maximum aggregation level, i.e. no explicit report of aggregation level.

### 2.2.4	Number of bits used for the report

RAN2 has confirmed that 8 bits are typically available for the DL quality report [7]. The main exception is the case when the minimum Msg3 size of 56 bits is used, in which case a special solution would be required to convey a DL quality report, and in this case the report size would be limited to 2 bits. RAN1 has agreed to specify a format with up to 8 bits for the EDT Msg3 case (and the connected-mode Msg3 case) but it is still FFS what format to specify for the non-EDT Msg3 case.

The reason to specify a 2-bit report rather than an 8-bit report in the non-EDT Msg3 case would be coverage related. However, it should be noted that while Msg3 in ordinary LTE is always a single subframe long, this is not necessarily the case in LTE-MTC (see Sections 6.2 and 8.0 in [8] for details). For example, when CE mode A is configured in a cell with the maximum PUSCH repetition factor set to 16 or 32 (rather than 8), the smallest PUSCH repetition factor that can be used for Msg3 is 16/8=2 and 32/8=4, respectively. Similarly, when CE mode B is configured in a cell with the maximum PUSCH repetition factor of 512, the smallest PUSCH repetition factor that can be used for Msg3 is 512/128=4. Furthermore, eNB can schedule significantly larger PUSCH repetition factors for Msg3 than these minimum values if needed for some PRACH CE levels. Since the transport block sizes used in the non-EDT case are small, this may not be necessary, though. Overall, this means that Msg3 is typically not the coverage bottleneck in LTE-MTC that it might be in ordinary LTE.

For many important configurations of the maximum PUSCH repetition factor in CE modes A and B, the smallest available number of PUSCH repetitions for Msg3 is so high that Msg3 should not become a coverage bottleneck.

There may of course be some configurations of the maximum PUSCH repetition factor and other relevant parameters that result in situations where the coverage is more sensitive to the size of Msg3. The question is then to what extent a 2-bit quality report in Msg3 would be so useful that it is simultaneously preferable to the 8-bit report and preferable to not implementing support for any quality report. In a cell where the RRC Suspend/Resume procedure with full Resume ID is supported, the minimum Msg3 size would be 72 bits, so it is not obvious that the 56-bit Msg3 size will be that much used in the future deployments.

Thus, the potential benefits with specifying a 2-bit report rather than an 8-bit report are debatable and do not seem to motivate the required efforts in terms of standardization, implementation and testing. Therefore, the straightforward approach is to specify the same reporting range for all PRACH CE levels as well as for the EDT and non-EDT cases, and not support DL quality reporting in Msg3 in the special case when the Msg3 TBS is too small to carry a useful DL quality report.

DL quality reporting range in Msg3 is the same (1, 2, 4, 8, 16, 32, 64, 128, 256) for all PRACH CE levels as well as for the EDT and non-EDT cases.

### 2.2.5	Frequency hopping case

The full range of repetition factors only requires 4 bits so there is room for additional information in an 8-bit report, for example indication of a UE-selected narrowband in the special case that the UE is configured with frequency hopping. However, if the more detailed frequency domain information would then only be available when frequency hopping is used in the cell, and only for the narrowbands that are involved in the frequency hopping pattern used during random access, the usefulness of the information is very limited. In practice the only practical use may be to inform eNB whether to dynamically activate/deactivate frequency hopping during the beginning of a session before updated information can be obtained via periodic or aperiodic CSI reporting.

The usefulness of Msg3 reporting of frequency domain information is very limited if the information is only available when frequency hopping is used in the cell, and only for the narrowbands that are involved in the frequency hopping pattern used during random access.

Based on the above observation, and for maximum simplicity and commonality across different configurations, we propose to just report the wideband (average) report.

The DL quality report only indicates the number of MPDCCH repetitions (i.e., no other information such as a UE-selected narrowband in the frequency hopping case).

## 2.3	Aperiodic quality reporting in connected mode (not in Msg3)

In the WID [1] one of the objectives is to specify aperiodic quality report in connected mode using same quality definition as in Msg3. Recall that aperiodic CSI reporting is supported by CE mode A, which is triggered by 1 bit in DCI. However, CSI reporting does not include number of repetitions for MPDCCH. A unified solution that applies for both CE mode A and CE mode B is preferred here, to simply the standardization efforts and implementations.

A unified solution that can be applied to both CE mode A and CE mode B is preferred for the aperiodic DL channel quality report in connected mode using same quality definition as in Msg3 in connected mode. This can both simplify the standardization efforts and implementations.

In connected mode, in the DL a UE is monitoring MPDCCH search space for DCI and/or receiving PDSCH. Due to the bandwidth limitations for Cat-M1 UEs, it cannot be assumed that it is possible to monitor more than one narrowband at the same time. Therefore, if the UE is instructed to monitor a narrowband different than the current one it is monitoring, a measurement gap needs to be provided. However, it is complicated to provide a measurement gap, as it fragments the DL resource. Also, for the UE is in CE mode B, the gap is expected to be long for the UE to achieve a required measurement accuracy. Therefore, from UE perspective, it is beneficial to perform measurement at the same narrowband it is currently operating on. Hence, we have the following proposal (which reuses the wording from a corresponding NB-IoT agreement).

For DL channel quality report in connected mode (other than Msg3), UE performs measurement on the narrowband(s) it is assigned to monitor in USS for MPDCCH and the associated PDSCH.

RAN1 has agreed that “for DL quality report in connected mode, the mechanism for triggering is based on one or more of […] DCI (e.g. UL grant, PDCCH order) [and] MAC CE”. Since a DCI based solution would probably require different solutions in CE mode A and B and furthermore make the connected mode case more different from the Msg3 case, we prefer MAC CE based triggering.

For DL channel quality report in connected mode (other than Msg3), triggering is MAC CE based.

RAN1 has already agreed that the “DL channel quality report in connected mode is transmitted via higher layer signalling, e.g. MAC CE or RRC message”.

## 2.4	Quality reporting in PUR

For the Rel-16 preconfigured uplink resource (PUR) feature [1], whether it is beneficial to include the DL channel quality report depends on the PUR scheme. For contention based PUR, it is better to keep the message small, and it is likely it will only be used for small data transmission in the UL. Therefore, including the DL channel quality report may not be beneficial. However, for dedicated PUR, there can be use cases that the eNB would like to setup connection to send UE DL data after the UE finishes its UL transmission. In such cases, inclusion of the DL channel quality reporting is useful.

Conclude that from RAN1 point of view, support of DL channel quality reporting in dedicated PUR transmission is feasible and beneficial but leave to RAN2 whether to introduce support for it or not.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	A unified solution that can be applied to both CE mode A and CE mode B is preferred for the DL channel quality report in Msg3. This can both simplify the standardization efforts and implementations.

Observation 2	Msg3 reporting of aggregation level will not make any practical difference when eNB configures Rmax for USS in Msg4.

Observation 3	Msg3 reporting of aggregation level will not provide any significant additional advantage over existing Rel-13 CSI reporting when eNB performs link adaptation for PDSCH.

Observation 4	For many important configurations of the maximum PUSCH repetition factor in CE modes A and B, the smallest available number of PUSCH repetitions for Msg3 is so high that Msg3 should not become a coverage bottleneck.

Observation 5	The usefulness of Msg3 reporting of frequency domain information is very limited if the information is only available when frequency hopping is used in the cell, and only for the narrowbands that are involved in the frequency hopping pattern used during random access.

Observation 6	A unified solution that can be applied to both CE mode A and CE mode B is preferred for the aperiodic DL channel quality report in connected mode using same quality definition as in Msg3 in connected mode. This can both simplify the standardization efforts and implementations.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For DL quality report in Msg3 for IDLE mode UEs, the narrowband(s) for downlink quality measurement are the narrowband(s) on which MPDCCH of RAR is monitored.

Proposal 2	If DL quality reporting is configured in SI and the scheduled Msg3 TBS is large enough, then the UE reports its DL quality in Msg3.

Proposal 3	For the DL quality reporting in Msg3 in CE mode A (PRACH CE levels 0 and 1), only repetition number is reported with a pre-defined maximum aggregation level, i.e. no explicit report of aggregation level.

Proposal 4	DL quality reporting range in Msg3 is the same (1, 2, 4, 8, 16, 32, 64, 128, 256) for all PRACH CE levels as well as for the EDT and non-EDT cases.

Proposal 5	The DL quality report only indicates the number of MPDCCH repetitions (i.e., no other information such as a UE-selected narrowband in the frequency hopping case).

Proposal 6	For DL channel quality report in connected mode (other than Msg3), UE performs measurement on the narrowband(s) it is assigned to monitor in USS for MPDCCH and the associated PDSCH.

Proposal 7	For DL channel quality report in connected mode (other than Msg3), triggering is MAC CE based.

Proposal 8	Conclude that from RAN1 point of view, support of DL channel quality reporting in dedicated PUR transmission is feasible and beneficial but leave to RAN2 whether to introduce support for it or not.

# References

RP-191356, “Revised WID: Additional MTC enhancements for LTE”

R1-1907630, “RAN1 agreements for Rel-16 Additional MTC Enhancements for LTE”

R2-1908188, “RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC”

R1-1907610, “Feature summary of DL quality report in MTC”

R1-1907631, “Remaining issues on DL quality report in MTC”

R1-1905593, “LS on channel quality report in Msg3 for NB-IoT and LTE-M”, RAN1

R2-1905471, “LS on quality report in Msg3 for LTE-M”, RAN2

TS 36.213 V15.4.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures (Release 15)”

TS 36.321 V15.4.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification (Release 15)”