3GPP TSG RAN WG1 Meeting #98	R1-1908048

Prague, Czech Republic, August 26 – 30, 2019

Agenda Item:	7.2.5.2

Source:	Huawei, HiSilicon, CAICT

Title:	Discussion on physical layer control procedures for NTN

Document for:	Discussion and Decision

Introduction

RAN#80 approved a new SI on solution evaluation for NR to support Non-Terrestrial Network [1]. The objectives of the SI for physical-layer and high-layer are reported as follows respectively.

Physical layer

Consolidation of potential impacts as initially identified in TR 38.811 and identification of related solutions if needed  [RAN1]:

- Physical layer control procedures (e.g. CSI feedback, power control)

- Uplink Timing advance/RACH procedure including PRACH sequence/format/message

- Making retransmission mechanisms at the physical layer more delay-tolerant as appropriate. This may also include capability to deactivate the HARQ mechanisms.

With respect to NTN physical layer control procedure, RAN1#97 made the following agreements:

Agreement:

- The need and the applicable scenarios for potential enhancements (with respect to the power control schemes in NR Rel-15) for both open-loop and closed-loop power control for NTN are to be studied.

- Study the performance of AMC in NTN considering at least the following solutions (some solutions may have no specification impact):

- Prediction-based link adaptation with prediction confidence level

- AMC with CQI reflecting only long-term fading

- Additional BLER targets for CQI reporting to limit number of retransmissions and latency

- CQI offset applied by gNB

- Finer granularity of CQI

- Prediction based CQI reporting

In this contribution, we provide some considerations on physical layer control for NTN.

Discussion

The long round-trip delay (RTD), large Doppler shift and variation are of the main characteristics in NTN that need be specifically considered in the system design. As shown in TR 38.821 [2], NTN should deal with maximum RTD (propagation delay only) up to 541ms, and the maximum user equipment (UE) speed can be up to 1000km/h. Considering these two key characteristics in NTN, the physical layer control procedures shall be re-evaluated, including adaptive modulation and coding (AMC), UL power control, HARQ procedure and transmission related timing.

Adaptive modulation and coding

AMC is one of the most key techniques to cope with the dynamic fading of wireless channels. In NR Rel-15, the gNB determines the modulation scheme and coding rate according to the channel state information (CSI) reported by the UE in the uplink, maximizing the throughput while maintaining the target BLER performance of the transmission.

However, due to the long RTD, the CSI feedback in NTN cannot be obtained in time, which limits the performance of the AMC to some extent. So it makes sense to consider prediction-based AMC scheduling. One study shows that in certain scenario, e.g. GEO, the CSI calculated based on downlink reference signal (RS) with large scale fading does not differ very much with small scale fading [3], so it is rational to do prediction based on the channel status report with large scale fading.

In addition, many existing prediction methods with different prediction ability have been studied in terrestrial system [4]. To guarantee link adaption performance in NTN, a proper model for prediction and method for generating approximate result to do prediction shall be re-considered, which depends on implementation.

Proposal 1: Prediction-based AMC scheme can be considered in certain scenario.

UL power control

To mitigate large scale shadowing and long distance path loss in NTN communication, a power control scheme is essential. Generally, uplink power control includes two components:

- Open loop control

In the open loop control scheme, the UE compensates the UL path loss based on DL RSRP measurements, it can be deployed without any system-wide consideration and additional resource, but the power control accuracy is a major concern;

- Closed loop control

For the closed-loop scheme, UE receives transmit power control command from NTN base station and modifies the transmitted power to compensate the path loss for uplink. As defined in 38.213, the UE determines the PUSCH transmission power as follows:

Where fb,f,c(i,l) is closed-loop related indication by DCI.

As discussed in [7], the near far effect is not significant in NTN, i.e. the pathloss difference between beam center and beam edge is very small. On the other hand, it may not be possible to track the fast fading via TPC command due to the large propagation delay in NTN. However, the closed-loop power control may still be useful to correct the other impairment such as UE power setting error etc. Therefore, the existing power control scheme seems sufficient for NTN.

Proposal 2: The existing NR Rel-15 power control scheme can be reused for NTN.

HARQ procedure

HARQ is a combination of high-rate forward error-correcting coding and ARQ error-control. With HARQ, the data transmission becomes more reliable and efficient. In NTN, the HARQ procedure shall be re-designed to accommodate the challenge of long RTD.

Many considerations and challenges of HARQ design are mentioned in previous proposals. Simply increasing the number of HARQ processes linearly maybe not feasible due to the restriction of UE’s memory and limited processing capabilities, new HARQ scheme including pre-active scheme shall be evaluated [5]. Besides, disabling HARQ has also been considered as an option, consequently, detailed disabling HARQ schemes and potential challenges brought to upper layer protocol need to be studied [5].

Transmission related timing

The timing relationships of UCI (uplink control information) transmission in NR PUCCH/PUSCH and of uplink data transmission in PUSCH have been specified in 38.212 and 38.214. However, long RTD may also affect the timing in physical control procedures, as described in [6]. Therefore, we suggested a constant timing offset to eliminate the issues.

Figure1 Beam-specific timing offset

Since satellites in difference orbit may cover an area from several hundred to thousand kilometers in diameter, depending on the relative position between UE and satellite, the required timing offset may be different. Beam-specific offset seems reasonable and can be notified in system information.

Proposal 3: A constant offset in transmission related timing can be considered in NTN.

Conclusions

In summary, we discuss some key considerations on data transmission related procedures for NTN communication and have the following observations:

Proposal 1: Prediction-based AMC scheme can be considered in certain scenario.

Proposal 2: The existing NR Rel-15 power control scheme can be reused for NTN.

Proposal 3: A constant offset in transmission related timing can be considered in NTN.

- References

RP-181370, Study on solutions evaluation for NR to support Non-Terrestrial Network, RAN#80.

TR 38.821, Solutions for NR to support non-terrestrial networks.

Y. Zheng, M. Dong, and J. Wu, “Channel quality approximation based on shadow fading model in GEO mobile satellite system”, Advanced Materials Research, vol. 791-793, pp. 1941-1944, Sep. 2013.

Y. Zheng, M. Dong, Y. Jin, and J. Wu, “CQI prediction for shadow fading in LTE-compatible GEO mobile satellite communications system”, Advanced Materials Research, vol. 756-759, pp. 2137-2141, Sep. 2013.

R1-1908050, Discussion on HARQ procedures for NTN, Huawei, HiSilicon, RAN1 #98, Prague, Czech Republic.

R1-1905095, On physical layer control procedures for NTN, Ericsson, RAN1 #96b, Xi’an, China.

R1-1905999, Discussion on cell measurement for mobility management in NTN, Huawei, HiSilicon, RAN1#97, Reno, USA