3GPP TSG RAN WG1 Meeting #98	R1-1908049

Prague, Czech Republic, August 26 – 30, 2019

Agenda Item:	7.2.5.3

Source:	Huawei, HiSilicon, CAICT

Title:	Discussion on Doppler compensation, timing advance and RACH for NTN

Document for:	Discussion and Decision

Introduction

In RAN #80 a new SI on solutions evaluation for NR to support Non-Terrestrial Networks (NTN) was approved [1]. The objectives of the SI for physical-layer are reported as follows.

Physical layer

Consolidation of potential impacts as initially identified in TR 38.811 [2] and identification of related solutions if needed [RAN1]:

- Physical layer control procedures (e.g. CSI feedback, power control)

- Uplink Timing advance/RACH procedure including PRACH sequence/format/message

- Making retransmission mechanisms at the physical layer more delay-tolerant as appropriate. This may also include capability to deactivate the HARQ mechanisms.

In RAN1#97, the following on DL synchronization, UL frequency compensation and PRACH design was agreed:

Agreement:

Performance evaluations of the synchronization for DL are encouraged. For these evaluations,

- For LEO systems, beam specific pre-compensation of the common frequency shift at satellite with respect to the spot beam center can be considered

Agreement:

For UL frequency compensation at least in LEO systems:

- Both open and closed-loop can be studied

- Beam specific post-compensation of common frequency offset at gNB can be considered

            - FFS: Further indication of common frequency offset

            - FFS: Signalling details

            - FFS: Compensation of common frequency offset at UE side

- For Open-loop method:

- Estimation of UE-specific frequency offset and pre-compensation at UE side can be conducted based on:

            - DL RSs

            - UE location and satellite ephemeris

            - FFS: Determination of UE location

Agreement:

The scenarios where the Rel-15 PRACH design is sufficient and the scenarios where an extended or new PRACH design is required should be identified as part of the study

The significant difference between NTN and terrestrial networks includes the large Doppler shift, large round-trip propagation delay (up to hundreds of milliseconds for GEO), large differential delay, and delay variation rate due to the fast motion of satellites especially in LEO scenarios. To support NTN in NR, the above factors need to be considered. In this contribution, we discuss some key aspects regarding frequency synchronization, Timing Advance (TA), preamble formats and timing issues in RACH procedures. The contributions of this paper include:

- Performance evaluation of DL synchronization based on the assumptions agreed in RAN1#97 (details are presented in Appendix A), is provided. Moreover, the potential UL frequency misalignment of LEO scenarios is also discussed.

- Issues of TA for sending preamble, TA in the Random Access Response (RAR) and TA adjustment are analyzed. Furthermore, potential solutions to these issues are listed.

- Rel-15 Preamble formats are discussed for NTN scenarios, and possible solutions are also enumerated. Besides, scheduling timing issues of RACH procedures are introduced at last.

Discussion

Consideration on DL timing and frequency acquisition & UL frequency misalignment

The performance evaluation for DL synchronization includes PSS detection, frequency tracking, and timing tracking with the frequency offset of LEO based NTN access.

PSS detection performance

The main challenge for PSS detection is the perceived frequency shift at the UE side. The overall frequency offset consists of four parts:

- UE crystal offset

- Satellite oscillator offset

- Doppler shift due to satellite movement

- Doppler spread and shift due to UE movement

The frequency offset due to the first three items can be applied over all multipath and therefore can be modelled separately from the fast fading channel. In the fourth item, the Doppler spread and shift due to UE movement are modelled in the fast fading channel. The frequency offset due to the first three items for LEO elevation angel 30 degree is summarized as follows.

Table 1. Total frequency offset at UE side w/o pre-compensation at satellite

| Satellite altitude (km) | Doppler at cell center | Satellite oscillator offset | UE crystal offset | UE perceived freqency offset |
| --- | --- | --- | --- | --- |
| 600 | 21.81ppm | [0.5ppm] | 10 ppm | 32.31ppm |
| 1200 | 20.93ppm |  |  | 31.43ppm |

Once the common Doppelr shift of a satellite beam is pre-compensated at the satellite with respect to spot beam center, the main part of the total frequency offset can be elimintated. The maximum residual Doppler happenes at the beam edge, and is a function of the cell size and the elevation angle. The overall residual frequency offset w.r.t the configuration in DL agreements (Appendix A) and beam sizes defined in “Set 2” are summarized as follows.

Table 2. Residual frequency offset at UE side w/ pre-compensation at satellite for “Set 2”, 30 dgree

| Satellite altitude (km) | Beam diameter (km) | Residual Doppler at cell edge | Satellite oscillator offset | UE crystal offset | UE perceived freqency offset |
| --- | --- | --- | --- | --- | --- |
| 600 | 90 for S band | 1.00ppm | [0.5ppm] | 10 ppm | 11.5ppm |
| 600 | 50 for Ka band | 0.54ppm |  |  | 11.0ppm |
| 1200 | 190 for S band | 1.02ppm |  |  | 11.5ppm |
| 1200 | 90 for Ka band | 0.26ppm |  |  | 10.8ppm |

Scenarios with the worst frequecy offset of each frequency band are selected for evaluation. For S band (2GHz), the PSS detection perfromance of SCS 15kHz and SCS 30kHz is shown in Figure 1 and Figure 2, respectively. The UE speed is assumed to be 3km/h. The TDL NTN model in TR38.811 is used as the fast fading model with 1T1R antenna configuration. A revised PSS dectetion algorithm [3] is used in the simulation. The PSS detection threshold is carefully selected to ensure the FAR is under 1%.

Figure 1. PSS detection rate (UE3km/h, 30deg, SCS15kHz, residual Doppler 1.54SCS)

Figure 2. PSS detection rate (UE3km/h, 30deg, SCS30kHz, residual Doppler 0.77SCS)

From the performance comparision in Figure 1 and Figure 2, we can see that PSS detection rates of SCS15kHz and SCS30kHz are almost the same under the NTN fading channels. The SNR point corresponding to 90% likelihood is slightly below -5dB.

Observation 1: The PSS detection rates of SCS15kHz and SCS30kHz are almost the same under the considered fading channels.

For Ka band (20GHz), the PSS detection perfromance of SCS 120kHz and SCS 240kHz are shown in Figure 3 and Figure 4, respectively. The UE speed is assumed to be 1000km/h. The “VSAT” antenna pattern is employed as a spatial filter over NTN CDL channel model. Due to the longer scanning distant of sliding window for high SCS numerology, the PSS detection threshold is a slightly strcicter than that for SCS15kHz and SCS30kHz.

Figure 3. PSS detection rate (UE1000km/h, 30deg, SCS120kHz, residual Doppler 1.84SCS)

Figure 4. PSS detection rate (UE1000km/h, 30deg, SCS240kHz, residual Doppler 0.92SCS)

Similarly, the PSS detection rates of SCS120kHz and SCS240kHz are almost the same under the considered NTN fading channels. The SNR point corresponding to 90% likelihood is slightly above -5dB, which is slightly worse compared with the performance of SCS15kHz and SCS30kHz.

Observation 2: The PSS detection rates of SCS120kHz and SCS240kHz are almost the same under the considered fading channels.

Observation 3: The SNR points corresponding to 90% likelihood for all SSB numerologies are about -5dB for the considered fading channels.

Frequency tracking performance

Once the PSS is successfully detected, a conventional fine frequency tracking can be implemented based on PSS and SSS or based on 2-column PBCH DMRS. The CDFs of residual frequency offset normalized by SCS at the SNR point corresponding to 90% PSS detection rate for SCS15kHz and SCS30kHz are shown in Figure 5 and Figure 6, respectively.

Figure 5. CDF of residual frequency offset (UE3km/h, 30deg, SCS15kHz, residual Doppler 1.54SCS)

Figure 6. CDF of residual frequency offset (UE3km/h, 30deg, SCS30kHz, residual Doppler 0.77SCS)

The CDFs of residual frequency offset for SCS120kHz and SCS240kHz are shown in Figure 7 and Figure 8, respectively.

Figure 7. CDF of residual frequency offset (LEO600km, UE1000km/h, 30deg, SCS120kHz, residual Doppler 1.84SCS)

Figure 8. CDF of residual frequency offset (UE1000km/h, 30deg, SCS240kHz, residual Doppler 0.92SCS)

The CDFs of residual frequency offset of different SSB numerologies in S-band and Ka-band are almost the same. In general, about 90% the residual frequency offset can be controlled within 0.03SCS. The PSS-SSS based method can provide better frequency tracking performance than the SSB-DMRS based method.

Observation 4: The relative residual frequency offset (normalized by SCS) of different SSB numerologies are almost the same under the considered fading channels.

Observation 5: The absolute residual frequency offset increases as the SSB SCS increases.

Observation 6: About 90% the residual frequency offset can be controlled within 0.03SCS.

Observation 7: The PSS-SSS based method can provide better frequency tracking performance than the PBCH-DMRS based method.

DL timing tracking performance

The fine timing tracking can also be implemented with different kinds of reference signals, such as PSS, SSS and PBCH DMRS. The CDF of the residual timing errors at SNR point corresponding to 90% PSS detection rate for SCS15kHz and 30kHz is shown in Figure 9 and Figure 10, respectively.

Figure 9. CDF of timing residual offset (UE3km/h, 30deg, SCS15kHz, residual Doppler 1.54SCS)

Figure 10. CDF of timing residual offset (UE3km/h, 30deg, SCS30kHz, residual Doppler 0.77SCS)

The CDFs of the residual timing errors for SCS120kHz and 240kHz in Ka-band are shown in Figure 11 and Figure 12, respectively.

Figure 11. CDF of timing residual offset (UE1000km/h, 30deg, SCS120kHz, residual Doppler 1.84SCS)

Figure 12. CDF of timing residual offset (UE1000km/h, 30deg, SCS240kHz, residual Doppler 0.92SCS)

From Figure 9 to Figure 12, we can see that the absolute timing residual offset reduces as the bandwidth of the reference sequences increase with the SCS value. The PSS-SSS based timing tracking method can provide better performance than PBCH-DMRS based method.

Observation 8: The absolute timing residual offset reduces as the SSB SCS increases.

Observation 9: The PSS-SSS based method can provide better timing tracking performance than the SSB-DMRS based method.

Although the instant timing tracking performance seems acceptable, the satellite motion will incur timing drift. For satellites with orbit altitude 600km, the relative moving speed of UE and satellite is about 7.4km/s (elevation angle = 10 degree). The corresponding relative position change between UE and satellite is about 7.4 meters in 1 ms time interval. The downlink timing drift in 1 ms is about 24.8ns, converting to a sampling point of about 48.8Tc. In NR, the minimum period of CSI-RS for timing tracking is 10ms, which means the timing drift between two timing adjustments is about 487.7 Tc. For transparent NTN scenarios, the maximum timing drift value is doubled. It is still possible for a “synchronized” UE to suffer from timing offset and even inter-symbol-interference during the interval of two reference signals if it simply holds the previous timing point. Therefore, the UE has to adjust the timing by itself even without the help of the reference signals to avoid possible inter-symbol-interference.

Observation 10: A UE needs to adjust the timing point autonomously between two pilot signals to avoid possible ISI due to DL timing drift in LEO.

Furthermore, the timing drift has impacts on the phase of the estimated channel even within a slot. In Figure 13, the relation between the phase and subcarrier is shown. Note that each symbol is with slightly different timing offset. Since the phase of each symbol and subcarrier is always changing, it is less likely to deploy a low complexity channel estimation algorithm and equalizer, unless the timing drift can be compensated.

Figure 13. Phase frequency characteristics with timing drift

Observation 11: A UE needs to compensate the phase change within a slot due to DL timing drift to ease low complexity channel estimation and equalizer.

Observation 12: It is necessary to acquire DL timing drift rate to avoid potential ISI and compensate phase change.

Proposal 1: Study the potential solutions for DL timing drift rate estimation.

Frequency misalignment issues

For LEO-based NTN scenarios, the main concern is the possible frequency misalignment due to the huge Doppler shift. We will analyse the potential UL frequency misalignment with two frequency alignment schemes.

Option 1: Figure 14 depicts the frequency alignment scheme in the terrestrial networks. The UE and BS oscillator offsets are assumed x1 ppm and x2 ppm, respectively. The frequency offset in the channel due to the relative movement of BS and UE are assumed x3 ppm. The local oscillator of UE is aligned to the received DL signal and the UL transmission frequency is shifted by the duplex distance. Considering the UE modulated carrier frequency should be accurate to within +/-0.1ppm according to TS38.101-1, the frequency offset of the received signal at the BS is about double Doppler shift, i.e. (2x3+0.1) ppm.

If the common part of the Doppler shift is pre-compensated for TX and post-compensated for RX at the BS side, the main part of x3 can be eliminated and only the residual Doppler part matters.

Figure 14. Illustration of UL frequency misalignment for LEO scenarios: option 1

Observation 13: Aligned to the frequency of DL signal, the UL frequency misalignment at the BS side is double the (residual) Doppler shift.

Option 2：The second option of frequency alignment scheme is shown in Figure 15. Since UE is assumed to be able to acquire the “measured frequency offset” in the DL synchronization process, it can shift the central frequency to the opposite side of the “measured frequency offset” for UL transmission. Then the overall frequency offsets of the received UL signal is double the oscillator errors, i.e. 2(x1+x2) ppm.

Figure 15. Illustration of UL frequency misalignment for LEO scenarios: option 2

Observation 14: With the UL frequency pre-compensation at the UE side, the UL frequency misalignment at the BS side is double the oscillator errors.

Provided that the pre-/post-compensation mechanism is assumed at the satellite payload side, the frequency offset at UL is summarized for the two options w.r.t the maximum Doppler shift (at the edge of nadir beams) shown in Table X.9 [11], as follows.

Table 3. UL frequency offset at satellite

| Beam configuration & max. residual Doppler at nadir beam |  | Option 1 | Option 2 |
| --- | --- | --- | --- |
| LEO 1200 km | beam diameter = 90 km (Set 1 - S-band) : 0.91 ppm | 1.92 ppm | 21 ppm |
|  | beam diameter = 40 km (Set 1 - Ka-band): 0.40 ppm | 0.90 ppm |  |
|  | beam diameter = 190 km (Set 2 - S-band) : 1.91 ppm | 3.92 ppm |  |
|  | beam diameter = 90 km (Set 2 - Ka-band) : 0.91 ppm | 1.92 ppm |  |
| LEO 600km | beam diameter = 50 km (Set 1 - S-band) : 1.05 ppm | 2.20 ppm |  |
|  | beam diameter = 20 km (Set 1 - Ka-band) : 0.42 ppm | 0.94 ppm |  |
|  | beam diameter = 90 km (Set 2 - S-band) : 1.88 ppm | 3.86 ppm |  |
|  | beam diameter = 50 km (Set 2 - Ka-band) : 1.05 ppm | 2.20 ppm |  |

For option 1, the residual UL frequency offset of Set 2 configuration is larger than that of the Set 1 due to the larger beam sizes. The worst UL frequency offset cases for S band and Ka band are about 3.92 ppm and 2.20 ppm, respectively. For larger beam diameter e.g. 200km, the UL frequency offset would be about 8ppm. For option 2, the UL frequency offset is 21ppm, regardless of the beam configurations. Both the values are far beyond the terrestrial UL frequency offset (0.1ppm + 2x0.32ppm) contributed by UE adjustment errors (0.1ppm) and UE movement (350km/h 0.32ppm).

Observation 15: The UL frequency offset in LEO is a challenge for RACH.

Proposal 2: Take the NTN specific UL frequency offset into the consideration of RACH design.

Consideration on timing advance

A UE shall adjust the timing of its UL transmissions based on a TA command so that the frame boundary of the UL signals from different UEs can be aligned at the gNB within the required accuracy.

Timing advance for sending preamble

One of the main differences between NTN and terrestrial networks is the large Round Trip Time (RTT) between the gNB and UE. As shown in Figure 16, received uplink signals are delayed because of large RTT. So the preamble detection window has to be delayed with the minimum round trip delay (RTD) in one beam. However, the minimum RTD is usually not multiple integral subframe duration or slot duration, which means that the preamble detection window boundary cannot be aligned with other time-synchronized UEs’ UL signals. So the extra interference between preambles and other UEs’ UL signals will be introduced. Therefore, to avoid the interference between preambles and other UEs’ UL signals, a TA may be utilized in sending preamble.

On the other hand, the round trip delay in NTN is much larger than terrestrial communications (~250 ms in GEO), and a significant increase of the number of RAR TA bits will be required. If the TA for sending preamble is applied, the TA in RAR only needs to indicate the remaining delay measured by detecting preambles, which can avoid extending RAR TA largely.

Figure 16. Illustration of round trip time and uplink timing

- Closed-loop

Satellites can broadcast a common TA to all UEs in one beam, which roughly compensates the received uplink signal RTT delay. A common TA can be calculated according to the RTT between the closest point within the beam to the satellite and the satellite/gateway. In Figure 17, for a regenerative satellite the common TA is 2*d1/c, where c is the speed of light. However, for a transparent satellite the common TA is 2*(d1+d2)/c. Therefore, regenerative and transparent satellites can broadcast the actual full common TA, i.e., 2*d1/c and 2*(d1+d2)/c to UEs in one beam, respectively.

Another optional method is that both regenerative and transparent satellites broadcast 2*d1/c as the beam specific common TA, while the gNB compensates the feeder link RTT delay (i.e., RTT between the satellite and the gateway) in the transparent satellite scenario [4]. As the broadcast TA is part of the actual full common TA in the transparent satellite scenario, it can be called as partial common TA.

Additionally, the full/partial common TA can be broadcasted in different signaling, such as indicated in SIB1, and in different forms, such as indicated by a table index.

Figure 17. Common TA calculation example

Observation 16: In the closed-loop method, the full common TA can be broadcasted by the gNB in both regenerative satellite and transparent satellite scenarios.

Observation 17: In the closed-loop method, the partial common TA can be broadcasted and the residual delay can be compensated by the gNB.

- Open-loop

Besides the closed-loop method, another approach is the open-loop method, where the UE does not need the broadcast common TA from satellites. If the UE can obtain its own position information and the satellite ephemeris data, it can calculate the required TA by itself before transmitting a preamble. In general, the calculated TA is more accurate than the broadcast common TA for the UE, and it can compensate the differential delay between the UE position and the reference point. Therefore, the TA for sending the preamble can be obtained by the open-loop method. However, the positioning error and ephemeris error will induce inaccurate TA, which needs to study further.

Observation 18: In the open-loop method, the UE can calculate the TA for transmitting preamble according to its position and the satellite ephemeris.

Proposal 3: To obtain the TA for transmitting a preamble, consider at least the following options:

Closed-loop method

Option 1: gNB broadcasts the full common TA.

Option 2: gNB broadcasts the partial common TA and the network compensates the residual delay.

FFS: Details on signaling and forms

Open-loop method

Option 3: The UE calculates the TA according to its position and the satellite ephemeris.

FFS: Effects of positioning error and ephemeris error

Timing advance command in RAR

During initial access, the gNB estimates the UE-specific TA based on the preamble sequence sent by UE, and sends the TA command to UE in the RAR. In NR, the TA command indicates the maximum index value of , where the granularity of TA for Subcarrier Spacing (SCS)  kHz is  with a time unit , where  Hz and  (ms). For SCS of 15 kHz, the TA command can indicate up to 2ms, corresponding to 300 km cell radius. However, this value will be scaled down with the increase of the SCS [5].

To extend the indication range of the initial TA, the expression of  can be multiplied by a scaling factor [6]:

where the granularity of extended TA is . For a fixed SCS, the accuracy of the extended TA indication is determined by the scaling factor .

A UE should meet the timing error requirement for an initial transmission defined in [7]. The reference point is the downlink timing of the reference cell minus , where the value of  given in [7] depends on the uplink transmission duplex mode and the frequency range (FR).

According to Table 7.1.2-1 in [7] (details are presented in Appendix B), the scaling factor  in the expression of  cannot be arbitrarily chosen since the error due to the limited granularity has to meet the timing accuracy requirement, especially for large SCS used in FR2. If the timing accuracy requirement cannot be met when adopting the scaling factor, increasing the bit width of the TA command can be considered.

Observation 19: The extended initial TA with the scaling factor  may not meet the timing accuracy requirement, especially for large SCS used in FR2.

Proposal 4: Increase the bit width of the TA command, if the timing accuracy requirement cannot be met when adopting the scaling factor.

Timing advance adjustment

UE should have the capability of tracking the frame timing change of the serving gNB. In NR, gNB sends a TA adjustment command to UE. Specifically, the TA adjustment command is an index value with , and a new TA is calculated by [8]. Hence, the UE adjusts its TA for data transmission with this new TA, and the maximum TA adjustment at a time is .

Intuitively, instead of adjusting UL transmissions based on the TA command, UEs with GNSS information is able to support open-loop TA adjustment with the knowledge of the serving satellite’s ephemeris. Generally, there is a meter-level error between the GNSS positioning information and the real position of a UE, which can be tolerated when considering the error requirement of TA adjusment. However, the positioning accuracy worsens in some cases, e.g. the UE is near buildings or the UE moves with varying acceleration. Therefore, the open-loop TA adjustment mechanism based on UE GNSS information and satellite ephemeris may not meet the TA adjustment accuracy requirement in certain cases.

Observation 20: Since the accuracy of positioning worsens in some cases, open-loop TA adjustment mechanism based on UE GNSS and satellite ephemeris may not meet the TA adjustment accuracy requirement in certain cases.

Proposal 5: It is necessary to retain the closed-loop TA adjustment mechanism due to positioning inaccuracy, even in case the GNSS of UE and the ephemeris of serving satellites are available.

In NTN, the fast motion of an LEO satellite causes a large timing drift, which is larger than that in terrestrial networks. According to Table 7.1-1 in [9], the timing drift experienced by a UE can be up to 40 s/s. Hence, the UE needs to update TA more frequently to cope with the large timing drift in NTN. Besides, there is misalignment between the indicated TA adjustment value and the actual TA value due to the large propagation delay.

If the aforementioned NR TA adjustment mechanism is used in NTN, the gNB will transmit the TA adjustment command to the UE much more frequently to maintain the uplink time synchronization for LEO cases. At the same time, the timing drift in the interval between two TA adjustment commands sent by the gNB should not be larger than the Cycle Prefix (CP) length at least. Provided that 40 µs/s timing drift exists in the worst LEO case, the requirement of the TA command transmission period is shown in Table 4.

Table 4. Requirement of the TA command transmission period in the worst LEO case (600km)

| Numerology (u) | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| SCS (kHz) | 15 | 30 | 60 | 120 |
| CP length (us) | 4.688 | 2.344 | 1.172 | 0.586 |
| TA command period (ms) | 117 | 58.6 | 29.3 | 14.7 |

As shown in Table 4, larger SCS requires shorter transmission period of TA commands. Moreover, if the timing error requirement described in [7] is concerned, the required TA command period will become shorter. Consequently, it can be concluded that if only using the existing NR TA adjustment mechanism to maintain the uplink timing synchronization, the gNB has to frequently send TA adjustment commands to each UE in LEO scenarios, which will introduce increased signaling overhead to the system.

Observation 21: To maintain the UL time synchronization in LEO scenarios, gNB has to frequently send TA adjustment commands to each UE, which will introduce increased signaling overhead to the system.

To reduce the signaling overhead, the UE could update the TA by itself based on the timing drift rate, which can be acquired from the gNB indication or the UE’s own estimate. To support a UE to implement TA self-adjustment, the following 3 options can be considered:

- The gNB indicates the timing drift rate to each UE. Before receiving the next TA adjustment command, the UE calculates the TA value based on the previous received TA adjustment command and the indicated timing drift rate. When receiving a new TA adjustment command, the UE updates the TA value based on the TA adjustment command. In consideration of the maximum change rate of timing drift rate in LEO cases and the timing error requirement described in [7], the timing drift rate can be indicated from every hundreds of microseconds to every seconds.

- The gNB broadcasts a cell/beam-specific common timing drift rate to UEs within a certain area. After the initial access, the UE uses the common timing drift rate to update the TA value before receiving a new TA adjustment command. The UE will adjust the timing drift rate based on subsequent TA commands. The common timing drift rate shared by all UEs within a certain area contributes to reducing signaling overhead. This method is more suitable for cases with smaller variation of the timing drift rate, e.g. small cell size at low elevation angles in regenerative cases.

- The UE can estimate the timing drift rate by itself with the help of downlink reference signals, such as PSS, SSS, DM-RS and CSI-RS which are described in [1]. Figure 4 in [3] shows the timing tracking performance comparison of different reference signals. Downlink timing tracking based on CSI-RS performs best at the same SNR.

It is obvious that signaling overhead and accuracy of aforementioned options are different. Therefore, which option is chosen needs further study. Additionally, the misalignment between the indicated TA adjustment command and the actual TA due to the long propagation delay can be relieved by pre-compensation at the gNB or post-compensation by the UE, because both the gNB and the UE can acquire the rough TA value.

Observation 22: Compared with sending TA commands frequently to the UE, signaling overhead can be greatly reduced if the UE is able to update the TA by itself.

Proposal 6: Besides utilizing the existing TA adjustment mechanism, UE may also implement self-adjustment of the TA value with the help of the timing drift rate. The following options can be considered:

Option 1: gNB indicates the timing drift rate to each UE.

Option 2: gNB broadcasts a cell/beam-specific common timing drift rate.

Option 3: UE estimates the timing drift rate by itself.

FFS: Details on signaling and forms

FFS: Overhead and accuracy

Consideration on RACH

A brief introduction of the 4-step contention-based random access procedure for NR is given in Figure 18.

Figure 18. The random access procedure in NR

Upon detection of the Synchronization Signal Block (SSB), a UE performs downlink synchronization and determines to use which preamble, which is also referred to as PRACH.

Step 1: A UE sends the preamble to the gNB for accessing the network within RACH slots.

Step 2: If the preamble is detected, gNB will transmit a RAR message to the UE, which contains the timing advance command, RA-RNTI and so on.

Step 3: The UE transmits its identity to the gNB, and requests the RRC connection.

Step 4: The gNB resolves any potential collision by transmitting the message to notify the UE if the initial access is successful.

Once the RACH procedure is completed, the UE will be in connected state and respond the HARQ-ACK for the data.

Preamble format

The key parameters of preamble formats include CP length, sequence length, SCS and sequence repetition number. Among them, the CP length, sequence length and sequence repetition number determine the beam/cell coverage. The SCS is related to the carrier frequency offset robustness. In the following, we discuss the applicability of NR preamble formats for NTN.

CP length

There are two types of preambles in NR Rel-15: long preambles and short preambles. The main difference between them is the length of preamble sequences, i.e., long preamble length LRA=839 and short preamble length LRA=139. Details of the preamble numerology can refer to Table 6.3.3.1-1 and Table 6.3.3.1-2 in [10].

The repetition of preamble sequences and the CP length limit the cell coverage of NR. The preamble sequence and CP lengths in time are detailed in Table 5. It can be seen that the CP length is no more than 684.37 us, which is enough for covering 100km cell radius in terrestrial networks. In NTN, the preamble CP is responsible to combat the corresponding large differential round trip delay of UL signals in the same beam/cell. However, the max differential delay time within a beam is 1.6 ms in the reference GEO scenario [9], which is much larger than 684.37 us. Therefore, the current NR preamble formats cannot support all NTN scenarios.

Table 5. Preamble parameters in different formats

| Format | LRA | ∆fRA (kHz) | Repetitions | CP length (us) | Preamble length (without CP)(us) |
| --- | --- | --- | --- | --- | --- |
| 0 | 839 | 1.25 | 1 | 103.12 | 800 |
| 1 | 839 | 1.25 | 2 | 684.37 | 1600 |
| 2 | 839 | 1.25 | 4 | 152.60 | 3200 |
| 3 | 839 | 5 | 4 | 103.12 | 800 |
| A1 | 139 | 15 | 2 | 9.37 | 133.33 |
| A2 | 139 | 15 | 4 | 18.75 | 266.67 |
| A3 | 139 | 15 | 6 | 28.12 | 400 |
| B1 | 139 | 15 | 2 | 7.03 | 133.33 |
| B2 | 139 | 15 | 4 | 11.71 | 266.67 |
| B3 | 139 | 15 | 6 | 16.41 | 400 |
| B4 | 139 | 15 | 12 | 30.47 | 800 |
| C0 | 139 | 15 | 1 | 40.36 | 66.67 |
| C2 | 139 | 15 | 4 | 66.67 | 266.67 |

To identify which NTN scenarios where the Rel-15 PRACH design is insufficient, the required minimum CP lengths in different scenarios need to be calculated and compared with the Rel-15 preamble formats. As shown in Figure 19, the distance for calculating the maximum differential delay time in one beam is related to the minimum elevation angle (MEA) in the beam, orbit altitude, and beam diameter. Meanwhile, the maximum differential delay time was computed by taking the earth curve into account for each scenario. Here, beam diameters in Table 6 are adopted, which refers to [2], [9] and [11].

The minimum required CP lengths for different minimum elevation angles and beam diameters are plotted in solid lines in Figure 20, Figure 21 and Figure 22. For comparing with Rel-15 preamble formats, CP lengths of different Rel-15 preamble formats are displayed as dashed lines.

Figure 19. Illustration of the distance for calculating the maximum differential delay time

Table 6. Reference beam foot print size

| Satellite orbit | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- |
| Satellite altitude | 35786 km | 1200 km | 600 km |
| Beam foot print size in diameter | 200~800 km | 40~300 km | 20~200 km |

Figure 20. Required minimum CP length in different minimum elevation angles and beam diameters for GEO scenarios

Figure 21. Required minimum CP length in different minimum elevation angles and beam diameters for LEO-1200 scenarios

Figure 22. Required minimum CP length in different minimum elevation angles and beam diameters for LEO-600 scenarios

From Figure 20, it is shown that in GEO scenarios only Rel-15 format 1 can support some beams whose MEA are larger than 60 degree and corresponding diameters are larger than 200~300 km. For LEO-1200 and LEO-600 scenarios in Figure 21 and Figure 22, the CP length of Rel-15 format 1 is feasible for beams whose diameters are less than 104 km. Overall, it can be easily seen that there is no preamble format whose CP length is sufficient for all NTN scenarios.

Observation 23: There is no Rel-15 preamble format whose CP length is sufficient for all NTN scenarios.

From Figure 20, Figure 21 and Figure 22, it can be seen that the minimum required CP length increases with the increasing of orbit altitude for the same MEA and beam diameter. For example, when the minimum elevation angle of the beam is 10 degree and the beam diameter is 200 km, required minimum CP lengths are 1.306 ms, 1.308 ms and 1.309 ms for LEO-600, LEO-1200 and GEO, respectively. Moreover, for the same orbit altitude and MEA, the required minimum CP length increases with the increasing beam diameter. For instance, in the GEO scenario (MEA=10 degree), required minimum CP lengths are 1.635 ms, 2.285 ms and 2.933 ms for 250 km, 350 km and 450 km cell radii, respectively. Besides, for the same orbit altitude, the required minimum CP length decreases with the increasing minimum elevation angle. Take the LEO-1200 scenario (diameter=300 km) as an example, and required minimum CP lengths are 1.957 ms, 1.844 ms and 1.661 ms for 10 degree, 20 degree and 30 degree, respectively.

Observation 24: From the perspective of limiting scenarios, increasing the supported minimum elevation angle or decreasing the supported maximum beam diameter can reduce the requirement of the preamble CP length.

Table 7. Supported minimum elevation angle of beam by Rel-15 preamble CP

| Format | Supported minimum elevation angle of beam (degree) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | GEO |  | LEO-1200 |  | LEO-600 |  |
|  | D=200 km | D=400 km | D=100 km | D=300 km | D=100 km | D=200 km |
| 0 | 85 | 86 | 79 | 81 | 77 | 77 |
| 1 | 59 | 74 | 10 | 63 | 10 | 52 |
| 2 | 83 | 85 | 75 | 78 | 72 | 74 |
| 3 | 85 | 86 | 79 | 81 | 77 | 77 |

To explicitly show the supported MEA of specific beams by Rel-15 preamble CP, the MEA is calculated and listed in Table 7. Refer to this table, when the diameter of the GEO beam is 200 km, the format 1 CP length can support MEA = 59 degree. So, if the supported MEA is increased to 59 degree, the existing Rel-15 format 1 can be used in GEO scenario. Consequently, the following observation is obtained.

Observation 25: The possible supported minimum elevation angles of different specific beams by Rel-15 preamble format 1 CP length are listed below:

| Format | Supported minimum elevation angle of beam (degree) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | GEO |  | LEO-1200 |  | LEO-600 |  |
|  | D=200 km | D=400 km | D=100 km | D=300 km | D=100 km | D=200 km |
| 1 | 59 | 74 | 10 | 63 | 10 | 52 |

When the supported MEA is determined, the maximum supported beam diameter of the corresponding beam by the Rel-15 preamble CP length can be calculated, which is calculated and listed in Table 8. For example, when the MEA of GEO beam is 10 degree, the format 1 CP length is sufficient for diameter = 104.39 km. Then, if the supported maximum beam diameter is decreased to 104.39 km, the existing Rel-15 format 1 can be used in GEO scenario. As a result, the following observation is obtained.

Observation 26: The possible supported maximum beam diameter of different specific beams by Rel-15 preamble format 1 CP length are 104.39 km (GEO, MEA=10 degree), 120.33 km (LEO-1200, MEA=30 degree) and 121.47 km (LEO-600, MEA=30 degree).

Table 8. Supported maximum beam diameter by Rel-15 preamble CP length

| Format | Supported maximum beam diameter (km) |  |  |
| --- | --- | --- | --- |
|  | GEO minimum elevation angle=10 degree | LEO-1200 minimum elevation angle=30 degree | LEO-600 minimum elevation angle=30 degree |
| 0 | 15.71 | 17.89 | 17.91 |
| 1 | 104.39 | 120.33 | 121.47 |
| 2 | 23.25 | 26.51 | 26.56 |
| 3 | 15.71 | 17.89 | 17.91 |

Another way to make use of the existing preamble CP length satisfy requirements of NTN scenarios is adopting a positioning method. UE can use its position information and the satellite ephemeris data to calculate its rough TA, so the requirement of CP length is reduced. However, the accuracy of positioning and ephemeris will induce inaccurate TA, which needs further study.

Observation 27: Utilizing a positioning method can reduce the requirement for the preamble CP length.

Even though the CP length is inefficient to address the uncertainties of the differential delay, the gNB can detect the preamble through multiple detections, which is illustrated in Figure 23. This method can make Rel-15 preamble format CP length work in NTN scenarios at the cost of higher computational complexity, increased FAR, and detection performance deterioration.

Figure 23. Illustration of multiple detection method

Observation 28: From the perspective of implementation, utilizing the multiple detection method can reduce the requirement for the preamble CP length.

To avoid increasing the computation complexity too much at the receiver, one simple way is to increase the CP length to support large beam diameter and small MEA. The required minimum CP lengths for some typical NTN scenarios are calculated and listed in Table 9. It is shown that the required shortest CP is 1.309 ms in the GEO scenario which is much longer than the Rel-15 preamble format 1 CP length. Therefore, to support the corresponding NTN scenario, the preamble CP length needs to be extended to corresponding required minimum CP length in Table 9.

Table 9. Required minimum CP length (MEA=10 degree)

| Required minimum CP length (GEO) |  | Required minimum CP length (LEO-1200) |  | Required minimum CP length (LEO-600) |  |
| --- | --- | --- | --- | --- | --- |
| D=100 km | D=400 km | D=100 km | D=300 km | D=100 km | D=200 km |
| 0.656 ms | 2.609 ms | 0.570 ms | 1.661 ms | 0.566 ms | 1.105 ms |

Overall, one could either make use of the existing preamble but limit the NTN deployment or extend the preamble design.

Proposal 7: Consider the following options in the study of preamble CP length for NTN:

Option 1: Utilize a positioning method

FFS: Effects of positioning error and ephemeris error

Option 2: Utilize the multiple detection method

Option 3: Extend the Rel-15 preamble CP length

FFS: the repetition method

Preamble detection performance

In this subsection, we evaluate the detection performance of some typical preamble formats in NTN scenarios. Simulation assumptions are according to the Table X.1, Table X.2 and Table X.11 from [11], which are listed in Appendix C. It is assumed that the CP is long enough to eliminate the impact of CP length, and the frequency offset is not considered in this evaluation part. So only the ZC sequence length and repetition number influence the detection performance. Long sequence formats 0~3 and short sequence formats B1, B4 are chosen, which include the largest repetition number. It is considered that one UE sends preamble to the gNB, and the antenna configuration is 1T1R.

The miss detention rate Pm is taken as the evaluation metric, which is defined as the ratio of the number of transmitted preambles that are not detected or detected as a different preamble to the total number of transmitted preambles. The false alarm rate (FAR) Pf is used to determine the detection threshold, which is defined as the ratio of the number of detected but not transmitted preambles to the total number of no transmitting preambles. Assuming the false alarm rate is 0.001.

Figure 24. Miss detection rate of long preamble detection (S band, 3 km/h)

Figure 25. Miss detection rate of short preamble detection (S band, 3 km/h)

Figure 26. Miss detection rate of short preamble detection (Ka band, 0 km/h, 1000 km/h)

Figure 24, Figure 25 and Figure 26 are the simulation results of preamble detection performance. When the required miss detection rate is 0.01, the required minimum SNR is listed in Table 10. Besides, the uplink received SNR based on link budget calculation is also listed in Table 10.

From Table 10, some short preamble formats and all long preamble formats can meet the SNR requirement of the LEO-1200 (30 degree) scenario in S band. For the GEO (10 degree) scenario in S band, long preamble formats can provide adequate link budget except for the format 3. Meanwhile, all short preamble formats cannot provide sufficient link budget for the GEO (10 degree) scenario in S band.

In Ka band, as the VSAT UE can have larger power and antenna gain, the received SNR is much larger than the handheld UE. For the GEO (10 degree) scenario in Ka band, the received SNR according to link budget calculation can be larger than 14 dB above required detection SNR. Therefore, for LEO-1200 (30 degree) and GEO (10 degree) scenarios in Ka band, the existing Rel-15 preamble formats may be reused with respect to SNR.

Table 10. Required minimum SNR and link budget of different formats in LEO-1200 and GEO

| Frequency band | Preamble format | SCS (kHz) | UE speed (km/h) | Required minimum SNR (dB) | LEO-1200 (30 degree) Link budget (set-1) Received SNR (dB) | GEO (10 degree) Link budget (set-1) Received SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| S band | 0 | 1.25 | 3 | -14.5 | -5.1 | -13.4 |
|  | 1 | 1.25 | 3 | -17.0 | -5.1 | -13.4 |
|  | 2 | 1.25 | 3 | -19.4 | -5.1 | -13.4 |
|  | 3 | 5 | 3 | -19.4 | -11.1 | -19.5 |
|  | B1 | 15 | 3 | -9.4 | -8.0 | -16.4 |
|  |  | 30 | 3 | -9.4 | -11.1 | -19.4 |
|  | B4 | 15 | 3 | -14.4 | -8.0 | −18.8 |
|  |  | 30 | 3 | -14.3 | -11.1 | −21.9 |
| Ka band | B1 | 60 | 0 | -9.3 | 29.7 | 17.6 |
|  |  | 60 | 1000 | -8.8 | 29.7 | 17.6 |
|  |  | 120 | 0 | -9.2 | 26.6 | 14.6 |
|  |  | 120 | 1000 | -8.7 | 26.6 | 14.6 |
|  | B4 | 60 | 0 | -14.3 | 29.7 | 17.6 |
|  |  | 60 | 1000 | -11.8 | 29.7 | 17.6 |
|  |  | 120 | 0 | -14.2 | 26.6 | 14.6 |
|  |  | 120 | 1000 | -13.5 | 26.6 | 14.6 |

Observation 29: Without consideration of frequency offset, phase noise and CP length,

- R-15 long preamble formats may work in the S band for LEO-1200 (30 degree) scenarios with respect to SNR.

- some of R-15 short preamble formats may work in the S band for LEO-1200 (30 degree) scenarios with respect to SNR.

- except for the format 3, R-15 long preamble formats may work in the S band for GEO (10 degree) scenarios with respect to SNR.

- R-15 short preamble formats cannot work in the S band for GEO (10 degree) scenarios with respect to SNR.

- R-15 short preamble formats may work in the Ka band for LEO (30 degree) and GEO (10 degree) scenarios with respect to SNR.

Subcarrier spacing

We have evaluated the CP length and detection performance of some typical Rel-15 preambles in NTN scenarios. However, the frequency offset from oscillators and movement, which is mentioned in section 2.1, is not considered in the above simulations and analyses. In practice, the residual frequency offset could have a big impact on the detection performance at the gNB. The SCS is related to the robustness to the residual frequency offset.

As mentioned in [12] and [13], the long preambles with restricted set type B can support residual frequency offset of [-2fscs, 2fscs] at most, where fscs is the preamble SCS. Meanwhile, the short preambles can support residual frequency offset of [-0.4fscs, 0.4fscs] at most. The supported residual frequency offsets by different preambles are listed in Table 11. For instance, the long preamble (SCS=1.25 kHz) can support frequency offsets of [-2.5 KHz, 2.5 KHz] with the restricted set type B.

According to Table 3, the residual UL frequency offsets with option 1 at the satellite side can be obtained and are applied in Table 12. Comparing the supported frequency offsets from Table 11 with residual UL frequency offsets from Table 3, the supported scenarios by existing different preambles at the nadir beam are listed in Table 12.

For example, when the beam diameter is 90 km in the LEO-1200 scenario, the UL frequency offset of Option 1 in S band is 1.92 ppm according to Table 3, i.e., 3.84 KHz, which is much larger than the supported frequency offset by the long preamble (SCS=1.25 KHz). Therefore, the long preamble (SCS=1.25 KHz) cannot work in this LEO-1200 scenario.

When employing the restricted set type A or the restricted set type B, the valid preamble CP for absorbing the large differential round trip delay of UL signals is shorter than the actual CP length in Table 5. For instance, the long preamble (SCS=5 KHz) format with the restricted set type B can only support 5.66 km diameter in LEO-1200 scenarios (30 degree). However, the actual CP length of the long preamble (SCS=5 KHz) can support 17.89 km diameter in LEO-1200 scenarios (30 degree).

Therefore, the issue of uplink frequency offset is a challenge in NTN scenarios. As an option for providing higher robustness to frequency offset, higher preamble SCS is preferred.

Observation 30: Considering the residual UL frequency offset, LEO-1200 (D=90 km) and LEO-600 (D=50 km) scenarios in Ka band cannot be supported by existing Rel-15 preambles.

Observation 31: To provide higher robustness to UL frequency offset, higher preamble SCS is preferred.

Table 11. Supported residual frequency offset by different preambles

| Format | SCS (KHz) | Supported residual frequency offset |  |  |
| --- | --- | --- | --- | --- |
|  |  | Unrestricted set | Restricted set type A | Restricted set type B |
| Long preamble (839) | 1.25 | [-0.5 KHz, 0.5 KHz] | [-1.25 KHz, 1.25 KHz] | [-2.5 KHz, 2.5 KHz] |
| Long preamble (839) | 5 | [-2 KHz, 2 KHz] | [-5 KHz, 5 KHz] | [-10 KHz, 10 KHz] |
| Short preamble (139) | 15 | [-6 KHz, 6 KHz] | N/A | N/A |
| Short preamble (139) | 30 | [-12 KHz, 12 KHz] | N/A | N/A |
| Short preamble (139) | 60 | [-24 KHz, 24 KHz] | N/A | N/A |
| Short preamble (139) | 120 | [-48 KHz, 48 KHz] | N/A | N/A |

Table 12. Supported scenarios by different preambles considering UL frequency offset at the nadir beam

| Orbit | Beam configuration Diameter(km) & frequency band | Unrestricted /restricted set | Long preamble (839) SCS (KHz) |  | Short preamble (139) SCS (KHz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.25 | 5 | 15 | 30 | 60 | 120 |
| LEO-1200 km | 90 km (Set 1 - S-band) | Unrestricted set | × | × | √ | √ | N/A | N/A |
|  |  | Restricted set type A | × | √ | N/A | N/A | N/A | N/A |
|  |  | Restricted set type B | × | √ | N/A | N/A | N/A | N/A |
|  | 40 km (Set 1 - Ka-band) | Unrestricted set | N/A | N/A | N/A | N/A | × | √ |
|  |  | Restricted set type A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | Restricted set type B | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 190 km (Set 2 - S-band) | Unrestricted set | × | × | × | √ | N/A | N/A |
|  |  | Restricted set type A | × | × | N/A | N/A | N/A | N/A |
|  |  | Restricted set type B | × | √ | N/A | N/A | N/A | N/A |
|  | 90 km (Set 2 - Ka-band) | Unrestricted set | N/A | N/A | N/A | N/A | × | × |
|  |  | Restricted set type A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | Restricted set type B | N/A | N/A | N/A | N/A | N/A | N/A |
| LEO-600 km | 50 km (Set 1 - S-band) | Unrestricted set | × | × | √ | √ | N/A | N/A |
|  |  | Restricted set type A | × | √ | N/A | N/A | N/A | N/A |
|  |  | Restricted set type B | × | √ | N/A | N/A | N/A | N/A |
|  | 20 km (Set 1 - Ka-band) | Unrestricted set | N/A | N/A | N/A | N/A | × | √ |
|  |  | Restricted set type A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | Restricted set type B | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 90 km (Set 2 - S-band) | Unrestricted set | × | × | × | √ | N/A | N/A |
|  |  | Restricted set type A | × | × | N/A | N/A | N/A | N/A |
|  |  | Restricted set type B | × | √ | N/A | N/A | N/A | N/A |
|  | 50 km (Set 2 - Ka-band) | Unrestricted set | N/A | N/A | N/A | N/A | × | × |
|  |  | Restricted set type A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | Restricted set type B | N/A | N/A | N/A | N/A | N/A | N/A |

Sequence repetition

As mentioned above, the CP length and preamble detection performance are not sufficient in some NTN scenarios. Here, we analyses a simple repetition method, which extends the CP length and sequence length.

As mentioned in subsection 2.3.1.1, extending the CP with the repetition method can be considered to compensate for the large differential delay in NTN. Meanwhile, to ensure that no other transmissions are received with the preamble at the same time by the satellite, longer Guard Time (GT) may be needed at the receiver. Basically, the GT is just a scheduling restriction in practice. Thus, the satellite can provide different GTs in different cells or beams, which depends on implementation. Overall, the maximum differential delay could change with different cells or beams. Hence, extending different CP and GT lengths could be adopted in different cells or beams.

As mentioned in subsection 2.3.1.2, detecting a preamble with high reliability is a challenge for a mobile UE with low transmit power in GEO scenarios. One straightforward way to enhance the preamble detection is to increase the repetition number of preamble sequences at the expense of longer RACH time.

Observation 32: Increasing the CP and GT lengths and the repetition number of preamble sequences may be considered for enhancing the preamble transmission in NTN.

The repetition method can also be applied to the CP extension. Figure 27 is an illustration of the repetition of CP and preamble sequences. It is assumed that there are 3.5 duplicate preamble sequences in CP. As mentioned above, by using the repetition for preamble sequences to increase the detection reliability, 8 duplicate preamble sequences in the PRACH are assumed in Figure 27.

If the CP length can cover the large differential delay among UEs in an NTN cell, all preambles with the long CP can be received within the detection window. However, for the far-user preamble, there may be two or three CP sequences locating in the detection window, and correspondingly two or more boundaries between the CP and the preamble sequences may be found. This may confuse the satellite when it intends to determine the UE-specific TA command. As a result, the satellite may not be able to send the correct TA indication to the far-UE. Consequently, the repetition method to revise preamble formats needs further study.

Figure 27. CP and preamble sequence repetition

Observation 33: With simple repetition method to extend the CP and preamble sequences lengths, it is not possible to find the exact boundary between the CP and preamble sequences.

Timing for transmitting Msg3 and ACK

In step 3 of RACH procedures, if in slot  UE receives a PDSCH with a RAR message for a corresponding PRACH transmission from gNB, UE transmits MSG3 on PUSCH in slot n∙2μPUSCH2μPDCCH+K2+∆ [8], where μPUSCH and μPDCCH are the subcarrier spacing configurations for PUSCH and PDCCH, respectively. The values of K2 and ∆ are given by Appendix D.

Once UE successfully decodes MSG4, UE sends HARQ ACK for the data after K1 slot. The value of K1 is specified in RRC parameter: dl-DataToUL-ACK in PUCCH-Config.

In the following the values of K2+∆ and K1 are discussed.

- From Table 6.1.2.1.1-2 and Table 6.1.2.1.1-3 in Appendix D, it can be seen that the maximum value of K_2+∆ is j+3+∆, where j and ∆ for different μ_PUSCH are respectively given by Table 6.1.2.1.1-4 and Table 6.1.2.1.1-5 in [14] (details are presented in Appendix D). In the case of μ_PUSCH=0 (SCS = 15kHz), the value of j+3+∆ equals to 6 slots. This means that the latest time for transmitting Msg3 is after 6 slots from receiving MSG2.

- The value of K1 is provided by [8]. As can be seen, the maximum value of K1 is 8 slots (for DCI format 1_0). This means that the latest time for sending HARQ ACK is after 8 slots from decoding MSG4.

In terrestrial system, the delay of 6 or 8 slots is sufficient for UE to complete the signal processing and timing advance procedures. However, it is inadequate to compensate for the hundreds of milliseconds propagation delay in NTN.

Observation 34: The delay for MSG3 on PUSCH and HARQ-ACK of MSG4 on PUCCH is insufficient.

Proposal 8: Constant offset should be considered for extending the delay of MSG3 on PUSCH and HARQ-ACK of MSG4 on PUCCH.

Conclusion

In this document, the following observations and proposals are presented:

Observation 1: The PSS detection rates of SCS15kHz and SCS30kHz are almost the same under the considered fading channels.

Observation 2: The PSS detection rates of SCS120kHz and SCS240kHz are almost the same under the considered fading channels.

Observation 3: The SNR points corresponding to 90% likelihood for all SSB numerologies are about -5dB for the considered fading channels.

Observation 4: The relative residual frequency offset (normalized by SCS) of different SSB numerologies are almost the same under the considered fading channels.

Observation 5: The absolute residual frequency offset increases as the SSB SCS increases.

Observation 6: About 90% the residual frequency offset can be controlled within0.03SCS.

Observation 7: The PSS-SSS based method can provide better frequency tracking performance than the PBCH-DMRS based method.

Observation 8: The absolute timing residual offset reduces as the SSB SCS increases.

Observation 9: The PSS-SSS based method can provide better timing tracking performance than the SSB-DMRS based method.

Observation 10: A UE needs to adjust the timing point autonomously between two pilot signals to avoid possible ISI due to DL timing drift in LEO.

Observation 11: A UE needs to compensate the phase change within a slot due to DL timing drift to ease low complexity channel estimation and equalizer.

Observation 12: It is necessary to acquire DL timing drift rate to avoid potential ISI and compensate phase change.

Observation 13: Aligned to the frequency of DL signal, the UL frequency misalignment at the BS side is double the (residual) Doppler shift.

Observation 14: With the UL frequency pre-compensation at the UE side, the UL frequency misalignment at the BS side is double the oscillator errors.

Observation 15: The UL frequency offset in LEO is a challenge for RACH.

Observation 16: In the closed-loop method, the full common TA can be broadcasted by the gNB in both regenerative satellite and transparent satellite scenarios.

Observation 17: In the closed-loop method, the partial common TA can be broadcasted and the residual delay can be compensated by the gNB.

Observation 18: In the open-loop method, the UE can calculate the TA for transmitting preamble according to its position and the satellite ephemeris.

Observation 19: The extended initial TA with the scaling factor  may not meet the timing accuracy requirement, especially for large SCS used in FR2.

Observation 20: Since the accuracy of positioning worsens in some cases, open-loop TA adjustment mechanism based on UE GNSS and satellite ephemeris may not meet the TA adjustment accuracy requirement in certain cases.

Observation 21: To maintain the uplink time synchronization, gNB has to frequently send TA adjustment commands to each UE in LEO scenarios, which will introduce increased signaling overhead to the system.

Observation 22: Compared with sending TA commands frequently to the UE, signaling overhead can be greatly reduced if the UE is able to update the TA by itself.

Observation 23: There is no Rel-15 preamble format whose CP length is sufficient for all NTN scenarios.

Observation 24: From the perspective of limiting scenarios, increasing the supported minimum elevation angle or decreasing the supported maximum beam diameter can reduce the requirement of the preamble CP length.

Observation 25: The possible supported minimum elevation angles of different specific beams by Rel-15 preamble format 1 CP length are listed below:

| Format | Supported minimum elevation angle of beam (degree) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | GEO |  | LEO-1200 |  | LEO-600 |  |
|  | D=200 km | D=400 km | D=100 km | D=300 km | D=100 km | D=200 km |
| 1 | 59 | 74 | 10 | 63 | 10 | 52 |

Observation 26: The possible supported maximum beam diameter of different specific beams by Rel-15 preamble format 1 CP length are 104.39 km (GEO, MEA=10 degree), 120.33 km (LEO-1200, MEA=30 degree) and 121.47 km (LEO-600, MEA=30 degree).

Observation 27: Utilizing a positioning method can reduce the requirement for the preamble CP length.

Observation 28: From the perspective of implementation, utilizing the multiple detection method can reduce the requirement for the preamble CP length.

Observation 29: Without consideration of frequency offset, phase noise and CP length,

- R-15 long preamble formats may work in the S band for LEO-1200 (30 degree) scenarios with respect to SNR.

- some of R-15 short preamble formats may work in the S band for LEO-1200 (30 degree) scenarios with respect to SNR.

- except for the format 3, R-15 long preamble formats may work in the S band for GEO (10 degree) scenarios with respect to SNR.

- R-15 short preamble formats cannot work in the S band for GEO (10 degree) scenarios with respect to SNR.

- R-15 short preamble formats may work in the Ka band for LEO (30 degree) and GEO (10 degree) scenarios with respect to SNR.

Observation 30: Considering the residual UL frequency offset, LEO-1200 (D=90 km) and LEO-600 (D=50 km) scenarios in Ka band cannot be supported by existing Rel-15 preambles.

Observation 31: To provide higher robustness to UL frequency offset, higher preamble SCS is preferred.

Observation 32: Increasing the CP and GT lengths and the repetition number of preamble sequences may be considered for enhancing the preamble transmission in NTN.

Observation 33: With simple repetition method to extend the CP and preamble sequences lengths, it is not possible to find the exact boundary between the CP and preamble sequences.

Observation 34: The delay for MSG3 on PUSCH and HARQ-ACK of MSG4 on PUCCH is insufficient.

Proposal 1: Study the potential solutions for DL timing drift rate estimation.

Proposal 2: Take the NTN specific UL frequency offset into the consideration of RACH design.

Proposal 3: To obtain the TA for transmitting a preamble, consider at least the following options:

Closed-loop method

Option 1: gNB broadcasts the full common TA.

Option 2: gNB broadcasts the partial common TA and the network compensates the residual delay.

FFS: Details on signaling and forms

Open-loop method

Option 3: The UE calculates the TA according to its position and the satellite ephemeris.

FFS: Effects of positioning error and ephemeris error

Proposal 4: Increase the bit width of the TA command, if the timing accuracy requirement cannot be met when adopting the scaling factor.

Proposal 5: It is necessary to retain the closed-loop TA adjustment mechanism due to positioning inaccuracy, even in case the GNSS of UE and the ephemeris of serving satellites are available.

Proposal 6: Besides utilizing the existing TA adjustment mechanism, UE may also implement self-adjustment of the TA value with the help of the timing drift rate. The following options can be considered:

Option 1: gNB indicates the timing drift rate to each UE.

Option 2: gNB broadcasts a cell/beam-specific common timing drift rate.

Option 3: UE estimates the timing drift rate by itself.

FFS: Details on signaling and forms

FFS: Overhead and accuracy

Proposal 7: Consider the following options in the study of preamble CP length for NTN:

Option 1: Utilize a positioning method

FFS: Effects of positioning error and ephemeris error

Option 2: Utilize the multiple detection method

Option 3: Extend the Rel-15 preamble CP length

FFS: the repetition method

Proposal 8: Constant offset should be considered for extending the delay of MSG3 on PUSCH and HARQ-ACK of MSG4 on PUCCH.

- References

RP-181370, “Study on solutions evaluation for NR to support Non-Terrestrial Network”, RAN#80.

3GPP TR 38.811 V15.0.0, “Study on New Radio (NR) to support non-terrestrial networks”.

R1-1905998, “Evaluation of DL synchronization and central frequency tracking for NTN”, Huawei, HiSilicon

R1-1906264, “Issues on Timing Advance and RACH for NTN”, Panasonic.

R1-1805095, “NR-NTN: Impact on initial TA during random access procedure”, Thales.

R1-1904650, “Doppler Compensation, Uplink Timing Advance, Random Access and UE Location in NTN”, Nokia.

3GPP TS 38.133 V15.5.0, “Requirement for support of radio resource management”.

3GPP TS 38.213 V15.5.0, “Physical layer procedures for control”.

3GPP TR 38.821 V0.4.0, “Solutions on NR to support non-terrestrial networks”.

3GPP TS 38.211 V15.5.0, “Physical channels and modulation”.

3GPP TSG RAN WG1 Meeting #97, “RAN1 Chairman’s Notes”.

R1-1609349, “Restricted sets of cyclic shifts for PRACH in high speed scenario”, Huawei

R1-1714661, “DRAFT WF on Restricted set for PRACH”, ZTE

3GPP TS 38.214 V15.5.0, “NR; Physical layer procedures for data”.

- Appendix A. Simulation assumptions for DL synchronization [11]

Table X.10 : LLS parameters for DL synchronization evaluation

|  | S-band | Ka-band |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 20 GHz |
| Channel Model | For GEO (optional) : Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle 10 deg For LEO : Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle [30] deg |  |
| Subcarrier Spacing(s) | 15kHz, 30kHz | 120kHz, 240kHz |
| DL RS | SSB |  |
| Antenna Configuration at the TRP (satellite) | 1Tx | 1Tx |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| UE speed | 3 km/h | 0 km/h, 1000 km/h |
| UE elevation angle | For GEO (optional) : 10°, For LEO : 30° |  |
| Frequency Offset | - UE crystal accuracy: 10 ppm - Satellite : oscillator accuracy values provided in Table X.9 - Doppler shift in channel due to satellite movement : max. Doppler shift values provided in Table X.9 - Doppler shift in channel due to UE movement : max. value to be computed based on the UE speed and the elevation angle Note 1 : The final frequency offset is the sum up of all the contributions Note 2 : Doppler spectrum on Rayleigh fading taps based on Jake model should be considered in addition to Doppler shift (see section 6.9.2 in TR 38.811) Note 3 : For a Rayleigh fading tap a minimum Doppler of 1 Hz should be considered. |  |
| Frequency drift | [Doppler rate values provided in Table X.9] |  |
| Phase noise model | FFS |  |
| Metrics | CDF of timing and frequency residual offset at SNIR point corresponding to 90% likelihood. Note 4: FAR requirement = 1% |  |

Note: The SNR range to be evaluated should be based on the link budget analysis for each channel

- Appendix B. NR specification for UE transmit timing [7]

The UE should meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. The reference point for the UE initial transmit timing control requirement is the downlink timing of the reference cell minus . The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell.

When the transmission timing error between the UE and the reference timing exceeds Te, the UE is required to adjust its timing to within Te. The reference timing will be  before the downlink timing of the reference cell.

Table 7.1.2-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (KHz) | SCS of uplink signals (KHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 |  |  |  |

- Appendix C. Simulation assumptions for PRACH performance evaluation [11]

Table X.11 : LLS parameters for PRACH performance evaluation

| Configurations | S-band | Ka-band |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 30 GHz |
| Channel Model | For GEO (optional) : Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle 10 deg For LEO : Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle [30] deg |  |
| Antenna Configuration at the TRP (satellite) | 1 Rx 2 Rx optional | 1 Rx 2 Rx optional |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| Frequency Offset | FFS |  |
| UE speed | 3 km/h | 0 km/h, 1000 km/h |
| Initial timing Offset (Note 1) | FFS |  |
| Phase noise model | FFS |  |
| PRACH design | Each company should provide details on configuration (i.e. format, SCS, N_CS, …). New formats are not precluded. |  |
| Metric | PRACH detection rate, FAR |  |
| Note 1: Ideal common delay compensation is assumed. |  |  |

Table X.1: Set-1 Satellite parameters for System Level Simulations

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35’786 km | 1’200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| Satellite EIRP density |  | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| Satellite beam diameter (Note 2) |  | 250 km | 90 km | 50 km |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 5 m | 0.5 m | 0.5 m |
| Satellite EIRP density |  | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| Satellite beam diameter (Note 2) |  | 110 km | 40 km | 20 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| G/T |  | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 5 m | 0.5 m | 0.5 m |
| G/T |  | 28 dB K-1 | 13 dB K-1 | 13 dB K-1 |
| Satellite RX max Gain |  | 62 dBi | 42 dBi | 42 dBi |
| Note 1: This value is equivalent to the antenna diameter to be used in Sec. 6.4.1 of TR 38.811. Note 2: This beam size refers to the Nadir pointing of the satellite |  |  |  |  |

Table X.2 : Set-2 Satellite parameters for System Level Simulations

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35’786 km | 1’200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 12 m | 1 m | 1 m |
| Satellite EIRP density |  | 53.5 dBW/MHz | 34 dBW/MHz | 28 dBW/MHz |
| Satellite Tx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |
| Satellite beam diameter (Note 2) |  | 450 km | 190 km | 90 km |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 2 m | 0.2 m | 0.2 m |
| Satellite EIRP density |  | 32 dBW/MHz | 2 dBW/MHz | -4 dBW/MHz |
| Satellite Tx max Gain |  | 50.5 dBi | 30.5 dBi | 30.5 dBi |
| Satellite beam diameter (Note 2) |  | 280 km | 90 km | 50 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band (i.e. 2 GHz) | 12 m | 1 m | 1 m |
| G/T |  | 14 dB K-1 | -4.9 dB K-1 | -4.9 dB K-1 |
| Satellite Rx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 2 m | 0.2 m | 0.2 m |
| G/T |  | 20 dB K-1 | 5 dB K-1 | 5 dB K-1 |
| Satellite Rx max Gain |  | 54 dBi | 34 dBi | 34 dBi |
| Note 1: This value is equivalent to the antenna diameter to be used in Sec. 6.4.1 of TR 38.811. Note 2: This beam size refers to the Nadir pointing of the satellite |  |  |  |  |

- Appendix D. Default PUSCH time domain resource allocation of 5G NR

The PUSCH time domain allocation is defined in [14].

Table 6.1.2.1.1-2: Default PUSCH time domain resource allocation A for normal CP

| Row index | PUSCH mapping type | K2 | S | L |
| --- | --- | --- | --- | --- |
| 1 | Type A | j | 0 | 14 |
| 2 | Type A | j | 0 | 12 |
| 3 | Type A | j | 0 | 10 |
| 4 | Type B | j | 2 | 10 |
| 5 | Type B | j | 4 | 10 |
| 6 | Type B | j | 4 | 8 |
| 7 | Type B | j | 4 | 6 |
| 8 | Type A | j+1 | 0 | 14 |
| 9 | Type A | j+1 | 0 | 12 |
| 10 | Type A | j+1 | 0 | 10 |
| 11 | Type A | j+2 | 0 | 14 |
| 12 | Type A | j+2 | 0 | 12 |
| 13 | Type A | j+2 | 0 | 10 |
| 14 | Type B | j | 8 | 6 |
| 15 | Type A | j+3 | 0 | 14 |
| 16 | Type A | j+3 | 0 | 10 |

Table 6.1.2.1.1-3: Default PUSCH time domain resource allocation A for extended CP

| Row index | PUSCH mapping type | K2 | S | L |
| --- | --- | --- | --- | --- |
| 1 | Type A | j | 0 | 8 |
| 2 | Type A | j | 0 | 12 |
| 3 | Type A | j | 0 | 10 |
| 4 | Type B | j | 2 | 10 |
| 5 | Type B | j | 4 | 4 |
| 6 | Type B | j | 4 | 8 |
| 7 | Type B | j | 4 | 6 |
| 8 | Type A | j+1 | 0 | 8 |
| 9 | Type A | j+1 | 0 | 12 |
| 10 | Type A | j+1 | 0 | 10 |
| 11 | Type A | j+2 | 0 | 6 |
| 12 | Type A | j+2 | 0 | 12 |
| 13 | Type A | j+2 | 0 | 10 |
| 14 | Type B | j | 8 | 4 |
| 15 | Type A | j+3 | 0 | 8 |
| 16 | Type A | j+3 | 0 | 10 |

Table 6.1.2.1.1-4: Definition of value j

| µPUSCH | j |
| --- | --- |
| 0 | 1 |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |

Table 6.1.2.1.1-5: Definition of value Δ

| µPUSCH | Δ |
| --- | --- |
| 0 | 2 |
| 1 | 3 |
| 2 | 4 |
| 3 | 6 |