3GPP TSG RAN WG1 Meeting #98	 R1-1908065

Prague, Czech Republic, August 26-30, 2019

Agenda Item:	7.2.8.1

Source:	Huawei, HiSilicon

Title:	Discussion on CSI enhancement

Document for:	Discussion and Decision

Introduction

Based on the WID of NR MIMO enhancements for Rel-16 in RAN meeting #80 [1], Rel-16 will specify overhead reduction, based on Type II CSI feedback, taking into account the tradeoff between performance and overhead as follows:

- Extend specification support in the following areas [1]

  - Enhancements on MU-MIMO support:

    - Specify overhead reduction, based on Type II CSI feedback, taking into account the tradeoff between performance and overhead

    - Perform study and, if needed, specify extension of Type II CSI feedback to rank >2.

It has been agreed at RAN1 #95 [2] that:

Agreement

For Rel-16 NR, agree on Alt1 (DFT-based compression) in Table 1 of R1-1813002 as the adopted Type II rank 1-2 overhead reduction (compression) scheme as formulated in Alt1.1 of R1-1813002

- Note: The same DFT-based compression scheme is extended for Type II port selection codebook

- Codebook subset restriction (CBSR) is supported when DFT-based compression is utilized for Type II codebooks with overhead reduction (compression) scheme

  - FFS: detailed signaling mechanism

- Note: Additional compression scheme(s) are not precluded

It has been agreed at RAN1 AH1901 [3] that:

Agreement

On FD compression unit, agree on Alt1 (PMI subband size = CQI subband size) as the default, along with Alt2.2 (PMI subband size = CQI subband size / R) as secondary

- The value of R is fixed to 2

- FFS: Whether secondary implies a separate UE capability or restricted use cases

  - Include issues such as limitation on the number of FD compression units, CPU occupation, latency constraint and/or BW constraint

- FFS: Whether FD compression unit is higher-layer configured or reported by the UE

It has been agreed at RAN1 96bis [4] that:

Agreement

Support L=6 for the following combinations of p and beta

- p value equals to 1/4, beta value equals to {1/4, ½, 3/4}

- p value equals to 1/2, beta value equals to 1/4

Above applies only for the case of 32 ports, rank 1 or 2, R=1

Note that the payload size for L=6 should not exceed that of Rel-15 type-2 codebook

The above feature is UE optional

FFS: Further specification support to relax UE processing complexity

It has been agreed at RAN1 97 [5] that:

Agreement

Alt3C (illustrated in the table below) is supported where the parameter p=v0 for RI=3-4 is higher-layer configured in conjunction with the parameter p=y0 for RI=1-2.

- The parameters (y0, v0)(y0,v0) take value from   12,14,14,14,14,18

| RI | Layer | L | p |
| --- | --- | --- | --- |
| 1 | 0 | x0 | y0 |
| 2 | 0 |  |  |
|  | 1 |  |  |
| 3 | 0 |  | v0 |
|  | 1 |  |  |
|  | 2 |  |  |
| 4 | 0 |  |  |
|  | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |

FFS: Possible down-selection on the FD combination parameters in RAN1#98

Agreement

For further details on the agreed UCI parameters in Table 1 of R1-1905629:

- The following FD basis subset selection scheme is supported:

  - For N3≤19N3≤19, one-step free selection (cf. Alt5.1 in RAN1#96bis) is used

  - For N3>19, IntS is window-based and fully parameterized with MinitialMinitial, indicating that the intermediate set consists of FD bases mod(Minitial + n, N3), n=0,1, …,  modMinitial+n,N3, n=0,1,⋯N3'-1

    - The value N3'=αM where  is higher-layer configured from two possible values

      - FFS (to be finalized in RAN1#98 Prague): the supported parameter combinations for (L, p, β, )L,p,β,α

  - The 2nd step subset selection is indicated by an X2-bit combinatorial indicator (for each layer) in UCI part 2

Agreement

In RAN1#98, finalize the values of  based on the following aspects

- Candidate values for  to be down selected/evaluated: at least {1.5, 2, 2.5}

  - The set of values is to be finalized via offline email discussion prior to RAN1#98

- Configuration of :

  - Whether it is independent of other FD compression parameters, or dependent on at least one of the other FD compression parameters, i.e. p (=y0, and/or v0 for RI=3-4), L, β, and/or R

  - Whether  is rank-specific or rank-common

  - Note: This is to be discussed along with the supported parameter combinations for (L, p, β, ) L,p,β,α

Agreement

In RAN1#98, decide if the specification will restrict the UE from reporting all “zero” in the bitmap for a polarization for each layer

In this contribution, we discuss some remaining issues for the Rel-16 codebook design, including codebook subset restriction, CSI omission, UCI reporting, port selection codebook, parameter combination reduction and UE capability. More details are in our companion paper [6].

Codebook subset restriction

Codebook subset restriction (CBSR) is an effective method to suppress the interference of adjacent cells through restriction of the direction and strength of some spatial beams. For Rel-15 Type II codebook, the CBSR is implemented in two dimensions:

- Restriction of spatial beam vectors. Part of O1O2 spatial beam vector groups are restricted, where each spatial beam group comprises N1N2 spatial beam vectors.

- Restriction of the amplitude of spatial beam vectors. Because weak spatial beams do not cause strong interference to the other cells, in order to improve the freedom of beam selection, additional restriction of the WB amplitude is introduced to each of the restricted spatial beam vectors.

In RAN1 #95, it has been agreed that CBSR is supported when DFT-based compression is utilized for Type II codebooks. Compared with Rel-15 Type II codebook, additional frequency basis vectors are introduced. The CBSR design for Rel-16 codebook can be categorized into two methods:

- Method 1: The spatial beam vectors and their corresponding amplitude of spatial-frequency coefficients are restricted.

- Method 2: The spatial beam vectors, FD basis vectors and their corresponding amplitude of spatial-frequency coefficients are restricted.

In Rel-15 CBSR for Type II codebook, restriction of SD basis is well understood. One benefit of CBSR is to control the direction and strength of spatial beams. For FD compressed codebook, the FD basis vectors correspond to the time domain taps of the fading channel. It is worth noting that the propagation delays involved in one beam is one of the basic characteristics of fading channels that have no direct relationship to SD beams. However the selection of FD basis cannot be restricted as the delay taps in time domain directly. Based on the agreement in last meeting, the strongest FD basis for each layer is fixed through cyclic shift and UE only reports the modulo-shifted indices in the FD basis indicator. This is because a cyclic-shift to the selected FD basis of a certain layer corresponds to a linear phase rotation due to the property of DFT bases and a phase rotation does not cause any performance variation. This means that the specified FD basis vectors cannot be restricted because they can be replaced by another set of vectors. When the normalization in spatial-frequency LC coefficients quantization and the phase correction proposed in [7] are further considered, the relationship between FD basis and channel delays becomes more complicated and unclear. Therefore it is hard to determine in advance which FD basis vector needs to be restricted in order to controlling spatial beams.

A simple and effective method of CBSR for compressed codebook is following the basic principles of existing CBSR for R15 Type II codebook without restriction of FD basis vectors. In R15 CBSR of Type II codebook, the strength of SD beam can be limited by restriction of maximum allowed WB amplitude. For the FD compressed codebook W=W1W2WfH, the coefficients corresponding to i-th spatial beam can be written in the form of vector as follows,

W2(i)=W2(i)WfH                                  (1)

where W2(i) is the i-th row vector of W2. Because of WfH is the normalized orthogonal basis matrix, the WB amplitude of i-th spatial beam can be upperbounded as

AiWB=1N3j=1MW2i,j2≤MN3maxj=1,..,MW2i,j2=MN3maxj=1,..,MW2i,j              (2)

where W2i,j is the element of W2 in i-th row and j-th column which corresponding to i-th SD beam vector and j-th FD basis vector, j=1, 2,…, M. Based on equation (2), the WB amplitude of one spatial beam can be directly restricted by the maximum quantized amplitude of coefficients corresponding to this beam, e.g. maxj=1,..,MW2i,j≤Pmax. Specifically, for the DFT-based compressed codebook, the CBSR can be implemented as:

- Restriction of spatial beam vectors. Part of SD beam vector groups are restricted.

- Restriction of the maximum quantized amplitude of coefficients corresponding to the SD beam vector in the restricted beam vector groups. The quantized amplitude is the product of reference amplitude and differential amplitude.

For example, as shown in Figure 1, the configured maximum allowed amplitude of the strongest spatial-frequency coefficients corresponding to Beam 1 and Beam 2 are Pmax1 and Pmax2, respectively. For M=4, each SD beam vector corresponds to 4 coefficients. All of the amplitude of M=4 spatial-frequency coefficients corresponding to beam 1 are less than Pmax1. Therefore, Beam1 can be used without any adjustment. However, Beam 2 cannot be used unless the amplitude of the coefficients corresponding to the first two FD basis vectors is reduced to Pmax2, because the amplitude of strongest coefficient corresponding to Beam 2 is larger than the maximum allowed value.

Figure 1. Illustration of CBSR of DFT-based compressed codebook

Based on analysis above, the higher layer parameter similar to the bitmap n1-n2‑codebookSubsetRestriction in R15 can be introduced to indicate the CBSR for DFT based compressed codebook. For the DFT-based compressed codebook, the CBSR can be implemented as:

- Part of SD beam vector groups are restricted. The UE shall be configured with restrictions for 4 SD basis vector groups g(k) indicated by binary sequence B1 in the same manner with R15 Type II codebook. Each restricted SD beam vector group has N1N2 restricted SD beams.

- Restriction of the maximum quantized amplitude of coefficients corresponding to the SD beam vector in the restricted beam vector groups. The bit sequence B2=B2(0)B2(1)B2(2)B2(3) is the concatenation of the bit sequences B2(k)=b2(k,2N1N2-1)⋯b2(k,0) corresponding to the 4 restricted SD beam groups. The pair of bits b2k,2N1x2+x1+1b2k,2N1x2+x1 in B2(k) indicate the maximum allowed amplitude Pmax(x1,x2) of strongest spatial-frequency coefficients corresponding to SD beam vector indexed by x1 and x2, where x1=0,1,…,N1-1, x2=0,1,…,N2-1.The restricted quantized amplitude is the product of reference amplitude and differential amplitude. The candidate value of Pmax(x1,x2) can be predefined in the similar approach as Table 1.

Table 1. Maximum allowed amplitude coefficients for restricted vectors

| Bits | Maximum Amplitude of Coefficient |
| --- | --- |
| 00 |  |
| 01 | 1/2 |
| 10 | 1/2 |
| 11 |  |

Observation 1: Restriction of WB amplitude corresponding to a spatial beam can be achieved just by restricting the amplitude of spatial-frequency coefficients corresponding to the beam. The bitmap parameter n1-n2-codebookSubsetRestriction in R15 can be reused to indicate the CBSR of compressed codebook by slightly updating the interpretation of the bit sequence of B2.

Proposal 1: For CBSR of FD compressed codebook, existing principle of CBSR for R15 Type II codebook should be reused so that only spatial beam vectors and their corresponding amplitude of spatial-frequency coefficients are restricted.

CSI omission

Similar with Rel-15 CSI, the PMI overhead of Rel-16 is rank dependent. For instance, the rank 2 overhead is almost two times of the rank 1 overhead. If the overhead to be reported exceeds the PUSCH resources allocated by the gNB, a part of CSI part 2 should be dropped to fit the allocated resources. Therefore, CSI omission rule should also be considered for the Rel-16 codebook.

A simple way of CSI omission is to drop some of the reported non-zero coefficients. Since the number of actual reported coefficients is indicated in CSI part 1, the gNB knows the part 2 payload after dropping exactly from CSI part 1 and the remaining non-zero coefficients are indicated by the bitmaps. It should be noted that such CSI omission rule has no spec impact and the coefficients to be dropped can be implemented by UE.

However, the bitmaps also use a lot of overhead in the CSI reporting. If only the non-zero coefficients are dropped and the sizes of bitmaps remain the same, the bitmaps may be rather “empty” to indicate only a few non-zero coefficients, which is a waste of overhead. A better way is to drop part of bitmap together with the non-zero coefficients. Since the SD basis indication is layer-common and FD basis indication is layer-independent, it is natural to drop some FD basis to reduce the size of bitmap without introducing additional signalling indication.

Since the number of non-zero coefficients is indicated in part 1, the actual number and overhead of reported non-zero coefficients in part 2 can be known by the gNB. However, the number of dropped FD basis and the size of bitmaps cannot be directly known according to the CSI part 1. A priority table can be introduced to solve this problem. Part of FD basis for larger layer index will be dropped firstly. With such a priority table known by both UE and gNB, the gNB can calculate exact part 2 overhead given RI and #NZC in part 1 for different dropped priority levels, then the gNB can determine the number of dropped priority levels with the allocated resource.

An evaluation result for CSI omission is provided. Suppose that the gNB allocates the PUSCH resource of rank 1 and UE reports rank 2. In this example, the configured (L, p, β) is (4, 0.5, 0.5). For the 13 subbands case, the value of M is 7 and the size of bitmap for rank 1 is 2*4*7=56 and K0=28. We consider two different dropping alternatives. Dropping Alt0 is the one without spec impact and drops NZC only. Dropping Alt1 follows the rule above with FD basis priority but drop different numbers of levels. Mi is the number of FD basis of layer i.

- Dropping Alt0: only NZC dropped, M0=M1=7

- Dropping Alt1: NZC and weaker half FD basis for layer 1 dropped, i.e., M0=7, M1=4

The overhead of rank 1 and different dropping alternatives are listed in Table 2. Alt0 and Alt1 have similar total overhead but different sizes of bitmaps. For the overhead restriction, two alternatives report different numbers of non-zero coefficients (18 and 22 for Alt0 and Alt1, respectively). The maximum reported coefficients can be computed by the total overhead and the overhead except NZCs. The overhead except NZCs can be uniquely determined by the dropping of FD basis.

Table 2. Overhead calculation of different dropping alternatives

|  | SD over-sampling | SD basis | FD basis | Bitmap | SCI | Reference amplitude | Coefficients | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RI=1 | 4 | 11 | 10 | 56 | 3 | 4 | 7*(28-1) =189 | 277 |
| RI=2, Alt0 | 4 | 11 | 10+10=20 | 56+56=112 | 3+3=6 | 4+4=8 | 7*(18-2) =112 | 273 |
| RI=2, Alt1 | 4 | 11 | 10+8=18 | 56+32=88 | 3+3=6 | 4+4=8 | 7*(22-2) =140 | 275 |

Performance of dropping Alt0 and Alt1 is shown in Fig. 2. To compare the performance, we suppose the CSI dropping happens in each transmission in the evaluation. According to the evaluation results, Alt 1 performs better than Alt0 with around 4% gain for mean UPT.

Figure 2. Performance of different dropping alternatives.

Another example is the gNB allocates the PUSCH resource of rank 3 and UE reports rank 4. Assume that the configured (L, p, v, β) is (4, 0.5, 0.25, 0.5). For the 13 subbands case, the number of FD basis per layer for rank 3 is 4 and K0=28. Dropping Alt0 is the one without spec impact and drops NZC only. Dropping Alt1 drops not only NZC but also part of FD basis for some layers.

- Dropping Alt0: only NZC dropped, M0=M1=M2=M3=4

- Dropping Alt1: NZC and weaker half FD basis for layer 2 and 3 dropped, i.e., M0=M1=4, M2=M3=2

The overhead of rank 3 and different dropping alternatives are listed in Table 3. Alt0 and Alt1 have similar total overhead but different sizes of bitmaps. For the overhead restriction, the two alternatives report different numbers of non-zero coefficients (50 and 55 for Alt0 and Alt1, respectively).

Table 3. Overhead calculation of different dropping alternatives

|  | SD over-sampling | SD basis | FD basis | Bitmap | SCI | Reference amplitude | Coefficients | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RI=3 | 4 | 11 | 8*3=24 | 32*3=96 | 3*3=9 | 4*3=12 | 7*(56-3) =371 | 527 |
| RI=4, Alt0 | 4 | 11 | 8*4=32 | 32*4=128 | 3*3=9 | 4*3=12 | 7*(50-4) =322 | 525 |
| RI=4, Alt1 | 4 | 11 | 8*2+7*2=30 | 32*2+16*2=96 | 3*3=9 | 4*3=12 | 7*(55-4) =357 | 526 |

Performance of dropping Alt0 and Alt1 for SU-MIMO is shown in Fig. 3. According to the evaluation results, Alt 1 performs better than Alt0 with around 2% gain for mean UPT.

Figure 3. Performance of different dropping alternatives.

In summary, for similar overhead, it is natural to see that the dropping alternative with most reported coefficients has the best performance, even though the coefficient selection is sub-optimal due to a smaller bitmap. Therefore, the CSI omission rule with both NZC and FD basis dropping has better performance than NZC dropping only. Another advantage of this omission rule is no additional signalling in needed and only a priority table is included in the spec.

Observation 2: CSI omission by dropping NZC only has no spec impact and can be achieved by UE implementation.

Observation 3: CSI omission by dropping both NZC and FD basis has around 2-4% performance gain over that by dropping NZC only, if assuming the same reporting overhead. A pre-defined priority table can be used to determine the payload of reporting after omission and derive omitted CSI contents by the gNB, and no further signalling is required.

Proposal 2: CSI omission rule by dropping both NZC and FD basis can be considered for Rel-16 Type II compression codebook.

UCI reporting

In Rel-15, 2-part UCI reporting was agreed for subband CSI reporting over PUCCH/PUSCH, wherein CRI/RI/CQI/LI as well as the indicator of the number of non-zero wideband amplitudes are reported in part 1 and the remaining PMI components are reported in part 2. Subband amplitude and phase of certain spatial beam/polarization would not be reported if the quantized wideband amplitude is 0 and the payload size of part 2 would be affected by the number of non-zeroes wideband amplitudes. The payload size of part 1 is fixed regardless of the value of RI/CRI to avoid blind detecting for part 1 at gNB side. Furthermore, the payload size of part 1 should be as small as possible for better encoding efficiency and the reliability of part 1 transmission. The gNB can know the payload size of part 2 after decoding part 1 and then part 2 can be relatively interpreted easily.

Restriction of all zero for a polarization

For the FD compression codebook, i=0RI-1KNZ,i≤2K0 coefficients can be reported at most, where K0=2LM*β and the minimum value of β is 1/4. Moreover, considering that CSI part 2 omission can be achieved through discarding some weak coefficients, the number of reported coefficients may further reduced. As a result, the bitmap can be sparse and contains a large number of zero elements.

For the DFT-based compressed codebook, reported coefficients of each layer depend entirely on the implementation of UE. As shown in Figure 4, it is feasible that no coefficient is reported for the weaker polarization, because K0 strongest coefficients may be concentrated in and then selected from one polarization. For another polarization, the coefficients are discarded although the magnitude may not be zero. Table 4 shows the probability that no coefficient is reported in one polarization with maximal rank=4. In the simulation, all coefficients are ranked by amplitude for each layer, and the maximum 2K0 coefficients are reported. The bandwidth is 10MHz and subband size is 4 RBs. L=2 and β=1/4 are used. The probability of selecting one out of two polarizations can be much higher for relatively small values of L, p and β. For L=2, {y0, v0}={1/4, 1/4}, β=1/4, the probability of no coefficient is reported in one polarization is up to 15.6%.

Figure 4. An example of reported coefficients being concentrated in one polarization (stronger coefficient is marked with larger index)

Table 4. The probability of no coefficient reporting in one polarization

| {y0, v0} | Probability of no coefficient is reported in one polarization |
| --- | --- |
| {1/2, 1/4} | 4.60 % |
| {1/4, 1/4} | 16.21 % |
| {1/4, 1/8} | 15.6 % |

To approximate the eigenvectors of the channel matrix by Type II codebook, if no coefficients reported in one polarization, the precoding matrix corresponding to that polarization is zero. The performance loss can be expected because one polarization is muted and the corresponding precoding matrix is far from the ideal eigenvectors. For Rel-16 DFT-based compression codebook, only the maximum number of reported coefficients is specified. UE has great flexibility in determining how many and which coefficients to be reported. As a result, different implementation can lead to huge performance differences. To prevent poor performance due to poor implementation, reporting mechanism should be developed to guarantee the minimum number of reported coefficients in each polarization. For example, at least one coefficient should be reported in each polarization. As a result, at least WB PMI with single beam could be reported for the weaker polarization.

Figure 5 shows the performance comparison of different implementations with L=2, beta=1/4. For implementation 1, coefficients for all the layers are ranked by amplitude, and the maximum 2K0 coefficients are reported. For implementation 2, at least one coefficient is reported in each polarization and each layer. Take {y0, v0}={1/4, 1/4} as an example, only 8 coefficients can be reported for all layers. For implementation 1, it has much higher probability that no coefficient reporting in one polarization. In some extreme cases, the coefficients in two polarizations may be all zero even if the actual amplitude larger than 0. Implementation 2 provides a much more reasonable solution of coefficient selection, which can take into account the performance of each polarization and each layer. It can be observed that implementation 2 achieves about 1%~2% performance gains compared with implementation 1. The gain is more obvious for small value of K0.

Figure 5. Performance comparison of different implementations

Proposal 3: For Rel-16 Type II compression codebook, at least one non-zero coefficient should be reported per polarization per layer.

Parameter design for FD basis indication for R=2

For FD basis indication for R=2, the window based design has been agreed. The length of window is N3'=αM, which is determined by a parameter α. Evaluation results show that the performance for α=1.5 has slight loss compared with that of α=2.0 and α=2.5, and the latter two perform very similar. Therefore there is no need to support more than one values for α. Since the overhead for α=2.0 is smaller than that for α=2.5, α=2.0 is preferred for window based FD basis indication for R=2. Please refer to [6] for more details.

Proposal 4: For the value of α, support single value α=2.0.

Port selection codebook design

Port selection codebook has been supported for Rel-15 Type II codebook. Compared with Type II codebook, the selected spatial beam matrix is replaced with a port selection matrix indicating the selected ports and the combination coefficients matrix remains the same for the port selection codebook.

As agreed in RAN1 #95 meeting, the same DFT-based compression scheme is extended for Type II port selection codebook. For the DFT-based type II compression codebook, the space-frequency matrix W can be represented and approximated by the following formulation

W=W1W2WfH

where W1 and Wf are composed of selected basis vectors from the spatial codebook and frequency codebook, respectively.

Therefore, for the DFT-based type II port selection codebook, the space-frequency matrix W can be represented and approximated by the following formulation

W=W1W2WfH

where W1 is composed of selected port selection vectors and Wf is composed of selected frequency domain basis vectors from the frequency codebook, respectively. The UE only needs to feedback the indices of selected port selection and frequency basis vectors, i.e. W1 and Wf, as well as the combination coefficients W2. An example of W1 with L=4 is depicted as follows:

W1=e1,e2,e3,e400e1,e2,e3,e4

where ei=0,0,…,1,…,0,0T is a port selection vector with only one element being 1 and the remaining elements being 0.

Proposal 5: Support Rel-16 port selection codebook by simply replacing W1 with a port selection matrix, similar with Rel-15 port selection codebook.

Value of N3

PMI subband size for R=2

For “PMI subband size = CQI subband size / 2”  and R=2, this may lead to ambiguity if the size of the first or last CQI subband is smaller than the configured CQI subband size, which is caused by the misalignment of BWP and subband boundaries. For the edge CQI subband whose size is smaller than the configured CQI subband, if the PMI subband size is half of the actual CQI subband band size, the actual frequency density of CSI-RS of one of the two PMI subbands may be smaller than the configured density, which is not preferred for channel estimation. A simple solution is the CQI subband is not divided into two PMI subbands if its size is smaller than the configured CQI subband size, as shown in Fig. 6.

Another benefit for this solution is that the maximum value of N3 will be 36 instead of 40. Since the maximum number of CQI subband is 19, the number of PMI subbands may be 37 or 38 if the edge CQI subband is divided into two PMI subbands, which will lead to N3=40 to be the multiple of 2, 3 and 5. If the edge CQI subbands with smaller sizes are not divided, there are at most 2*17+2=36 PMI subbands. Reducing such a large candidate N3 value may be beneficial for UE implementation.

Proposal 6: For an edge CQI subband with the size smaller than the CQI subband size configured for CSI subband reporting, the UE should report single subband PMI for that edge CQI subband if R=2.

Figure 6. Illustration of CQI subband division.

Different padding schemes

When NSB*R > 13 the following alternatives are listed, where NSB is the number of CQI subbands. Evaluation results show that there is around 1-2% difference among different padding schemes. However, N3=NSB*R without padding seems to have the better performance over all of the padding schemes. Please refer to [6] for detailed evaluation results.

Proposal 7: For the value of N3 when NSB*R > 13, N3=NSB*R without padding is preferred to avoid risks of fragmented UE implementation and performance loss.

Parameter combination reduction

The overhead of the DFT-based compression codebook is determined by three parameters L, p and β with part of or all parameters configured by gNB. However, not all the combinations are necessary because some of them have similar overhead and performance. It is necessary to remove the redundancy of parameter combinations.

For rank 3-4 cases, the value of p is determined by a pre-defined table, in which p is different for rank 1-2 as y0 and rank 3-4 as v0, respective. It has been agreed that the candidate values of (y0, v0) is (1/4, 1/8), (1/4, 1/4) and (1/2, 1/4). Further down-selection can be considered.

In our companion paper [6], the evaluation results for all supported combinations are shown with performance and overhead. According to the results, we have the following proposed parameter table, shown in Table 5.

Table 5. Proposed parameter combinations for different ports and ranks.

| Index | L | p = y0 (rank 1-2) | p = v0 (rank 3-4) | β |
| --- | --- | --- | --- | --- |
| 0 | 2 | 1/4 | 1/8 | 1/4 |
| 1 | 2 | 1/4 | 1/8 | 1/2 |
| 2 | 2 | 1/2 | 1/4 | 1/2 |
| 3 | 4 | 1/4 | 1/8 | 1/4 |
| 4 | 4 | 1/4 | 1/8 | 1/2 |
| 5 | 4 | 1/2 | 1/4 | 1/2 |
| 6 | 6 | 1/4 | - | 1/2 |
| 7 | 6 | 1/4 | - | 3/4 |

Proposal 8: Parameter combination reduction for (L, p, β) should take into account numbers of CSI-RS ports, rank 1-4, performance difference and reporting payload. Only (y0, v0) = (1/4, 1/8) and (1/2, 1/4) are supported.

UE capability

Recall the UE capability reporting for Type II codebook in Rel-15. Four-component UE feature in Rel-15 includes:

- Comp.1: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously

- Comp.2: Parameter “Lx” (number of beams) in codebook generation, where x is index of Tx ports, corresponding to 4, 8, 12, 16, 24 and 32 ports

- Comp. 3: Support amplitude scaling type

- Comp. 4: Support amplitude subset restriction level.

In the above UE feature parameters, the first component determines the implementation complexity of channel estimation at UE, the second component determines the complexity of eigendecomposition (EVD) in PMI calculation, the third component determine the complexity of LCC calculation and the last one is related to codebook subset restriction (CBSR).

DFT-based compression codebook in Rel-16 is enhanced from Type II codebook in Rel-15, therefore the above UE feature parameters in Rel-15 can be used as the start point of UE capability reporting for DFT-based compression codebook in Rel-16.

In RAN1 #96bis, the following was agreed on the candidate value of L:

Agreement Support L=6 for the following combinations of p and beta - p value equals to 1/4, beta value equals to {1/4, ½, 3/4} - p value equals to 1/2, beta value equals to 1/4 Above applies only for the case of 32 ports, rank 1 or 2, R=1 Note that the payload size for L=6 should not exceed that of Rel-15 type-2 codebook The above feature is UE optional FFS: Further specification support to relax UE processing complexity

For some UE implementation methods, the value of L, i.e., the number of SD basis, determines the dimension of effective covariance matrix and therefore determines the computational demands of EVD at UE. Hence, in the above agreement, the support of L=6 will be reported by UE as an optional feature.

Besides the value of L, the number of EVD operations at the UE highly affects the computational complexity of DFT-based compression codebook. The number of EVD operations equals the number of FD compression units. In Rel-16, the FD compression unit can be CQI subband size or half of it, i.e. PMI subband size = CQI subband size / R with R=1 or 2. For each PMI subband, a time-consuming eigendecomposition is needed to obtain the un-quantized precoding matrix. The computational complexity for PMI calculation mainly depends on the total number of EVD operations, which is roughly same as the number of FD compression units. i.e., R×NSB, where NSB denotes the total number of “1” in the higher layer parameter csi-ReportingBand. If the value of R is configured as 2, required computational complexity for PMI quantization is roughly doubled.

To ease UE implementation, CSI processing timing may be further relaxed, but it would inevitably lead to complicated and unnecessary specification changes. Instead a new UE capability of the supported number of FD compression units S can be signaled to the gNB and then the gNB will guarantee that the configured number of FD compression units N3 ≤S is satisfied. Note that we consider the signaling of the supported number of FD compression units S rather than the signaling of supporting R = 2 is reported because the former enable the flexibility in scheduling at the gNB. For example, if one UE can support 19 EVD operations in one CSI reporting, then it can only inform gNB that R=2 is not supported. In this context, gNB can only configure the PMI subband size same as the CQI subband size. It prevents the gNB from configuring UE to perform the more precise PMI estimation on some particular FD compression units with a half of CQI subband size. Actually, the number of EVD operations highly determining the PMI calculation complexity cannot really be characterized by the value of R. When the number of selected subband NSB is small, e.g., 3, or 4, the number of FD compression units and the corresponding number of EVD operations under R = 2 are 6 or 8, which is much smaller than 19 and within UE’s capability.

For value range of the supported number of FD compression units S, we propose to allow the small minimum value, e.g. 8, to avoid the significant increase of the requirement of UE computing capability due to several new features with high additional complexity such as codebook of Rank = 3/4 and L = 6 in Rel-16. Furthermore, according to section 2.2, the maximum number of PMI subbands is 36 if R=2. Therefore, the range of supported number of FD compression units for UE capability reporting can be from 8 to 36.

Proposal 9: The supported number of FD compression units is reported by UE capability of Rel-16 codebook, whereas candidate values of the number ranges from 8 to 36.

In addition, the maximum number of Tx ports in a CSI-RS resource X, the existing UE capability of L and the new UE capability of S heavily influence the implementation complexity from channel estimation and EVD operations. The above three capabilities of X, L and S may be mutually dependent. The supported maximum number of Tx ports in a CSI-RS resource X and maximum number of FD compression units S under L=4 must be different from that under L=2, except the advanced UE who can support PMI estimation with X = 32, L = 4 and S=38. Hence it is reasonable for UE to a set of combinations of X, L and S.

Proposal 10: A number of supported CB parameter combinations, i.e.{X, L, S}, are reported by UE capability of Rel-16 codebook, including the number of CSI-RS ports in a CSI-RS resource X, the number of SD beams L, and the number of FD compression units S.

Conclusions

The contribution discusses the codebook design or enhancement for Rel-16, based on which the following observations and proposals are made.

Observation 1: Restriction of WB amplitude corresponding to a spatial beam can be achieved just by restricting the amplitude of spatial-frequency coefficients corresponding to the beam. The bitmap parameter n1-n2-codebookSubsetRestriction in R15 can be reused to indicate the CBSR of compressed codebook by slightly updating the interpretation of the bit sequence of B2.

Observation 2: CSI omission by dropping NZC only has no spec impact and can be achieved by UE implementation.

Observation 3: CSI omission by dropping both NZC and FD basis has around 2-4% performance gain over that by dropping NZC only, if assuming the same reporting overhead. A pre-defined priority table can be used to determine the payload of reporting after omission and derive omitted CSI contents by the gNB, and no further signalling is required.

Proposal 1: For CBSR of FD compressed codebook, existing principle of CBSR for R15 Type II codebook should be reused so that only spatial beam vectors and their corresponding amplitude of spatial-frequency coefficients are restricted.

Proposal 2: CSI omission rule by dropping both NZC and FD basis can be considered for Rel-16 Type II compression codebook.

Proposal 3: For Rel-16 Type II compression codebook, at least one non-zero coefficient should be reported per polarization per layer.

Proposal 4: For the value of α, support single value α=2.0.

Proposal 5: Support Rel-16 port selection codebook by simply replacing W1 with a port selection matrix, similar with Rel-15 port selection codebook.

Proposal 6: For an edge CQI subband with the size smaller than the CQI subband size configured for CSI subband reporting, the UE should report single subband PMI for that edge CQI subband if R=2.

Proposal 7: For the value of N3 when NSB*R > 13, N3=NSB*R without padding is preferred to avoid risks of fragmented UE implementation and performance loss.

Proposal 8: Parameter combination reduction for (L, p, β) should take into account numbers of CSI-RS ports, rank 1-4, performance difference and reporting payload. Only (y0, v0) = (1/4, 1/8) and (1/2, 1/4) are supported.

Proposal 9: The supported number of FD compression units is reported by UE capability of Rel-16 codebook, whereas candidate values of the number ranges from 8 to 36.

Proposal 10: A number of supported CB parameter combinations, i.e.{X, L, S}, are reported by UE capability of Rel-16 codebook, including the number of CSI-RS ports in a CSI-RS resource X, the number of SD beams L, and the number of FD compression units S.

- References

RP-181453, Samsung, “WI Proposal on NR MIMO Enhancements”, 3GPP RAN#80, La Jolla, USA, 11-14 June, 2018.

“RAN1 Chairman’s Notes”, Spokane, USA, November 12-16, 2018.

“RAN1 Chairman’s Notes”, Taipei, January 21-25, 2019.

“RAN1 Chairman’s Notes”, Xi’an, China, April 8-12, 2019.

“RAN1 Chairman’s Notes”, Reno, USA, May 13-17, 2019.

R1-1909319, Huawei, HiSilicon, “Other issues on NR eMIMO in R16”, Prague, Czech Republic, August 26th – 30th, 2019.

R1-1906032, Huawei, HiSilicon, “Discussion on UE complexity of DFT-based compression codebook”, Reno, USA, May 13th-17th, 2019.

- Appendix I

| Parameters | Dense Urban (Macro layer only) |
| --- | --- |
| Duplex mode | FDD |
| Carrier frequency | 4GHz |
| Subcarrier spacing | 15kHz for 10MHz |
| System bandwidth | 10MHz (13 subbands, 4 PRBs for each subband) |
| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites |
| Channel model | SCM-3D-UMa |
| Inter-BS distance | 200m |
| Minimum distance | 35m |
| BS antenna height | 25m |
| BS Tx power | 41dBm |
| BS antenna configuration | (M, N, P, Mg, Ng, Mp, Np) = (8,8,2,1,1,2,8), (dH, dV) = (0.5, 0.8) λ |
| UE antenna configuration | (M, N, P, Mg, Ng, Mp, Np) = (1,1,2,1,1,1,1) / (1,2,2,1,1,1,2); the polarization angles are 0 and 90 |
| UE distribution | 80% indoor, 3km/h; 20% outdoor, 30km/h |
| UE receiver | MMSE-IRC |
| MIMO scheme | SU/MU-MIMO switch for rank 1-2 design; SU-MIMO for rank 3-4 design |
| Scheduler | PF |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes |
| Feedback assumption | Realistic |
| Channel estimation | Realistic |