3GPP TSG RAN WG1 Meeting #98	R1-1908108

Prague, Czech Republic, August 26-30, 2019

Agenda Item:	7.2.2.1.3

Source:	Huawei, HiSilicon

Title:	UL channels and signals in NR unlicensed band

Document for:	Discussion and Decision

Introduction

In the previous meeting [1], the following agreements have been achieved:

Support enhancement of Rel-15 PUCCH formats PF0 and PF1 as follows:

- Mapping to physical resources of one full interlace in 20 MHz.

  - FFS: Sequence type and mapping considering the following alternatives:

    - Alt-1: Repetition of the length-12 Rel-15 PF0 and PF1 sequence in each PRB of an interlace with mechanism to control PAPR/CM considering the following alternatives

      - Alt-1a: Cycling of cyclic shifts across PRBs

      - Alt-1b: Phase rotation across PRBs of an interlace where the phase rotation is can be per RE or per PRB

    - Alt-2: Mapping of different length-12 Rel-15 PF0 and PF1 sequences to the PRBs of an interlace based on different group number u (range is 0 .. 29)

    - Alt-3: Mapping of a single long sequence to the PRBs of an interlace

  - FFS: Impact due to guardbands

- Note: Decisions on the above should be based on at least performance using the agreed MCL metric and specification impact

- Note: Interlaced PF2 and 3 are not enhanced to support 1-2 bit payloads

For enhanced Rel-15 PF3 supporting interlaced mapping, do not replace DFT-s-OFDM with CP-OFDM.

In this contribution we investigate the remaining issues for UL physical channels, i.e., PUSCH, PUCCH, and SRS, including interlace design for 60 kHz SCS, resource allocation, PUCCH formats and waveform adaptation.

NR-U PUSCH Design

Interlace for 20 MHz channel bandwidth with 60 kHz SCS

For 60 kHz SCS, either M=2 or M=3 interlaces could be considered from the perspective of OCB requirement and we prefer M=3 as it provides finer granularity in frequency multiplexing. Depending on whether 24 or 25 PRBs will be available in 20 MHz [2], the number of PRBs per interlace, N, can be either 8 or 9. It should be noted that for N=8, just counting the number of PRBs results in 79.2% OCB. However, this is too crude a computation of the OCB, considering that out-of-band emissions should be taken into account. Therefore, the OCB will be met in practice for N=8.

Proposal 1: For 20 MHz channel bandwidth with 60 kHz SCS, the PRB interlaces are defined by:

- For a channel bandwidth of 24 PRBs: M=3 interlaces, each with N=8 PRBs

- For a channel bandwidth of 25 PRBs: M=3 interlaces, each with N=8 or N=9 PRBs

Interlace for 10 MHz channel bandwidth

NR supports 10 MHz channel bandwidth with 52, 24 and 11 PRBs for 15, 30 and 60 kHz SCS, respectively. In LTE eLAA, 10 MHz channel bandwidth is defined with 50 PRBs for 15 kHz SCS and with M=5 interlaces, each containing N=10 PRBs. This is not in accordance with the NR agreement to use M=10 interlaces with N=10 or 11 PRBs for 15 kHz SCS. The WID states that air-interface optimizations specific to 10 MHz should not be made. Therefore, if interlaces should be defined for 10 MHz channel bandwidth, we suggest that the number of interlaces also is M=10 for 15 kHz SCS and M=5 for 30 kHz SCS. The corresponding values of N would then be N=5 or 6 and N=4 or 5, respectively. It is noted that the use case for 10 MHz bandwidth in NR-U is limited, and the OCB requirement for 10 MHz channel bandwidth is not mandatory.

Proposal 2. For 10 MHz channel bandwidth, the PRB interlaces are defined by:

- For 15 kHz SCS: M=10 interlaces, each with N=5 or N=6 PRBs

- For 30 kHz SCS: M=5 interlaces, each with N=4 or N=5 PRBs

- For 60 kHz SCS: M=3 interlaces, each with N=3 or N=4 PRBs

UL interlace for bandwidth greater than 20MHz

The working assumption was achieved to use same spacing (M) between consecutive PRBs in an interlace for all interlaces regardless of carrier BW. For 15 kHz SCS, M = 10 and for 30 kHz SCS, M = 5 for all bandwidths. When UE is configured with wideband BWP, it is beneficial for a UE to transmit on part of BWP either due to LBT failure on some LBT bandwidth or due to temporary small uplink packets. Restricting PUCCH within LBT bandwidth is also necessary to ensure the reliability. It is efficient for gNB to schedule PUCCH and PUSCH of different UEs in the same interlace on different LBT bandwidth. Therefore, partial interlace allocation and transmission in the units of LBT bandwidth should be supported.

One concern on the working assumption is whether the partial interlace transmission can still meet the OCB requirement on the corresponding LBT bandwidth. Some companies also mentioned that the channel raster may affect the channel occupancy for partial interlace, however, as illustrated in Figure 1, we find that no matter how the channel raster is defined, the partial interlace located in one LBT bandwidth could always be a whole 20 MHz interlace (In RAN4 90bis meeting, many companies have proposed the possible RF reference frequencies for NR-U [3], e.g. 5190 MHz (ARFCN=746000), 5210.01 MHz (ARFCN=747334) etc., and same assumptions are made in the following). In Rel-15, to determine the resource grid, Point A expressed by an ARFCN is used as a common reference point, besides, common resource blocks (CRBs) are also defined to determine the PRB positions for a carrier. Furthermore, the mapping between channel raster and resource element has also been defined in TS 38.104 [4] as the following:

Table 5.4.2.2-1: Channel Raster to Resource Element Mapping

| Resource element index | 0 | 6 |
| --- | --- | --- |
| Physical resource block number |  |  |

Therefore, using the number of PRBs, NRB, for the carrier, we can find the RE index k located on the channel raster and determine the point A which aligns the CRBs to PRB nPRB of RE k. In Figure 1(a), a 40 MHz CC/BWP (5170 MHz~5210 MHz) with 15 kHz SCS and NRB=216 is considered. It can be found that the nearest point A should be ARFCN=744668. Under this assumption, we could obtain the location for CRB#0. Assume that the parameter offsetToPointA = 3 PRBs (Note that it should be in units of resource block assuming 15 kHz SCS for FR1), then starting point of the transmission bandwidth (PRB#0) could also be determined. The LBT bandwidth is assumed to be 20 MHz, e.g. LBT bandwidth #1: 5170 MHz~5190 MHz, LBT bandwidth #2: 5190 MHz~5210 MHz. Correspondingly, the location for PRBs in each LBT bandwidth can be obtained. It can be found that for each LBT bandwidth, at most 107 PRBs with 15 kHz SCS are available. If guardbands were configured as defined in 38.104 [4], 0.4525 MHz for 20 MHz carrier, at least 104 available PRBs for each LBT bandwidth are available and there are still 10 or 11 PRBs for each partial interlace, which could obviously meet the OCB requirement.

Similar observations for carrier bandwidth greater than 20 MHz with 30 kHz SCS could be found in Figure 1(b). As defined in 38.104 [4], the guardband is 0.805 MHz for 20 MHz carrier with 30 kHz SCS. There are still at least 50 available PRBs for each LBT bandwidth (10 PRBs occupying 16.2 MHz for each partial interlace within LBT bandwidth) and could still meet the OCB requirement.

Note that the spectrum utilization for 20 MHz with 60 kHz SCS is still under discussion in RAN4, one potential option is that NRB=25 and the guradband is 1 MHz. Based on this assumption we can obtain the wideband interlace design for 60 kHz SCS, as shown in Figure 1(c). It can be found that if guardbands were configured, at least 24 PRBs for each LBT bandwidth are available.

(a) 15 kHz SCS

(b) 30 kHz SCS

(c) 60 kHz SCS

Figure 1. Examples of partial interlaces for bandwidth greater than 20 MHz.

Proposal 3: Partial interlace allocation in the units of LBT bandwidth should be supported.

Proposal 4: The working assumption on UL interlace design should be confirmed.

Based on the discussion above, the following PRB-based interlace structure in Table 1 should be supported.

Table 1: PRB-based interlace structure for bandwidth larger than 20 MHz.

| SCS [kHz] | 40 MHz |  |  | 60 MHz |  |  | 80 MHz |  |  | 100MHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | M | N | NRB | M | N | NRB | M | N | NRB | M | N |
| 15 | 216 | 10 | {21,22} | N.A |  |  | N.A |  |  | N.A |  |  |
| 30 | 106 | 5 | {21,22} | 162 | 5 | {32,33} | 217 | 5 | {43,44} | 273 | 5 | {54,55} |
| 60 | 51 | 3 | 17 | 79 | 3 | {26,27} | 107 | 3 | {35,36} | 135 | 3 | 45 |

Proposal 5: UL interlace design in Table 1 should be supported.

Requirement on DFT-S-OFDM

NR Rel-15 supports CP-OFDM and DFT-S-OFDM in UL.   In order to achieve low complexity DFT process, the number of allocated PRBs should be a product of the integers 2^n1, 3^n2, and 5^n3. Considering the agreed number of PRBs per interlace in non-uniform interlace design, additional restriction on the resource allocation is required to apply DFT-S-OFDM. For example, UE should not be scheduled on the interlace with 11 PRB or one PRB in the 11-PRB interlace should not be used.

DFT-S-OFDM is used in the coverage limited scenario because it has relatively lower PAPR and thus requires less power backoff when localized PRB allocation is adopted. However, considering the non-contiguous PRB allocation per interlace, reduction of the PAPR is limited. Considering the transmit power limitation per device according to the regulation, it is not necessary for UE to use DFT-S-OFDM over bandwidth larger than 20MHz.

Observation 1: The usage of DFT-S-OFDM is restricted in NR-U. No additional work more than NR Rel-15 is required in NR-U WI.

Different numerologies between UL and DL

It is identified beneficial to configure same numerology for all DL signals/channels or UL signals/channel in some scenarios from the perspectives of UE implementation complexity, specification impact and measurement gap. Whether configuring DL and UL signal/channels with the same numerology or not is FFS. Considering different hardware limitation and channel access mechanism in DL and UL, we believe it is still beneficial to have different numerology configuration for DL and UL transmission. One use case is to configure smaller subcarrier spacing in UL than DL. The larger power boost from interlace with smaller SCS could provide UE additional coverage. The larger SCS in DL would also reduce the FFT calculation at UE side to decode DL channel. On the contrary, considering limited number of start points in PUSCH compared with PDSCH, configuring larger SCS for UL than DL could alleviate the performance loss in UL when coexisting with WiFi systems, as observed in Figure 2. Configuring PUSCH with 60 kHz SCS can also help PDSCH with 30 kHz SCS, because PUSCH occupies less time resource to finish same amount of traffic. NR Rel-15 already provided flexibility to configure different numerology for different scenarios. NR-U could adopt most of design without additional standard and implementation effort.

Figure 2. NR-U DL and UL performance with different SCS when coexisting with WiFi

Proposal 6: Different UL and DL numerologies should be supported in NR-U to cater for various hardware limitation and channel access mechanism between uplink and downlink in NR-U.

Multiple start points of PUSCH

Similar to FeLAA, it has been identified to be beneficial for the NR-U design to not require the UE to change a granted TBS for a PUSCH transmission depending on the LBT outcome. Two options were identified

-	Option 1: PUSCH(s) as in Rel-15 NR

-	Option 2: Multiple starting positions in one or multiple slot(s) are allowed for PUSCH(s) scheduled by a single UL grant (i.e., not a configured grant) and one of the multiple PUSCH starting positions can be decided depending on LBT outcome.

The difference between them is that for Option 1 the partial PUSCH (including the starting point and ending point) is indicated by gNB (similar to PUSCH mode 2 in FeLAA), while for Option 2 the partial PUSCH depends on the on the LBT outcome from UE (similar to PUSCH mode 1 in FeLAA).

Note that, PUSCH mode 3 in FeLAA is used for ending partial PUSCH which has already been supported in NR Rel-15, and there is no need to make further enhancement.

In NR-U, the SCS of 15 kHz, 30 kHz and 60 kHz are supported for PUSCH and PUCCH. Assuming one starting point per slot, 30 kHz and 60 kHz SCS equivalently provide 2 times and 4 times access opportunities compared with 15 kHz SCS. Option 1 should be enough for PUSCH using large SCS, e.g. 60 kHz. Considering the obvious performance gap between 60 kHz and 15/30 kHz SCS as shown in Figure 2, increasing number of starting point for 15 kHz and 30 kHz SCS is still beneficial. Considering the front loaded DMRS in PUSCH, it can be used by gNB to identify the start point of PUSCH. In order to avoid additional latency to regenerate PUSCH, the scrambling sequence of DMRS should be independent of the symbol index, and several OFDM symbols at the end of PUSCH should be punctured as plotted in Figure 3.

Proposal 7: For PUSCH starting positions in NR-U:

- For 60 kHz SCS, PUSCHs as in Rel-15 is enough (option 1).

  - No additional mechanism is specified to determine the starting position.

- For 15 and 30 kHz SCS, more than one starting positions in a slot is necessary (option 2).

  - PUSCH DMRS can be used by gNB to detect the starting point.

  - OFDM symbols are punctured from the end to fit the PUSCH into a slot.

  - FFS: number of starting positions in a slot.

Figure 3. Example of multiple starting points for PUSCH.

PUSCH frequency resource allocation

The frequency Resource Allocation (RA) can be considered to consist of allocation of PRBs within the allocated sub-bands and allocation of the sub-bands. A sub-band would typically be of 20 MHz bandwidth on which the LBT procedure is performed (i.e., LBT bandwidth). Figure 4 shows an example of a carrier consisting of 2 sub-bands of which only 1 is used for transmission. We refer to this as a partial interlace allocation, i.e., within a sub-band a UE uses all the PRBs of the interlace while certain sub-bands are not allocated.

Figure 4. Example of partial interlace allocation for one UE where only the PRBs in the first LBT sub-band are allocated from the interlace.

Allocation of resources within the allocated sub-bands

According to TR38.889, a waveform contiguous in frequency may be adequate in some scenarios. This would especially be applicable when the 2 MHz OCB exception rule can be exercised or in regions where there are no OCB requirements. The NR Rel-15 RA schemes are useful in order to provide finer resource granularity than interlaced based allocations and  there appears to be no technical arguments for excluding Type-0 and Type-1 in NR-U, since the UE will support them. Notably, whether to use the legacy PUSCH RA schemes was discussed already for LTE eLAA and the decision not allow them in unlicensed spectrum, was merely to follow the wording in the WI description to have a single solution. It is noted that a dynamic switch (by the DCI) between Type-0 and Type-1 PUSCH RA can be configured in NR Rel-15, which could be supported for NR-U, as well as a dynamic switch between any of Type-0 and Type-1 and the new interlaced based RA scheme.

Proposal 8. Either of the NR Rel-15 Type-0, the NR Rel-15 Type-1 and the NR-U interlaced PUSCH RA schemes can be semi-statically configured. Dynamic switching can be configured between either of the RA schemes:

- NR Rel-15 Type-0 and NR Rel-15 Type-1

- NR Rel-15 Type-0 and the NR-U interlaced resource allocation

- NR Rel-15 Type-1 and the NR-U interlaced resource allocation

The frequency domain RA field size would thus depend on the configuration, which is already the case for DCI format 0_1. If dynamic switching is configured, the frequency domain RA field size corresponds to the largest one of the configured RA schemes plus one bit. Otherwise, it corresponds to that of the semi-statically configured RA scheme.

Considering using a Resource Indication Value (RIV), for 15 kHz SCS there are M=10 interlaces and log2(MM+12)=6 bits in the DCI would suffice and also provide 9 spare RIVs which could be used for non-contiguous interlace allocations. A bitmap would in this case require 10 bits (i.e., 67% more RA overhead). For 30 kHz SCS there are M=5 interlaces and log2(MM+12)=4 bits in the DCI would suffice and also provide 1 spare state. A bitmap would in this case require 5 bits (i.e., 25% more RA overhead). Interlacing results in wideband transmission with similar performance for all interlaces, thus the excessive RA overhead from a bitmap is not motivated.

Proposal 9. The resource allocation scheme for interlaced PUSCH is defined by:

- An RIV denoting the starting interlace index and the number of contiguous interlaces

- Spare RIVs defining non-contiguous interlace allocations

Notably, there is no need to change the NR Rel-15 VRB-to-PRB mapping for this resource allocation scheme. The handling of non-uniform interlaces (e.g., N=10 or N=11 PRBs) could be straightforwardly defined in the specification and does not need to involve the resource allocation signaling, e.g., for 15 kHz SCS in a 20 MHz carrier (or BWP) the first 6 interlaces are defined to contain 11 PRBs etc.

It can be found that there is only a small number of non-contiguous interlace allocations which offer larger transmit power than contiguous interlaces, e.g., for 30 kHz SCS power gains only exist for 20 and 30 allocated PRBs. Table 2 contains the power gain for non-contiguous interlace allocations which result in a larger transmit power than what could be obtained by allocation of contiguous PRB interlaces. Thus, spare RIVs could be allocated to these. Here, an allocation is denoted by PRB indices i over  a set of M PRBs, where i=0,1,…,9 for 15 kHz and i=0,1,…,4 for 30 kHz. Each allocation could further be cyclically shifted while obtaining the same power gain, e.g., {0,5}, {1,6}, {2,7}, {3,8} and {4,9} offer 1.2 dB gain. Note that power gains are primarily present for 15 kHz SCS and if an interlace with N=11 PRBs (for 15 kHz) or N=6 PRBs (for 30 kHz) is used, the gain from a non-contiguous allocation decreases. Moreover, power backoff has not been considered, which would decrease the gain even further.

Table 2. Maximum power gain of non-contiguous interlace allocations which provide larger transmit power than that of contiguous interlace allocations. The number of PRBs is N=10 and N=5, for 15 kHz and 30 kHz SCS, respectively, no power backoff is used and 10 dBm/1 MHz constraint is assumed.

| Number of PRBs | 15 kHz SCS |  | 30 kHz SCS |  |
| --- | --- | --- | --- | --- |
|  | PRB allocation | Power gain [dB] | PRB allocation | Power gain [dB] |
| 20 | {0,5} | 1.2 | {0,2} | 0.6 |
| 30 | {0,4,8},{0,4,7} | 1.8 | {0,2,4} | 1.4 |
| 40 | {0,1,5,6} | 2.0 | - | - |
| 50 | {0,2,4,6,8} | 2.2 | - | - |
| 60 | {0,1,2,5,6,7} | 2.0 | - | - |
| 70 | {0,3,4,5,7,8,9}, {0,3,4,5,6,8,9}, {0,2,4,5,7,8,9}, {0,2,4,5,6,8,9}, {0,2,4,5,6,7,9}, {0,2,3,5,6,8,9} | 0.9 | - | - |
| 80 | {0,1,2,3,5,6,7,8} | 0.9 | - | - |

It has been proposed to support adding/removing some PRBs to/from interlaces with the argument to introduce finer resource allocation granularity. However, by borrowing PRBs from another interlace, the resource allocation granularity may decrease for a UE which is allocated that other interlace. Hence, overall it is not clear what the benefit is and how the scheduler could use this. Moreover, it also has to be assured that the interlace still fulfills the OCB requirements, even if some of its PRBs has been removed. Essentially, adding/removing PRBs to/from an interlace is equivalent to a new interlace structure. Hence, the RIV-based method is sufficient without modifications.

Allocation of the sub-bands

For the wideband operation, as discussed in Section 2.2, the partial interlace allocation in the units of LBT sub-band should be supported. For the resource allocation, besides the RIV indication described above, the information for scheduled sub-band(s) should also be indicated. According to the RIV indication, UE could determine the interlace allocations, and with the help of the scheduled sub-band information, UE could determine the partial interlace allocations, i.e., which PRB(s) in each allocated interlace is(are) scheduled.

Proposal 10. For resource allocation in wideband operation, indication of the scheduled sub-band(s) should be provided.

NR-U PUCCH Design

Considerations on the reuse of Rel-15 NR PUCCH Formats 2 and 3

For both NR PUCCH Formats 2 and 3, if the payload size is large, CSI bits are dropped to preserve the HARQ-ACK bits which are more important. The number of PRBs used is determined by the payload size and the effective coding rate. As such, the smaller the payload size is the fewer the number of PRBs occupied. Given that transmissions occur on the unlicensed channel in an opportunistic manner based on the LBT results, the NR-U gNB would often multiplex multiple PDSCHs into the DL burst. Also, the UE would combine delayed HARQ-ACK bits with current ones. As such, NR-U UEs would be often reporting HARQ-ACK using large codebooks as shown in Figure 5. The codebook size is even larger if CBG-based and/or cross-carrier HARQ feedback is used. Note that CRC bits are attached only to large payloads (> 11 bits) for which Polar codes are used, otherwise Reed-Muller codes are used with small payloads without CRC attachments.

Unlike other NR PUCCH formats, Format 2 only uses OFDM without DFT precoding at the cost of increased cubic metric given its use as is without an interlaced waveform. However, the transmission spans only 1-2 symbols and typically occurs at the end of the transmission slot unless configured otherwise. Such a short PUCCH duration enables NR-U to exploit NR’s bi-directional slot formats or the agreed bi-directional MCOTs with single or multiple switching points for timely feedback with increased channel access opportunities. As concluded in the SI phase [5], transmission of HARQ A/N for the corresponding data in the same shared COT is identified as beneficial.

The long PUCCH duration of PUCCH Format 3, on the other hand, has the advantage of better coverage. In addition, DFT precoding is applied post modulation in PUCCH Format 3 to reduce the cubic metric and thus reduce the power backoff required for the PA.

Nevertheless, NR-U PUCCH formats should be able to fulfil the minimum OCB requirements when transmitted independently on the unlicensed channel without frequency multiplexing with PUSCH. Although up to 16 PRBs can be configured, a contiguous bandwidth of such PRBs is assumed with NR PUCCH Formats 2 and non-hopping Format 3 which will not satisfy the regular OCB requirement using a SCS less than 120 kHz (assuming a 20 MHz channel and all 16 PRBs are used). Alternatively, the UE can be allowed to occasionally violate the regular OCB requirements using a BW of at least 2 MHz within COT. In such case, at least contiguous 12, 6 and 3, PRBs would be used with the SCS of 15 kHz, 30 kHz and 60 kHz, respectively. If repetition/padding of UCI bits is to be avoided, this in fact imposes more constraints on the minimum number of PRBs to be allocated to Format 2 and non-hopping Format 3, and thus, on the minimum payload each can handle. Such constraints however are further relaxed as the SCS   is increased.

As per the NR specifications in TS 38.213, if a UE transmits a PUCCH with HARQ-ACK information bits and  bits using PUCCH format 2 or PUCCH format 3 in a PUCCH resource that includes  PRBs, the UE determines a number of PRBs  for the PUCCH transmission to be the minimum number of PRBs, that is smaller than or equal to a number of PRBs  provided respectively by higher layer parameter nrofPRBs of PUCCH-format2 or nrofPRBs of PUCCH-format3 and starts from the first PRB from the number of PRBs, that results to  and if , the UE transmits the PUCCH over  PRBs. Therefore, by setting  to the minimum numbers of 12, 6 and 3 PRBs, the minimum UCI payload sizes can be estimated as follows corresponding to the SCS of 15 KHz, 30 KHz and 60 KHz, respectively,

Table 3. Minimum UCI payload to satisfy the 2 MHz OCB using PF2 and PF3

|  | SCS | Min PRBs | SCs/PRB | UCI Symbols | Qm | r | Min UCI Payload | Modulation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PF2 | 15KHz | 12 | 8 | 1 | 2 | 0.08 | 15 | QPSK |
|  | 30KHx | 6 | 8 | 1 | 2 | 0.08 | 7 |  |
|  | 60KHz | 3 | 8 | 1 | 2 | 0.08 | 3 |  |
| PF3 | 15KHz | 12 | 12 | 3 | 1 | 0.08 | 21 | BPSK |
|  | 30KHz | 6 | 12 | 3 | 1 | 0.08 | 15 |  |
|  | 60KHz | 3 | 12 | 3 | 1 | 0.08 | 6 |  |
|  | 15KHz | 12 | 12 | 3 | 0.5 | 0.08 | 16 | pi/2-BPSK |
|  | 30KHz | 6 | 12 | 3 | 0.5 | 0.08 | 8 |  |
|  | 60KHz | 3 | 12 | 3 | 0.5 | 0.08 | 3 |  |

Where in the analysis of Table 2,, , , and  are set to (8, 1, 2, 0.08) for PUCCH format 2 and (12, 3, 1(0.5), 0.08) PUCCH format 3 accounting for DMRS locations, the least number of symbols carrying UCI,  available modulation, and the minimum supported code rate.

Although the option of frequency hopping in PUCCH Format 3 was intended for exploiting the frequency diversity, it can be used in NR-U operations to relax the requirements to fulfil the 2 MHz OCB over the two hops of the transmission slot. Therefore, the minimum number of PRBs can be reduced to the half for Format 3 if the frequency hopping option is configured. In such case, at least one symbol per hop will be dedicated to carry the DM-RS whereas critical UCI bits such as HARQ-ACK, SR, and CSI part1, are jointly encoded and mapped to the symbols around it.

Figure 5. NR PUCCH Formats 2 and 3 employed for NR operations in the unlicensed spectrum.

From the above discussion, we observe that NR-U can support the reuse of NR Rel-15 PUCCH Formats 2 and 3 without enhancements with considerations on the minimum UCI payload required to fulfil the 2 MHz OCB requirement at the given SCS. It can be concluded from Table 2 as well that such a reuse does not directly support the transmission of small UCI payloads (1 or 2 bits) without enhancement.

Proposal 11: NR Rel-15 PUCCH formats 2 and 3 can be reused in NR-U under the 2 MHz OCB requirement with the following considerations:

- A minimum of 12, 6, and 3 PRBs should be configured with the SCS of 15 KHz, 30 KHz and 60 KHz, respectively.

- To ensure that the UE occupies the configured minimum number of PRBs, the corresponding minimum UCI payload needs to be specified. Configure PUCCH Format 3 with the frequency hopping option to satisfy the 2 MHz OCB requirement over the two hops of the transmission slot.

Extensions/Enhancements of PUCCH Design for NR-Unlicensed

As discussed in Section 2, for the most part, PUCCH is one UL PHY channel to follow an interlace-based design approach. This also allows for simple frequency multiplexing of concurrent PUCCH transmissions from multiple UEs. Nevertheless, further enhancements can be applied to a basic interlace design such as employing FH to meet OCB requirement per slot/subframe, or PRB-specific processing to reduce the resultant PAPR/CM as discussed in the Subsection 3.2.1.

As per earlier agreements [5], the NR-U operating bandwidth can be an integer multiple of 20 MHz unit channels on which LBT can be performed individually. As such, the actual available bandwidth for UE could be different from the scheduled bandwidth by gNB due the uncertainty of LBT. Therefore, NR-U PUCCH should be confined within the minimum nominal channel bandwidth, e.g. 20 MHz in 5 GHz band.

Proposal 12: NR-U PUCCH should be confined within the minimum nominal channel bandwidth, i.e. 20 MHz.

Proposal 13: When the UE is configured to use a wideband PRB-based interlace in a wideband BWP, the PUCCH can be confined within the bandwidth of LBT bandwidth by exploiting the partial interlace.

As agreed in previous RAN1 meetings, flexible number of OFDM symbols, flexible payload size, user multiplexing, number of formats are aspects that can be considered for interlace waveform based PUCCH design. In addition, a prudent choice of an interlace-based design for PUCCH needs to consider other factors such as the scalability over various transmission bandwidths, and the potential increase in PAPR/CM, especially when repetition of a small sized UCI is needed.

A PSD constraint will limit coverage of DL signals/channels with smaller bandwidths, hence, UL coverage of an interlaced PUCCH may not always be the bottleneck and it would then not be needed to configure a 14 OFDM symbol duration. However, a new PUCCH format should be able to offer robustness against channel delay spread and UE velocity. For this, a UE multiplexing order of 1 should be supported (i.e., no CDM on a PRB-interlace), which is also favorable for accommodating the larger anticipated UCI payloads in NR-U due to the LBT failures. Notably, NR PUCCH format 3 and 4 offer UE multiplexing order of 1, 2 and 4 for a duration of 4-14 OFDM symbols on up to 16 PRBs. Thus even if a new NR-U PUCCH format uses the 10/11 PRBs of an interlace, when it is configured with a duration closer to 4 than 14 OFDM symbols, the PUCCH overhead can be comparable to that of a long PUCCH format 3. Hence, we see that a UE multiplexing factor of 1 will still be useful for NR-U PUCCH, while it may not preclude also supporting larger multiplexing factors.

Proposal 14: A new PRB-interlaced PUCCH format should at least support a UE multiplexing order of 1 on a PRB-interlace (i.e., no CDM).

Usage for NR-U PUCCH formats

Similarly with licensed band, for NR-U operation, the PUCCH formats should be capable to carry all the UCI combinations and an agreement is reached in last meeting [1], which introduced following cases to be considered for the PUCCH format design:

- HARQ-ACK prior to dedicated PUCCH resource configuration

- HARQ-ACK, SR, CSI and combinations thereof after dedicated PUCCH resource configuration

When a UE has not been provided dedicated PUCCH resource configuration, the UE would use common PUCCH resources configured by higher layer signaling to report at most 1 bit ACK/NACK for Msg.4. If the SR is transmitted alone and then 1 bit is enough to represent SR information status as per the current Scheduling Request on-off mechanism. For SPS PDSCH without a dynamic PDCCH indication, 1 or 2 HARQ-ACK information bits are commonly used because of the periodical feedback. One the other hand, if taking CSI reports into account, the total payload size would increase much more than 2 bits especially for the subband CSI wherein the two CSI parts are both reported.

Therefore, NR-U PUCCH formats should be capable to convey these various payload sizes. However, for NR-U operation, the SI phase concluded in [5] that only NR Rel-15 PUCCH Formats 2 and 3 can be supported (reused) without a PRB-based interlaced transmission, because several contiguous PRBs can be allocated for one PUCCH resource to meet the OCB requirements. While reusing PF 2/3 cannot be suitable for the small payload cases as discussed in Section 3.1, other enhanced formats supporting PRB-based interlaced transmission should be further considered, for example, enhanced PF 0/1 and/or enhanced PF 2/3.

Enhanced PUCCH format 0/1

We evaluate the following enhancements for PUCCH format 0 according to the simulation assumptions in Table 6 in Appendix, which also contains Figure 6~Figure 13  with the detailed simulation results.

- Alt. 1a: The initial cyclic shift for PRB k is m0k=k, k=0,1,…,10.

- Alt. 1b: The sequence in PRB k is multiplied with the value A(k) where:

  - A=[1 1 1 i 1 -1 -i i i -i] for an interlace of 10 PRBs

  - A=[ 1 -1 1 1 -1 1 1 1 -1 -1 -1]  for an interlace of 11 PRBs

- Alt. 2: The allocation of sequence group u∈0,1,…,29 to PRB k is given by the 30x11 matrix B in Appendix.

- Alt. 3: A ZC sequence of length NZC=113 is extended to length-120 for an interlace of 10 PRBs and a ZC sequence of length NZC=131 is extended to length-132 for an interlace of 11 PRBs.

Table 4 shows that the 95-percentile CM is higher for Alt. 3 while being similar for Alt. 1a, 1b and 2, albeit with a noticeable gain for 11 PRBs and 15 kHz SCS for Alt. 1b.

Table 4. The 95-percentile Cubic Metric for interlaces with different number of PRBs.

| Alternative | Cubic Metric [dB] |  |
| --- | --- | --- |
|  | 10 PRB interlace | 11 PRB interlace |
| Alt. 1a | 1.83 | 1.64 |
| Alt. 1b | 1.95 | 1.27 |
| Alt. 2 | 1.88 | 1.93 |
| Alt. 3 | 2.95 | 2.77 |

The results are summarized in Table 5, which shows that, with the exception of 10 ns delay spread, Alt. 3 is significantly worse than the other alternatives. Alt. 1b and Alt. 2 have comparable MCL and are slightly better than Alt. 1a. While Alt. 1b might be acceptable, our preference is Alt. 2 since it offers better performance and utilizes only existing NR Rel-15 signals.

Proposal 15: For enhanced PUCCH format 0 and 1, Alt. 2 is adopted, i.e., different length-12 Rel-15 PF0 and PF1 sequences are mapped to the PRBs of an interlace.

Table 5. Required SNR to reach 1% probability of missed ACK and the corresponding Maximum Coupling Loss, for the different enhanced PUCCH format 0/1 alternatives.

| Alternative | SCS [kHz] | Delay spread [ns] | 10 PRBs |  | 11 PRBs |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SNR [dB] | MCL [dB] | SNR [dB] | MCL [dB] |
| Alt. 1a | 15 | 10 | -4 | 130.45 | -4.45 | 131.31 |
| Alt. 1a | 15 | 100 | -7.9 | 134.35 | -8.2 | 135.06 |
| Alt. 1a | 30 | 10 | -3.7 | 130.15 | -4.05 | 130.91 |
| Alt. 1a | 30 | 100 | -7.6 | 134.05 | -7.95 | 134.81 |
| Alt. 1b | 15 | 10 | -4.1 | 130.55 | -4.4 | 131.26 |
| Alt. 1b | 15 | 100 | -8 | 134.45 | -8.15 | 135.01 |
| Alt. 1b | 30 | 10 | -3.6 | 130.05 | -4.1 | 130.96 |
| Alt. 1b | 30 | 100 | -7.5 | 133.95 | -7.8 | 134.66 |
| Alt. 2 | 15 | 10 | -4.2 | 130.65 | -4.6 | 131.46 |
| Alt. 2 | 15 | 100 | -8 | 134.45 | -8.25 | 135.11 |
| Alt. 2 | 30 | 10 | -3.8 | 130.25 | -4.05 | 130.91 |
| Alt. 2 | 30 | 100 | -7.5 | 133.95 | -7.9 | 134.76 |
| Alt. 3 | 15 | 10 | -5.9 | 132.35 | -6.25 | 133.11 |
| Alt. 3 | 15 | 100 | 1 | 125.45 | 1.1 | 125.76 |
| Alt. 3 | 30 | 10 | -4.8 | 131.25 | -5.6 | 132.46 |
| Alt. 3 | 30 | 100 | 1.5 | 124.95 | 1.6 | 125.26 |

To increase enhanced PF0 UE multiplexing capacity, CDM including frequency domain OCC and CS shift could be considered. For the frequency domain OCC, one solution is to apply RE-level multiplexing between different UEs, However, the length of the base sequence will be shortened along with the augment of UE multiplexing capacity. Once the sequence becomes shorter, the performance of cross-correlation between sequences degrades as well. Another option to increase the number of UEs on one PUCCH resource is to allocate different CSs to different UEs. While it should be known that in Rel-15, to represent 2 bits HARQ-ACK information, 4 CS values (0, 3, 6, 9) have been used. If considering SR and HARQ-ACK multiplexing scenario, additional 4 CS values (1, 4, 7, 10) are also occupied. Therefore, it is difficult to use different CS values to represent different UEs because of limited remaining CS values currently.

Observation 2: Further study is needed whether the UE multiplexing capacity for enhanced PF0 should be increased, e.g., by frequency domain CDM or mechanisms using different CS values.

Enhanced PUCCH format 2/3

Since the baseline PF 2/3 are multi-PRB formats, a mapping rule to the physical PRBs of one or more interlaces can be introduced. As agreed in the SI phase [5], the nominal number of PRBs per interlace (N) can be similar for each SCS (in a given bandwidth) at least for 15 and 30 kHz SCS. Given that the PUCCH transmission using ePF2/3 is expected to fully occupy the PRBs of at least one interlace, the higher layer parameter nrofPRBs in case of ePF2/3 should be thus configured to be at least N PRBs. Whereas, instead of indicating the index PUCCH-starting-PRB, the gNB indicates the index of the first allocated interlace i.

Following the same UE procedure discussed in Section 3.1, the minimum number of PRBs determined by the UE based on the UCI payload, , could be however larger or smaller than N. Therefore, when the UCI payload is large enough such that   > N, then ( – N) PRBs can be multiplexed with the N PRBs of the indicated interlace i, e.g., by partially or fully occupying the PRBs of the subsequent interlace(s) starting from i+1.

However, for smaller UCI payloads that are insufficient for the PUCCH transmission to occupy the PRBs of a full interlace, repetition of PRBs from the determined   PRBs should be applied to occupy the remaining PRBs of interlace i.

Proposal 16: For enhanced PUCCH formats 2/3, introduce a physical mapping rule to the physical PRBs of one or more interlaces.

Proposal 17: For transmission of UCI payloads that are insufficient for the enhanced PUCCH format 2/3 to occupy the PRBs of a full interlace, repetition of PRBs from the minimum number of PRBs determined by the UE should be applied.

Enhanced SR mechanism

As the analysis of Section 3.2.1 showed, 1 bit SR can be carried on at least a full interlace for 20 MHz subband, that means 10/11 PRBs are configured to transmit just this 1 bit information. If taking the UE multiplexing into account, the number of PRBs is still redundant and resources are wasted. So the resources may provide a chance to convey more information related to UL resource request. Generally, once a gNB receives the SR from a UE, the gNB will allocate a resource for the UE so that the UE transmits its corresponding buffer status report back to the gNB. After that, when the gNB has a good understanding of how much data the UE needs to transmit. An appropriate UL resource and reasonable MCS are indicated to the UE. On unlicensed band, the procedure might take a quite long time and be at a high risk to lose the channel because of frequent DL-UL switching. One possible improvement for this resource request mechanism on unlicensed band is to merge these two steps into one; for example, using SR resource to transmit BSR. When gNB receives the SR, BSR is also acknowledged in the meantime. In this way, SR resource is utilized efficiently and reduce the risk for losing channels.

Another issue for PUCCH transmission on unlicensed band is the channel occupancy. Similar to HARQ-ACK information, SR might not be transmitted either because of LBT failure. Following the current restriction for SR, at most one PUCCH resource for SR is configured per BWP for a logical channel. When a SR cannot be transmitted on time, one possible solution is delaying the SR transmission to the future occasions, as the occasions are configured periodically. While that will introduce a high latency for UL resource request, especially that the periodicity is configured very long, for example tens of slots. On the contrary, if the periodicity is too short, PUCCH resources may be wasted because once the resources are scheduled, they cannot be used by other UEs arbitrarily. If one SR configuration is not required to correspond to only one PUCCH resources, which means different PUCCH resources can configured for a single SR transmission, more opportunities are provided and the transmission latency might be decreased.

Therefore, the current UL scheduling request mechanism is not suitable on unlicensed band and relevant improvement should be considered to provide more SR transmission opportunities and reduce the transmission latency.

Proposal 18: The mechanism for Scheduling Request in Rel-15 should be enhanced when Scheduling Request is transmitted on unlicensed band and following aspects can be considered for improvement:

- SR resource efficiency

- SR transmission opportunities

SRS design

The Rel-15 SRS is by default supported for NR-Unlicensed and would be functional, e.g., a wideband SRS would fulfil OCB requirements and obtain high TX power under a PSD constraint. Alternatively, frequency hopping may fulfil OCB requirements. The SRSs could be multiplexed by FDM through frequency hopping and/or different frequency combs or by CDM through different cyclic shifts of the ZC sequence. It has also been agreed that any UL symbol could be configured to carry SRS for NR-Unlicensed. This is motivated in order to fill the DL/UL switching gap or to mitigate LBT failure by having more SRS resources. Thus the following WI objective is fulfilled:

SRS including the introduction of additional flexibility in configuring/triggering SRS in line with agreements during the study phase.

The motive for specifying an additional SRS waveform, i.e., following a PRB based interlace, is primarily to ease multiplexing with PRB-based interlaced PUCCH and PUSCH. However, given that the SRS only occupies 1, 2 or 4 OFDM symbols and that it could be aperiodically triggered, the gNB has means for handling multiplexing. A PRB-based interlaced SRS would not have any substantial benefit in terms of fulfilling OCB requirements or PSD constraints compared to a wideband comb-2 or comb-4 SRS. With similar sequence length and using orthogonal SRS multiplexing, the SRS capacity is also not expected to be fundamentally different. It was agreed that the PRACH, which is a more critical signal and of longer duration, will not use a PRB-based interlaced waveform. It should be noted that this agreement came after almost a year of discussions and with a substantial evaluation campaign. A PRB based interlaced SRS would be a new waveform requiring the same effort. Therefore, within the scope of NR-Unlicensed, a PRB based interlaced SRS could be down-prioritized. Nevertheless, it should be acknowledged that SRS enhancements may be needed for other reasons in the future not being limited to unlicensed operation, e.g., it is known that massive MIMO is sensitive to outdated channel estimates. This requires frequent SRS transmissions and thus SRS capacity may become an issue, especially for DL heavy UL/DL configurations.

Conclusions

In this contribution, we discussed the design considerations of the UL physical channels, i.e. PUSCH, PUCCH, and SRS, including interlace-based design, PUCCH formats and waveform adaptation. The following observations and proposals were made:

For the PUSCH design:

Observation 1: The usage of DFT-S-OFDM is restricted in NR-U. No additional work more than NR Rel-15 is required in NR-U WI.

Proposal 1: For 20 MHz channel bandwidth with 60 kHz SCS, the PRB interlaces are defined by:

- For a channel bandwidth of 24 PRBs: M=3 interlaces, each with N=8 PRBs

- For a channel bandwidth of 25 PRBs: M=3 interlaces, each with N=8 or N=9 PRBs

Proposal 2. For 10 MHz channel bandwidth, the PRB interlaces are defined by:

- For 15 kHz SCS: M=10 interlaces, each with N=5 or N=6 PRBs

- For 30 kHz SCS: M=5 interlaces, each with N=4 or N=5 PRBs

- For 60 kHz SCS: M=3 interlaces, each with N=3 or N=4 PRBs

Proposal 3: Partial interlace allocation in the units of LBT bandwidth should be supported.

Proposal 4: The working assumption on UL interlace design should be confirmed.

Proposal 5: UL interlace design in Table 1 should be supported.

Proposal 6: Different UL and DL numerologies should be supported in NR-U to cater for various hardware limitation and channel access mechanism between uplink and downlink in NR-U.

Proposal 7: For PUSCH starting positions in NR-U:

- For 60 kHz SCS, PUSCHs as in Rel-15 is enough (option 1).

  - No additional mechanism is specified to determine the starting position.

- For 15 and 30 kHz SCS, more than one starting positions in a slot is necessary (option 2).

  - PUSCH DMRS can be used by gNB to detect the starting point.

  - OFDM symbols are punctured from the end to fit the PUSCH into a slot.

  - FFS: number of starting positions in a slot.

Proposal 8. Either of the NR Rel-15 Type-0, the NR Rel-15 Type-1 and the NR-U interlaced PUSCH RA schemes can be semi-statically configured. Dynamic switching can be configured between either of the RA schemes:

- NR Rel-15 Type-0 and NR Rel-15 Type-1

- NR Rel-15 Type-0 and the NR-U interlaced resource allocation

- NR Rel-15 Type-1 and the NR-U interlaced resource allocation

Proposal 9. The resource allocation scheme for interlaced PUSCH is defined by:

- An RIV denoting the starting interlace index and the number of contiguous interlaces

- Spare RIVs defining non-contiguous interlace allocations

Proposal 10. For resource allocation in wideband operation, indication of the scheduled sub-band(s) should be provided.

For the PUCCH design:

Observation 2: Further study is needed whether the UE multiplexing capacity for enhanced PF0 should be increased, e.g., by frequency domain CDM or mechanisms using different CS values.

Proposal 11: NR Rel-15 PUCCH formats 2 and 3 can be reused in NR-U under the 2 MHz OCB requirement with the following considerations:

- A minimum of 12, 6, and 3 PRBs should be configured with the SCS of 15 KHz, 30 KHz and 60 KHz, respectively.

- To ensure that the UE occupies the configured minimum number of PRBs, the corresponding minimum UCI payload needs to be specified.

- Configure PUCCH Format 3 with the frequency hopping option to satisfy the 2 MHz OCB requirement over the two hops of the transmission slot.

Proposal 12: NR-U PUCCH should be confined within the minimum nominal channel bandwidth, i.e. 20 MHz.

Proposal 13: When the UE is configured to use a wideband PRB-based interlace in a wideband BWP, the PUCCH can be confined within the bandwidth of LBT bandwidth by exploiting the partial interlace.

Proposal 14: A new PRB-interlaced PUCCH format should at least support a UE multiplexing order of 1 on a PRB-interlace (i.e., no CDM).

Proposal 15: For enhanced PUCCH format 0 and 1, Alt. 2 is adopted, i.e., different length-12 Rel-15 PF0 and PF1 sequences are mapped to the PRBs of an interlace.

Proposal 16: For enhanced PUCCH formats 2/3, introduce a physical mapping rule to the physical PRBs of one or more interlaces.

Proposal 17: For transmission of UCI payloads that are insufficient for the enhanced PUCCH format 2/3 to occupy the PRBs of a full interlace, repetition of PRBs from the minimum number of PRBs determined by the UE should be applied.

Proposal 18: The mechanism for Scheduling Request in Rel-15 should be enhanced when Scheduling Request is transmitted on unlicensed band and following aspects can be considered for improvement:

- SR resource efficiency

- SR transmission opportunities

- References

RAN Chairman’s Notes, 3GPP TSG RAN Meeting #97, Reno, USA, May 13 – 17, 2019.

Huawei, HiSilicon,  “Spectrum utilization in 20MHz channel bandwidth for 60 kHz SCS” , R1-1907539, Reno, USA, May 13-17, 2019.

Huawei, HiSilicon, “Channel arrangement for NR-U”, R4-1905480, Reno, USA, May 13 – 17, 2019

3GPP TS 38.104, “Base Station (BS) radio transmission and reception”.

3GPP TR 38.889, “Study on NR-based access to unlicensed spectrum”.

Huawei, HiSilicon, “Performance evaluation of PUCCH”, R1-1907485, Reno, USA, May 13-17, 2019.

- Appendix

Table 6. Simulation assumptions for enhanced PUCCH format 0/1.

| Property | Value |
| --- | --- |
| Carrier frequency | 5 GHz |
| Channel bandwidth | 20 MHz |
| Channel model | TDL-C |
| Delay scaling | 10 ns, 100 ns |
| Antenna configuration at BS* | (M,N,P) = (1,1,2) with omni-directional antenna element |
| Antenna configuration at UE | Single omni-directional antenna element |
| Antenna port virtualization | No beamforming and no beam selection |
| Frequency offset | 0 ppm |
| UE speed | 3 km/h |
| Subcarrier spacing | 15/30 kHz |
| Number of code-division multiplexed users if applicable | 1 user |
| Interference assumption | No inter-cell interference |
| Number of OFDM symbols | 2 |
| Payload | 1 bit |
| Frequency hopping | No |
| * See Table 7-1 of R1-1704144 |  |

- The 30x11 matrix B contains the sequence group indices for each PRB. For example, for sequence group u=3, the 4th row of the matrix gives the values of u to be used in each PRB. For an interlace of N PRBs, the first N columns are used.

B=0628221610141929102111425217262221613421614252210111229141731241614161612720441811161241331221651001623121176141662211591423266497212528223231514168011102541624718289231222631954820100116112124472611112872516414137181712109311828251816013281919142414181714141818254913231623015172214112126426416817161582341242617182018252143182614181419230121201671319161801194287214201618314272150268218172822262318197922232425262728291725191214273101941021221142415151672825152216194260131516142319142519614129441616426212229162215710282515285426426101213291814434245217412

- 15kHz SCS, DS 10ns, 10PRBs/interlace		(b)  15kHz SCS, DS 100ns, 10PRBs/interlace

(c) 30kHz SCS, DS 10ns, 10PRBs/interlace	  	(d)  30kHz SCS, DS 100ns, 10PRBs/interlace

Figure 6. SNR operation points for Alternative 1a (10PRBs/interlace)

(a)15kHz SCS, DS 10ns, 10PRBs/interlace		(b)  15kHz SCS, DS 100ns, 10PRBs/interlace

(c) 30kHz SCS, DS 10ns, 10PRBs/interlace	  	(d)  30kHz SCS, DS 100ns, 10PRBs/interlace

Figure 7. SNR operation points for Alternative 1b(10PRBs/interlace)

- 15kHz SCS, DS 10ns, 10PRBs/interlace		(b)  15kHz SCS, DS 100ns, 10PRBs/interlace

(c) 30kHz SCS, DS 10ns, 10PRBs/interlace	  	(d)  30kHz SCS, DS 100ns, 10PRBs/interlace

Figure 8. SNR operation points for Alternative 2(10PRBs/interlace)

- 15kHz SCS, DS 10ns, 10PRBs/interlace		(b)  15kHz SCS, DS 100ns, 10PRBs/interlace

(c) 30kHz SCS, DS 10ns, 10PRBs/interlace	  	(d)  30kHz SCS, DS 100ns, 10PRBs/interlace

Figure 9. SNR operation points for Alternative 3 (10PRBs/interlace)

(a) 15kHz SCS, DS 10ns, 11PRBs/interlace		(b)15kHz SCS, DS 100ns, 11PRBs/interlace

(c) 30kHz SCS, DS 10ns, 11PRBs/interlace	  	(d)  30kHz SCS, DS 100ns, 11PRBs/interlace

Figure 10．SNR operation points for Alternative 1a (11PRBs/interlace)

(a)15kHz SCS, DS 10ns, 11PRBs/interlace		   (b)15kHz SCS, DS 100ns, 11PRBs/interlace

(c) 30kHz SCS, DS 10ns, 11PRBs/interlace	          	(d)  30kHz SCS, DS 100ns, 11PRBs/interlace

Figure 11. SNR operation points for Alternative 1b (11PRBs/interlace)

(a)15kHz SCS, DS 10ns, 11PRBs/interlace	(b)15kHz SCS, DS 100ns, 11PRBs/interlace

(c) 30kHz SCS, DS 10ns, 11PRBs/interlace	  (d)  30kHz SCS, DS 100ns, 11PRBs/interlace

Figure 12. SNR operation points for Alternative 2 (11PRBs/interlace)

(a) 15kHz SCS, DS 10ns, 11PRBs/interlace		(b)15kHz SCS, DS 100ns, 11PRBs/interlace

(c) 30kHz SCS, DS 10ns, 11PRBs/interlace	               (d)  30kHz SCS, DS 100ns, 11PRBs/interlace

Figure 13. SNR operation points for Alternative 3 (11PRBs/interlace)