3GPP TSG RAN WG1 Meeting #98	　	                                 R1-1908120

Prague, Czech Republic, 26th Aug. – 30th Aug. 2019

Source:	Mitsubishi Electric

Title:	Views on physical-layer procedures for NR positioning

Agenda Item:	7.2.10.4 Necessity and details for physical-layer procedures to support UE/gNB measurements

Document for: 	Discussion/Decision

Introduction

In RAN1 #97 meeting, procedures related to NR positioning was agreed [1]. In this contribution, we explain several proposals regarding the procedure for NR positioning. In Section 2, proposals related to spatial relation among DL RS for positioning and between DL-RS and SRS are described. In Section 3, proposals related to positioning for IDLE/INATIVE UEs are described. Finally, the contribution is concluded in Section 4.

Proposals related to beam management and beam sweeping

In RAN1 #97 meeting, different mechanism to assist UE with RX beamforming was discussed. In particular, the following agreement was made.

Agreement: For positioning purposes, to assist UE to perform Rx beamforming, the following options are supported (options are from previous related agreement in RAN1#96bis): - Option 1: The DL PRS can be configured to be QCLed Type D with a DL Reference Signal from a serving or neighboring cell. - Discuss further which DL reference signal can be used (e.g., SSB, CSI-RS, DL-PRS). - Option 2: The UE may perform a Rx beam sweeping on DL PRS resources which are transmitted with the same downlink spatial domain transmission filter. - Option 3: The UE may use a fixed Rx beam to receive DL PRS resources which are transmitted with different downlink spatial domain transmission filter. - FFS: Whether this option can be achieved by Option 1 - FFS on specification impacts.

From the agreement, we support to relate DL PRS spatially (QCL-D) with other RS. Particularly, DL PRS can be QCLed Type D with CSI-RS and DL-PRS. Since CSI-RS is used for fine beam management, relating DL-PRS with CSI-RS will be beneficial for UE to perform efficient beam management. Relating two DL-PRS is also beneficial since it is possible that two transmission sources of DL-PRS are co-located and they experience similar channel characteristics. Thus, we make the following proposals based on Option 1.

Proposal 1 The DL PRS can be configured to be QCLed Type D with at least CSI-RS and DL-PRS from a serving or neighboring cell

Agreement: For positioning purposes, with regards to the UL SRS transmission power, support at least the following in addition to the existing Rel-15 behaviour: - Support configuring a DL reference signal of a neighbouring cell to be used as DL path loss reference for the purpose of SRS power control. - FFS: Which reference signals can be used (e.g. a CSI-RS, or a SSB, or DL PRS). - Study further the UE configuration signalling and procedure. - FFS on a fallback procedure if the UE is not able to obtain the pathloss reference. - FFS on number of measurements for pathloss.

Similar agreement, shown above, is made for SRS beam management. In this case, spatial relationship between SRS and DL-RS is discussed. For the same reason explained earlier for DL-PRS, having spatial relationship with CSI-RS and DL-PRS is beneficial since both RS have fine angular granularities. For SRS aimed at neighbouring cells, accurate alignment with DL RS will become critical for power consumption and maximizing coverage. Thus we make the following proposal based on Option 1 above:

Proposal 2: Support configuration of a spatial relation between a reference DL RS from serving or neighbouring cells and the target SRS where a reference DL RS is either CSI-RS or DL-PRS.

Proposals related to INACTIVE or IDLE UEs

In addition, positioning for INACTIVE or IDLE UEs should be considered considering devices used in factory environment. Power saving is a critical feature in IoT devices and positioning while devices are in INACTIVE or IDLE mode should be considered. Such specification will be useful for devices which need to conserve energies. For general DL and UL positioning techniques, broadcast information based positioning process for IDLE or INACTIVE devices is considered here. Periodical monitoring of SIB by IDLE or INACTIVE UEs can be used as opportunities to initiate positioning. For example, the positioning request can be included in SIB in PDSCH from which UE initiates the positioning procedure. In addition, paging information monitored by UE can also contain a trigger for positioning procedure. PRACH sent by UE can be dedicated for positioning as well. For example, since random access procedure is not necessary for positioning, RAR from gNB may not be needed. Finally, the aforementioned monitoring can be done without moving to the RRC_CONNECTED mode.

Based on the above, the following proposals are made.

Proposal 3: Positioning for INACTIVE or IDLE UEs should be considered

Proposal 4: Positioning request in SIB and/or paging information should be supported for IDLE or INACTIVE UEs. FFS details of the request

Conclusion

In this contribution we made the following proposals:

Proposal 1 The DL PRS can be configured to be QCLed Type D with at least CSI-RS and DL-PRS from a serving or neighboring cell

Proposal 2: Support configuration of a spatial relation between a reference DL RS from serving or neighbouring cells and the target SRS where a reference DL RS is either CSI-RS or DL-PRS.

Proposal 3: Positioning for INACTIVE or IDLE UEs should be considered

Proposal 4: Positioning request in SIB and/or paging information should be supported for IDLE or INACTIVE UEs. FFS details of the request

- References

- [1]	RAN1 Chairman’s Note, RAN1 #97, Reno, USA, May, 2019.