3GPP TSG RAN WG1 Meeting #98							R1-1908155

Prague, CZ, August 26th – 30th, 2019

Source:	vivo

Title:	QoS management for sidelink

Agenda Item:	7.2.4.6

Document for:	Discussion and Decision

- Introduction

The V2X WI has been approved in RAN#83 meeting [1]. The objectives include the support of QoS management, as well as the congestion control, in NR sidelink:

1. NR sidelink: Specify NR sidelink solutions necessary to support sidelink unicast, sidelink groupcast, and sidelink broadcast for V2X services, considering in-network coverage, out-of-network coverage, and partial network coverage. - Congestion control [RAN1, RAN2] 4. Specify support for QoS management [RAN2, RAN3, RAN1]

In this contribution, we provide our view on these objectives for NR sidelink.

- Discussion

  - Congestion control

Congestion control has been investigated in LTE V2X. It has been agreed that LTE V2X sidelink congestion control is the starting point for defining NR sidelink congestion control in the previous RAN1 meeting [2].

Agreements: - LTE V2X sidelink congestion control is the starting point for defining NR sidelink congestion control. Agreements: - Higher-layer reporting of CBR to the gNB is supported for RRC_CONNECTED UEs.

Therefore, the concept of CBR/CR measurement defined in LTE can be the starting point for NR V2X. However, given that many new features are introduced in NR, the definition of CBR/CR should be modified accordingly.

Channel busy ratio (CBR)

In LTE V2X, CBR measurement is introduced separately for PSCCH and PSSCH respectively, and is defined as the portion of sub-channels/resources in the resource pool whose S-RSSI measured by the UE exceeds a (pre-)configured threshold sensed over a fix measurement window. Similarly, in NR V2X, CBR measurement can also be defined for PSCCH and PSSCH respectively. This is beneficial in NR V2X because the PSCCH is transmitted in a broadcast manner for sensing by nearby UEs, while the PSSCH may be transmitted only to the target UE (i.e., for unicast). As a result, different Tx power or precoding may be applied to PSCCH and PSSCH respectively. Therefore, CBR can be largely different between these two channels and should not be measured together. Otherwise, it may not work well if only the control channel is congested, while the CBR of the channels (control + data) are still under the threshold.

Beside PSCCH and PSSCH, PSFCH is introduced in NR V2X. Thus, whether CBR is defined for PSFCH should be discussed. Given that implicit mapping rule is agreed for PSFCH resource determination, it is likely that the congestion situation of PSFCH is dependent on the associated channels (PSCCH and PSSCH). Therefore, it seems not necessary to define CBR for PSFCH.

Proposal 1: CBR measurement is defined separately for PSCCH and PSSCH, respectively. It is not necessary to introduce CBR measurement for PSFCH.

Another issue of CBR measurement is the duration of the measurement window to be used in NR V2X. Considering that SCSs other than 15 kHz are supported in NR, the 100 ms of measurement window in LTE can be revisited.  However, for the sake of integrating the NR V2X into the existing ETSI ITS DCC (distributed congestion control) framework, it is desirable to maintain the same measurement window duration (i.e., 100 ms).

Proposal 2: CBR measurement is taken over the measurement window of 100 ms.

Channel occupancy ratio (CR)

In LTE V2X, CR is defined as the total number of sub-channels used for its transmissions in subframes [n-a, n-1] and granted in subframes [n, n+b] divided by the total number of configured sub-channels in the transmission pool over [n-a, n+b]. UE is configured with a CR limit and ensures the acquired sidelink resources would never exceed the CR limit for a given CBR range. The same principle can be reused for NR V2X, while the definition of the measurement window [n-a, n+b] can be determined once the sensing window is clearly defined.

Proposal 3: The CR measurement in LTE can be reused for PSSCH in NR V2X, while the measurement window is FFS.

  - QoS signaling

In the case of congestion control in sidelink mode 2, to perform inter-UE contention for resources, a UE should be able to identify the QoS attribute for each packet from the other UEs during sensing. As a result, the UE can preempt the resource reserved for lower priority packet, while avoid violating the resource of higher priority packet. More details are discussed in [3]. Therefore, the QoS attribute should be indicated in SCI.

The Per-Flow QoS model is agreed by SA2 for unicast, broadcast and groupcast transmissions [4], and PC5 5QI (PQI) value instead of PPPP/PPPR is used to represent QoS requirements. Further, the Priority Level of PC5 QoS characteristics has the same format and meaning as that of the PPPP:

The Priority Level has the same format and meaning as that of the ProSe Per-Packet Priority (PPPP) defined in TS 23.285 [8]. NOTE: Using the same format for Priority Level and PPPP provides better backward compatibility. The Priority Level shall be used to different treatment of V2X service data across different mode of communication, i.e., broadcast, groupcast, and unicast. In case when all QoS requirements cannot be fulfilled for all the PC5 service data, the Priority Level shall be used to select for which PC5 service data the QoS requirements are prioritized such that a PC5 service data with Priority Level value N is prioritized over PC5 service data with higher Priority Level values, i.e. N+1, N+2, etc (lower number meaning higher priority).

Therefore, at least the Priority Level can be delivered to physical layer and included in SCI.

Proposal 4: At least the Priority Level of PC5 QoS should be indicated in SCI.

  - QoS handling

According to the conclusions in SA2 TR 23.786, for Key Issue #4 (Support of PC5 QoS framework enhancement for eV2X), depending on RAN WG decisions, it is concluded that Solution #19 in clause 6.19 is used as the baseline for normative work, with the working assumption that VQI can be supported by NR PC5. A new set of VQIs would be defined in normative phase [5]. The key part of how PC5 QoS is handled is abstracted as below.

##### 6.19.2.1.3 QoS handling for eV2X communication When PC5 unicast is used for the transmission of eV2X messages, the following principles are applied for both network scheduled operation mode and UE autonomous resources selection mode: - PC5 QoS parameters defined in clause 6.19.1.2 applies to the eV2X communication over PC5. - The eV2X message is sent on the PC5 QoS flow established using the procedure described in clause 6.19.2.1.2. - The mapping of application layer eV2X message to PC5 QoS parameters is based on the PC5 QoS rule. When the network scheduled operation mode is used, following additional principles apply: - UE provides PC5 QoS parameter information to the gNB for resources request. - When the gNB receives a request for PC5 resource from a UE, the gNB can authorize the requested PC5 QoS parameter based on the PC5 QoS parameters received from AMF. - gNB can use the PC5 QoS parameter information for PC5 QoS handling. When the autonomous resources selection mode is used, following additional principle applies: - The UE can use the PC5 QoS parameter for PC5 QoS handling based on the provisioned information described in clause 6.19.2.1.1.

From the above-highlighted text, it can be observed that the node performing the PC5 QoS handling is determined by the resource allocation mode used for the traffic. To be more specific, in mode-1 gNB is responsible for guaranteeing the required PC5 QoS requirement, while in mode-2, UE is responsible for guaranteeing the required PC5 QoS requirement.

Observation 1: In mode-1, gNB is responsible for guaranteeing the required PC5 QoS, while in mode-2 UE is responsible for guaranteeing the required PC5 QoS.

From the performance perspective, it is assumed that mode-1 resource allocation where gNB acting as a scheduling node can ensure better scheduling performance than that of mode-2 resource allocation. The main reason is that in mode-2 UE can only discover surrounding UEs in the proximity and gather some local information. However, in mode-1, gNB can collect information from a large amount of UEs, and this information can be further utilized to achieve a globally optimal solution for the scheduling algorithm. A comparison simulation is conducted and demonstrated in Figure 1.

Figure 1 Average PRR (packet reception ratio) performance of groupcast in Freeway scenario

Moreover, mode-2 sensing and resource allocation operations have their performance constraints due to hidden UE, collision/interference, and half-duplex issues. To achieve comparable spectrum efficiency as mode-1 operation, some tradeoff should be made, e.g., by consuming more system resource such as bandwidth resource. However, bandwidth resource is precious. It is usually much cost-effective to use mode-1 when the very stringent PC5 QoS is required for NR V2X. If the mode-2 operation is not enforced to guarantee the same level of PC5 QoS requirement as using mode-1, the mode-2 design can be much simplified.

Therefore, it seems that relying on mode-1 operation to achieve stringent QoS requirements is a reasonable assumption. That is also the rationale why most companies consider various QoS requirements for dual configuration of different sidelink modes, for the use case that a UE is configured to perform both mode-1 and mode-2 sidelink operations [6]. In other words, different modes can be used to guarantee different PC5 QoS requirements, and the dual configuration of different sidelink modes is especially beneficial when new service data with very stringent PC5 QoS requirement arrives.

Observation 2: Different resource allocation modes can be used to guarantee different PC5 QoS requirements, while mode-1 can guarantee more stringent PC5 QoS requirements than mode-2 operation.

Given the above observations, the condition and mechanism to determine a specific resource allocation mode, for the sake of fulfilling the QoS requirement should be studied. The relationship between resource allocation mode and PC5 QoS range may depend on deployment scenarios, applicable regulations, etc. It is beneficial that the UE AS layer maintains a mapping between resource allocation mode and PC5 QoS range based on UE configuration. In such way, when new service or traffic is arriving, the AS layer can be involved to choose a suitable resource allocation mode for each transmission based on the PC5 QoS requirement.

Proposal 5: UE can be configured with a mapping between resource allocation mode and PC5 QoS range.

The Uu and/or PC5 load status can vary a lot, considering the dynamic wireless environment and UE mobility. Under this premise, UE configured with a single-mode may not able to guarantee the required PC5 QoS. For example, when a UE is using mode-1 while the Uu interface is overloaded, the gNB may have trouble to send the SL grant properly to the UE. However, it is reasonable to assume that the gNB has enough information about the Uu load status; therefore, the decision of configuring UE to activate mode-2 can be left to network implementation.

Observation 3: For a UE using mode-1 operation, whether the UE is configured to activate mode-2 can be left to network implementation.

On the other hand, when a UE is using mode-2, and the PC5 interface is congested, the UE may not be able to select enough resource to guarantee the required PC5 QoS. Nevertheless, it is still promising that UE can use mode-1 instead to meet the required PC5 QoS. Hence, mode trigger condition can be introduced in this scenario. Specifically, when the mode-2 UE is in RRC Idle or Inactive, the UE can trigger RRC establishment or resume procedure to enter RRC Connected and request gNB to activate mode-1. When the mode-2 UE is in RRC Connected firstly, UE can directly request gNB to activate mode-1. After receiving a request from the UE, the gNB further decides whether to accept or reject this request and responses to the UE.

Proposal 6: When RRC Idle/Inactive UE using mode-2 cannot guarantee the required PC5 QoS, UE can trigger RRC establishment/resume procedure to enter RRC Connected state and request gNB to activate mode-1.

Proposal 7: When RRC Connected UE using mode-2 cannot guarantee the required PC5 QoS, UE can request gNB to activate mode-1.

- Conclusion

In the contribution, we provide our view on the support of QoS management, as well as the congestion control in NR sidelink, and observe that,

Observation 1: In mode-1, gNB is responsible for guaranteeing the required PC5 QoS, while in mode-2 UE is responsible for guaranteeing the required PC5 QoS.

Observation 2: Different resource allocation modes can be used to guarantee different PC5 QoS requirements, while mode-1 can guarantee more stringent PC5 QoS requirements than mode-2 operation.

Observation 3: For a UE using mode-1 operation, whether the UE is configured to activate mode-2 can be left to network implementation.

Based on these observations, we propose that,

Proposal 1: CBR measurement is defined separately for PSCCH and PSSCH, respectively. It is not necessary to introduce CBR measurement for PSFCH.

Proposal 2: CBR measurement is taken over the measurement window of 100 ms.

Proposal 3: The CR measurement in LTE can be reused for PSSCH in NR V2X, while the measurement window is FFS.

Proposal 4: At least the Priority Level of PC5 QoS should be indicated in SCI.

Proposal 5: UE can be configured with a mapping between resource allocation mode and PC5 QoS range.

Proposal 6: When RRC Idle/Inactive UE using mode-2 cannot guarantee the required PC5 QoS, UE can trigger RRC establishment/resume procedure to enter RRC Connected state and request gNB to activate mode-1.

Proposal 7: When RRC Connected UE using mode-2 cannot guarantee the required PC5 QoS, UE can request gNB to activate mode-1.

- Reference

- RP-190766, “New WID on 5G V2X with NR sidelink”, RAN#83, Shenzhen, March 2019.

- Chairman Notes, 3GPP TSG RAN WG1 #97, Reno, May 2019.

- R1-1908150, “Discussion on mode 2 resource allocation mechanism”, vivo, Prague, August 2019.

- R1-1905944, “LS response on unicast, groupcast and broadcast in NR sidelink”, Reno, May 2019.

- TR 23.786, “Study on architecture enhancements for EPS and 5G System to support advanced V2X services”, V1.1.0, 2019-01.

- [104#59][NR/V2X], RAN2 email discussion for resource allocation.