3GPP TSG RAN WG1 Meeting #98	R1-1908206

Prague, CZ, 26th – 30th August, 2019

Title:	Discussion on enhancement of initial access procedures for NR-U

Source: 	ZTE, Sanechips

Agenda Item:	7.2.2.2.2

Document for:	Discussion and Decision

Introduction

In RAN #82 meeting, the new WI proposal on “NR-based Access to Unlicensed Spectrum” (NR-U) was approved [1]. This WI targets to specify NR enhancements for a single global solution framework operating in unlicensed bands, e.g., 5GHz and 6GHz. In the NR-U WID, it was described about the specifying support for initial access, random access, scheduling request (SR) and RLM/RRM. During NR-U SI stage, some agreements have been reached and can be regarded as a starting point for NR-U WID.

In RAN1 #96 meeting [2], some agreements on enhancement of initial access procedures for NR-U were reached as follows:

Agreement:

- At least the functionalities of Rel-13 LTE-LAA RSSI and channel occupancy reporting as a baseline should be supported

- FFS:

  - Enhanced RSSI metrics, for e.g., sub-band-level interference measurements in a wideband operation scenario

  - Reporting of a new medium contention/load metric other than channel occupancy

  - Any modification of the parameters of the Rel-15 SMTC for operation in unlicensed spectrum

Agreement:

- For a given cell, UE may assume SS/PBCH blocks in the same candidate position within the DRS transmission window are QCL across DRS transmission windows

  - Alt1: The PBCH DMRS sequence index is also the same

  - Alt2: The PBCH DMRS sequence index may be different

  - Note: The first candidate position of the DRS transmission window is located at the first half slot of a half frame

- FFS: QCL assumption for SSBs in different candidate positions within a DRS transmission window and across DRS transmission windows

Agreement:

- An RLM measurement window for serving cell RLM measurements based on SSBs in the DRS is supported for in-sync and out-of-sync evaluations.

  - FFS: How RLM measurement window is indicated or determined and relation to DRS transmission window

  - FFS: Whether or not an SSB can fall outside the measurement window and, if so, whether it can be used for in-sync and out-of-sync evaluations.

  - FFS: Any relationship of RLM measurements based on CSI-RS to the measurement window.

- FFS: Mechanism to handle missing RLM-RS due to LBT failure

In RAN1 #96bis meeting [3], some agreements on enhancement of initial access procedures for NR-U were reached as follows:

Agreement:

The maximum DRS transmission window duration is 5 ms.

- The maximum number of candidate SSB positions within a DRS transmission window, Y, is selected as Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS.

  - Note: The number of starting points for DRS transmissions with the 5 ms window that can use a Cat. 2 LBT is to be discussed further as part of channel access discussions.

- FFS: If the DRS transmission window is configurable, and if yes, how to configure and indicate the window, including the range of configurable values.

Agreement:

For a given cell, the UE may assume that the PBCH DMRS sequence index is the same for SS/PBCH blocks that are transmitted at the same candidate positions across DRS transmission windows.

Agreement:

UE determines serving cell timing from the detected SSB candidate position, where the SSB candidate positions within the DRS transmission window are indexed from 0,…,Y-1 (Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS).

In RAN1 #97 meeting [4], some agreements on enhancement of initial access procedures for NR-U were reached as follows:

Agreement:

The mechanism to determine serving cell timing is as follows:

- The SS/PBCH block position index within a DRS transmission window is detected using a combination of PBCH DMRS sequence index and 1 bit/2 bits for 15 kHz SCS/30 kHz SCS of the 3 available bits in the PBCH payload (not in MIB) originally used in Rel-15 FR2 for MSB SSB index

- 10-bits SFN and half-frame indicator are indicated as in Rel-15

- PBCH payload size is not increased compared to Rel-15

  - FFS: Whether reuse of other available bits in PBCH payload is also required for timing determination

- If the UE is required to perform PBCH decoding of neighbor cell(s) (e.g., in asynchronous deployments), an explicit time allowance for acquisition of SSB index is provided to the UE

Agreement:

LBT category for msg 3 initial transmission is provided to the UE in RAR

Agreement:

Reply to the RAN2 LS informing them of the following:

- RAN1 has made the following agreement which facilitates COT sharing between Msg2 and Msg3:

  - LBT category for msg 3 initial transmission is provided to the UE in RAR

- Multiple msg3 tx opportunities with a single or multiple RARs in the time domain is feasible from a RAN1 perspective but there is no consensus at this time in RAN1 to support this. RAN1 will continue discussions on the support of multiple msg3 tx opportunities.

Agreement:

For a serving cell, UE may assume a QCL relation between SS/PBCH blocks which are detected across DRS transmission windows and have the same value of modulo(A, Q), once Q is known to the UE

- FFS: A is the SSB candidate position index and/or PBCH DMRS sequence index

- FFS: How Q is indicated or determined

- FFS: Restriction on the range of Q.

Note: Neighbor cell RRM measurements will be addressed separately

Agreement:

For SSB-based RLM, UE may assume the RLM measurement window to be the same as the DRS transmission window.

- Note: This implies that the SSB-based RLM-RS cannot fall outside the measurement window

- FFS: Whether and how DRS transmission window is configured to the UE

In RAN #84 plenary meeting, the views on essential/optimization items on each WGs for on time completion of NR-U WI was presented in [5]. Wherein, the essential/optimization items in “7.2.2.2.2 Enhancements to initial access procedure” are as follows:

Essential

- Timing recovery from SSB

- Cross SSB QCL relationship by Q and A: Which A, how to indicate or determine Q, range of Q, how to determine Q for RRM

- Type0-PDCCH monitoring

- RLM enhancement: IS and OOS

Optimizations

- Msg1 enhancements with more opportunities in freq domain (over multiple LBT subbands) and time domain

- RRM enhancement including subband level RSSI and channel occupancy measurement

In this contribution, we will share our views on enhancement of initial access procedures for NR-U, including discovery reference signal (DRS), enhancement of RACH procedure, and RLM/RRM.

Discovery reference signal (DRS)

SS/PBCH Block transmission mechanism

Considering the uncertainty of the channel availability due to LBT, one or more SS/PBCH Blocks (SSBs) may not be able to be transmitted at the predefined position. In order to handle reduced SSB opportunities due to LBT failure, three candidate SSB mechanisms in RAN1 #94 meeting were identified to increase SS/PBCH Block transmission opportunities.

Alt-1: Shift SSB(s) in time to the next transmission instance.

According to the conclusion of the last meetings, we can know that the number of candidate SSB positions Y is 10 for 15 kHz SCS or 20 for 30 kHz SCS. The maximum number of actual transmitted SSBs X is 8. Here we assume the number of actual transmitted SSB is 4. Alt1’s method is illustrated in Figure 1 where it is observed that if the gNB does not obtain the ownership of the channel before the candidate SSB #0 position, it will drop the current SSB#0 transmission opportunity, while trying to transmit SSB#0 on the next candidate SSB position such as SSB#1 in Figure1. If the gNB performs LBT successfully before candidate SSB#1 position, it transmits SSB#0, SSB#1, SSB#2 and SSB#3 on the candidate SSB#1, SSB#2, SSB#3 and SSB#4 positions in turn. Wherein, the shift granularity is one SSB or half-slot.

Further, the first actually transmitted SSB position depends on the moment of LBT success. In order to acquire time/frame synchronization, the gNB needs to inform the offset value in the time domain between the original SSB position and actual transmitted SSB position to UE. As shown in Figure 1, at least 3bits is needed to indicate offset value when the number of candidate SSB positions Y is 10 for 15 kHz SCS and the number of actual transmitted SSBs is 4, while the bit overhead required to indicate offset will be larger when Y is 20 for 30 kHz and the number of actual transmitted SSB is 4. Besides, if UE is configured to monitor a particular SSB index for RLM, due to LBT failure, UE could not detect the configured SSB index in the fixed SSB position, which will impact on RLM/RLF.

Figure 1 Shift SSB(s) in time to the next transmission position

Alt-2: Cyclically wrap the SSBs dropped due to LBT failure around to the end of the burst set transmission.

The assumed conditions are similar to Alt-1. Different from Alt-1, the relationship between the actually transmitted SSB index and candidate SSB position is fixed. As shown in Figure 2, each SSB index has at least two times transmission opportunities. That is to say, if the gNB does not obtain the ownership of the channel before the candidate SSB #0 position, then it give up SSB #0 transmission in the candidate SSB #0 position and try to perform LBT before the candidate SSB #1 position. If LBT is successful, gNB transmits SSB#1, SSB#2, SSB#3 and SSB#0 on the candidate SSB#1, SSB#2, SSB#3 and SSB#4 positions.

In order to obtain timing information, UE only needs to obtain the number of actual transmitted SSB that can be indicated by gNB using RMSI. Each SSB index of transmission position can be determined by candidate position index of the detected SSB mod the number of actual transmitted SSB.

Compared with Alt-1, Alt-2 has less specification impact and bit overhead. Wherein, at most 3 bits are needed to indicate the number of actual transmitted SSB,

Figure 2 Cyclical wrapping the SSB to the end of the burst set transmission

Alt-3: Network to flexibly position SSB index and indicate the timing information.

For Alt-3, it will allow the network to flexibly transmit SSB index and indicate the frame timing information.. But compared with Alt-1 and Alt-2, more signaling bits are required for indicating the association of SS/PBCH blocks between different DRS occasions and frame timing and actual transmitted SS/PBCH Block index and so on, yet with unclear benefit of such flexibility. More specification modification is needed.

Based on the above analysis, Alt-2 is preferred to be used for SS/PBCH block transmission in NR-U considering the required signaling bits and specification impact.

Proposal 1: Considering the overhead and specification impact, Alt-2 method that cyclically wraps around the dropped SSBs due to LBT failure should be supported for NR-U.

Timing Acquisition and QCL Assumption

RAN1 #96bis has agreed that UE determines serving cell timing from the detected SSB candidate position, where the SSB candidate positions within the DRS transmission window are indexed from 0,…,Y-1 (Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS). RAN1 #97 further agreed that the SS/PBCH block position index within a DRS transmission window is detected using a combination of PBCH DMRS sequence index and 1 bit/2 bits for 15 kHz SCS/30 kHz SCS of the 3 available bits in the PBCH payload (not in MIB) originally used in Rel-15 FR2 for MSB SSB index.

According to above agreements, the problem of timing acquisition from SSB has been completely solved if the index of one SSB transmitted at a candidate position is the same as the position index, and there are no remaining problems. Here's an example of how to get the timing from SSB.

Base on the previous conclusion, the timing information can be obtained by formula , wherein, t is the candidate position index of the detected SSB, Stx is the number of transmitted PBCH DMRS sequence, which is fixed at 8, cycle index c is indicated in PBCH payload as Rel-15 FR2, s is PBCH DMRS sequence index, wherein, s{0, 1, 2, 3, 4, 5, 6, 7}.

In order to illustrate the above method, the following table is provided as references:

Table 1 timing determination by PBCH DMRS sequence and PBCH Payload for 15 kHz SCS

| P | DRS transmission window (5ms) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| t | SSB #0 | SSB #1 | SSB #2 | SSB #3 | SSB #4 | SSB #5 | SSB #6 | SSB #7 | SSB #8 | SSB #9 |
| c | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| s | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 |
| Notes: - P denotes parameters. - The number of candidate SSB position, Y, is 10 for 15 kHz SCS within 5ms DRS transmission window. - 1 bit for cycle index c is needed, so existing 3 bits in PBCH Payload is sufficient to indicate cycle index c. - Take yellow highlighted column as an example to illustrate how to determine timing information, if UE detects PBCH sequence index s as 2 and read PBCH payload content c as 0, then the candidate SSB position index of the detected SS/PBCH block can be determined by formula =8*0+2=2. |  |  |  |  |  |  |  |  |  |  |

Table 2 Timing determination by PBCH DMRS sequence and PBCH Payload for 30 kHz SCS

| P | DRS transmission window (5ms) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| t | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| c | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 |
| s | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 |
| Notes: - P denotes parameters. - The number of candidate SSB position, Y, is 20 for 30 kHz SCS within 5ms DRS transmission window. - Only 2 bits for cycle index c are needed, so existing 3 bits in PBCH Payload is sufficient to indicate cycle index c. - Take yellow highlighted column as an example to illustrate how to determine timing information, if UE detects PBCH sequence index s as 3 and read PBCH payload content c as 1, then the candidate SSB position index of the detected SS/PBCH block (t)can be determined by formula =8*1+3=11. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation 1: According to previous agreements, the problem of timing acquisition from SSB has been completely solved, if the index of an SSB transmitted at a SSB candidate position is the same as the position index that is a natural choice.

Another issue is how to determine QCL assumption. For a serving cell, UE may assume a QCL relation between SS/PBCH blocks which are detected across DRS transmission windows and have the same value of modulo(A, Q), once Q is known to the UE. Wherein A can be defined as the SSB candidate position index or PBCH DMRS sequence index. For Q = 1, 2, 4, 8, any one of SSB position index and sequence index is a right definition for A. However, for Q = 3, 5, 6, 7, A can only be interpreted as SSB position index, since the correct QCL relation can not be obtained from the formula modulo(DMRS sequence index A, Q).  Thus, no matter which value Q is configured, A can be interpreted as SSB candidate position index. Wherein, the range of Q should be consistent with that of R15. We should not restrict on the range of Q.

Proposal 2: UE assumes a QCL relation between SS/PBCH blocks which are detected across DRS transmission windows and have the same value of modulo(A, Q)

- The range of Q  should not be restricted, and it could be consistent with that of Rel-15 NR.

- A should be interpreted as SSB position index. It is not feasible to define A as PBCH DMRS sequence index if Q = 3, 5, 6, 7.

DRS Transmission in Time Domain

According to our companion contribution on initial access signals and channels [4], it can be known that the basic unit of DRS transmission can be half-slot for the sake of symmetrical design within a slot, which can be labeled as DRS burst. Due to the uncertainty of the channel availability, DRS burst may not be transmitted on the fixed candidate DRS burst positions within DRS transmission window due to LBT failure. In order to increase DRS burst transmission chances, Alt2-like method can be adopted. As shown in Figure3, assuming that the number of candidate SSB position within DRS transmission is 20 for 30 kHz SCS, the number of the actual transmitted SSB is 8. If LBT success before the DRS burst #3 position, then gNB can transmit DRS burst #3, #4, #5, #6, #7, #0, #1, #2 on the candidate DRS burst #3, #4, #5, #6, #7, #8, #9, #10 position, respectively(as Case-1 in Figure3). A special case, if there is an empty symbol at the end of the previous DRS burst or not all entire DRS burst can be transmitted within one channel occupation time or beam switching occurs, then gNB may need to perform a LBT operation. If LBT failure before the current DRS burst, then gNB can try to transmit it on the next candidate DRS burst position within DRS transmission window, e.g., DRS burst index #1 can try to be transmitted on the candidate DRS burst #17 position as Case-2 in Figure3.

Figure 3 Cyclical wrapping the DRS burst to the end of the DRS burst set transmission

Another enhancement method of DRS transmission that configuration multiple candidate DRS transmission window within a period of DRS. Wherein, the following methods can obtain multiple candidate DRS transmission window: directly add more candidate DRS transmission window within a period of DRS, or by configuring dual periodicities for DRS transmission (e.g., including a long periodicity and a candidate short periodicity). Wherein, multiple candidate DRS transmission window can be configured by semi-static mode.

For the latter as shown in Figure 4, a long periodicity and a candidate short periodicity are respectively configured as 80 ms and 20 ms. The DRS transmission window over short periodicity are used to increase DRS transmission opportunities when the DRS transmission at long periodicity occasion fails. If DRS transmission at long periodicity occasion is successfully transmitted, NR-U gNB can skip the following short periodicity occasions until the next long periodicity occasion. In other words, in a (long) period, there can be multiple candidates for DRS transmission or multiple DRS transmission window.

Figure 4 Multiple candidate half-frame windows/DRS transmission window within a DRS periodicity

Proposal 3: In order to increase DRS transmission opportunities, multiple candidate DRS transmission window can be configured within a period of DRS transmission, e.g., configure dual periodicities for DRS transmission window.

LBT for DRS burst (set)

Based on the conclusion RAN1#96bis, Cat-4 LBT is to be used for DRS duty cycle when DRS transmission is longer than 1/20 or the total of DRS duration is larger than 1 ms. While Cat-2 LBT can be performed only for DRS duty cycle smaller than 1/20 or the total of DRS duration smaller than 1ms. For this agreement, our understanding is that Cat-4 LBT is just used before the DRS multiplexed with unicast date are transmitted. Besides, Cat-4 LBT can also be used before DRS burst set starts transmission, wherein, duration of DRS burst set > 1 ms or duty cycle of DRS burst set in a DRS period >1/20.

Besides, Cat-2 LBT can be used for the following situations:

Case 1: Within DRS burst set, if there is an empty symbol/gap between DRS bursts (half-slot), Cat-2 LBT can try to be performed as shown in Figure 6.

Figure 6 An example of LBT for DRS burst set and DRS burst

Case 2: Within DRS transmission window, if a new DRS burst set is initiated by gNB and its duration is smaller than or equal to 1 ms and the total duty cycle of DRS transmission in a DRS burst set period is not greater than 1/20, Cat-2 LBT can try to be adapted before this new DRS burst set as shown in Figure 7. In another case, if gNB uses Cat-4 LBT to detect channel busy all the time and only a few SSB candidate positions remain before the ending of DRS transmission window, it can be considered to use Cat-2 LBT for DRS burst set transmission.

Figure 7 An example of LBT for DRS burst set and DRS burst transmission (total duty cycle >1/20)

Case 3: Before DRS burst set that only occupies 1ms channel time, which is similar to LTE-LAA DRS. Cat-2 LBT can be considered, as shown in Figure 8. However, if the length of DRS burst set is greater than 1 ms or duty cycle is greater than 1/20 in the next period of DRS burst set, then Cat-4 LBT should be performed.

Figure 8 An example of LBT for DRS burst set >= 1 ms

Case 4: If the adjacent two SSBs(DRS bursts(half-slot)) within DRS burst set is non-QCL’d relationship and there is a gap between non-QCL’d SSB or beam switching occurs, then Cat-2 LBT can be used in order to fast access channel and accurately access current channel status. Preferably, directional LBT can be performed for beam-based SSB transmission within DRS burst.

Case 5: Cat-2 LBT mechanism can also be considered for the wideband or CA scenarios, which is similar to LTE-LAA multiple CCs (e.g., Type A/B) manner.

Based on the above analysis, the LBT mechanism for DRS transmission should be considered according to different situations.

Proposal 4: Cat-4 LBT can be used for the following cases:

- Before the DRS multiplexed with unicast date are transmitted (Agreeded in RAN1#96).

- Before DRS burst set starts transmission and DRS burst set duration is larger than 1ms or the total duty cycle of DRS transmission in DRS burst set period is larger than 1/20.

Proposal 5: Cat-2 LBT can be considered to be used in the following situations:

- Case 1: Within DRS burst set, if there is an empty symbol/gap between DRS bursts (half-slot),.

- Case 2: Within DRS transmission window, if a new DRS burst set is initiated by gNB and its duration is smaller than or equal to 1ms and the total duty cycle of DRS transmission in a DRS burst set period is not larger than 1/20, or if gNB uses Cat-4 LBT to detect channel busy all the time and only a few SSB candidate positions (e.g., 2 SSBs) remain before the ending of DRS transmission window.

- Case 3: Before DRS burst set that only occupies 1ms channel time, which is similar to LTE-LAA DRS.

- Case 4: If the adjacent two SSBs (DRS bursts (half-slot)) within DRS burst set is non-QCL’d relationship and there is a gap between non-QCL’d SSB.

- Preferably, directional LBT can be performed for beam-based SSB transmission within DRS burst.

- Case 5: for case where multiple sub-band case in the wideband operation or multiple carriers/BWPs case.

Enhancement of RACH procedure

RACH Resource

In RAN1 #94bis meeting, the following several options were proposed to handle the issue of reduced transmission opportunities due to LBT failure for Msg 1 transmission in four-step RACH procedure.

- Frequency-domain enhancement

  - Multiple PRACH resources across multiple LBT sub-bands/carriers for both contention-free and contention-based RA

- Time-domain enhancements

  - For connected mode UE, scheduling of PRACH resources via DCI.

    - Triggered PRACH within TXOP can use a new resource

  - For idle mode UE, scheduling of PRACH resources via paging

    - Note: potential inefficiency in network resource due to paging across multiple cells

  - Additional, new RACH resources are used immediately following detection of DRS transmission

  - Multiple PRACH transmissions before Msg2 reception in RAR window for initial access

    - Number of allowed transmissions is pre-defined or indicated, e.g., in RMSI

    - FFS: How to handle potential multiple RARs to same UE

  - Group wise SSB-to-RO mapping by frequency first-time second manner, where grouping is in time domain

If PRACH resource is allowed to be configured per sub-band/carrier, UE can perform LBT operation for RACH over multiple sub-bands in parallel, which is beneficial to increase the success probability of Msg 1 transmission. In the case when LBT within one of multiple sub-bands in a BWP /carriers is performed successfully, UE will need to complete the whole RACH procedure on LBT successful sub-band/carrier. Optionally, each message can be transmitted on which sub-band in a BWP/carrier depends on LBT results of sub-band/carrier.

Proposal 6: RACH resources can be configured over multiple LBT sub-bands within a BWP where each RACH resource is limited within an LBT sub-band.

Option 2-a is a method worth considering since it enables Msg 1 transmission within a gNB initiated COT. Further, it is better to improve resource efficiency within a gNB initiated COT. In order to reduce the risk of losing the channel, option 2-a can be used in conjunction with option1-a.

Proposal 7: Scheduling of PRACH resources via DCI (option2-a) can be considered for NR-U.

As pointed out in the meeting conclusion, scheduling of PRACH resources via paging can be inefficient since the same paging message is transmitted by multiple cells in same paging area.

Proposal 8: Scheduling of PRACH resources via paging (option2-b) should not be supported for NR-U.

For option2-c, new RACH resources immediately following detection of DRS transmission are very limited when other signals/channel such as RMSI PDCCH/PDSCH and so on were filled within the blank symbols of DRS or the number of actual transmitted SS/PBCH blocks within DRS is up to 8. Based on this, UE will face high collision probability if RACH resources immediately following detection of DRS transmission is used.

Proposal 9: New RACH resources immediately following detection of DRS transmission (option2-c) should not be supported for NR-U.

For option2-d, assuming that multiple Msg 1 transmission opportunities were configured before Msg 2 reception in RAR window, if UE perform LBT success in the one of multiple Msg1 transmission opportunities, one or more RACH resources after Msg1 transmitted successfully may be unnecessary. So allocating multiple RACH resources seems inefficient.

Proposal 10: Multiple PRACH transmissions before Msg 2 reception in RAR window (option2-d) should not be supported for NR-U.

For option2-e, Group wise SSB-to-RO mapping by frequency first-time second manner can be achieved by Rel-15 NR design. For example, the number of SSBs is 4, the number of PRACH resources in the frequency domain is 4, and the number of SSBs per RO is 1. Then one SSB can get multiple ROs in the time domain as shown in Figure 9.

Figure 9 SSB-to-RO mapping by frequency first - time second manner.

Proposal 11: Group wise SSB-to-RO mapping by frequency first-time second manner can be achieved by existing Rel-15 NR design.

Random access Response

For RAR, if the preamble was correctly detected, the gNB will respond a RAR message to UE. If LBT of Msg 2 (RAR) fails within the RAR window, then this means that the random access fails. In order to improve success probability of RAR transmission, a straightforward way is to extend maximum RAR window size. However, if RAR window is extended too large, it will also impact random access latency. For example, when a RAR message cannot be transmitted due to either LBT failure or poor channel condition, UE will have to wait for a longer time until RAR window expires. Therefore, considering random access latency and the 10ms length of SFN, we think the RAR window size of around 40 ms is suitable. Given that 40 ms constitutes 4xMCOT of 10ms, the gNB will have enough opportunity to send RAR (i.e. up to 3 other nodes can acquire the channel before the gNB and still the RACH procedure can succeed). Note that the current assumption to use only 20 ms as the extended RAR window is too short since it only allows up to 1 other nodes to acquire the channel before the gNB acquires it. On a high loaded channel, it is very likely that the gNB will miss the transmission window for RAR in this case. So, allowing at least up to 40 ms provides a good tradeoff between latency and the channel access success probability.Besides, multiple RAR transmission opportunities method can also be considered to improve the probability of successful RAR transmission.

Proposal 12: In order to improve success probability of RAR transmission, some enhanced methods can be considered as follows:

- Extend maximum RAR window size but it is suggested that maximum size is not larger than 40ms.

- Configure multiple RAR transmission opportunities.

## Msg3

As agreed in previous meetings, RAN1 can facilitate COT sharing between Msg2 and Msg3, and multiple msg3 Tx opportunities with a single or multiple RARs in the time domain is feasible from the perspective of RAN1, but there is still no consensus in RAN1 #97 meeting. Therefore, RAN1 will continue to discuss whether multiple msg3 Tx opportunities should be supported.

The enhancement of  Msg3 is not a essential problem which must be discussed. The success rate of  a single Msg3 can be improved by means of implementation or multiple msg3 Tx opportunities with a single RAR in the time domain is suitable. So we recommend that the enhancement of  Msg3 should not be considered in Rel-16 NR-U.

Proposal 13: The enhancement of  Msg3 should not be considered in Rel-16 NR-U.

## LBT for PRACH

In order to improve the probability of successful random access procedure and consider fairness coexistence with other systems such as Wi-Fi, Cat-4 LBT with priority class 1 or 2 should be considered. In particular, for two-step RACH scenarios, Msg A may include Msg 1 and Msg 3 that belong to the four-step RACH procedure. The total duration of Msg A in the time domain may exceed 1 ms. It is inappropriate to adapt a DRS-like LBT mechanism, e.g., Cat 2 LBT. In summary, Cat 4 LBT with priority class 1 or 2 should be considered for four-step RACH or two-step RACH.

Proposal 14: Cat-4 LBT with priority class 1 or 2 should be considered for four-step RACH or two-step RACH.

RLM/RRM

- RLM/RLF

In some cases, if the channel is heavily loaded, then DL LBT failures happen with very high probability. Hence, gNB cannot ensure the service quality for UE. Hence, a new mechanism to trigger RLF is necessary in this case to avoid the UE being stuck on a problematic frequency for a prolonged period.

In conclusion, if UE can distinguish LBT failures from poor channel quality condition for transmission of RLM-RS, a mechanism to trigger RLF when necessary due to LBT failures can be considered. For example, when the number of LBT failures for transmission of RLM-RS within a certain period reaches a threshold, a RRC reestablishment procedure can be triggered by UE.

Proposal 15: A new mechanism that the number of consecutive DL LBT failures reaches a threshold in a given period of time can be introduced to trigger RRC reestablishment procedure.

For RRM measurement, sub-band level of the RSSI measurement can be configured as a measurement metric. UE can perform measurement and report the results for each sub-band. And gNB can determine the sub-band-level interference based on each measurement reporting, or the overall wideband-level interference combing with multiple measurement reporting. Thus, sub-band level of RSSI measurement can be achieved by existing RSSI measurement configuration.

Proposal 16: It is unnecessary to introduce a new RSSI measurement for sub-band-level due to it has been supported by existing RSSI measurement configuration.

In LTE-LAA, RSSI measurement was introduced to help eNB detection all possible potential signals from all devices belonging to the same or different operators/RATs on the configured resources and provide a reference for carrier selection. But it treats the signals from the devices of intra-RAT as the interference. In order to avoid this issue, some the effective interference cancellation mechanisms can be considered to be introduced such as COMP. Based on this, it is identified that a measurement to exclude the signals from intra-RAT as the interference is beneficial.

Besides, another type of RSSI measurement is that only detection the signals transmitted from the devices not belonging to the serving operator and the devices of other RATs. In other words, this type of RSSI does not calculate the energy derived from NR-U devices of the same operator. The RSSI type reflects the interference from non-serving cell/operator, which can be achieved by configuration two different types of measurement resources such as Zero power (ZP) resources and Non-zero power (NZP) resources. The RSSI type is specifically useful for frequency reuse among the NR-U devices of the same operator.

Proposal 17: A new type RSSI measurement that only measures signal energy from the devices of non-serving operators and other RATs can be introduced for NR-U to improve the efficiency of frequency reuse.

For the RSSI measurement timing, some parameters such as the periodicity, offset and duration should be configured. Wherein, the duration of SMTC in Rel-15 NR is composed of a set of the consistent subframe. However, in sake of the power consumption, a little measurement samples should be supported in NR-U, e.g., symbol-level based RSSI measurement. Hence, we suggest that the duration of RSSI measurement can be composed of a set of the consistent symbols.

Proposal 18: The duration of RSSI measurement is configured based on symbol level.

Conclusion

In this contribution, we share our views on enhancement of initial access procedures for NR-U. We have the following observation and proposals:

Observation 1: According to previous agreements, the problem of timing acquisition from SSB has been completely solved, if the index of an SSB transmitted at a SSB candidate position is the same as the position index that is a natural choice.

Proposal 1: Considering the overhead and specification impact, alt-2 method that cyclically wraps around the SSBs dropped due to LBT failure to the end of the burst set transmission should be supported for NR-U.

Proposal 2: UE assumes a QCL relation between SS/PBCH blocks which are detected across DRS transmission windows and have the same value of modulo(A, Q)

- The range of Q  should not be restricted, and it could be consistent with that of Rel-15 NR.

- A should be interpreted as SSB position index. It is not feasible to define A as PBCH DMRS sequence index if Q = 3, 5, 6, 7.

Proposal 3: In order to increase DRS transmission opportunities, multiple candidate DRS transmision window can be configured within a period of DRS transmission, e.g., configure dual periodicities for DRS transmission window.

Proposal 4: Cat4 LBT can be used for the following cases:

- Before the DRS multiplexed with unicast date are transmitted (Agreeded in RAN1#96).

- Before DRS burst set starts transmission and DRS burst set duration is larger than 1ms or the total duty cycle of DRS transmission in DRS burst set period is larger than 1/20.

Proposal 5: Cat2 LBT can be considered to be used in the following situations:

- Case 1: Within DRS burst set, if there is an empty symbol/gap between DRS bursts (half-slot),.

- Case 2: Within DRS transmission window, if a new DRS burst set is initiated by gNB and its duration is smaller than or equal to 1ms and the total duty cycle of DRS transmission in a DRS burst set period is not larger than 1/20, or if gNB uses Cat-4 LBT to detect channel busy all the time and only a few SSB candidate positions (e.g., 2 SSBs) remain before the ending of DRS transmission window.

- Case 3: Before DRS burst set that only occupies 1 ms channel time, which is similar to LTE-LAA DRS.

- Case 4: If the adjacent two SSBs (DRS bursts(half-slot)) within DRS burst set is non-QCL’d  relationship and there is a gap between non-QCL’d SSB.

- Preferably, directional LBT can be performed for beam-based SSB transmission within DRS burst.

- Case 5: for case where multiple sub-band case in the wideband operation or multiple carriers/BWPs case.

Proposal 6: RACH resources can be configured over multiple LBT sub-bands within a BWP where each RACH resource is limited within an LBT sub-band.

Proposal 7: Scheduling of PRACH resources via DCI (option2-a) can be considered for NR-U.

Proposal 8: Scheduling of PRACH resources via paging (option2-b) should not be supported for NR-U.

Proposal 9: New RACH resources immediately following detection of DRS transmission (option2-c) should not be supported for NR-U.

Proposal 10: Multiple PRACH transmissions before Msg 2 reception in RAR window (option2-d) should not be supported for NR-U.

Proposal 11: Group wise SSB-to-RO mapping by frequency first-time second manner can be achieved by existing Rel-15 NR design.

Proposal 12: In order to improve success probability of RAR transmission, some enhanced methods can be considered as follows:

- Extend maximum RAR window size but it is suggested that maximum size is not larger than 30ms.

- Configure multiple RAR transmission opportunities.

Proposal 13: The enhancement of  Msg3 should not be considered in Rel-16 NR-U.

Proposal 14: Cat-4 LBT with priority class 1 or 2 should be considered for four-step RACH or two-step RACH.

Proposal 15: A new mechanism that the number of consecutive DL LBT failures is detected in a given period of time can be introduced to trigger RRC reestablishment procedure.

Proposal 16: It is unnecessary to introduce a new RSSI measurement for sub-band-level due to it has been supported by existing RSSI measurement configuration.

Proposal 17: A new type RSSI measurement that only measures signal energy from the devices of non-serving operators and other RATs can be introduced for NR-U to improve the efficiency of frequency reuse.

Proposal 18: The duration of RSSI measurement is configured based on symbol level.

Reference

- 3GPP, RP-182878, New WID on NR-based Access to Unlicensed Spectrum, Qualcomm, RAN#82, December, 2018.

- Draft_Minutes_report_RAN1 #96_v1.0, February, 2019.

- Draft_Minutes_report_RAN1 #96bis, April, 2019.

- Draft_Minutes_report_RAN1 #97_v1.0, May, 2019

- RP-191581, Guidance on essential functionality for NR-U, Qualcomm

- 3GPP, R1-1908202, Considerations on initial access signals and channels for NR-U, ZTE, Sanechips, RAN1 #98 August, 2019.

- ETSI EN 301 893 V2.1.1, Harmonized European Standard, 5GHz RLAN, 2017-05.