3GPP TSG RAN WG1 #98 																		R1- 1908232

Prague, Czech Republic, 26th  – 30th August 2019

Source: 	InterDigital Inc.

Title:	On the Performance of Multi-TRP/Panel Transmission

Agenda item:	7.2.8.2

Document for:	Discussion and Decision

# Introduction

In WG1 Meeting #97 [1] and subsequent email discussion [2], RAN1 continued the discussion on multi-TRP transmission. Based on the discussion the following agreements were reached:

- For multi-PDCCH based multi-TRP operation, increase the maximum number of CORESETs per “PDCCH-config” to 5, according to UE capability - FFS: How to define capability per TRP - Study whether enhancement of reducing PDCCH blocking rate, e.g. Hash function enhancement, and UE complexity is needed, e.g. taking into account overbooking PDCCH candidates and blind detection reduction per TRP/CORESET group. - For separate ACK/NACK feedback for PDSCHs received from different TRPs, the UE should be able to generate separate ACK/NACK codebooks identified by an index, if the index is configured and applied across all CCs - FFS: for the index per TRP basis, e.g. a higher layer signalling index, PRI in L1, CORESET group ID, slot or subslot index in L1 - Support joint HARQ-Ack feedback for PDSCHs received from different TRPs where multiple DCIs are used - When the PUCCH resources are on the different slots, which are indicated by PDSCH-to-HARQ_feedback timing indicator fields of multiple DCIs for different TRPs, both type-1 HARQ-ACK codebook and type-2 HARQ-ACK codebook are supported. - FFS, additional specification impact from Rel-15 - Note that it can include other M-DCI NCJT NW implementation cases in Rel-16 - The index to be used to generate separated ACK/NACK codebook is a higher layer signalling index per CORESET - Note that the index may not be configured for scenarios if there is no ambiguity of codebook generation at the UE, e.g. slot based PUCCH resource allocation per TRP - This does not preclude configuring the index for other purposes - Further clarify details on how to generate separated ACK/NACK codebook by email discussion including how to use such an index - Further clarify details on how to generate joint ACK/NACK codebook by email discussion including whether/how to use such an index - Email discussion on generation of separated ACK/NACK codebook and joint ACK/NACK codebook - by 31st of May (Min, Huawei) - Support following principles for DMRS port indication design for NCJT transmission based on single-PDCCH multi-TRP, at least for single front-load symbol and eMBB - Antenna port field size is the same as Rel-15, at least for DCI format 1-1 - At least support following layer combinations from two TRPs indicated by antenna port field: - 1+1, 1+2, 2+1, 2+2 for single CW and SU, at least for DCI format 1-1 - To be evaluated to determine whether introducing following design principles for DMRS entries in RAN1#98: - 1+3 and/or 3+1 - MU cases, i.e. between NCJT UE+NCJT UE and NCJT UE+S-TRP UE - Two CWs for the case of total layers of NCJT reception more than 4 - For M-TRP based URLLC, support both 2a and 2b - Scheme 2a and 2b have separate UE capabilities. - For scheme 2b, - Additional UE capability is specified to inform the gNB whether the UE can support CW soft combining - Support up to two-layer transmission - In the case of one layer, up to two CBs per CW - In the case of two layers, one CB per CW - FFS: Support of multi-DCI based FDM scheme with repetition (to be concluded in RAN1#98) - FFS: Support of independent MCS selection for each TRP - For single-DCI based M-TRP URLLC schemes 3 & 4, support following design with respect to - The maximal number of transmission layers per transmission occasion, down-select one from the following options: - Option 1: up to single layer transmission - Option 2: up to two layers transmission - PDSCH repetition indication mechanism: - Number of repetitions, down-select one from following options: - Option 1: Dynamic indication - Option 2: High-layer configured as Rel-15 - At least for eMBB with M-DCI NCJT in order to generate different PDSCH scrambling sequences, support enhancing RRC configuration to configure multiple dataScramblingIdentityPDSCH - FFS details including how to associate dataScramblingIdentityPDSCH with TRPs - For rate matching mechanism used for multi-DCI based multi-TRP/panel transmission, support following enhancements: - For LTE CRS, extending lte-CRS-ToMatchAround to be configured with multiple CRS patterns in a serving cell - FFS: Whether/how they apply to one or multiple CRS patterns per PDSCH - FFS: Whether/how it is applied to single DCI based NCJT - For single-DCI based M-TRP URLLC schemes 3 & 4, support following design with respect to - Resource allocation in time domain: - n FFS for further details of the signaling, e.g. starting from the signaling mechanism of slot aggregation in Rel-15 - n FFS: whether a minimal gap between PDSCH mini-slot/slot groups is needed - n FFS: whether the same number of symbols should be indicated for each repetition - n FFS: whether/how to handle the time domain resource allocation considering slot boundary or DL/UL switch in a slot - Resource allocation at frequency domain: - Same frequency domain resource allocation across repetitions as Rel-15 - For the number of TCI states across PDSCH repetitions, down-select one from following options: - Option 1: up to 2 - One TCI codepoint can indicate up to 2 TCI states as already agreed in Rel-16 for eMBB - Option 2: up to 4 - u Option 2-1: One TCI codepoint can indicate up to 4 TCI states - u Option 2-2: New field in DCI (or reuse one or more existing fields in DCI) for indication. - For example, TCI states and RV sequences are jointly preconfigured and the combination of TCI states/RV sequences is jointly indicated in DCI. One codepoint in joint field is to indicate up to 4 TCI states and corresponding RV sequences. - RV sequences for PDSCH repetitions - Option 1: support Rel-15 RV sequences at least - FFS whether additional RV sequence(s), e.g {0,0,0,0}, {0,3,0,3},{0,3,2,1}, is needed, and whether/how a RV sequence applied to the UE is per TRP - Option 2: RV sequences are preconfigured by higher layer without restriction of specific orders in spec. - How to map RVs in RV sequences and indicated TCI states to transmission occasions taking into account - whether the number of transmission occasions is dynamically indicated or higher layer configured. - n whether the selected RV sequence depends on the number of TCI state(s) indicated in the codepoint. - n whether channel estimation interpolation across mini-slots/slot with the same TCI index - LDPC base graph and TBS shall be same across repetition.

In our previous contributions [3, 4, 5, 6] we discussed the application of multi-dimensional (MD) modulation for URLLC and multi-TRP transmissions. A multi-transmission system based on MD can be implemented over time or spatial domains following one of the transmission schemes proposed for multi-TRP transmission and readily combined with NR HARQ process to improve the performance. Preliminary results in [3, 4, 5, 6] showed that we can achieve significant performance gains over basic repetition mechanism. In this contribution, we provide our updated link-level simulation results considering simulation parameters for multi-TRP based URLLC. Furthermore, we provide our views on joint ACK/NACK transmission.

# Transmission schemes

High-level descriptions of multi-TRP transmission schemes have been discussed and agreed in recent RAN1 meetings [1, 2, 7, 8, 9, 10] The aim of each scheme is to extract diversity gains, as the signals sent by different TRPs are subject to different channel conditions. The identified schemes assume different time-frequency resource allocation approaches to multiplex or superimpose signals from multiple TRPs. In this section, we provide our views on the different schemes and the application of MD modulation to improve the gains of multi-TRP transmissions.

Scheme 1 is a form of spatial division multiplexing, thus the same time-frequency resources are used for transmissions/repetitions. Scheme 1 could be more efficient in resource utilization. This is similar to multi-layer MIMO transmission but unlike in tradional approach the layers are transmitted from different TRPs. If the transmissions from TRPs are different, an MMSE-IRC receiver may be required to perform spatial separation of the received signal prior to the detection. The benefit over single TRP multi-layer transmission stems from the channel variations which are less correlated and provide more opportunities to enhance reception. As signals from all the TRPs are received simultaneously, no additional transmission delay is introduced compared to each TRP occupying one transmission slot after the other. Further this scheme does not demand additional spectrum compared to multiple TRPs occupying different frequency bands. One of the main drawbacks of Scheme 1 is that it requires tight coordination between multiple TRPs to ensure transmissions are synchronized and that they are scheduled on the same set of time-frequency resources. Also the performance gains will be highly sensitive to backhaul delay. As interlayer interference can reduce the performance gains, coordinating DMRSs to ensure accurate channel estimations at the UE will be required.

Scheme 2 is based on frequency division multiplexing (FDM); signals from multiple TRPs are sent on the same time instant but on different frequency resources. The bandwidth allocation for the transmission block could be contiguous or interleaved between repetitions. Different spectrum locations can provide additional diversity in frequency selective channels. Contrary to Scheme 1, the resource assignment is orthogonal which avoids the issue with interlayer interference. Scheduling is also flexible since different PRBs can be chosen for the different TRPs as long as they are not overlapping; however, it comes at the cost of taking more frequency-resources for transmission. The bandwidth per transmission does not change but the set of non-overlapped bandwidth regions used by multiple coordinated TRPs can occupy a larger part of the spectrum.

Scheme 3 relies on time division multiplexing (TDM). Multiple repetitions are sent by TRPs using different OFDM symbols within the same slot. For example, with two repetitions, OFDM symbols 1:N1 could be occupied by TRP1 and OFDM symbols N1+1:N2 are occupied by TRP2 where N2 is no greater than the number of OFDM symbols in a slot for data transmission. This scheme allows to complete transmitting small packets within a slot on orthogonal time resources. It benefits from additional diversity in time selective channels if the channel varies within a mini-slot occupied by each TRP. However, to harness time diversity, enhancement to DMRS design will be necessary such that channel variations within a slot are properly captured. Scenarios with small packet sizes and demanding short round trip delay are the most likely to benefit from this setup. Tight synchronization is needed between multi-TRPs to align transmission on non-overlapping OFDM symbols.

Scheme 4 is another TDM scheme in which multiple repetitions are sent by TRPs in different time slots. It is similar to Scheme 3 but with larger inter-repetition periodicity. The benefit of allowing larger periodicity is to still take advantage of time diversity in more slowly-varying fading channels. However, larger periodicity also affects the latency. To process received signals, UE needs to wait multiple slots durations to receive signals from all the TRPs. Similar to Scheme 2, it also requires more resources since each transmission requires its own slot with associated control information.

MD-based Transmissions

In summary, all schemes could benefit from the spatial diversity provided by multiple TRPs. However, we can further improve the performance of any of the above schemes by applying MD modulation. MD-based modulation offers additional gains over the simple repetition mechanism by changing the distance properties of the resulted moduclation scheme over multiple symbols. A multi-TRP system based on MD can be implemented over time or spatial domains. In a multi-TRP system, MD modulation transmission can be spanned over the available TRPs where each TRP transmits the payload according to its assigned configured modulation parameters.

MD modulation can be easily enabled by a simple modification of the constellation used by different TRPs. This could be achieved by using a lookup table or a similar mechanism along with the existing Rel-15 modulation procedure without a significant impact on specifications. The constellation choice could be semi-statically preconfigured at the gNB during initial access or it could be determined when multi-TRP transmission is activated. Alternatively, it could be dynamically signaled to each UE as part of the DCI scheduling the repetitions across multiple TRPs.

A brief discussion on how multi-dimensional modulation can be used to enhance the reliability of Rel-16 URLLC and multi-TRP transmissions is given in Appendix B

# Performance evaluation

In this section, we present our link-level evaluation results considering a 2-TRP system. We consider Schemes 2a, 3, and 4 and compare the BLER performance achievable by using 16QAM constellation from NR specifications for both TRPs against 16QAM 2-dimentional (2D) modulation. The 16QAM 2D modulation scheme we considered (see Figure 1) is designed such that TRP1 (primary TRP) uses Rel.15 16QAM constellation from NR specifications and TRP2 uses a modified version of it. With such a modulation design, multi-TRP transmission with multi-dimensional modulation can be implemented with minimal changes to the existing specifications and also helps multi-TRP to single TRP switching easily. Table 1 in Appendix A specifies the complete set of simulation parameters used in this contribution.

Figure 1: NR-16QAM based 2-dimensional modulation design

3.1 BLER with Ideal Channel Estimates

| Figure 2: BLER in TDL-C channels with ICE, Code rate = 0.44, 2-D modulation vs NR-NR repetition | Figure 3: BLER in TDL-C channels with RICE, Code rate = 0.12, 2-D modulation vs NR-NR repetition |
| --- | --- |

3.1 BLER with Real Channel Estimates

| Figure 4: BLER in TDL-C channels with RCE, Code rate = 0.44, 2-D modulation vs NR-NR repetition | Figure 5: BLER in TDL-C channels with RCE, Code rate = 0.12, 2-D modulation vs NR-NR repetition |
| --- | --- |

Figures 2-5 compare BLER performance of a 2 TRP system with NR-NR and 2D modulation. Both ideal channel estimates (ICE) and front loaded DMRS based real channel estimates (RCE) are considered. The baseline constellation is 16QAM, where for NR-NR case the Rel-15 NR constellation is used for both transmissions. However, for the case labelled 2D, the first transmission uses Rel-15 NR constellation definition, but the second transmission is based on the modified constellation as shown in Figure 1. As demonstrated in Figures 2 and 4, multi-TRP transmission using 2D modulation results in a clear and significant performance gain of around 0.8 dB compared to the NR-NR case at high coding rate (CR = 0.44 in our test setup). Figures 2 and 5 also show that considerable performance gains of around 0.2 dB can be achieved by 2D modulation over NR-NR case when the coding rate is low (CR = 0.12 in our test setup). As for RCE compared to ICE, a degradation of ~0.9 dB is observed which equally affects both 2D and NR-NR cases.

Observation 1: Multi-dimensional modulation achieves significant gain over all transmitters using the same modulation scheme.

Note that as mentioned previously, with 2-dimensional modulation used in the simulation, TRP1 still uses Rel-15 16QAM given by NR specifications. The only modification required at the transmitter is for TRP2 to use a different bits-to-constellation mapping to achieve this performance gain. Thus the performance gains observed in our simulation results could be obtained with very limitited impact on the existing specifications and transmitter complexity.

Observation 2: Multi-dimensional modulation can be implemented in multi-TRP transmission with very limited impact on the current specification.

Observation 3: Multi-dimensional modulation is applicable to the considered multi-TRP transmission schemes 2,3, and 4.

Based on the observations and the discussion, following proposal is made:

Proposal 1: RAN1 considers use of multi-dimensional modulation for URLLC and Multi-TRP transmissions.

# Joint ACK/NACK in M-DCI

In the last meeting, and the follow up email discussions, joint ACK/NACK transmission for M-DCI multi-TRP transmission was discussed. In an M-DCI based multi-TRP/panel transmission, a UE receives CORESET and PUCCH resource configuration per TRP. Then in each slot, the UE monitors the CORESET for each TRP to receive and decode DCIs for each TRP and determine K1 values (K1TRP1,K1TRP2). From the RRC configured time offset values for each TRP, the actual time offset are determined. Now, depending on the decoded K1 and their corresponding configured time offset values, the recommended PUCCH locations may not be same and the resulting ambiguity for joint transmission need to be resolved.

If the determined PDSCH to ACK/NACK transmission offset by each DCI points to a same PUCCH opportunity, then UE can multiplex and transmit both ACK/NACKs in that slot. However, as shown in Figure 6, if K1 values point to different slots for PUCCH transmission, then the ACK/NACK is transmitted according  to  PUCCH corresponding to the later timing offset.

Further, if K1 values are defined interms of minislots (especially applicable to transmission scheme 3 where multiple TPRs multiplex their transmissions over mini-slots) the same principle discussed could be used. In this case, time offselt could be defined in terms of number of mini-slots instead of number of slots.

Proposal 2: In M-DCI multi-TRP transmission, if the determined PDSCH to ACK/NACK transmission offsets are different, joint ACK/NACK to be transmitted according to the DCI with the later time offset.

Figure 6 – Joint ACK/NACK transmission

# Conclusions

In this contribution, we discussed the application of multi-dimensional modulation for URLLC and multi-TRP transmission. Based on the link-level simulation results, it was observed that multi-dimensional modulation provides significant performance gains compared to using the same modulation scheme by all TRPs. Also it has been observed that multi-dimensional modulation can be adapted in multi-TRP transmission without significant impact on the current NR specifications. Based on the discussion, and the simulation results provided, the following observations and proposals were made:

Observation 1: Multi-dimensional modulation achieves significant gain over all transmitters using the same modulation scheme.

Observation 2: Multi-dimensional modulation can be implemented in multi-TRP transmission with very limited impact on the current specification.

Observation 3: Multi-dimensional modulation is applicable to the considered multi-TRP transmission schemes 2,3, and 4.

Proposal 1: RAN1 considers use of multi-dimensional modulation for URLLC and Multi-TRP transmissions.

Proposal 2: In M-DCI multi-TRP transmission, if the determined PDSCH to ACK/NACK transmission offsets are different, joint ACK/NACK to be transmitted according to the DCI with the later time offset.

# References

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #97, Reno, USA, May 2019.

- Email discussion [97-NR-09]  “Details of different alternatives on URLLC for M-TRP”.

- R1-1900808, “Enhanced Reliability for Multi-TRP Transmission”, InterDigital Inc., 3GPP TSG RAN WG1 Ad-Hoc Meeting #1901, Taipei, Taiwan, January 2019.

- R1-1902614, “Performance Evaluation of Multi-TRP Schemes”, InterDigital Inc., 3GPP TSG RAN WG1 Meeting #96, Athens, Greece, February-March 2019.

- R1-1904860, “Link-level Evaluation of Multi-TRP Schemes”, InterDigital Inc., 3GPP TSG RAN WG1 Meeting #96b, Xi’an, China, April 2019.

- R1-1906860, “Link-level Evaluation for MTRP Transmission”, InterDigital Inc., 3GPP TSG RAN WG1 Meeting #97, Reno, USA, May 2019.

- Email discussion [96b-NR-06] “Simulation parameters for multi-TRP based URLLC scheduled by single DCI”.

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #96b, Xi’an, China, April 2019.

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #96, Athens, Greece, February 2019.

- Chairman’s Notes, 3GPP TSG RAN WG1 Ad-hoc Meeting #1901, Taipei, Taiwan, January 2019.

# Appendix A

- Table 1: Link-Level Simulations Assumptions

| Parameters | Value / Assumption |
| --- | --- |
| Carrier Frequency | 4 GHz |
| Waveform (data part) | CP-OFDM |
| Channel coding | NR-LDPC |
| Numerology (data part) | SCS = 15 kHz, #OS = 14 |
| Allocated bandwidth | 8 PRBs |
| Modulation | TRP1: NR-16 QAM, TRP2: modified 16 QAM modulation (See Figure 1) |
| TBS per UE | 16 bytes + CRC (Coding rate 0.12), 62 bytes + CRC (Coding rate 0.44) |
| BS antenna configuration | 4Tx |
| UE antenna configuration | 4 Rx |
| Transmitter Precoding | Precoder cycling (granularity = 2 PRBs) |
| Propagation channel & UE velocity | TDL-C 100 ns in TR38.901, 3km/h (11 Hz) |
| Max number of HARQ transmission | 1 |
| Channel estimation | Ideal channel estimation (ICE) and real channel estimates (RCE) |
| DMRS overhead | (3 dB power boosting), front loaded |
| Number of layers per TRP | 1 |
| Inter TRP frequency offset | 0 Hz |

# Appendix B

# Multi-Dimensional Modulation for Enhanced Reliability

For NR Rel-16, further enhancement for URLLC reliability/robustness with multi-TRP/panel/beam is considered. An efficient approach to improve the performance of a multi-transmission system can be based on multi-dimensional modulation (MD). MD offers additional gains over the simple repetition mechanism. A multi-transmission system based on MD can be implemented over time or spatial domains. In a multi-TRP system, multi-modulation transmission can be spanned over the available TRPs where each TRP transmits the payload according to its assigned configured modulation parameters.

B.1 Enhanced Reliability for URLLC

To enhance the reliability and robustness of the transmission, the overall transmission strategy should have a better leverage on diversity. The notion of diversity may include various transmission aspects such as spatial and time domain. However, simple spatial and time diversity alone may not be sufficient for achieving high reliability transmission.

Figure B.1 shows the basic repetition mechanism to enhance the transmission reliability where each re-transmission is a replica of the first transmission. In NR Rel-15, the repetition mechanism is implemented across several slots as shown in Figure B1. At the receiver, by combining received replicas of the original transmission, the demodulation and detection performance can be improved through SNR enhancement.

Figure B.1

To further augment the performance of the system, MD can be used where a different constellation mapping for each transmission is adopted. Figure B.1 shows implementation of an MD-based transmission where each bit group for symbol mapping is interleaved by a secondary interleaver per transmission. As a result, as shown in Figure B.2, transmission of each bit group is mapped on a different constellation point per transmission. The interleaver can be designed to enhance detection capability by maximizing distance metrics such as a vector distance, Euclidean distance, product distance, etc. As demonstrated in Figure B.2, for an exemplary transmission of “0011” bits, different 16QAM symbols are used to improve detection likelihood.

B.2 Enhanced Reliability by Multi-TRP

In a multi-TRP deployment, instead of time, multi-dimensional modulation is applied across the spatial domain. As such, interleaver definition, i.e., effective constellation is varied per TRP. Figure B.3 and B.4 show different examples of a multi-TRP transmission assuming orthogonal and non-orthogonal resources, respectively.  In either case, UE performs a joint demodulation and detection such as Maximum Likelihood detection (ML).

Figure B.2

x

Figure B.3

Figure B.4