3GPP TSG RAN WG1 Meeting #98  			                                                                 	R1-1908336

Prague, Czech Republic, August 26th – 30th, 2019

Agenda Item:	7.2.1.2.2

Source:	Charter Communications

Title:	On NR-U DL Signals and Channels

Document for:	Discussion/Approval

# Introduction

The NR-U WI is in progress in RAN1 [1]. In the RAN1 #97 meeting, the following agreement was reached on DL signals and channels [2]:

Agreement:

- Additional durations from 2 to 13 (other than 2, 4 and 7 which are already supported in Rel-15) for PDSCH mapping type B are supported.

  - FFS: A duration of 14

  - FFS: DMRS aspects for the additional durations (Reuse Rel-15 patterns or define new patterns for these additional durations)

- Capability signalling will be defined for UEs to indicate which specific subset of durations are supported by the UE.

- FFS: Some duration(s) are mandatory for the UE to support for initial access

- FFS: PDSCH processing times for the new durations (reuse the values for durations of 2/4/7 or define new ones)

- Note: PDCCH monitoring occasions will be discussed separately as part of the NR-U WI

- Note: This agreement does not by itself imply an increased requirement in PDSCH processing and decoding capabilities compared to Rel-15

This contribution discusses open issues as captured above and in [3].

# NR-U DL Signals and Channels

For convenience, our views on various issues are encapsulated in the following Table 1.

Table 1: Views on DL Signals and Channels

| Topic | Position | Comments |
| --- | --- | --- |
| PDSCH transmission | DMRS aspects: Reuse Rel-15 patterns as follows: durations 2,3 – DMRS for duration 2; durations 4,5,6 – DMRS for duration 4; durations 7,8,9,10,11 – DMRS for duration 7; durations 12,13 – Type A DMRS mapping Initial access: all possible PDSCH durations do not need to be mandatory for reception of msg2, msg4, msgB, PDSCH processing times: Reuse existing processing times. | The main motivations for all possible non-slot durations is to increase the number of DL starting points, and to better adapt to LTE spectrum sharing (if applicable). Since spectrum sharing and high mobility are both not applicable for NR-U, it is more efficient to reuse existing DMRS mappings. In terms of initial access, PDSCH durations of 2/4/7 already provide sufficient flexibility for DL transmission. The limiting factor is more likely to be the PDCCH monitoring flexibility. |
| DL burst detection | Based on wideband DM-RS. The detection of DM-RS triggers further processing of configured CORESETs. | Reduction of PDCCH monitoring led to 5%-85% gains in Rel-16 Power Saving. The implication is that wideband DM-RS is transmitted at least in every DL slot that contains PDCCH. |
| PDCCH monitoring | Phase A: Use Rel-15 configured periodicity Phase B: Based on GC-PDCCH Phase C: Based on DCI in either GC-PDCCH or UE-specific search space |  |
| DMRS sequence generation | Support time-independent initialization of DMRS for PDCCH and PDSCH |  |
| COT structure indication | Indicate at least the following in addition to SFI: - Duration of COT - Allowance of CG PUSCH | Occupied sub-bands can be deduced from wideband DM-RS, similar to single sub-band case. |
| CSI-RS | Increase opportunities for AP-CSI-RS to mitigate impact of DL LBT failure | This could be achieved via a separate AP-CSI-RS configuration, for example |
| Wideband operation | CORESET does not need to be configured within the bandwidth of sub-band LBT | Better to handle this in AI 7.2.2.2.5 |

# Summary

In this contribution we commented on DL signals and channels for NR-U. The corresponding proposals are captured in Table 1 of Section 2.

# References

- RP-182878, New WID on NR-based Access to Unlicensed Spectrum, RAN#82

- Chairman’s Notes, RAN1#97, May 2019.

- R1-1907705, Feature lead summary for NR-U DL Signals and Channels, Motorola Mobility, Lenovo