3GPP TSG RAN WG1 #98	                                                       R1-1908378

Prague, Czech, August 26th – 30th, 2019

Source: MediaTek Inc.

Title: CSI enhancement for MU-MIMO support

Agenda Item: 7.2.8.1

Document for:  Discussion

# Introduction

One remaining issue in MU-CSI is padding/segmentation schemes for NSB×R>13.

# Padding schemes

The following agreement was made online on UCI design:

Agreement

On the value of N3 for NSB×R>13:

- For Alt1 (padding), consider only extrapolation-based scheme and decide on the final specific design alternatives in RAN1#97 (Reno) for down selection in RAN1#98 (Prague)

Alt1 (padding) is described as follows:

Notation: - Ns: the number of FD compression units (derived from the number of CSI subbands and R) before padding. - N3: the smallest integer which is a multiple of 2, 3 or 5 and is larger than Ns. - {Xk, 1≤k≤Ns}: The precoding coefficient of the unit k in the original Ns units. - θ=N3-Ns: extrapolation length - Y0,…,Yθ-1: padding coefficients Procedure: ⌈N3-Ns2⌉ units are prepended to the original Ns units; and ⌊N3-Ns2⌋ units are appended to the original Ns units, resulting in: - When θ=3: [Y1, Y2, X1,X2,…,XNs, Y0 ] - When θ=2: [Y1, X1,X2,…,XNs, Y0 ] - When θ=1: [Y0 , X1,X2,…,XNs] Padding signal (extrapolation): - Alt 1.1: Average: Yt= (X1+X2+…+XNs)/Ns, t=0,1,…,θ-1 - Alt 1.2: Raised cosine extrapolation: Yt= αtXNs+1-αtX1, t=0,1,…,θ-1 - When θ=3: α0=0.8536, α1=0.5, α2=0.1464 - When θ=2: α0=0.75, α1=0.25 - When θ=1: α0=0.5 - Alt 1.3: Repetition: - If θ=3 or 2, Y0=XNs, Y1=Y2=X1 - If θ=1, Y0=X1 - Alt 1.4: Zero padding: Yt= 0, t=0,1,…,θ-1

Offline agreement: On Alt1 (padding) for N3, for evaluation purposes, select one of Alt1.1, 1.2, 1.3, and 1.4 in RAN1#98 (Prague) as described in the above table.

### Simulation evaluations

We focus on the scenarios below:

- R=1;

- 1 subband padding;

- 16 ports (N1=4, N2=2)

- L=4

- NSB×R=14, 17, and 19.

Performance with NSB×R=14

|  | 5-percentile throughput gain | 50-percentile throughput gain |
| --- | --- | --- |
| Non-padding | 0% | 0% |
| Alt. 1-1 | 4.9% | 1.2% |
| Alt. 1-2 | 0.1% | -0.6% |
| Alt. 1-3 | 1.7% | -1.2% |
| Alt. 1-4 | 0.2% | -3.7% |

Performance with NSB×R=17

|  | 5-percentile throughput gain | 50-percentile throughput gain |
| --- | --- | --- |
| Non-padding | 0% | 0% |
| Alt. 1-1 | 0% | 0% |
| Alt. 1-2 | -0.83% | 1.4% |
| Alt. 1-3 | -2.26% | -1.3% |
| Alt. 1-4 | -4.0% | -0.66% |

Performance with NSB×R=19

|  | 5-percentile throughput gain | 50-percentile throughput gain |
| --- | --- | --- |
| Non-padding | 0% | 0% |
| Alt. 1-1 | 1.2% | 1.0% |
| Alt. 1-2 | 1.5% | 0% |
| Alt. 1-3 | 0% | 1.0% |
| Alt. 1-4 | -2.65% | -1.0% |

From the evaluation, we can see Alt. 1-1 provides gains/no loss for the evaluated cases compared with no-padding.

Observation: Alt. 1-1 (average padding) provides gains/no loss compared with no-padding.

# Conclusion

In this contribution, we provide our view on the remaining issue concerning padding/segmentation. We have

Observation: Alt. 1-1 (average padding) provides gains/no loss compared with no-padding.