3GPP TSG RAN WG1 #97                                                            R1-1908385

Prague, Czech Republic, August 26th – 30th, 2019

Agenda item:	7.2.2.2.1

Source: 	MediaTek Inc.

Title: 	Discussion on Channel Access Procedure

Document for:	Discussion and decision

# Introduction

In this contribution, we share some observations on ETSI BRAN LBT regulation and further proposal on RACH, PUCCH, and PUSCH on top of what has been agreed.

# Discussion on LBT for LBE operation

## Cat2 LBT design

Three alternatives were captured in the previous RAN1 meeting for Cat2 LBT detailed design.

Agreement: (RAN1 #97) Select one of the following alternatives for Cat2 LBT in a 16 us gap. - Alt 1: The 16us measurement period is split into two slots with the first slot having a duration 7us and second slot having a duration of 9us. - Energy measurement is done in the 9us slot with the measurement including averaging for at least 4 us in any portion of the slot. LBT is said to be successful if the measured energy is lower than the ED threshold. - Alt 2: The 16us measurement period is split into two slots with the first slot having a duration 7us and second slot having a duration of 9us. - Energy measurement is done in both the 7us and 9us slot with the measurement including averaging for at least 4 us in any portion of each slot. LBT is said to be successful if the measured energy is lower than the ED threshold in both slots. - Alt 3: Energy measurement is done in any portion of the 16 us duration including averaging for at least 4 us. LBT is said to be successful if the measured energy is lower than the ED threshold.

Figure 1: Timing of competing ETSI devices in a 16us to 25us gap

If a devices of any type (LBE or FBE) is occupying the channel and then presents a gap of 25us or less, there is no utility in this devices performing a Clear Channel Assessment (CCA) on the Operating Channel during a single observation slot of 9us. The only other device that could feasibly gain clear access to the channel in this timeframe is a Frame Based Equipment (FBE) device with the highly improbable event that the Fixed Frame Period (FFP) of the FBE device aligns with this gap, made more improbable by the fact that coexisting FBE and LBE devices will inherently self-synchronize. Any other competing LBE devices will not have sufficient time to gain clean access to the channel. Only class 3 and class 4 LBE devices that draw q = 0 for their random back-off period could respond in time and these would subsequently collide with the device that previously had channel access. An ideal timing diagram of what CCA activity can occur in a 16us to 25us gap by competing ETSI devices is shown in Figure 1.

Having analysed the above, we think the chances may be slim that other devices than the initiating device and the responding device can perform LBT successfully within 16us to 25us gaps and grab the channel.

Observation 1: The chances may be slim that other devices than the initiating device and the responding device can occupy the channel by performing a successful LBT within 16us to 25us gaps.

Based on the above analysis, we think Alt 3 captured in RAN1 #97 meeting should be supported for Cat2 in a 16us gap which can leave the implementation details to the UE that needs to take processing time and timing advance into account.

Proposal 1: For Cat2 LBT in a 16us gap, support that energy measurement is done in any portion of the 16 us duration including averaging for at least 4 us. LBT is said to be successful if the measured energy is lower than the ED threshold.

## LBT for RACH Messages

Unlink in NR licensed spectrum, channel access to unlicensed spectrum is regulated by listen-before-talk (LBT) clear channel assessment. Due to LBT, the overall access latency is expected to increase in a RACH procedure. In order to mitigate the impact of LBT for a procedure, one straightforward solution is to reduce the number of steps in the procedure which results in the proposal of 2-step RACH. But since the 4-step RACH still serves as the fall-back mechanism for 2-step RACH and is the baseline for NR-U, enhancement on 4-step RACH for NR-U should be discussed. In this section, we share our views on channel access procedures for RACH messages in both 4-step RACH (Msg1, Msg2, Msg3 and Msg4) and 2-step RACH (MsgA and MsgB).

Based on regulation, a COT acquired by an initiating device can be shared with a responding device so that LBT requirement within a shared COT can be alleviated. Therefore, if two or more steps belonging to a same procedure (e.g. Msg1/2/3/4 in 4-step RACH) can be transmitted in the same COT, the overall LBT duration for the procedure is expected to be reduced compared with that when separate COTs are initiated for each step in the procedure. Hence, NR-U should strive to transmit two or more steps belonging to a same procedure in the same COT to reduce LBT overhead.

Observation 2: If two or more steps belonging to a same procedure (e.g. Msg1/2/3/4 in 4-step RACH) can be transmitted in the same COT, the overall LBT overhead in the procedure can be reduced.

Observation 3: Sharing an UE-initiated COT with gNB is supported by regulation.

Proposal 2: NR-U should strive to minimize the overall LBT overhead in a RACH procedure (instead of the LBT duration for an individual RACH message).

In a 4-step contention-based random access, after UE transmits Msg1 on PRACH, the UE has to start monitoring Msg2 (random access response, RAR). And when a UE receives a RAR that includes the preamble index that UE has selected for Msg1 transmission, the UE has to transmit Msg3 with its ID on the UL grant that is carried by RAR. Finally, the base station responds with Msg4 for the UE to carry out contention resolution. Note the timeline of messages in the 4-step RACH has been discussed in NR. Since gNB has higher computational capability and often demands much shorter processing time, the timing gap between an uplink transmission and a downlink reception (mainly determined by UE processing time) is often longer than the timing gap between a downlink signal reception and an uplink transmission (mainly determined by gNB processing time). For example, the timing gap between Msg1 transmission and Msg2 reception can be as short as one OFDM symbol while the timing gap between Msg2 reception and Msg3 transmission is N1+N2+0.5msec which is about 2.5msec and 1.5msec for SCS=15kHz and SCS=30kHz, respectively. Therefore, it is reasonable that NR-U should support that gNB can transmit RAR in the UE-initiated COT that has been acquired by UE(s) for PRACH transmission(s) when PRACH is not done within a gNB-initiated COT. Similarly, it would be beneficial in terms of the overall RACH latency and LBT overhead reduction if transmission of Msg4 in the same COT as Msg3 is supported in NR-U.

Observation 4: In NR, the timing gap between Msg1 transmission and Msg2 reception is much shorter than that between Msg2 reception and Msg3 transmission because gNB has higher computational capability. It is hence beneficial to support Msg2 transmission in the same COT as Msg1.

As illustrated in Figure 2, if PRACH is transmitted outside of gNB-initiated COTs, then we propose that the COT acquired by UE(s) can be shared with gNB for immediate random access response (RAR). Within the UE-initiated COT, gNB can apply one-shot LBT or even no-LBT for the RAR transmission.

Figure 2: UE(s) applies Category 4 LBT for PRACH transmission outside of gNB-initiated COTs. Then the COT can be shared with gNB for transmitting random access response (RAR) to enhance the 4-step RACH in NR-U.

Based on the above reasoning and observations, we have the following proposals.

Proposal 3: NR-U supports to share a UE-initiated COT with gNB for RACH message transmissions.

### CAPC determination for RACH in NR-U

3GPP 38.321 standards have introduced differentiated Random Access (RA) procedure, with two major priority classes:

- High Priority RA: RA initiated for

  - Beam failure recovery

  - Handover

- Low Priority RA: RA initiated for all other reasons

  - Initial Access

  - Timing Alignment (Out of Sync UE)

  - RRC Reconfiguration etc.

High priority random access procedure could be identified by configuring power ramping priority and the back off parameters, associated with the random access process. Following the standard guidelines, we can argue that RACH over an unlicensed carrier (in NR-U) could be of two major types:

- High priority – triggered by beam failure recovery or handover

- Low priority – triggered by initial access, timing alignment, RRC reconfiguration

Observation 5: Random Access over NR is classified into two major types: (1) High Priority (beam failure recovery, handover) and (2) low priority (initial access, timing alignment, and RRC reconfiguration).

Based on this observation we propose that CAPC for RACH messages in NR-U should be based on the purpose (reason) for RACH triggering. Subsequently, we propose to explore differentiated Random Access, mentioned in 3GPP 38.321 for determination of CAPC during Random Access in NR-U. High priority CAPC should be assigned for RACH triggered for beam failure recovery and handover. The CAPC for other reasons for RACH should be assigned with low priority. We assume a table, which maps RACH differentiation to different CAPC values. We assume that lower the CAPC, higher the priority.

- Such a table could be configured and signaled by RRC.

- Alternatively, it could be hardcoded and used in specifications

| Purpose for RACH | CAPC (for RACH) |
| --- | --- |
| Beam Failure Recovery | 1 (High Priority) |
| Handover |  |
| All other reasons for RACH | 2 (Low Priority) |

Table 1: CAPC Determination for RACH

Proposal 4: CAPC for RACH message should be based on the purpose for RACH.

Proposal 5: High priority CAPC should be chosen for Handover and Beam Failure Recovery and low priority for other use cases.

## LBT for PUCCH

Since both stand-alone and dual-connectivity are supported in NR-U, PUCCH needs to be supported on unlicensed bands. We hence need to decide what LBT category and channel access priority class (CAPC) that UE applies to PUCCH transmission.

Similar to the discussion on channel access procedure for RACH, we think one-shot or no-LBT can be applied to PUCCH when it is transmitted within a gNB-initiated COT. On the other hand, if it is transmitted outside of gNB-initiated COTs, it is beneficial to support both one-shot LBT and Cat.4 LBT for PUCCH transmission. When Cat.4 is applied, UE can share this UE-initiated COT with gNB. For example, if UE transmits PUCCH for Scheduling Request, then gNB can transmit PDCCH within this UE-initiated COT immediately after it detects the transmitted PUCCH for SR as illustrated in Figure 3. When gNB transmits PDCCH and/or PDSCH within a UE-initiated COT, no-LBT or one-shot LBT can be applied to reduce the overall latency and LBT overhead for a Scheduling Request procedure.

Proposal 6: Cat.4 LBT is applied by UE to initiate a COT for PUCCH transmission and this UE-initiated COT can be shared with gNB for downlink transmission. For the downlink transmission in UE-initiated COTs, LBT is decided by

- Cat.1 immediate transmission is applied when the gap from the end of the PUCCH transmission to the beginning of the downlink transmission is up to 16usec.

- Cat.2 LBT is applied when the gap from the end of the PUCCH transmission to the beginning of the downlink transmission is larger than 16usec but not more than 25usec.

- FFS the case when the gap is larger than 25use.

Figure 3: When Category 4 LBT is applied to PUCCH outside of gNB-initiated COTs, this UE-initiated COT can be shared with gNB to reduce LBT overhead and the overall latency for example for a SR procedure.

### LBT type for HARQ-A/N feedback

In RAN1#AH1901 meeting, there is a conclusion was made for that gNB requesting HARQ-ACK feedback outside of the gNB-initiated COT by PDSCH-to-HARQ-timing-indicator can be supported in NR-U without enhancement to NR Rel-15. However, RAN1 should further discuss the details for determining the LBT type for the UE transmission of a HARQ-ACK feedback falling outside of the gNB-initiated COT. In our views, if a HARQ-ACK feedback for PDSCH(s) is scheduled outside of the gNB-initiated COT, the LBT type could be indicated by the gNB through the DCI(s) scheduling the PDSCH(s), or determined by the UE based on the indicated channel occupancy information, or both.

Proposal 7: To determine the LBT type for the UE transmission of a HARQ-A/N feedback falling outside of the gNB-initiated COT, the following mechanisms could be considered:

- Indication by the gNB through the DCI(s) scheduling the PDSCH(s)

- UE determination based on the indicated channel occupancy information

- Both of the above

### CAPC determination for Scheduling Request

Category 4 LBT requires determination of CAPC, where lower CAPC values reflect higher priority. We propose multiple options for estimating CAPC values corresponding to SR. We prefer gNB to explicitly configure the CAPC values for SR. However, UE can use UL Logical Channel Priorities to estimate the CAPC values. As a third alternative, uplink QCI could also be used to estimate the CAPC of SR in NR-U. If UL logical channel priorities or UL QCI are used, a table mapping the UL LCP priorities or UL QCI to CAPC need to be configured and signaled by RRC. Alternatively, the table could be hardcoded and used in specifications

Proposal 8: CAPC for UL SR could be based on multiple options:

- Network (gNB) can explicitly configure CAPC for UL SR transmissions.

- UE can map UL Logical channel priority values to CAPC for SR.

- Alternatively, UE can use UL QCI to determine the CAPC for SR.

## CAPC Determination for PUSCH

In LTE LAA, generally speaking, there are two mechanisms that can be used by the network to control the CAPC to be used by the UE for uplink transmissions. For dynamic scheduling, the UL grant issued by the eNB indicates both the type of LBT (Type 1 or Type2) as well as the CAPC (in case of Type 1) that is to be used by the UE for PUSCH transmission.

On the other hand, for configured grants, i.e., Type 1 autonomous uplink (AUL) channel access, the eNB signals the CAPC for each logical channel, and the UE selects the highest CAPC of the logical channel with data in the MAC PDU formed after executing LCP.

Observation 6: In LTE LAA, the CAPC used by the UE is either explicitly provided by the eNB (dynamic scheduling) or derived from a mapping from logical channels to CAPC (AUL).

For dynamic scheduling, the eNB is expected to choose a suitable CAPC based on the latest BSR and received uplink traffic, as mandated by the following excerpt from 3GPP TS36.300:

For uplink LAA operation, the eNB shall not schedule the UE more subframes than the minimum necessary to transmit all the traffic corresponding to the selected Channel Access Priority Class or lower (i.e., with a lower number in the Table 5.7.1-1), than the:

- Channel Access Priority Class signaled in UL grant based on the latest BSR and received uplink traffic from the UE if type 1 uplink channel access procedure (see section 15.2.1.1 of [6]) is signalled to the UE;

In our opinion, this approach for choosing CAPC suffers from several shortcomings. It is possible that the eNB does not have an accurate picture of the buffer status of the UE, and may not be fully able to predict the QoS class of the data that is eventually transmitted over unlicensed carrier. For example, current LCP rules do not always result in the most QoS sensitive data to be selected for transmission (In order to ensure fairness, the LCP mechanism prevents higher priority logical channels from exhausting every grant from the eNB based on PBR and BSD parameters). Also, the QoS sensitive data may have been sent over licensed carrier before LBT is successful, leaving relatively QoS tolerant data for transmission over unlicensed carrier. QoS sensitive data is also more likely to have been discarded as a result of PDCP discard. In these cases the UE may end up using high priority CAPC for sending low priority data, which is not desirable. Similarly, it is possible that new data arrives after the UE sends an SR but before it receives a grant. If the new data happens be of high priority, and the grant indicates a low priority CAPC, then the UE is forced to contend for channel access with low priority defeating the purpose of QoS differentiation afforded by multiple CAPCs.

Observation 7: The current mechanism where the eNB selects CAPC with dynamic grants prevent the UE from choosing the appropriate uplink LBT CAPC that is consistent with the QoS requirements of the data that is actually transmitted.

The current mechanism also imposes an unnecessary computation burden on the eNB in that it needs to keep track of BSR and uplink received data for purposes of CAPC calculation. Finally, with AUL, there is already a mechanism that relies on the UE to select the correct CAPC value based on the contents of the data to be transmitted. This mechanism is completely under network control since it is the network that maps logical channels to CAPC values. For these reasons, it seems that there is some benefit in considering a CAPC determination mechanism at the UE that is common for dynamic and configured grants.

Proposal 9: NR-U should consider developing a common CAPC selection mechanism for uplink dynamic and configured grants based on a mapping from logical channels to CAPC.

# ETSI BRAN LBT regulations

In the ETSI BRAN regulations [1] there are two methods of adaptivity (channel access mechanism) specified, one of which is Load Based Equipment (LBE) and the other of which is Frame Based Equipment (FBE).

## ETSI Load Based Equipment (LBE) Adaptivity

It should be noted that ETSI has left the Adaptivity (Channel Access Mechanism) procedure unspecified for Load Based Equipment (LBE) initiating devices that have transmission gaps of greater than 25us. The relevant text for this is copied below.

If a Channel Occupancy consists of more than one transmission the transmissions may be separated by gaps. The Channel Occupancy Time is the total duration of all transmissions and all gaps of 25 μs duration or less within a Channel Occupancy and shall not exceed the maximum Channel Occupancy Time in table 7 and table 8. The duration from the start of the first transmission within a Channel Occupancy until the end of the last transmission in that same Channel Occupancy shall not exceed 20 ms.

…

The Channel Access Engine can have multiple transmissions without performing an additional CCA on this Operating Channel providing the gap in between such transmissions does not exceed 16 μs. Otherwise, if this gap exceeds 16 μs and does not exceed 25 μs, the Initiating Device may continue transmissions provided that no energy was detected with a level above the ED threshold defined in clause 4.2.7.3.2.5 for a duration of one Observation Slot.

…

The Responding Device that does not proceed with such transmissions within 16 μs after the last transmission from the Initiating Device that issued the grant, shall perform a Clear Channel Assessment (CCA) on the Operating Channel during a single observation slot within a 25 μs period ending immediately before the granted transmission time. If energy was detected with a level above the ED Threshold defined in clause 4.2.7.3.2.5, the Responding Device shall proceed with step 3). Otherwise, the Responding Device shall proceed with step 2).

It should be further noted that there could be more than one reason why an ETSI LBE initiating device might pause for more than 25us, but one such reason could simply be that the responding devices for which the pause was designated simply did not respond. Another reason for gaps exceeding 25us could be that the initiating devices has no data to send but does, however, have disperse control frames that need to be sent at specific points in time.

One could assume that initiating devices are allowed to proceed in the same manner as responding devices in this respect and perform a Clear Channel Assessment (CCA) on the Operating Channel during a single observation slot within a 25 μs period ending immediately before the resumed transmission time. However, since this is not actually specified by ETSI, it would be unwise to proceed with this assumption. The other assumption is that the initiating device is allowed to resume transmission after a successful category-4 Listen-Before-Talk (cat-4 LBT) procedure. But this is equivalent to starting a new COT, which would be the prudent approach if cat-4 LBT is mandated.

On the other hand, ETSI has provisioned for extending COT sizes for ETSI class 1 and class 2 LBE devices. The text for this is copied here:

The maximum Channel Occupancy Time (COT) of 6 ms may be increased to 8 ms by inserting one or more pauses. The minimum duration of a pause shall be 100 μs. The maximum duration (Channel Occupancy) before including any such pause shall be 6 ms. Pause duration is not included in the channel occupancy time.

ETSI does not, however, provide a procedure on how to recommence transmission after inserting at least a 100us pause.

Observation 8: ETSI has not specified how initiating devices should handle gaps in its own transmission that are larger than 25us.

Proposal 10: Draft a Liaison statement to ETSI BRAN to seek clarification on how initiating devices should handle gaps in its own transmission that are larger than 25us or draft a change request to ETSI EN 301 893 to specifically cover this missing case.

### Greedy source traffic model (full buffer)

In Figure 5 we show the relative device utilizations of a channel in which these devices are competing for the same channel. Utilization here is defined as the amount of time a device gained exclusive access to the shared channel resource as a fraction of the total simulation time. This provides statistical probabilities of gaining access that also take into account the length of time allotted after gaining clear access. The traffic model for each device is assumed fully loaded with data always available to be sent. In all cases one of the devices is an LBE class 1 device. The relative utilization between ETSI LBE class 4 or class 3 and class 1 devices in Figure 5 demonstrate that ETSI LBE class 1 cannot expect to compete with these devices. In the IEEE 802.11 specifications, the equivalent class is referred to as the Background (BK) access class. When specifying what channels and signals use ETSI class 1, 3GPP should be cognizant of the fact that ETSI LBE class 1 was targeted for background low priority data with no reasonable expectation of timely delivery if competing with any other LBE device of a higher priority class.

It should not come as a surprise that ETSI LBE class 1 devices perform as they do if one considers the ideal timing of this class relative to any other ETSI LBE class. This can be seen in Figure 4.

Figure 4: Ideal timing of all ETSI LBE priority class devices' access engines

The first two dotted vertical red lines counting from the left in the timing diagram of Figure 4 show the average and worst case delay of an ETSI LBE class 4 access engine before assuming clear access to the shared channel when taking account of enforced pause, prioritization period and random back-off procedure period. It should be noted that both these lines fall well within the prioritization period of an ETSI LBE class 1 device. This ensures that the ETSI LBE class 1 device never completes its prioritization period and therefore never gets to decrement its random back-off counter, leaving it with zero channel utilization. The third vertical dotted red line shows the worst case ETSI LBE class 3 timing which only just overlaps with the ETSI LBE class 1 device’s prioritization period, giving it just one opportunity to decrement its random back-off counter, but only when an ETSI LBE class 3 device draws its largest back-off value, which will only occur 1 in 8 or 12.5% of the time. Couple these two probabilities together using the average random back-off count value for ETSI LBE class 1 of 1 in 7.5 and one gets an approximate chance of 1/8*1/7.5 = 1.67% of winning, before taking account of collisions, CCA times and COT time differences.

Only, in the rare event that exponential back-off takes an ETSI LBE class 3 or ETSI LBE class 4 device to a contention window (CW) of equal value to an ETSI_LBE class 1 device will the ETSI class 1 device gain some tiny proportion of the utilization (less that 0.01%). Note that this will only occur if the ETSI LBE class 3 or ETSI LBE class 4 device is contending and more importantly colliding with another device of similar priority.

Figure 5: ETSI LBE class 1 utilization relative to other ETSI LBE classes, all devices acting as greedy sources.

All simulation parameters used to generate Figure 5 and all subsequent figures are detailed in Table 2.

Table 2: Simulation parameters

| Class | p0 | CWmin | CWmax | Maximum COT |
| --- | --- | --- | --- | --- |
| 4 | 1 | 3 | 7 | 2ms |
| 3 | 1 | 7 | 15 | 4ms |
| 2 | 3 | 15 | 63 | 6ms |
| 1 | 7 | 15 | 1023 | 6ms |
| CCA slot [9us] fraction |  |  |  | 4us |
| Traffic model |  |  |  | Full buffer |
| Exponential back-off |  |  |  | Enabled |
| Simulation length |  |  |  | 20 000 000 000us |
| Timing |  |  |  | Ideal |

The use of the “full buffer” traffic model can be justified in this instance as we are trying to establish theoretical statistically fair use specifically in a coexistence scenario. There would be no point in including time periods where competing devices were not actually competing because they had no data to send, which would be the case for more real world traffic models.

A fractional period of 4us of a 9us CCA observation slot was used to ascertain if the channel was free, since both IEEE 802.11 and 3GPP LAA nominally use 4us for this. It is unclear what the ETSI BRAN expectation of this should be. Since ideal timing was used, this will only affect collision measurements between ETSI LBE and ETSI FBE devices. Ideal timing implies no propagation delays, no processing delays and no observation slot timing differences were modelled. Modelling non-ideal time will result in slightly increased collisions.

If any collisions were detected amongst competing devices, the channel time for those collisions was not counted towards the utilization totals for the colliding devices in Figure 5, while any non-colliding portions of a collided COT were counted towards devices’ totals. In Figure 5 the different shades of the same color depict this where the brighter color depicts the total accumulated time of COTs that did not collide at all while the darker shade of color depicts the total accumulated time of these non-collided COT portions. No assumptions were made about the ability to decode these collided COTs. The simulation is a measure of “exclusive use allocation” of channel resource time, without being technology specific about how well this time was used by any one device.

Observation 9: ETSI LBE class 1 devices should not have any reasonable expectation of channel utilization when any other higher priority LBE devices are competing for the same channel, including access engines enabled within the same device.

Proposal 11: When specifying what NR-U channels or signals should use ETSI LBE class 1, 3GPP should not have any expectation of timely delivery of these channels or signals. FFS – which channels or signals can tolerate this level of uncertainty?

Building on the results from Figure 5 and Observation 9 we now show results for neighboring ETSI LBE device classes competing for access to the channel. We place two devices in a simulation with differing priority class where the devices differ by just one level of access class, i.e. class 1 vs. class 2, class 2 vs. class3 and class 3 vs. class 4. We can see from Figure 6 that lower priority class devices are being locked out of the channel by higher priority devices by significant proportions. The same use of color sharing to depict collision free COTs and on non-collided COT portions as described in Figure 5 is again used here. If we consider that these competing devices are likely from different competing networks there is a considerable advantage to using a higher priority class to send data (even when accounting for the shortened COT lengths). This could be construed as unfair coexistence.

Figure 6: Utilization of a single ETSI LBE device when competing with a single ETSI LBE device of neighboring higher priority class with all devices acting as greedy sources.

Observation 10: A single higher priority access class device has an unfair advantage over a single lower priority access class device. The unfairness grows as the priority class difference grows.

To extend the results from Figure 6 we next look at larger device collections of neighboring priority classes. We do this to see if the unfairness shown in Figure 6 is perpetuated to larger numbers of devices or if the situation gets better or worse for the device set under consideration. In this simulation setup an equal number of devices of two different but neighboring priority classes are placed to compete for the channel. The size of the set is swept from a single device of each priority class (representing the same results depicted in Figure 6) through to a total of 64 devices of each priority class, making a combined simulation set of 128 devices. The aggregate channel utilization of each priority class type is measured for each set of device classes

What the results show is that the unfairness of the higher priority device depicted in Figure 6 is considerably magnified with increasing set size, with the higher priority device class set almost completely dominating the channel. What is also telling though is that the aggregate utilization of the combined set of both device classes in every case asymptotically approaches a total utilization of far less than 50%. This is demonstrated by the aggregate collisions extending well beyond 50%, and in the case class 2 competing with class 3 devices approaching almost 80% in collisions. This is an extraordinary price to pay for priority. The waste of precious bandwidth through useless collisions is far greater than the benefit of priority.

Observation 11: A collection of higher priority access class devices has complete dominance in the channel over an equal sized collection of lower priority access class devices. The unfairness grows wider as the size of the device set grows.

Figure 7: Aggregate utilization of a collection of ETSI LBE devices of one class competing with a collection of ETSI LBE devices of neighboring higher priority class with all devices acting as greedy sources.

It is clear from Figure 7 that a collection of ETSI LBE higher priority class devices has a detrimental effect on an equal sized collection of lower priority class devices. This is expected, given the differing priority, what is unexpected is the severity of the higher priority devices’ ability to curtail access to the channel by the lower priority devices as the collection size grows. What is also likewise unexpected is the amount of the channel that is lost to collisions as the collection of devices grows.

Next we explore what effect just a single ETSI LBE higher priority device has on the ability of a collection of lower priority class devices’ aggregate ability to gain access to the channel. We do this for a varying set size of ETSI LBE class 2 devices competing against just a single ETSI LBE class 3 device and then again for a single ETSI LBE class 4 device. It can be seen from Figure 8 that even a single device of higher priority class is able to dominate the channel even when competing against a significantly larger set of lower priority devices where its aggregate utilization of the entire lower priority set never breaches the utilization of the single higher priority device. Again, the aggregate utilization of the combined set of both device classes in every case asymptotically approaches a total utilization of far less than 50%, as demonstrated by the aggregate collisions extending well beyond 50%.

Figure 8: Channel utilization for a collection of 1 to 128 competing ETSI LBE class 2 devices competing against a single ETSI LBE class device of higher priority class (class 3 or class 4) with all devices acting as greedy sources.

In IEEE 802.11 specifications there is a notion of what type of traffic should be allowed to use different access priorities. In IEEE 802.11 these are, in ascending order of priority, BK (background, equivalent to class 1), BE (best-effort, equivalent to class 2), VI (video, equivalent to class 3) and VO (voice, equivalent to class 4). In 3GPP and in ETSI EN 301 893 there is no such notion of traffic type to priority class mapping. As a result, over time, this could be abused to gain unfair access to the channel in light of the results shown in Figure 1, Figure 5, Figure 6, Figure 7 and Figure 8.

Observation 12: There is no ETSI restriction on which ETSI priority class to use for what traffic.

Next we look at what sort of theoretical channel utilization to expect from a large collection of simultaneously competing ETSI LBE devices of a particular class. The results of this can be seen in Figure 9. In these simulations devices of just one particular type are placed in competition with each other and then the aggregate channel utilization is measured. No other devices, even of differing class, are introduced into the mix. From Figure 9 we can see that for all but ETSI LBE class 1 device collections, the channel utilization becomes very poor, even for relatively small device collections, with ETSI LBE class 3 and class 4 device collections dropping to 30% utilization for a collection of more than 20 devices. Even ETSI LBE class 1 eventually converges on a more than 50% collision rate (less than 50% utilization).

Figure 9: Channel utilization for a collection of 1 to 128 competing ETSI LBE devices of various classes with all devices acting as greedy sources.

Observation 13: ETSI BRAN defined LBE adaptivity results in excessive waste of precious bandwidth due to excessive collisions with competing devices of the same or different device class.

Proposal 12: Research alternative adaptivity mechanisms that are less wasteful of bandwidth when considering the 6GHz spectrum allocations.

This channel bandwidth waste comes at a bigger, but hidden cost of delay to a successful channel access. The collision rate in Figure 9 is so severe that is actually unnecessarily lengthens the periods between successful accesses to the competing channel, thus ironically delaying data that is meant to be prioritized. This is demonstrated in Figure 10 for ETSI LBE class 2 devices. For each device in the simulated collection of Figure 9 a histogram of delay between successful (non-collided) access attempts was kept. All histograms from all devices in the collection were collated and plotted in Figure 10. It should be noted that the x-axis of Figure 10 is on a log scale to emphasize the delay. It shows quite clearly that the delay spread is markedly affected by multiple decades of increase during periods of congestion that are disproportionate compared to the collection size. The average delay between successful access attempts of the same ETSI LBE class 2 device is an extraordinary 6.15s for 128 competing ETSI LBE class 2 devices, with a worst case observed delay of 49.67s.

Figure 10: Cumulative distribution of channel access delays for 1 to 128 ETSI class 2 devices

Observation 14: ETSI BRAN defined LBE adaptivity results in excessive access delay due to excessive collisions with competing devices of the same device class.

Proposal 13: Research alternative adaptivity mechanisms that limit excessive average delay when considering the 6GHz spectrum allocations.

### Poisson traffic

We now briefly turn our attention to Poisson distributed traffic models where we simulate the identical case to those presented in Figure 7 except the greedy source traffic model is substituted for a Poisson traffic model. All other simulation assumptions listed in Table 2 are the same as before. We choose two different average data packet arrival times for the Poisson distributed traffic model, the first is a 20ms average arrival time while the second is a 100ms average arrival time. The same average data arrival time is applied to all devices in the simulation. For reference the same greedy source plots from Figure 7 are included on these Poisson traffic plots where the average data packet arrival time is depicted as 0ms, implying that the data arrival rate in infinite and as such data is constantly arriving and the buffers are always running full. The data packet is assumed sufficiently large to fill an entire COT of the relevant ETSI LBE priority class.

Figure 11: Aggregate utilization of a collection of ETSI LBE devices of one priority class competing with a collection of ETSI LBE devices of neighboring higher priority class with all devices having data packet arrival times following a Poisson distribution all with the same average arrival times.

Observation 15: Poisson distributed traffic models behave similar to the greedy source traffic model and eventually, with enough device load, converge on the same utilization exhibited by a greedy source traffic model.

Lastly we try to show what we believe to be sub-optimal with the current adaptivity procedure as defined in ETSI EN 301 893. In this last simulation set we simulate a collection of 16 devices of either ETSI LBE class 2 or ETSI LBE class 3 device priorities and we then vary the data packet arrival rate of the Poisson distribution from 1 packet/sec up to some maximum where convergence to a steady-state of full-buffer is achieved. The packet data arrival rate in simply the inverse of the average data packet arrival time from the previous results and discussions relating to Figure 11.

It can be seen from Figure 12 that adaptivity as defined by ETSI in EN 301 893 holds relatively steady as the channel load rises due to increased data packet arrival rates, until is peaks at some maximum where the channel reaches capacity, but then worryingly it proceeds to drop from the maximum achieved capacity and eventually asymptotically converges on an aggregate channel utilization well below the peak. This same steady-state asymptotic value can be read off Figure 9 for the 16 device count case and shows that at this rate of data packet arrival and all higher rates, the devices have effectively become greedy sources with data arriving faster than it can be sent over the channel.

Figure 12: Aggregate utilization of a collection of 16 ETSI LBE devices of one priority class all having the same data packet arrival rate for a Poisson distributed traffic model.

The two worrisome aspects of Figure 12 are that the peak is muted relative what can be achieved with changes in adaptivity and that there is a precipitous drop from this peak that varies considerably based on ETSI LBE priority class, device count and average data packet arrival rate.

Observation 16: Adaptivity as defined by ETSI in EN 301 893 is sub-optimal when one considers the cases where the channel is close to reaching capacity through to the case where the channel is congested with all competing devices having a full data buffer. It is exactly this region where adaptivity should be saving the channel from collapse where in fact it is partly responsible for such a collapse.

Observation 10: Coexistence is only possible today due to traffic patterns not fully occupying buffers. Currently we rely on the aggregate effect of the underlying adaptivity probabilities convolved with the traffic model probabilities. But there is no ETSI restriction on what class to use for which traffic and there is no ETSI restriction on traffic patterns (except in the case of short control signaling). This is unsustainable. Future, more data hungry, traffic patterns will render adaptivity ineffective. We are already seeing this in deployments. We cannot predict the future, but we can regulate for better adaptivity.

Proposal 14: Research alternative adaptivity mechanisms that prevent precipitous channel collapse around the region of congestion when considering the 6GHz spectrum allocations.

We shouldn’t consider ETSI EN 301 893 defined adaptivity as completely hopeless or broken and should at least acknowledge credit that the currently defined procedures does hold the channel up through periods of light congestion. The alternative, although not recommended, would be to not implement any form of adaptivity and allow devices to simply transmit whenever they had data to send without any regard for other devices that may be doing the same. We show this in Figure 13 compared against the same plot of ETSI LBE class 3 from Figure 11. An equal sized collection of devices with a Poisson distributed traffic model that are not following any LBT procedures clearly do not perform as well as those following ETSI EN 301 893 adaptivity procedures. Interestingly the resulting aggregate utilization for the no-LBT case appears to follow a gamma distribution.

Figure 13: Aggregate utilization of a collection of 16 ETSI LBE class 3 devices compared to the same collection not following any LBT procedures, with all devices in both simulations having the same data packet arrival rate for a Poisson distributed traffic model.

Observation 17: Adaptivity as defined by ETSI in EN 301 893 is a good starting base for adaptivity to be defined in the 6GHz spectrum, but it does need some changes to counteract poor channel utilization during times of congestion and unfair access of higher priority classes. It is unlikely that these changes would result in something that is backwards compatible to EN 301 893 so any first draft of ETSI EN 303 687 should also include these changes.

Consider the case of ETSI LBE class 3 priority class utilization as shown in Figure 12 and Figure 13. If the aggregate utilization were to be maintained at its peak of approximately 80% even through a congested channel, rather than precipitously dropping down to less than 40% utilization, this would be equivalent to adding a whole new tranche of spectrum of equal size to what is already under consideration in 2.4GHz, 5GHz and 6GHz. This should be an attractive prospect to both regulators and all vested SDOs.

## NR-U Load Based Equipment (LBE) with Symbol Aligned Adaptivity

It has been discussed offline whether NR-U should adjust its adaptivity alignment such that the end of any LBT procedure aligns with a certain symbol boundary. This implies in most cases inserting a longer than required by ETSI regulations pause before progressing with its prioritization period followed by its random back-off period. The length of the additional pause will depend both on the sub-carrier spacing (SCS) and the desired symbol boundary to align to. Suggestions of single-symbol, two-symbol and four-symbol alignment have been discussed.

The rationale for doing such alignment would be to avoid the use of any length reservation signal after a successful LBT procedure but before any transmissions started on the COT. In Figure 14 we present the utilization results of such a symbol-aligned device competing with another like-for-like ETSI LBE device that was not undergoing NR-U symbol alignment, such as an IEEE 802.11 device. The legend describes the ETSI LBE class of device as well as the number of symbols it aligned to. For example LBE #3,2 implies an ETSI LBE class 3 device that aligned to NR-U 2-symbol boundaries at the relevant SCS. The SCS was swept from 15kHz to 60kHz and is shown on the x-axis.

Figure 14: Channel utilization of an ETSI LBE device with NR-U symbol alignment

For equally fair utilization when competing with just one other device of the same class, one should expect exactly 50% utilization. We can observe from Figure 14 that utilization is somewhat acceptable, where we get approximately 40% utilization, for ETSI LBE class 1 and class 2 devices with NR-U single-symbol alignment provided we do not use 15kHz SCS. Likewise, for ETSI LBE class 1 and class 2 devices with NR-U two-symbol alignment provided we only use 60kHz SCS. All other cases show drastically reduced utilization numbers, with the extreme left lower corner showing < 5% channel utilization for ETSI LBE class 4 devices with NR-U single-symbol alignment. The situation is significantly worse for 2- and 4- symbol alignment.

Observation 18: An ETSI LBE device with NR-U symbol alignment does not perform well against an ETSI LBE device, such as an IEEE 802.11 device, as defined in the ETSI BRAN regulations.

Proposal 15: Start the LBT procedure as per the ETSI BRAN regulations and use a cyclic prefix extension of fractional symbol length, based on active SCS, to bring the COT to symbol alignment. The maximum length of such an extension signal should be less than 78.428us.

## ETSI Frame Based Equipment (FBE) Adaptivity

It has been proposed by many companies that ETSI FBE should only be used in one of two scenarios, either environments where there would be reasonable expectation of isolation from other competing equipment, for example a factory floor, or in new greenfield spectrum not yet deployed for use by any technology, for example the 6GHz spectrum. The rationale behind the arguments has always been that ETSI FBE devices do not coexist well with ETSI LBE devices in that ETSI LBE devices will always dominate. However, with judicious choice of a Fixed Frame Period for ETSI FBE equipment, it is entirely possible for an ETSI FBE device to gain reasonable utilization of a channel when competing with an ETSI LBE device of any class. For example, a choice of Fixed Frame Period (FFP) of 2007us achieves utilizations as shown in Figure 15.

Figure 15: ETSI FBE utilization relative to competing ETSI LBE device classes

Observation 19: An ETSI FBE device can coexist with ETSI LBE devices

ETSI has defined ETSI FBE devices in such a manner that greedy source ETSI FBE devices cannot coexist with each other without either completely colliding with each other all of the time or one ETSI FBE device always completely dominating over the other ETSI FBE devices. This is under the assumption that they share the same FFP. If they do not share a common FFP, then the ETSI FBE device with the smaller FFP will always dominate at a ratio of approximately 90.5% : 4.5%. The only way that ETSI FBE devices can coexist in the regulations as they are currently written is if all ETSI FBE devices coordinate with each other and share out the channel resource in some form of fair sharing scheme, such as a round-robin scheme. An open question would be then whether this coordination is considered a single network, or disjoint networks that are coordinated in the interests of coexistence.

Observation 20: An ETSI FBE device cannot coexist with another ETSI FBE device unless they coordinate with each other when they grab access to the channel in some sort of supervisory manner. Inherent in this observation is that they already align their Fixed Frame Periods (FFPs).

One way for ETSI FBE devices to coexist is to follow a random procedure similar to ETSI LBE, except in this instance within a defined FFP whereby on a COT-by-COT basis an FBE device would be allowed to start its COT at any 9us observation slot offset within its FFP (currently ETSI EN 301 893 mandates that it shall always start its COT at the beginning of its FPP) provided it found the channel idle in the 9us observation slot immediately prior to it starting its transmission in a COT. The 100us minimum or 5% of the FPP time left idle would then account for the sum of the idle periods both at the start and at the end of the FFP (currently ETSI EN 301 893 mandates that the full idle period is always at the end of the FPP). This would add some random variability to the FBE COT and therefore allow other competing FBE devices a chance to gain access sooner if they drew a higher value for their observation slot number. What ths effectively does it randomly “float” the COT within the FFP. The number of 9us observation slots would obviously be bounded by the minimum 100us or 5% idle time occupancy, whichever is the greater. A timing diagram of this floating-FBE proposal can be seen in Figure 16.

Proposal 16: Draft a Liaison statement to ETSI BRAN to suggest a change to the adaptivity for FBE devices such that they can fairly coexist with each other without coordination. FFS – what sort of adaptivity for FBE results in fair use.

Figure 16: Floating-FBE proposal.

With some small changes to ETSI FBE adaptivity (that does not include coordination) it is possible to achieve much higher levels of utilization for FBE than even ETSI LBE class 1, previously shown in Figure 9. The same data of Figure 9 is replotted in Figure 17 with a modified ETSI FBE adaptivity utilization added. It shows that this modified ETSI FBE adaptivity converges on an aggregate utilization above 80%.

Figure 17: Channel utilization for a collection of 1 to 128 competing ETSI FBE devices, compared to ETSI LBE devices of various classes

Figure 18: Cumulative distribution of channel access delays for 1 to 128 modified ETSI FBE devices

To complete the comparison, the same cumulative distribution of delay spread that was shown for ETSI LBE class 2 in Figure 10 is shown for this modified ETSI FBE adaptivity in Figure 18. Figure 18 clearly shows a more favorable delay spread than that of Figure 10, even in very congested scenarios. The average delay between successful access attempts of the same ETSI LBE class 2 device is 912ms for 128 competing ETSI LBE class 2 devices, with a worst case observed delay of 18.6s.

Observation 21: Modified ETSI FBE can achieve extremely good channel utilization and delay spread, even without coordination.

# Conclusion

In this contribution, we have observed the following.

Observation 1: The chances may be slim that other devices than the initiating device and the responding device can occupy the channel by performing a successful LBT within 16us to 25us gaps.

Observation 2: If two or more steps belonging to a same procedure (e.g. Msg1/2/3/4 in 4-step RACH) can be transmitted in the same COT, the overall LBT overhead in the procedure can be reduced.

Observation 3: Sharing an UE-initiated COT with gNB is supported by regulation.

Observation 4: In NR, the timing gap between Msg1 transmission and Msg2 reception is much shorter than that between Msg2 reception and Msg3 transmission because gNB has higher computational capability. It is hence beneficial to support Msg2 transmission in the same COT as Msg1.

Observation 5: Random Access over NR is classified into two major types: (1) High Priority (beam failure recovery, handover) and (2) low priority (initial access, timing alignment, and RRC reconfiguration).

Observation 6: In LTE LAA, the CAPC used by the UE is either explicitly provided by the eNB (dynamic scheduling) or derived from a mapping from logical channels to CAPC (AUL).

Observation 7: The current mechanism where the eNB selects CAPC with dynamic grants prevent the UE from choosing the appropriate uplink LBT CAPC that is consistent with the QoS requirements of the data that is actually transmitted.

Observation 8: ETSI has not specified how initiating devices should handle gaps in its own transmission that are larger than 25us.

Observation 9: ETSI LBE class 1 devices should not have any reasonable expectation of channel utilization when any other higher priority LBE devices are competing for the same channel, including access engines enabled within the same device.

Observation 13: ETSI BRAN defined LBE adaptivity results in excessive waste of precious bandwidth due to excessive collisions with competing devices of the same or different device class.

Observation 14: ETSI BRAN defined LBE adaptivity results in excessive access delay due to excessive collisions with competing devices of the same device class.

Observation 18: An ETSI LBE device with NR-U symbol alignment does not perform well against an ETSI LBE device, such as an IEEE 802.11 device, as defined in the ETSI BRAN regulations.

Observation 19: An ETSI FBE device can coexist with ETSI LBE devices

Observation 20: An ETSI FBE device cannot coexist with another ETSI FBE device unless they coordinate with each other when they grab access to the channel in some sort of supervisory manner.

Observation 21: Modified ETSI FBE can achieve extremely good channel utilization and delay spread, even without coordination.

Based on the observations, we have the following proposals.

Proposal 1: For Cat2 LBT in a 16us gap, support that energy measurement is done in any portion of the 16 us duration including averaging for at least 4 us. LBT is said to be successful if the measured energy is lower than the ED threshold.

Proposal 2: NR-U should strive to minimize the overall LBT overhead in a RACH procedure (instead of the LBT duration for an individual RACH message).

Proposal 3: NR-U supports to share a UE-initiated COT with gNB for RACH message transmissions.

Proposal 4: CAPC for RACH message should be based on the purpose for RACH.

Proposal 5: High priority CAPC should be chosen for Handover and Beam Failure Recovery and low priority for other use cases.

Proposal 6: Cat.4 LBT is applied by UE to initiate a COT for PUCCH transmission and this UE-initiated COT can be shared with gNB for downlink transmission. For the downlink transmission in UE-initiated COTs, LBT is decided by

- Cat.1 immediate transmission is applied when the gap from the end of the PUCCH transmission to the beginning of the downlink transmission is up to 16usec.

- Cat.2 LBT is applied when the gap from the end of the PUCCH transmission to the beginning of the downlink transmission is larger than 16usec but not more than 25usec.

- FFS the case when the gap is larger than 25use.

Proposal 7: To determine the LBT type for the UE transmission of a HARQ-A/N feedback falling outside of the gNB-initiated COT, the following mechanisms could be considered:

- Indication by the gNB through the DCI(s) scheduling the PDSCH(s)

- UE determination based on the indicated channel occupancy information

- Both of the above

Proposal 8: CAPC for UL SR could be based on multiple options:

- Network (gNB) can explicitly configure CAPC for UL SR transmissions.

- UE can map UL Logical channel priority values to CAPC for SR.

- Alternatively, UE can use UL QCI to determine the CAPC for SR.

Proposal 9: NR-U should consider developing a common CAPC selection mechanism for uplink dynamic and configured grants based on a mapping from logical channels to CAPC.

Proposal 10: Draft a Liaison statement to ETSI BRAN to seek clarification on how initiating devices should handle gaps in its own transmission that are larger than 25us or draft a change request to ETSI EN 301 893 to specifically cover this missing case.

Proposal 11: When specifying what NR-U channels or signals should use ETSI LBE class 1, 3GPP should not have any expectation of timely delivery of these channels or signals. FFS – which channels or signals can tolerate this level of uncertainty?

Proposal 12: Research alternative adaptivity mechanisms that are less wasteful of bandwidth when considering the 6GHz spectrum allocations.

Proposal 13: Research alternative adaptivity mechanisms that limit excessive average delay when considering the 6GHz spectrum allocations.

Proposal 15: Start the LBT procedure as per the ETSI BRAN regulations and use a cyclic prefix extension of fractional symbol length, based on active SCS, to bring the COT to symbol alignment. The maximum length of such an extension signal should be less than 78.428us.

Proposal 16: Draft a Liaison statement to ETSI BRAN to suggest a change to the adaptivity for FBE devices such that they can fairly coexist with each other without coordination. FFS – what sort of adaptivity for FBE results in fair use

# Reference

[1] ETSI EN 301 893 V2.1.1 (2017-05)