3GPP TSG RAN WG1 #98			          R1-198388

Prague, Czech, August 26th – 30th, 2019

Agenda Item:	7.2.2.2.4

Source: 	MediaTek Inc.

Title:	Discussion on NR-U configured grant

Document for: 	Discussion

Introduction

In RAN1#96 [1] and RAN1#97 [2] meetings, the following agreements were made:

Agreement RAN1#96: For PUSCH transmitted using CG, CBG-based retransmission is supported at least by using dedicated scheduled resource allocated by an UL grant. - FFS: CBG-based retransmission using a configured grant - Note: Include this agreement in an LS to RAN2 informing them of relevant RAN1 agreements Agreement RAN1#96: For initial transmission on configured grant resource, HARQ retransmission on configured grant resource upon configured grant timer expiration (assume NACK if no ACK is received) is supported - Note: Include this agreement in an LS to RAN2 informing them of relevant RAN1 agreements Agreement RAN1#96: When a UE initiates a channel occupancy with a transmission using a configured grant, it can signal at least the following - The duration that the gNB is allowed to transmit in the channel occupancy initiated by the UE - FFS: - How the duration is signalled - Whether the UE should signal continued use of the COT for its own transmissions - LBT priority class Agreement RAN1#97: For configured grant time domain resource allocation, the mechanisms in Rel-15 (both Type 1 and Type 2) are extended so that the number of allocated slots following the time instance corresponding to the indicated offset can be configured - FFS: How to indicate multiple PUSCHs within a slot. Agreement RAN1#97: UE can only start transmissions accessing transmission opportunities provided by a configured grant at the configured/indicated starting position.

In this contribution, we will discuss the following issues for NR-U configured grant:

- CBG-based retransmission on CG resource

- Downlink feedback information

- Multiple active CG configurations

- COT sharing with CG transmission

Discussion

CBG-based retransmission on CG resource

One of the important motivations to enable CBG-based retransmission on CG resource is that the UE can only retransmit some CBGs rather than the entire TB when some symbols of a CG transmission may be dropped due to the transmission later than the configured/indicated starting based on LBT outcome. However, in previous meeting, it was agreed that flexible starting positions within a CG transmission are not allowed in NR-U, and we don't see other essential purposes. To avoid additional overhead due to enabling CBG-based retransmission on CG resource, e.g., DFI payload size would increase significantly in order to provide HARQ A/N per CBG instead of just per TB, CBG-based retransmission should be only supported by using dedicated scheduled resource allocated by an UL grant.

Proposal 1: Support CBG-based retransmission using ONLY dedicated scheduled resource allocated by an UL grant.

Downlink feedback information

During the NR-U SI phase [3], supporting explicit HARQ-ACK feedback included in Downlink Feedback Information (DFI) for CG transmissions has been agreed for NR-U operation. In FeLAA, a HARQ-ACK bitmap in AUL-DFI is adopted for conveying TB-level A/N for all HARQ processes, where the bitmap size is equal to the number of UL HARQ processes. Since CBG-based retransmission on CG-PUSCH is no longer necessary, the same HARQ-ACK bitmap can be used for conveying TB-level A/N feedback in CG-DFI for NR-U.

Proposal 2: Support CG-DFI containing a HARQ-ACK bitmap including TB-level A/N for all UL HARQ processes configured for NR-U CG.

Since NR Rel-15 does not support gNB transmitting HARQ-ACK feedback to UE corresponding to PUSCH, no PUSCH-to-HARQ-feedback-timing related parameter is defined. In some proposals, gNB should provide a minimum PUSCH-to-HARQ-feedback timing to UE. In presence of a minimum PUSCH-to-HARQ-feedback timing, for example, UE can assume that the HARQ-ACK feedback in a received CG-DFI only corresponds to PUSCHs that were completely transmitted before that timing. However, we don't see a necessity to introduce this parameter. If a CG-DFI can be transmitted as flexible as an UL grant from gNB, gNB can transmit HARQ-ACK feedback in the CG-DFI to UE at any time it want. Then, gNB can decide to transmit HARQ-ACK feedback ONLY after the configured/scheduled PUSCHs are received and decoded, and misalignment between gNB and UE can be naturally avoided.

Observation 1: The necessity of defining a minimum PUSCH-to-HARQ-feedback-timing parameter is unclear.

In NR Rel-15, DCI format 0_0 and format 0_1 with CRC scrambled by CS-RNTI are used for providing an UL grant for CG retransmission (by setting NDI = 1) or activating/releasing CG Type 2 (if the defined validation rule is satisfied). Similarly, CG-DFI can be also transmitted via DCI format 0_0 and/or format 0_1 scrambled by CS-RNTI. We see that, after CG transmission occasions, it is natural to receive DCI(s) with CRC scrambled by CS-RNTI at UE for either providing an UL grant for a certain HARQ process with a CG transmission, or a HARQ-ACK bitmap for all HARQ processes configured for CG.

In one of possible options to transmit CG-DFI on a DCI scrambled by CS-RNTI, a new validation rule can be defined for differentiating that the DCI is transmitted for providing CG-DFI or other purposes, and some fields (other than the special fields for validation) can be re-interpreted as a HARQ-ACK bitmap if validation of CG-DFI is achieved.

Proposal 3: CG-DFI is transmitted via DCI format 0_0 and/or format 0_1 scrambled by CS-RNTI.

- FFS: How to differentiate a DCI is transmitted for CG-DFI or other purposes?

- FFS: How to convey the content(s) included in CG-DFI?

Multiple active CG configurations

In RAN1#95 meeting [4] within the NR URLLC SI, it is agreed that multiple active CG configurations for a given BWP of a serving cell should be supported at least for different services/traffic types and/or for enhancing reliability and reducing latency. In NR-U, for a BWP configured with multiple 20 MHz LBT subbands, it is beneficial to assign different active CG configurations to on these20 MHz LBT subbands to increase channel access opportunities. Moreover, since HARQ process ID, NDI, and RV of a CG transmission would be carried in CG-UCI and transmitted together, NR-U has a good flexibility to support multiple active CG configurations in a BWP.

Proposal 4: Multiple active CG configurations per cell per BWP should be supported in NR-U

COT sharing with CG transmission

In FeLAA, eNB is allowed to share an eNB-initiated COT with AUL transmission in subframes belonging to the UL subframes only if the COT is acquired using Cat.4 LBT with the largest priority class value, and the COT sharing is enabled/disabled via C-PDCCH signalling. Whereas, the UEs with data to transmit can share the gNB- initiated COT and either access the channel immediately, if the AUL transmission can start after gap that is less than 16 μsec, or using Cat.2 LBT otherwise. We see that a similar operation can be also supported in NR-U for CG transmission. The COT sharing information could be conveyed in DCI transmitted on GC-PDCCH together with other CO-related control information for e.g., time/frequency domain channel occupancy structure.

Proposal 5: CG transmission within a gNB-initiated COT is supported in NR-U, and the COT sharing information could be conveyed by GC-PDCCH.

Conclusion

The proposals and observations made in this contribution are summarized below:

Proposal 1: Support CBG-based retransmission using ONLY dedicated scheduled resource allocated by an UL grant.

Proposal 2: Support CG-DFI containing a HARQ-ACK bitmap including TB-level A/N for all UL HARQ processes configured for NR-U CG.

Observation 1: The necessity of defining a minimum PUSCH-to-HARQ-feedback-timing parameter is unclear.

Proposal 3: CG-DFI is transmitted via DCI format 0_0 and/or format 0_1 scrambled by CS-RNTI.

- FFS: How to differentiate a DCI is transmitted for CG-DFI or other purposes?

- FFS: How to convey the content(s) included in CG-DFI?

Proposal 4: Multiple active CG configurations per cell per BWP should be supported in NR-U

Proposal 5: CG transmission within a gNB-initiated COT should be supported in NR-U, and the COT sharing information could be conveyed by GC-PDCCH.

- References

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #96, Athens, Greece

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #97, Reno, USA

- TR 38.889, Study on NR-based access to unlicensed spectrum

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #95, Spokane, USA