3GPP TSG-RAN WG1 #98 Meeting	R1-1908404

Prague, Czech Republic, 26th August – 30th August, 2019

Agenda item:		7.2.10.1

Source:	MediaTek Inc.

Title:	DL RS design for NR positioning

Document for:		Discussion

# Introduction

The DL PRS design has reached the following agreement in the previous two meetings,

Agreement: DL PRS Resource is described by at least the following parameters - DL PRS Resource ID (previously agreed) - Sequence ID (previously agreed) - Comb Size-N - RE Offset in frequency domain - FFS whether this offset is a single value or multiple values - Starting slot and symbol of DL PRS Resource - FFS whether it can be represented by time offset with respect to some reference - Number of symbols per DL PRS Resource (Duration of DL PRS Resource) - Quasi-colocation information (QCL with other DL reference signals) - FFS QCL type and source reference signals - FFS: Number of Tx Ports - FFS: Power offset b/w DL PRS and SSB - FFS: Transmission bandwidth and starting PRB with respect to Point A - FFS: Numerology Note: RAN1 to discuss further whether some of the above parameters belong to Resource Set configuration and are applied to all Resources within a Resource Set or not. Agreement: - Number of symbols for DL PRS Resource is configurable from the following set {2, 4, 6} - FFS: Inclusion of other values in the set including values in {1, 3, 8, 12} - DL PRS Resource comb-N value is configurable from the set {2, 4, 6} - FFS: Inclusion of other values in the set including values in {1, 8, 12} - Note: The dependence between the number of symbols and the comb size should be considered when considering the inclusion of additional values in the sets for these parameters Agreement: RAN1 to select one of the following alternatives for periodic DL PRS Resource Allocation at RAN1#98: - Alt.1 - Periodicity of DL PRS Resource is configured at DL PRS Resource Set level - Common period is used for DL PRS resources within a DL PRS Resource Set - Alt.2 - Periodicity of DL PRS Resource is configured at DL PRS Resource level - Different periods can be used for DL PRS resources within a DL PRS Resource Set Agreement: DL PRS occasion is one instance of periodically repeated time windows (consecutive slot(s)) where DL PRS is expected to be transmitted - FFS: If this definition is introduced for muting or DL PRS resource allocation or both Agreement: - A DL PRS Resource Set is defined as a set of DL PRS Resources, where each DL PRS Resource has a DL PRS Resource ID - The DL PRS Resources in a DL PRS Resource set are associated with the same TRP - A DL PRS Resource ID in a DL PRS Resource set is associated with a single beam transmitted from a single TRP (A TRP may transmit one or more beams) - Note: This does not have any implications on whether the TRPs and beams from which signals are transmitted are known to the UE. Agreement: - DL PRS sequence is generated using a Gold sequence generator as defined in TS 38.211 Section 5.2.1 - FFS: cinit value for DL PRS - QPSK modulation is used for the DL PRS signal transmitted using CP-OFDM - FFS: Whether a DL PRS sequence generated using a different mechanism is also specified Agreement: Comb-N resource element pattern per DL PRS Resource is supported to map DL PRS sequence to resource elements in frequency domain - Comb-N pattern can shift in frequency domain across symbols within DL PRS Resource - FFS values for N. The potential values for down-selection are provided in the set {1,2,3,4,6,8,12} - FFS relationship between N and number of symbols per DL PRS Resource - FFS support of staggered and non-staggered patterns and exact definition of staggered pattern Agreement: DL PRS configuration including DL PRS transmission schedule is to be indicated to the UE for DL PRS positioning measurements - The UE is not expected to perform any blind detection of DL PRS configurations Agreement: Number of DL PRS Sequence IDs is at least 4096 Agreement: Support of numerologies (CP length and sub-carrier spacing) for the DL PRS is the same as for data transmissions in Rel-15. Agreement: DL PRS muting is supported. The UE is expected to be indicated when the DL PRS is muted.

Based on the above, the remaining issues can be organized as follows,

- DL RS structure including

  - RS frequency domain density

  - RS time domain symbol number

  - RS frequency shift pattern across symbols

  - The mapping to the symbols in a slot

  - Supported number of antenna port

  - Occasion (or a burst) structure

- Scrambling sequence and initial seed generation

- Signalling structure issues

In this contribution, we provide our view for the listed issues.

# DL RS structure

## 2a Frequency domain density, time domain symbol number and frequency shift pattern across symbols

The agreement related to this category is listed below,

Agreement: - Number of symbols for DL PRS Resource is configurable from the following set {2, 4, 6} - FFS: Inclusion of other values in the set including values in {1, 3, 8, 12} - DL PRS Resource comb-N value is configurable from the set {2, 4, 6} - FFS: Inclusion of other values in the set including values in {1, 8, 12}

For a DL PRS resource, we don’t see the need of building up the correspondence that the number of symbols for a DL PRS resource be the same as the frequency domain density in terms of number of combs. In our view, the two most important factors for the DL PRS design should be,

- The overall received SINR improvement which can be further categorized as

  - The extent of RX processing gain through interpolation across symbols for received SINR improvement

  - The extent of TX processing gain through power boosting

  - The mechanism for interference avoidance

- The timing search range without ambiguity

In [1], our investigation leads to the following interesting conclusions,

- When full power boosting is applied, the received SINR enhancement is highly dependent on the number of symbols for transmission, and is less dependent on the comb structure in frequency domain. (observation 2d-4 and 2d-5)

In above, the full power boosting means, all the REs without PRS transmission in the same symbol within the transmission bandwidth are not used for any transmission and their default power are borrowed to boost PRS. So basically for the comb structure with more combs, for example comb-12 structure, the maximum power boosting level on the PRS would be larger. (10*log10(12) = 10.79dB )

In LTE, RAN4 replied the LS [2] regarding CSI-RS boosting impact on BS transmission distortion. RAN4 gave the following response:

While the exact impact due to CSI RS boosting is BS implementation dependent, RAN WG4 observed the following: - for N = 4 the related the impacts on BS RF performance are negligible. - for N = 8 (9 dB CSI RS boosting) some impact due to PA non-linearity is expected which would require either a more linear transmitter or RF output power reduction. RAN WG4 concluded that unless sufficient gain can be justified to offset the disadvantages brought by CSI RS boosting by 9 dB (N = 8) outlined above, there is a preference to avoid CSI RS boosting by 9 dB (N = 8).

For comb-12 PRS, if such boosting level can’t be attained, the received SINR could be worse. It is because the gain of SINR enhancement only comes from the TX processing gain by power boosting. The RX processing gain through interpolation on same subcarrier across symbols may not be obtained since the comb-12 signal requires 12 symbols to occupy all the subcarriers within the transmission bandwidth, leaving no room for interpolation.

One potential advantage for the comb-12 structure is to provide better interference avoidance by interleaved FDM (IFDM) manner of allowing 12 orthogonal transmissions. However, interference avoidance can be realized by multiple ways, for example by TDM transmission, by proper beamforming and by existing muting mechanism.

The disadvantage due to the advantage of allowing 12 orthogonal transmissions is the limited RX processing gain. Hence, comb-12 is not preferred. Instead, comb-8 structure could be a better solution if we inspect these factors of interference avoidance, attainable power boosting level and RX processing gain altogether. Remember that comb-8 structure has been agreed for uplink PRS transmission.

Frequency shift pattern across symbols will impact both the timing search range and RX processing gain. Let N denote the number of symbols for a PRS resource transmission, where N can be further equal to N1+N2. The RS allocation in the N1 symbols determines the observation range without ambiguity for unknown timing delay measurement. N1 is a value of greater than or equal to 1. The design of RS allocation in the N1 symbols can consider that,

- The occupation of the subcarriers of the RS in any symbol in the N1 symbols doesn’t overlap with that in other symbols of the N1 symbols. The RS are allocated in each RB and are uniformly spanned within the transmission bandwidth in each symbol of the N1 symbols

The RS allocation in the N2 symbols determines the UE processing gain regarding the received SINR improvement. N2 is a value of greater than or equal to 0, and N2 can also be larger than N1. The design of RS allocation in the N2 symbols can consider that,

- The occupation of the subcarriers of the RS in the N2 symbols is the subset of the occupation of the subcarriers of the RS in the N1 symbols. The RS is uniformly spanned within the transmission bandwidth in each symbol in the N2 symbols

The advantages of such structure are,

- The filter coefficients for interpolation across symbols can be the same for all symbols

- Provide flexibility on PRS transmission. For example,

  - For the small area deployment due to the potentially short time delay, there is no need to allocate PRS on all the subcarriers within the transmission bandwidth in order to achieve whole symbol observation range. The RS allocation in the first N1 symbols can determine this

  - The network can increase the number in N2 symbols to improve the accuracy due to SINR improvement

Based on the above, we propose the following table as the feasible combination to form the RS structure.

| Row | Comb structure | Total symbols (N=N1+N2) | N1 (symbol number to determine observation range) | N2 (symbol number determine processing gain) | number of allowed transmission within a slot |
| --- | --- | --- | --- | --- | --- |
| 1 | 8 | 12 | 6 | 6 | 1 |
| 2 | 6 | 12 | 6 | 6 | 1 |
| 3 | 6 | 8 | 5 | 3 | 1 |
| 4 | 4 | 6 | 3 | 3 | 2 |
| 5 | 4 | 4 | 3 | 1 | 3 |
| 6 | 2 | 3 | 2 | 1 | 4 |
| 7 | 2 | 2 | 2 | 0 | 6 |
| Ref: LTE | 6 | 8 | 5 | 3 | 1 |

We propose the following RS frequency shift pattern across symbols, as shown in Fig. 2a-1. The advantage of the pattern is, the occupied subcarriers in the first two symbols are equally spaced with the spacing reduced to Sf/2. This may facilitate the timing search by the UE due to the reduction of half of the false peaks by observing the first two symbols while other symbols are still not received. The corresponding equation for frequency shift may be expressed as,

- ( l*Sf/2 + l/2 + vshift) mod Sf

Where Sf is the comb number, vshift is frequency shift configured by the higher layer and l has range [0, N1-1]. The frequency shift pattern in the N2 symbols follows that in the first N1 symbols.

Fig. 2a-1: frequency shift pattern (the occupied subcarriers in the first two symbols are equally spaced with the spacing reduced to half of the original spacing)  (comb-6  comb-3 in this figure)

Proposal 2a-1: We don’t see the need of building up the correspondence that the number of symbols for a DL PRS resource be the same as the frequency domain density in terms of number of combs

Proposal 2a-2: 8 and 12 number of symbols for DL PRS resource can also be supported, due to our investigation that when full power boosting is applied, the received SINR enhancement is highly dependent on the number of symbols for transmission, and is less dependent on the comb structure in frequency domain

Proposal 2a-3: Comb-12 structure for downlink is not preferred due to the two reasons

- may not attain full power boosting

- limited RX processing gain

Proposal 2a-4: Comb-8 structure for downlink can be considered due to the three reasons

- may attain full power boosting

- better RX processing gain than comb-12 structure

- comb-8 has been supported for uplink RS

Proposal 2a-5: We propose the following table as the feasible combination to form the RS structure

| Row | Comb structure | Total symbols (N=N1+N2) | N1 (symbol number to determine observation range) | N2 (symbol number determine processing gain) | number of allowed transmission within a slot |
| --- | --- | --- | --- | --- | --- |
| 1 | 8 | 12 | 6 | 6 | 1 |
| 2 | 6 | 12 | 6 | 6 | 1 |
| 3 | 6 | 8 | 5 | 3 | 1 |
| 4 | 4 | 6 | 3 | 3 | 2 |
| 5 | 4 | 4 | 3 | 1 | 3 |
| 6 | 2 | 3 | 2 | 1 | 4 |
| 7 | 2 | 2 | 2 | 0 | 6 |
| Ref: LTE | 6 | 8 | 5 | 3 | 1 |

Proposal 2a-6: 3 number of symbols for DL PRS resource can also be supported, since one additional symbol for comb-2 signal can further provide RX processing gain for SINR improvement

Proposal 2a-7: We consider the frequency shift pattern to be able to reduce the RS spacing to half of the original spacing in the first two symbols. The corresponding equation for frequency shift for first N1 symbols may be expressed as

- ( l*Sf/2 + l/2 + vshift) mod Sf

And the frequency shift pattern in the N2 symbols follows that in the first N1 symbols. N is the total symbols equal to N1+ N2

## 2b Concern on using comb-1

The comb-1 RS occupies all the subcarriers within the transmission bandwidth in one symbol. The use case in FR2 for comb-1 RS transmission deserves more discussion. Fig. 2b-1 shows the layout of indoor office scenario. UE may receive the signal from 120 meters away. The propagation time delay for 120 meters is around 0.4us. The OFDM symbol time period for SCS=120KHz is 8.3us. This means, for the indoor office scenario, there is no need to observe the whole OFDM symbol time for the time delay measurement.

The potential use case of the comb-1 RS is to combine two RS resources with different densities in a RS block. The first RS resource with the comb-1 structure is transmitted at a subband within the transmission bandwidth, followed by the second RS resource with sparser density which spans uniformly within the transmission bandwidth. The subband comb-1 RS can be used for the coarse timing search, and the wideband RS with sparser density can be further utilized for the timing refinement. Fig. 2b-2 gives a simple example. The comb-1 signal transmitted from different TPs may occupy the non-overlapping subband.

Fig. 2b-1, layout of indoor office scenario (TR 38.901)

Fig. 2b-2, proposed structure for comb-1 transmission

Proposal 2b-1: Consider the structure of the combination of two RS resources with different densities for positioning purpose. The first RS resource with the comb-1 structure is transmitted at a subband within the transmission bandwidth, followed by the second RS resource with sparser density which spans uniformly within the transmission bandwidth

## 2c Transmission burst (or occasion) structure

In LTE, a transmission burst may contain consecutive subframes with PRS. It is assumed in LTE that the PRS transmission on consecutive subframes are to go through the same transmission filter. So the UE can combine the received signal across subframes for improving the detection.

In NR, the analog beamforming within a transmission burst can be considered. Namely, the PRS block allocated at different time period can apply different transmission filter. Same transmission filter can also be applied for some PRS blocks to facilitate the combination. This means, we may consider the use case of beam sweeping for multiple times within a transmission burst. Then, the PRS transmission through both same transmission filter and through different transmission filter maybe together within a transmission burst.

Then simply defining “repetition on” and “repetition off” may not be enough. We may further consider one extra parameter: “periodicity for repetition off”. For example, if there are 8 beams, and the beam sweeping is performed twice, then there are totally 16 PRS blocks within a transmission burst. With such an indication on “periodicity for repetition off” = 8, the UE can know how to combine the signal and how to adjust the receiving beamformer.

Proposal 2c-1: Consider to add the parameter of “periodicity for repetition off” as the signalling for the use case of beam sweeping for multiple times within a transmission burst

## 2d Number of antenna ports

We mentioned in [3] the potential of LOS detection when the PRS transmission comes from two polarizations. The LOS detection could be a complicated mechanism. It may not be simply relied on a single approach. The interband measurement as proposed in [4] could also be an approach. But it is doubtful on the detection rate by simply based on this single approach.

Proposal 2d-1: Support PRS transmission through two antenna ports, at least for the on-demand mode

# Sequences

There is no further agreement in previous meeting. The agreement related to this category two meetings ago is listed below,

Agreement: Number of DL PRS Sequence IDs is at least 4096 Agreement: - DL PRS sequence is generated using a Gold sequence generator as defined in TS 38.211 Section 5.2.1 - FFS: cinit value for DL PRS - QPSK modulation is used for the DL PRS signal transmitted using CP-OFDM - FFS: Whether a DL PRS sequence generated using a different mechanism is also specified

The pseudo-random sequences are defined by a length-31 Gold sequence. The initial seed is represented by 31 bits. Hence the total number of the initial seeds is 231.

The NR CSI-RS considers 1024 resource IDs and different sequence for each symbol in a frame. So this needs 1024x14x160 ( slightly greater than 221) different seeds for SCS=240KHz.

The LTE PRS supports 4096 resource IDs. If NR also considers the same ID number and applies to all the symbols in a frame by different seeds, the required seed number is around 223 seeds, which is still below 231.

We propose the easy extension by modifying the seed generation equation for CSI-RS to support 4096 ID number for positioning purpose. Then we have

Another consideration that the PRS seed generation is backward compatible to CSI-RS. The CSI-RS seed generation is shown below,

For notational convenience, let

The CSI-RS seed generation can then be re-written as,

To extend the CSI-RS seed generation to support more ID number, let’s redefine y as shown below, and x is unchanged. An offset should be added in order to avoid overlapping on the seed value for ID index >= 1024. Now we have,

The offset can be given by

Through the simulation, a= 229 is feasible.

Proposal 3-1: Consider the easy extension by modifying the seed generation equation for CSI-RS to support 4096 ID number for positioning purpose. Then we have

Proposal 3-2: Consider that the PRS seed generation is backward compatible to CSI-RS. Then the PRS seed generation can be as a function of

- nid % 1024, and

- nid/1024

# Signalling issues

It is understood that, a DL PRS resource denotes a beam for PRS transmission from a TP. A TP can be logically linked to a DL PRS resource set. The common configurations among the resources (beams) can actually be configured at the resource set level for saving the higher layer signalling effort. We don’t see that there is particular need to configure the beams with different periods. So we support

- Periodicity of DL PRS Resource is configured at DL PRS Resource Set level

We also don’t see the need to have the linkage regarding power offset between DL PRS and SSB. The reason is, the power offset may not be related to the timing estimation.

Proposal 4-1: Support that “Periodicity of DL PRS Resource is configured at DL PRS Resource Set level”

Proposal 4-2: The linkage regarding power offset between DL PRS and SSB is not needed

# Conclusion

Based on the above, we have,

Proposal 2a-1: We don’t see the need of building up the correspondence that the number of symbols for a DL PRS resource be the same as the frequency domain density in terms of number of combs

Proposal 2a-2: 8 and 12 number of symbols for DL PRS resource can also be supported, due to our investigation that when full power boosting is applied, the received SINR enhancement is highly dependent on the number of symbols for transmission, and is less dependent on the comb structure in frequency domain

Proposal 2a-3: Comb-12 structure for downlink is not preferred due to the two reasons

- may not attain full power boosting

- limited RX processing gain

Proposal 2a-4: Comb-8 structure for downlink can be considered due to the three reasons

- may attain full power boosting

- better RX processing gain than comb-12 structure

- comb-8 has been supported for uplink RS

Proposal 2a-5: We propose the following table as the feasible combination to form the RS structure

| Row | Comb structure | Total symbols (N=N1+N2) | N1 (symbol number to determine observation range) | N2 (symbol number determine processing gain) | number of allowed transmission within a slot |
| --- | --- | --- | --- | --- | --- |
| 1 | 8 | 12 | 6 | 6 | 1 |
| 2 | 6 | 12 | 6 | 6 | 1 |
| 3 | 6 | 8 | 5 | 3 | 1 |
| 4 | 4 | 6 | 3 | 3 | 2 |
| 5 | 4 | 4 | 3 | 1 | 3 |
| 6 | 2 | 3 | 2 | 1 | 4 |
| 7 | 2 | 2 | 2 | 0 | 6 |
| Ref: LTE | 6 | 8 | 5 | 3 | 1 |

Proposal 2a-6: 3 number of symbols for DL PRS resource can also be supported, since one additional symbol for comb-2 signal can further provide RX processing gain for SINR improvement

Proposal 2a-7: We consider the frequency shift pattern to be able to reduce the RS spacing to half of the original spacing in the first two symbols. The corresponding equation for frequency shift for first N1 symbols may be expressed as

- ( l*Sf/2 + l/2 + vshift) mod Sf

And the frequency shift pattern in the N2 symbols follows that in the first N1 symbols. N is the total symbols equal to N1+ N2

Proposal 2b-1: For comb-1 signal usage. Consider the structure of the combination of two RS resources with different densities for positioning purpose. The first RS resource with the comb-1 structure is transmitted at a subband within the transmission bandwidth, followed by the second RS resource with sparser density which spans uniformly within the transmission bandwidth

Proposal 2c-1: Consider to add the parameter of “periodicity for repetition off” as the signalling for the use case of beam sweeping for multiple times within a transmission burst

Proposal 2d-1: Support PRS transmission through two antenna ports, at least for the on-demand mode

Proposal 3-1: Consider the easy extension by modifying the seed generation equation for CSI-RS to support 4096 ID number for positioning purpose. Then we have

Proposal 3-2: Consider that the PRS seed generation is backward compatible to CSI-RS. Then the PRS seed generation can be as a function of

- nid % 1024, and

- nid/1024

Proposal 4-1: Support that “Periodicity of DL PRS Resource is configured at DL PRS Resource Set level”

Proposal 4-2: The linkage regarding power offset between DL PRS and SSB is not needed

# Reference

[1] R1-1904500

[2] R4-102236

[3] R1-1901816

[4] RP-190861, Qualcomm

# Appendix

The reply LS from RAN4 for the concern on the level of CSI-RS boosting is attached below,

3GPP TSG-RAN WG4 (Radio) Meeting #55 R4-102236 Montreal, Canada, May 10 – 14, 2010 Title: Reply LS on CSI RS boosting impact on BS transmission distortion Response to: R4-101853 (R1-101692) Release: Rel-10 Work Item: Enhanced downlink multi-antenna transmission for LTE Source: RAN WG4 To: RAN WG1 Cc: - Contact Person: #### Name: Peter Muszynski Tel. Number: +358 40 502 7939 E-mail Address: peter.muszynski@nsn.com #### Name: Ronnie Zhang E-mail Address: wrzhang@huawei.com Attachments: none 1. Overall Description: RAN WG4 would like to thank RAN WG1 for their LS in R4-101853 entitled “LS on CSI RS boosting impact on BS transmission distortion”. The LS contains two questions and RAN WG4 would like to provide the following answers: Q1: For a BS transmitting at an antenna with maximum power when all the REs have the same EPRE and using a PA corresponding to practical implementations, how is the noise level per RE due to non-ideal PA behavior on PDSCH REs, CSI RS REs and silent REs affected when the CSI RS is boosted in the above mentioned manner or conceivable manners using boosting values as implied by N equal to 2, 4, or 8? The power boosted REs can occur at one or two OFDM symbols in as often as every 5 ms. For the question particularly related to noise level due to the CSI RS boosting, RAN WG4 considered the following aspects: - Phase noise: the OFDM sub-carriers are orthogonal only when they are sampled at the right time. Phase noise will translate the sampling error as interference between sub-carriers. As power boosting is applied to CSI RS, the neighbouring sub-carriers would experience increased noise from the sub-carriers where CSI RS is placed. - In band distortion caused by PA inter-modulation products: when the IMD products are considered in terms of sub-carrier, the IMD products caused by the power boosted sub-carriers where CSI RS are located would generate additional noise to adjacent sub-carriers. The following RF requirement specification limits the in-band noise level: - The worst case average TX noise (in dB), as generated per TX antenna, can be upper bounded by the EVM requirement (TS36.104, clause 6.5.2) according to the expression 20*log(EVM % / 100). This results in a worst case aggregated TX noise, across any RE, of 20*log(0.175 [QPSK]) + 10*log(N) relative to the average EPRE per TX antenna. For the question related to the impact on different REs, RAN WG4 concluded the following: - The mentioned types of REs suffer more noise if they are closer to REs with power boosting. Q2: Are there any other foreseen impacts on other RF performance parameters, e.g. emissions, in relation to the CSI RS design problem that RAN1 should be aware of? There will be out-of-band inter-modulation products generated due to CSI RS power boosting. With CSI RS boosting it will be more difficult for the BS to meet operating band unwanted emissions (TS36.104, clause 6.6.3) and in particular, some of the regional emission limits (FCC limits, TS36.104, clause 6.6.3.3). As most of these unwanted emission requirements have a regulatory background they cannot be relaxed and hence must be met with better BS RF performance. In this respect RAN WG4 investigated the impacts due to CSI RS boosting considering practical TX spectrum shaping filters and non-linear PA [1]. While the exact impact due to CSI RS boosting is BS implementation dependent, RAN WG4 observed the following: - for N = 4 the related the impacts on BS RF performance are negligible. - for N = 8 (9 dB CSI RS boosting) some impact due to PA non-linearity is expected which would require either a more linear transmitter or RF output power reduction. RAN WG4 concluded that unless sufficient gain can be justified to offset the disadvantages brought by CSI RS boosting by 9 dB (N = 8) outlined above, there is a preference to avoid CSI RS boosting by 9 dB (N = 8). 2. Actions: To RAN2: RAN4 kindly asks RAN1 to consider the above information in its further work. 3. Date of Next RAN WG4 Meetings: 3GPPRAN4#3 28 Jun - 2 Jul 2010 Bratislava, SK 3GPPRAN4#56 23 - 27 Aug 2010 Madrid, ES 4. References: - R4-101796, CSI RS boosting impact on BS transmission distortion, Nokia Siemens Networks

The below is the interference analysis session as shown in previous contribution R1-1904500.

Let’s consider 19 sites, each with 3 sectors (cells). Then totally there are 57 cells for transmission. Assume that the 57 cells are transmitted together in the same slot. So under the same RS overhead including both symbols for RS transmission and muted symbols, we get the following statistics, - Consider 1 RS block in a slot, 12 symbols for this block and comb-6 RS structure. This is approximately the same as the LTE PRS structure. For each comb index, there would be, on the average, 57/6 = 9.5 cells for simultaneous transmission - Consider 2 RS blocks in a slot, 6 symbols for each RS block, and comb-6 RS structure. For each RS block, 28.5 (57/2) cells on the average are transmitted and other cells are muted. For each comb index in a RS block, there would be, on the average, 28.5/6 = 4.75 cells for simultaneous transmission - Consider 3 RS blocks in a slot, 4 symbols for each RS block, and comb-4 RS structure. For each RS block, 19 (57/3) cells are transmitted and other cells are muted. For each comb index in a RS block, there would be, on the average, 19/4 = 4.75 cells for simultaneous transmission - Consider 4 RS blocks in a slot, 3 symbols for each RS block, and comb-3 RS structure. For each RS block, 14.25 (57/4) cells on the average are transmitted and other cells are muted. For each comb index in a RS block, there would be, on the average, 14.25/3 = 4.75 cells for simultaneous transmission - Consider 6 RS blocks in a slot, 2 symbols for each RS block, and comb-2 RS structure. For each RS block, 9.5 (57/6) cells on the average are transmitted and other cells are muted. For each comb index in a RS block, there would be, on the average, 9.5/2 = 4.75 cells for simultaneous transmission - Consider 12 RS blocks in a slot, 1 symbol for each block, and comb-1 RS structure. For each RS block, 4.75 (57/12) cells on the average are transmitted and other cells are muted. For the only one comb index in a RS block, there would be, on the average, 4.75/1 = 4.75 cells for simultaneous transmission The more analysis is tabulated below, Number of RS block in a slot; Number of symbols in a block; Comb structure per symbol; Average loading (collided cells) per comb under 57 cells transmission. 1; 12; Comb-6; 9.5 cells; As benchmark. 2; 6 or 5; Comb-6; 4.75 cells. 2; 6 or 5; Comb-4; 7.125 cells. 3; 4; Comb-4; 4.75 cells. 3; 4; Comb-3; 6.33 cells. 4; 3; Comb-3; 4.75 cells. 4; 3; Comb-2; 7.125 cells. 5; 2; Comb-2; 5.7 cells. 6; 2; Comb-2; 4.75 cells. 12; 1; Comb-1; 4.75 cells. 12; 1; Comb-2; 2.375 cells. We further derive the following observations, - The theoretical average loading (number of colliding cells) for each comb is the same under a given number of transmitted cells among these cases: 1-symbol comb-1, 2-symbol comb-2, 3-symbol comb-3, 4-symbol comb-4 and 6-symbol comb-6 transmission - The TDM transmission in a slot may reduce the interference level