3GPP TSG RAN WG1 Meeting #98  	R1- 1908410

Prague, Czech, August 26th – 30th, 2019

Agenda Item: 7.2.6.3

Source: MediaTek Inc.

Title: On repetition schemes for NR PUSCH

Document for: Discussion and Decision

# Introduction

A work item on physical layer enhancements for NR URLLC has been approved in RAN#83 with the following objective for PUSCH [1];

- Specification of PUSCH enhancements for both grant-based PUSCH and configured grant based PUSCH [RAN1] - For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots

RAN1 has agreed during RAN1#97 meeting to adopt a repetition scheme for NR PUSCH with the following details:

One or more actual PUSCH repetitions in one slot, or two or more actual PUSCH repetitions across slot boundary in consecutive available slots, is supported using one UL grant for dynamic PUSCH, and one configured grant configuration for configured grant PUSCH. - The number of the repetitions signaled by gNB represents the “nominal” number of repetitions. The actual number of repetitions can be larger than the nominal number. - FFS dynamically or semi-statically signalled for dynamic PUSCH and type 2 configured grant PUSCH - The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first “nominal” repetition. - FFS the detailed interaction with the procedure of UL/DL direction determination - The time domain resources for the remaining repetitions are derived based at least on the resources for the first repetition and the UL/DL direction of the symbols. - FFS the detailed interaction with the procedure of UL/DL direction determination - If a “nominal” repetition goes across the slot boundary or DL/UL switching point, this “nominal” repetition is splitted into multiple PUSCH repetitions, with one PUSCH repetition in each UL period in a slot. - Handling of the repetitions under some conditions, e.g., when the duration is too small due to splitting, is to be further investigated in the WI phase. - No DMRS sharing across multiple PUSCH repetitions - The maximum TBS size is not increased compared to Rel-15. - FFS: L > 14 - S+L can be larger than 14 - FFS: The bitwidth for TDRA is up to 4 bits. - Note: different repetitions may have the same or different RV. For option 4, dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH is supported for PUSCH enhancements. The dynamic indication can be enabled or disabled by the gNB. - FFS the exact signaling method - FFS the exact DCI format(s) - FFS the exact mechanism to enable or disable - FFS the DCI activating type 2 configured grant PUSCH

In this contribution, we discuss the remaining details, in terms of frequency hopping and segmentation pattern, for the agreed repetition scheme for NR PUSCH.

# Discussion

During RAN1#97 meeting, RAN1 has agreed to adopt a repetition scheme for NR PUSCH. Generally, the agreed repetition scheme combines the following two features;

Mini-slot level repetition: the same TB is repeated within a slot.

Multi-segment transmission: if one of the repetitions crosses the slot boundary, the repetition is segmented into two repetitions, each in one slot.

There were two main objectives behind adopting the repetition scheme for PUSCH are;

- Diversity gains: with mini-slot based repetitions it is possible to provide more frequency diversity compared to intra-slot FH, i.e. more than one frequency hop per slot.

- Alignment delay reduction: in NR Rel-15, PUSCH is not allowed to cross the slot boundary, which creates scheduling delays when the gNB wants to schedule an UL grant that crosses the slot boundary.

Mini-slot repetition aim to achieve diversity (in frequency, precoder, TRP) within a slot. NR Rel-15 supports intra-slot frequency hopping (FH) which provides two frequency hops in a slot, and with mini-slot repetitions more than two frequency hops could be achieved. To achieve more diversity with the agreed repetition scheme compared to Rel-15 intra-slot FH, the number of frequency hops within should be allowed to be larger than two.

Proposal 1: Support more than two frequency hops within a slot for PUSCH repetition.

For intra-slot FH in Rel-15, the FH pattern is repeated across slots when the transmission spans multiple slots. This limits the gain that can be achieved from FH. Also, there is no added complexity to the UE or the gNB by changing the FH pattern across slots.

Proposal 2: Support different hopping pattern per slot for multi-slot PUSCH transmission.

On the other hand, the main objective of multi-segment transmission is to reduce the alignment delay. As shown in our paper [2] on the latency analysis, not allowing the PUSCH to cross the slot boundary represents a bottleneck for the transmission incurred latency. However, some of the segmentation patterns could impact the performance due to the extra DMRS overhead. As one example, splitting a 2-symbols PUSCH could result in effectively losing the two symbols.

Proposal 3:  Adopt at least the following restrictions on the possible segmentation patterns for the agreed repetition pattern;

- The UE is not expected to segment a PUSCH that has a length less than 4 symbols.

- The UE is not expected to segment a PUSCH that result in 1-symbols PUSCH.

Regarding the possible length for the time domain allocation (L), there is no need to support L > 14. Given that total time domain resources for the PUSCH transmission (K*L, where K is the number of repetitions) could be significantly larger than 14 symbols.

Observation 1:  There is no clear motivation for having L > 14.

# Conclusion

In this contribution, we discussed the repetition scheme for NR PUSCH and we have the following observations and proposals:

Proposal 1: Support more than two frequency hops within a slot for PUSCH repetition.

Proposal 2: Support different hopping pattern per slot for multi-slot PUSCH transmission.

Proposal 3:  Adopt at least the following restrictions on the possible segmentation patterns for the agreed repetition pattern;

- The UE is not expected to segment a PUSCH that has a length less than 4 symbols.

- The UE is not expected to segment a PUSCH that result in 1-symbols PUSCH.

Observation 1:  There is no clear motivation for having L > 14.

# References

- RP-190726, “New WID: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC)”, Huawei, HiSilicon, TSG-RAN#83, March, 2019.

- R1-1903706 “Study and evaluation of scheduling, HARQ and CSI processing timeline”, MediaTek, RAN1#96.