3GPP TSG RAN WG1 Meeting #98																			R1-1908445

Prague, CZ, August 26th – 30th, 2019

Agenda item:	6.2.1.3

Source:	Samsung

Title:	Scheduling of multiple transport blocks for MTC

Document for:	Discussion/Decision

# 1. Introduction

Scheduling of multiple transport blocks with single DCI or without DCI for SC-PTM and unicast was introduced in Rel-16 MTC to reduce overhead of control signaling and improve peak data rate. The feasibility and possible solutions were discussed in RAN1#97 meeting with the following:

Working assumption

- For unicast, scheduling of initial and retransmission TB(s) within one DCI is supported.

- Till the RAN1#98 meeting, consider potential simplifications that can help achieve a tradeoff between scheduling flexibility and DCI size. For example:

  - Configurable maximum number of TBs per grant

  - Maximum size of DCI

  - Joint coding of DCI fields

  - Reduced TBS choices

  - Reduced resource allocation choices

  - Reduced/eliminated RV field

Working assumption

For unicast, scheduling gaps for multiple transport blocks is supported and a scheduling gap can be configured by [RRC and/or DCI]

- The support of scheduling gaps is UE optional feature regardless of the support of multiple TBs

- FFS: Details on the scheduling gap such as duration, applicability, etc

Agreement

For scheduling of multiple TBs with SC-MTCH, scheduling gaps are supported.

Agreement

For scheduling of multiple TBs with SC-MTCH, a DCI field indicates the number of scheduled TBs.

In this contribution, the design of scheduling multiple DL/UL transport blocks with single DCI for MTC is discussed.

# 2. Scheduling of multiple transport blocks for unicast

For dynamic scheduling in legacy LTE, one DCI grant allocates resource for a single transport block. Multi-subframe scheduling (MSF) for LAA uplink transmission was introduced in Rel-14 to enable scheduling of multiple UL transport blocks. One MSF DCI schedules consecutive subframes for PUSCH transmission with single transport block per subframe (two transport blocks per subframe is also supported). Some scheduling information that introduced or modified to support MSF in the DCI was listed as the following:

- The number of actual scheduled subframes in the DCI;

- HARQ process ID for every subframe: HARQ ID for the first subframe is indicated in DCI, and the HARQ IDs for the subsequent subframes are consecutive with the indicated HARQ ID (modulo max number of HARQ processes);

- 1-bit RV value per scheduled subframe;

- 1-bit NDI value per scheduled subframe.

Figure 1 MSF for LAA uplink

Similarly, MSF-like mechanism can be considered for MTC to support scheduling of multiple transport blocks within one DCI. A new DCI format carrying resource allocation for multiple transport blocks can be introduced, in which the number of actual scheduled transport blocks and the scheduling information for each transport block are indicated.

Handling of HARQ retransmission is the key issue for scheduling of multiple transport blocks.

Mixed initial transmission and retransmission in single DCI should be supported to reduce control signaling overhead. Otherwise, the retransmissions of scheduled multiple transport blocks need fallback to legacy single-TB scheduling with multiple DCI grants, which will significantly reduce the gain of multi-TB scheduling. Furthermore, compared with scheduling initial transmission and retransmission with two separate DCIs, mixed initial transmission and retransmission with single DCI still can reduce the overhead of control signal.

In legacy MTC system, HARQ retransmission for PUSCH is triggered by UL grant, in which NDI field is used to indicate HARQ-ACK feedback implicitly. This mechanism can be easily reused in DCI design of multi-TB scheduling. A NDI bitmap with 1 bit per transport block can be carried in DCI to indicate the initial transmission or retransmission for each transport block.

Proposal 1: Confirm the following working assumption made in RAN1#97:

- For unicast, scheduling of initial and retransmission TB(s) within one DCI is supported.

Proposal 2: NDI bitmap with 1 bit per TB in UL/DL grant is used to schedule the initial transmission or retransmission of multiple UL/DL transport blocks.

When multiple transport blocks corresponding to multiple HARQ processes are scheduled, in order to support flexible scheduling, dynamic indication of HARQ ID of each actual scheduled TB should be designed rather than using fixed mapping between transport block and HARQ ID.

The overhead of HARQ ID indication needs to be carefully controlled to avoid DCI size increasing. Similarly as LAA MSF, one simple solution is to explicitly indicate the HARQ ID of first actual scheduled transport block in DCI grant, and the HARQ ID of subsequent transport blocks are consecutive (modulo maximum number of HARQ processes), which is easy to be derived at UE side.

Proposal 3: The HARQ ID corresponding to first actual scheduled TB is explicitly indicated in DCI grant, and the HARQ IDs of the subsequent HARQ processes are consecutive.

For DL transmission, since HARQ-ACK feedback for DL is explicitly indicated on PUCCH, the PUCCH resource of HARQ-ACK feedback of multiple blocks should also be considered. If HARQ-ACK bundling or multiplexing is used, the legacy mechanism of ACK/NACK resource scheduling could be reused. Otherwise if separately encoded HARQ-ACK feedback e.g. bitmap-based ACK/NACK feedback is configured, the PUCCH resource of first TB can be derived based on the resource of last DL transport block, and the following TBs could use consecutive PUCCH resources. For example, the scheduling delay between PUCCH and the last DL transport block can be indicated in DCI grant for PUCCH resource scheduling.

Proposal 4: The PUCCH resource allocation corresponding to HARQ-ACK feedback of multiple DL transport blocks could:

- Reuse legacy PUCCH resource allocation if HARQ-ACK bundling or multiplexing is used;

- Otherwise, PUCCH resource of the first scheduled TB is derived based on PDSCH resource of last DL TB, and the subsequent TBs use consecutive PUCCH resources.

Explicit UL HARQ-ACK feedback was introduced in Rel-15 MTC for early termination of PUSCH transmission by indicating an unused state in DCI. It can be further extended to indicate ACK/NACK for any ongoing PUSCH transmission in order to improve the efficiency of scheduling of multiple UL transport blocks. For FD-FDD UE support scheduling of multiple transport blocks, the transmission of one given transport block can also be early terminated to reduce power consumption.

Observation 1: For FD-FDD UE, it is beneficial to support early termination of PUSCH when multiple UL transport blocks are scheduled via single DCI.

# 3. DCI detection and design

If the enhancement on scheduling of multiple transport blocks in a single DCI is supported, new DCI format(s) needs to be introduced carrying the scheduling information of each transport block. The new DCI format could explicit indicates the following information (which is different with in legacy formats):

- Number of actual scheduled transport blocks: the size of each field can be calculated based on the number of actual scheduled TBs. Padding bits might be used when the actual scheduled number is smaller than the maximum number.

- HARQ ID: As discussed in section 2, one possible solution is to indicate a reference HARQ ID in DCI, and the HARQ IDs for all transport blocks scheduled in the DCI is calculated by the reference HARQ ID. For example, the reference HARQ ID can be the HARQ ID of the first transport block, and the HARQ ID of the subsequent transport blocks are consecutive.

- Resource assignment and MCS: It is reasonable to assume the multiple transport blocks corresponding to the same MTC traffic, and transmitted in a short duration with similar channel status. Therefore, these parameters should be common for all scheduled transport blocks rather than TB-specific, similarly as using common repetition as agreed in RAN1#96bis.

- TB-specific NDI and RV: NDI bitmap with 1 bit per transport block can be indicated in DCI as discussed in Section 2.

For RV, both common RV and TB-specific RV can be considered. One common RV field can be applied to all TBs to reduce overhead. Joint coding of NDI and RV can also be considered, e.g. RV can be derived based on NDI. If there could be enough bits, RV of each transport block could also use bitmap-based indication. In addition, for the purpose of overhead reduction, the supported number of RV values can be further reduced, e.g. to 2 states.

Proposal 5: The new DCI format scheduling multiple transport blocks for unicast could explicitly indicate at least the following information:

- Number of actual scheduled transport blocks

- HARQ ID of first actual scheduled TB

- Resource assignment/MCS, which are common for all TBs

- TB-specific NDI

- Common RV, or TB-specific RV derived by NDI

No increasing of blind detection cost is agreed as a basic metric in RAN1#94b meeting that UE should only monitor one DCI size in the UE specific search space.

In order to support fallback (to scheduling of a single transport block in one DCI), one possible solution is that UE needs to monitor both legacy and new DCI formats. Therefore, the size of new DCI format needs to be aligned to the size of at least one legacy DCI format. UE can distinguish the formats by using different search space, RNTI, or by format differentiation field that explicitly indicated in DCI. eNodeB will use both new and legacy DCI formats to schedule UL/DL transmission, thus the fallback can be supported very simply at any time.

Since the DCI might carry TB-specific fields, e.g. RV and NDI, and additional fields such as number of actual scheduled transport blocks, the size of new DCI format is expected to be increased. For the purpose of alignment with legacy DCI formats, some legacy fields could be removed or with reduced size in the new DCI format. For example, scheduling of multiple transport blocks is used for large data packet, then choosing small TBS or smaller number of subcarriers will lead to dividing the large data packet into more transport blocks, which seems unreasonable. Therefore some lower values in MCS field and resource assignment field can be removed to reduce size of the fields. Moreover, some parameters e.g. scheduling delay can be fixed or configured by RRC for the feature of scheduling multiple transport blocks, and the corresponding field is not carried in new DCI format.

Otherwise, add 1 or 2 padding bits in legacy DCI formats to achieve the alignment between legacy and new DCI formats is also considerable. These padding bits can be reserved for potential use in future releases.

Another possible solution is, when scheduling of multiple transport blocks is enabled, UE stops blind decoding of legacy DCI formats and only detects new DCI format. Compared with co-existed legacy DCI and new formats, no restriction on DCI size due to size alignment will happen. However, with this solution it is hard to fallback to legacy scheduling, and if only one TB needs to be scheduled, the new DCI format with increased size will introduce a waste of radio resource and UE power, and then the gain of enabling scheduling of multiple transport blocks can hardly be guaranteed.

Proposal 6: Remove part of legacy values of transmission parameters to support size alignment from legacy DCI format and DCI format scheduling multiple TBs.

# 4. Conclusion

Based analysis above, the following observations and proposal are provided:

Observation 1: For FD-FDD UE, it is beneficial to support early termination of PUSCH when multiple UL transport blocks are scheduled via single DCI.

Proposal 1: Confirm the following working assumption made in RAN1#97:

- For unicast, scheduling of initial and retransmission TB(s) within one DCI is supported.

Proposal 2: NDI bitmap with 1 bit per TB in UL/DL grant is used to schedule the initial transmission or retransmission of multiple UL/DL transport blocks.

Proposal 3: The HARQ ID corresponding to first actual scheduled TB is explicitly indicated in DCI grant, and the HARQ IDs of the subsequent HARQ processes are consecutive.

Proposal 4: The PUCCH resource allocation corresponding to HARQ-ACK feedback of multiple DL transport blocks could:

- Reuse legacy PUCCH resource allocation if HARQ-ACK bundling or multiplexing is used;

- Otherwise, PUCCH resource of the first scheduled TB is derived based on PDSCH resource of last DL TB, and the subsequent TBs use consecutive PUCCH resources.

Proposal 5: The new DCI format scheduling multiple transport blocks for unicast could explicitly indicate at least the following information:

- Number of actual scheduled transport blocks

- HARQ ID of first actual scheduled TB

- Resource assignment/MCS, which are common for all TBs

- TB-specific NDI

- Common RV, or TB-specific RV derived by NDI

Proposal 6: Remove part of legacy values of transmission parameters to support size alignment from legacy DCI format and DCI format scheduling multiple TBs.

# Reference

[1] RP-191356, Revised WID on Rel-16 MTC

[2] Chairman’s note of RAN1#97

[3] R1-1906894, Scheduling of multiple TBs for MTC, Samsung