3GPP TSG RAN WG1 98			R1-1908500

Prague, Czech Republic, 26th – 30th August, 2019

Agenda item:	7.2.8.1

Source: 	Samsung

Title: 	Feature lead summary: agreements on MU-MIMO CSI until RAN1#97

Document for:	Discussion and Decision

# Introduction

In this summary, the agreements made on Rel.16 NR_eMIMO MU-MIMO CSI to-date are compiled and systematically arranged ([1][2][3]). The main purpose is to identify missing parts, if any, as well as to aid specification editors. Therefore:

- Only agreements pertinent to specification are included.

- For a given topic/component, the captured agreement represents the most mature form. Redundancies (including most FFS items that have been agreed) are removed.

- Open issues (including missing items) are identified with yellow highlight

Summary

## Structure of DFT-based compression

For Rel-16 NR, agree on DFT-based compression as the adopted Type II rank 1-2 overhead reduction (compression) scheme

- Wf is determined from a set of predefined DFT vectors.

- PMI feedback: W1 (wideband), W2, possibly a small number of parameters for Wf (wideband)

- Detailed formulation is given below

- Note: The same DFT-based compression scheme is extended for Type II port selection codebook

- Codebook subset restriction (CBSR) is supported when DFT-based compression is utilized for Type II codebooks with overhead reduction (compression) scheme

  - FFS: detailed signaling mechanism

- Precoders for a layer is given by size-P×N3 matrix W=W1W2WfH - P=2N1N2= #SD dimensions - N3= #FD dimensions - Precoder normalization: the precoding matrix for given rank and unit of N3 is normalized to norm 1/sqrt(rank) - Spatial domain (SD) compression - L spatial domain basis vectors (mapped to the two polarizations, so 2L in total) selected - Compression in spatial domain using W1=v0v1…vL-100v0v1…vL-1, where vii=0L-1 are N1N2×1 orthogonal DFT vectors (same as Rel. 15 Type II) - Frequency-domain (FD) compression - Compression via Wf=fk0 fk1…fkM-1, where fkmm=0M-1 are M size-N3×1 orthogonal DFT vectors - Number of FD-components M is configurable - Note: The terminology “SD-compression” and “FD-compression” are for discussion purposes only and are not intended to be captured in the specification

## Basic compression parameters

Spatial domain compression parameters

On the values of L, support

- L={2,4}

- L=6 for the following combinations of p and beta

  - p value equals to 1/4, beta value equals to {1/4, ½, 3/4}

  - p value equals to 1/2, beta value equals to ¼

  - Above applies only for the case of 32 ports, rank 1 or 2, R=1

  - Note that the payload size for L=6 should not exceed that of Rel-15 type-2 codebook

  - The above feature is UE optional

  - FFS: Further specification support to relax UE processing complexity

- [Inferred from late agreement on (L,p)] The value of L is higher-layer configured

Inherited from Rel.15: N1, N2 (along with the values) are higher-layer configured. (O1,O2)=(4,4).

Frequency domain compression parameters

On the choice of oversampling factor O3, agree on O3=1

- The rotation factor q3 is therefore not needed

On FD compression unit, agree on {PMI subband size = CQI subband size} as the default, along with {PMI subband size = CQI subband size / R} as secondary

- The value of R is fixed to 2

- FFS: Whether secondary implies a separate UE capability or restricted use cases

  - Include issues such as limitation on the number of FD compression units, CPU occupation, latency constraint and/or BW constraint

- The FD compression unit parameter R is higher-layer configured

On the value of M (the number of FD compression units), M=p×N3R where.

- The value of M is higher-layer configured [via R and p]

Values of N3:

- For R∈1,2 and NSB is # CQI subbands, when NSB×R≤19,

  - FFS: when NSB×R>19

Parameter setting across layers and RI values

The parameter R is layer-common and RI-common

For the higher-layer setting of SD/FD basis parameters (L, p):

- The parameter p=v0 for RI=3-4 is higher-layer configured in conjunction with the parameter p=y0 for RI=1-2.

- The parameters (y0, v0)(y0,v0) take value from {(1/2,1/4),(1/4,1/4),(1/4,1/8)}

12,14,14,14,14,18

| RI | Layer | L | p |
| --- | --- | --- | --- |
| 1 | 0 | x0 | y0 |
| 2 | 0 |  |  |
|  | 1 |  |  |
| 3 | 0 |  | v0 |
|  | 1 |  |  |
|  | 2 |  |  |
| 4 | 0 |  |  |
|  | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |

Subset selection (as a means of compression)

Terms:

- “FD basis subset selection” refers to the selection of M out of N3 FD DFT vectors

- “Coefficient subset selection” refers to the selection of KNZ (# non-zero coefficients) out of 2LM where KNZ ≤ K0

On basis/coefficient subset selection for RI=1, the following is supported:

- Common selection for all beams with size-K0 subset of 2LM reported

  - Note: Wf=fk0 fk1…fkM-1, i.e. Mi=M ∀i and ki,mm=0Mi-1 are identical (i.e., ki,m=km, i=0,…,2L-1), and kmm=0M-1 are all selected from the index set {0,1,…,N3-1} from the same orthogonal basis group

  - The value of K0 is configured via higher-layer signaling

    - The number of reported non-zero coefficients can be smaller than or equal to K0

- Size-K0K0 subset design: Unrestricted (polarization-independent) subset selection which requires a size-2LM bitmap in UCI part 2

- The value of K0:  K0=β×2LM where  (the value of K0 is higher-layer configured via ).β∈18,14,12,34

On subset selection for RI=2:

- SD basis selection (selection of L out of N1N2 SD DFT vectors) is layer-common

- Layer-independent FD basis subset selection

- Layer-independent coefficient subset selection

- The size-K0 subset design for layer 0 is also applied to layer 1

- K0 is the maximum number of non-zero coefficients for each layer.

On RI=3-4 extension:

- SD basis subset selection is layer-common

- Layer-independent FD basis subset selection

- Layer-independent coefficient subset selection

- With the agreed total max # NZ coefficients across all layers ≤ 2K0 where the K0 value (hence β) set for RI{1,2}, KNZ,i≤K0 (the number of non-zero coefficients per layer shall be less than or equal to K0) as long as i=0RI-1KNZ,i≤2K0

  - No further restriction on the maximum number of non-zero coefficients per layer

The following FD basis subset selection scheme is supported:

- For N3≤19N3≤19, one-step free selection is used:

  - FD basis subset selection indicator is per layer where it is log2N3-1Mi-1-bit indicator, (i=0,1,…,(RI-1))

- For N3>19, IntS is window-based and fully parameterized with MinitialMinitial, indicating that the intermediate set consists of FD bases mod(Minitial + n, N3), n=0,1, …,  modMinitial+n,N3, n=0,1,⋯N3'-1

  - The value N3'=αM where  is higher-layer configured from two possible values

  - Finalize the values of  via down selection from {1.5, 2, 2.5}, FFS =3 as an additional candidate

- The 2nd step subset selection is indicated by an X2-bit combinatorial indicator (for each layer) in UCI part 2

Supported parameter combinations

FFS (to be finalized in RAN1#98 Prague): the supported parameter combinations for (L, p, β, )L,p,β,α

## LCC quantization

On LCC quantization, denote the LC coefficient associated with beam l ∈0,1,…,2L-1 and frequency unit m ∈0,1,…,M-1 as cl,m, and the strongest coefficient (out of the KNZ non-zero coefficients reported using the bitmap) as cl*,m*.

UE reports the following for the quantization of the non-zero coefficients in W2

- (For RI=1) A log2KNZ-bit indicator for the strongest coefficient index l*,m*

  - Strongest coefficient cl*,m*=1 (hence its amplitude/phase are not reported)

- Two polarization-specific reference amplitudes:

  - For the polarization associated with the strongest coefficient cl*,m*=1, since the reference amplitude = 1, it is not reported

  - For the other polarization, reference amplitude is quantized to 4 bits

    - The alphabet is {1,1214,1414,1814,…,121414 , 0} (-1.5dB step size)

- For cl,m,  l,m≠(l*,m*):

  - For each polarization, differential amplitudes of the coefficients calculated relative to the associated polarization-specific reference amplitude and quantized to 3 bits

    - The alphabet is {1,12,12,122,14,142,18,182} (-3dB step size)

    - Note: The final quantized amplitude pl,m is prefl,m×pdiffl,m and the coefficient is given by cl,m=preflL×pl,m×φ(l,m)

- Each phase is quantized to either 8PSK (3-bit) or 16PSK (4-bit) (configurable)

On “zero” in the reference amplitude value set, “zero” is removed and the associated code point is designated as “reserved”.

- Note: “Reserved” implies that the associated code point is not used in reference amplitude reporting or, at least in Rel-16, any other purpose(s)

For RI∈2,3,4, different layers are independently quantized.

## UCI parameters

The UCI consists of two parts:

- Information pertaining to the number(s) of non-zero coefficients is reported in UCI part 1

  - Note: This does not imply whether this information consists of single or multiple values

- The payload of UCI part 1 remains the same for different RI value(s)

- Bitmap is used to indicate non-zero coefficient indices

| Parameter | Location | Details/description |
| --- | --- | --- |
| # NZ coefficients | UCI part 1 | RI ({1,…, RIMAX}∈1,…,RIMAX) and KNZ,TOTKNZ,TOT (the total number of non-zero coefficients summed across all the layers, where KNZ,TOTKNZ,TOT {1,2,…, 2K0}KNZ,TOT∈1,2,…,2K0 are reported in UCI part 1 - FFS: If the total number of non-zero coefficients are jointly encoded with M’ (if supported) or independently encoded |
| Wideband CQI | UCI part 1 | Same as R15 |
| Subband CQI | UCI part 1 | Same as R15 |
| Bitmap per layer | UCI part 2 | RI=1-2: for layer l, size-2LM For RI=3-4, bitmaps, each with size-2LMi (i=0,1,…, RI-1, where i denotes the i-th layer) are reported in UCI part 2 - FFS: size-2LMi2LMi-1 (i=0,1,…, RI-1i=0,1,…,RI-1, where i i denotes the ii-th layer) are reported in UCI part 2 |
| Strongest coefficient indicator (SCI) | UCI part 2 | RI=1: the existing agreement on SCI: “A log2KNZ-bit indicator for the strongest coefficient index l*,m*” RI>1: See Table below |
| SD basis subset selection indicator | UCI part 2 | SD basis subset selection indicator is a -bit indicator. Details follow Rel.15 |
| FD basis subset selection indicator | UCI part 2 | See Table below |
| LC coefficients: phase | UCI part 2 | Quantized independently across layers |
| LC coefficients: amplitude | UCI part 2 | Quantized independently across layers (including reference amplitude for weaker polarization, for each layer) |
| SD oversampling (rotation) factor q1, q2 | UCI part 2 | Values of q1, q2 follow Rel.15 |

| SCI and FD basis subset selection indicator |  |
| --- | --- |
| SCI for RI>1 | Per-layer SCI, where SCIi is a log22L–bit (i=0,1,…,(RI-1)) indicator. The location (index) of the strongest LC coefficient for layer i before index remapping is (li*,mi*), SCIiindicates li*, and mi* is not reported |
| Index remapping | For layer i, the index mi of each nonzero LC coefficient cli,mi is remapped with respect to mi* to mi such that mi*=0. The FD basis index kmi associated to each nonzero LC coefficient cli,mi is remapped with respect to kmi* to kmi such that kmi*=0. The sets cli,mi≠cli*,0 and kmi≠0 are reported. Informative note (for the purpose of reference procedure): The index li,mi of nonzero LC coefficients is remapped as (li,mi)→(li,mi-mi*modMi). The codebook index associated with nonzero LC coefficient index li,mi is remapped as kmi→kmi-kmi*modN3. |
| Combinatorial indicator for N3≤19 | log2N3-1Mi-1 bits |
| Combinatorial indicator for N3>19 | log2N3'-1Mi-1 bits |
| Minitial | Reported in UCI part 2, details on bitwidth and possible values are FFS |

# References

- RP-182320, RAN#82, Samsung, Status report to TSG: Enhancements on MIMO for NR

- RP-190149, RAN#83, Samsung, Status report to TSG: Enhancements on MIMO for NR

- RP-191187, RAN#84, Samsung, Status report to TSG: Enhancements on MIMO for NR