3GPP TSG RAN WG1 RAN1 Meeting #98	R1-1908508

Rrague, CZ, August 26th – 30th, 2019

Agenda Item:		7.2.9.4

Source:		Samsung

Title:		On UE assistance information

Document for:		Discussion and decision

# Introduction

In RAN#84, the WID for UE power saving is revised and updated. As in [1], it was agreed to specify the power saving technique of UE adaptation to the maximum number of MIMO as below:

4) Specify, if agreed, the mechanism to provide UE assistance information [RAN2, RAN1]:

- Study and select among the following UE assistance information for RAN2 by RAN#85:

power preferred information (baseline LTE PPI in a well-defined manner) and UE's preference on C-DRX, BWP and SCell configuration [RAN2].

NOTE: additional UE assistance information for RAN1-specific power saving techniques to indicate what setting of parameters related to this Work Item will lead to power savings for the UE can be included if agreed in RAN1.

In this contribution, we consider UE assistance information to assist UE adaptation schemes in our companion contributions [2-4].

# UE assistance information

From a UE power saving perspective, it is preferable for a UE to turn on its RF and baseband as needed according to the practical traffic demand and real-time channel condition. Compared with a gNB, a UE is immediately aware of its mobility/battery status and changes on link characteristics. These factors make the UE more capable of making an accurate adaptation request for a configuration of parameters for power saving purpose. For some adaptive parameters, such as the activated CCE ALs or (de)activation on CORESET, it may even not be possible for the gNB to determine a reconfiguration without any feedback on PDCCH CSI from UE.

UE assistance information can be transmitted through MAC CE or PUCCH depending on the latency requirements.

## 2.1 Preferred values for adaptive configuration parameter

During SI phase of UE power saving, a variety of UE adaptation schemes triggered by L1 power saving signal/channel have been studied and showed to achieve remarkable power saving gains in RRC_CONNECTED state. To improve the efficiency of adaptation schemes, the indicated applicable value for adaptive parameter(s) can be determined based on assistance information on preferred values transmitted from the UE. Therefore, the first type of UE assistance information we considered is UE’s preferred values for different adaptive configuration parameters.

Preferred processing timeline relaxation

For adaptation on processing timeline or cross-slot scheduling, a unified processing delay, Kmin,, applicable for minimum values for K0/K2, aperiodic CSI-RS triggering offset and SRS triggering offset, can be indicated by a scheduling DCI for power saving purpose [2].

To assist the adaptation on Kmin, a list of candidate values for Kmin can be determined based on UE assistance information on its preferred applicable values. For example, to reduce signaling overhead without sacrificing power saving gain, two candidates, i.e. a default candidate and a non-zero candidate, can be considered. A UE can report at least its preferred value for the non-zero candidate based on UE capability or its needs for power saving. The default value can be zero or the minimum applicable value in configured TDRA tables. In this case, a binary bit in scheduling DCI is enough to trigger UE adaptation on Kmin. For another example, UE can report more than one preferred candidate values for Kmin, the indicated Kmin in a scheduling DCI from gNB is selected from one of the preferred candidate values.

The list of candidate values can be updated by UE on-demand through assistance information report via MAC CE.

Proposal 1: Enable UE assistance information on one or more preferred values for unified minimum applicable value Kmin regarding PDCCH processing timeline relaxation.

Proposal 2: Kmin indicated by a scheduling DCI is selected from one of the preferred values reported by UE.

Preferred maximum MIMO layers

For adaptation on maximum MIMO layers proposed in our companion contribution [3], a maximum MIMO layer for PDSCH/PUSCH, Lmax,, can be configured per BWP. UE is able to adapted to different maximum MIMO layer based on BWP switching. A gNB can trigger adaptation on Lmax based on the real-time DL traffic or RI in CSI report. Alternatively, gNB can trigger the adaptation on Lmax based on the feedback from UE. For example, a UE can indicate the preferred maximum MIMO layers based on its implementation or need on activated number of RX antennas.

Proposal 3: UE reports assistance information on preferred maximum MIMO layers.

Preferred adaptation on PDCCH monitoring

For adaptation on PDCCH monitoring as proposed in our companion contribution [4], the relevant adaptive configuration parameters can be PDCCH monitoring periodicity, and (de)activated CORESETs/CCE ALs. It’s beneficial for UE to report its preferred values for the associated adaptive parameters. A gNB can use the preferred values as candidate values, and indicate one of the preferred values to UE through physical layer signal/channel.

Proposal 4: Enable UE assistance information on preferred PDCCH monitoring adaptation candidates, at least for

- PDCCH monitoring periodicity/duration

- (de)activated CORSETs

- (de)activated CCE ALs

## 2.2 PDCCH CSI report

For some adaptation request, such as activated CCE ALs, and deactivation of all search space sets in CORESET p, a real-time channel condition of PDCCH is needed. Therefore, it’s preferable for UE to report PDCCH CSI in order for gNB to trigger an optimal adaptation on PDCCH monitoring configuration.

As the valid time of CSI is short, CSI for PDCCH should be reported through a PUCCH/PUSCH. The PDCCH CSI can be reported similar to PDSCH CSI in periodic/semi-persistent/aperiodic manner. When a power saving signal/channel is transmitted in multi-beam manner, the DMRS of PDCCH based power saving signal/channel can be used as RS resources for PDCCH CSI report.

Proposal 5: Enable UE assistance information of PDCCH CSI report for adaptation in respective CORESETs.

## 2.3 Preferred power saving signal/channel configuration

A preferred configuration for power saving signal/channel depends on the channel condition such as delay spread, Doppler shift, etc. Therefore, support for UE assistance information can be considered for the preferred power saving signal/channel parameters as they cannot be always accurately estimated by the gNB, for example in FDD as the DL channel can be different than the UL channel. Moreover, a gNB has less motivation to implement such parameter estimation than a UE and this may make a deployment of power saving signal/channel less likely.

For the UE assistance information about preferred power saving signal configuration, reporting can be done by higher layers, such as MAC CE.

Proposal 6: Enable UE assistance information on preferred search space set/CORESET configuration for power saving signal/channel, such as CORESET BW, PDCCH monitoring periodicity, DMRS precoder granularity.

## 2.4 UE triggered power saving mode

To reduce the signaling overhead on UE assistance information about UE’s preferred adaptations, a UE can send an assistance information about its preferred power saving mode/state in general. In this case, NW can enable different power saving schemes accordingly to meet the actual needs of UE.

As illustrated in Figure 1, a gNB can provide UE a configuration of different power saving modes/states it can supports first. Different power saving modes/states can associated with different power saving schemes, or indicate different power saving levels. The number of power saving modes/states can be extended further in further release if more stringent requirements appears. In next step, a UE can feedback its preference on preferred power saving mode/state according to its needs and real-time channel conditions. For example, when the battery level is very low, a UE can send assistance information through a MAC CE to request for the highest level of power savings. In the last step, gNB can enable corresponding power saving schemes or trigger UE adaptation associated with UE’s preferred power saving mode. The last step about gNB triggered UE adaptations is currently under discussion in the agenda item of PDCCH based power saving signal/channel.

FIGURE 1: Procedure of UE triggered power saving mode

Proposal 7: Support UE triggered power saving mode switching.

# Conclusions

This contribution considers UE assistance information for power saving schemes in NR. Our proposals are as follows.

Proposal 1: Enable UE assistance information on one or more preferred values for unified minimum applicable value Kmin regarding PDCCH processing timeline relaxation.

Proposal 2: Kmin indicated by a scheduling DCI is selected from one of the preferred values reported by UE.

Proposal 3: UE reports assistance information on preferred maximum MIMO layers.

Proposal 4: Enable UE assistance information on preferred PDCCH monitoring adaptation candidates, at least for

- PDCCH monitoring periodicity/duration

- (de)activated CORSETs

- (de)activated CCE ALs

Proposal 5: Enable UE assistance information of PDCCH CSI report for adaptation in respective CORESETs.

Proposal 6: Enable UE assistance information on preferred search space set/CORESET configuration for power saving signal/channel, such as CORESET BW, PDCCH monitoring periodicity, DMRS precoder granularity.

Proposal 7: Support UE triggered power saving mode switching.

# References

[1] RP-191607, New WID: UE Power Saving in NR

[2] R1-1908506, “Cross-slot scheduling power saving techniques” Samsung, RAN1#98, August, 2019

[3] R1-1908607, “UE adaptation to maximum number of MIMO layers” Samsung, RAN1#98, August, 2019

[4] R1-1908505, “PDCCH-based power saving signal/channel” Samsung, RAN1#98, August, 2019