3GPP TSG RAN WG1 #98	        		             R1-1908516

Prague, Czech Republic, August 26th – 30th, 2019

Agenda item:	7.2.13.3

Source: 	Samsung

Title: 				    On SCell Activation/Deactivation

Document for:	Discussion and Decision

# Introduction

This contribution considers potential mechanisms for enabling fast access to deactivated or “dormant” SCells for a UE.

# Reducing Latency for SCell Access

Reduced SCell access has been considered either by reducing activation time for a deactivated SCell or by utilizing BWP configurations and BWP switching to functionally achieve an operation of a “dormant cell” as

in LTE.

The latency of the MAC-CE based SCell activation in Rel-15 is the sum of:

- THARQ: time between PDSCH with MAC CE and corresponding HARQ-ACK.

- Tactivation_time: time required for RF warm up, AGC setting and time-frequency tracking.

- TCSI_Reporting: time until CSI-RS reception, UE processing time for CSI measurement, generation of CSI report, PUCCH transmission with CSI report.

Several mechanisms have been proposed to reduce each of the above components

- THARQ: Use DCI instead of MAC CE

  - Requires substantial reduction in Tactivation_time and TCSI_Reporting to offer any meaningful gains. Also, MAC CE processing delay is only 0.5 msec.

  - Presumes no HARQ-ACK feedback for the DCI activating the SCell. This is problematic because the HARQ-ACK codebook is based on the number of configured cells, there is no differentiation between NACK and DTX. The gNB has no way of “quickly” knowing whether the UE transmits NACK for an incorrectly decoded TB or because the UE failed to detect the DCI format for SCell activation - then there is a waste of PDSCH transmissions that increases as the number of activated SCells increases.

  - Cannot offer the flexibility of MAC CE for SCell activation/deactivation in any meaningful way.

  - Requires changes to Rel-15 DCI formats and additional signaling overhead.

- Tactivation_time: This is the largest component in SCell activation delay and Tactivation_time = TSMTC_SCell + 5msec [1]

  - A typical value for TSMTC_SCell is 20 msec (to balance overhead from SS/PBCH blocks and required time for time-frequency sync) while the 5 msec is needed for MAC-CE processing delay, RF warm up/AGC tracking, and to cover SMTC window uncertainty

  - Proposed mechanisms to reduce Tactivation_time include

    - Use A-CSI-RS instead of SS/PBCH blocks for AGC and time-frequency tracking. This represents additional signaling overhead (PDCCH and A-CSI-RS), new UE measurement requirements, new DCI format design and, although it can eliminate the “TSMTC_SCell + 5msec” component, a delay for CSI-RS transmission/measurements remains and optimizations for CSI reporting are needed.

    - Keep the RF on for deactivated SCells, at least for intra-band CA, to avoid RF warm up time. This practically defeats the purpose of deactivating an SCell.

- TCSI_Reporting: This component can be assumed unchanged relative to Rel-15 possible values.

From the above, the following observations are made:

- Using DCI/L1 instead of MAC CE for SCell activation/deactivation does not provide a material benefit.

- Using A-CSI-RS instead of SS/PBCH blocks for AGC and time-frequency tracking may materially reduce SCell activation time but requires broad specification support (DCI format design, A-CSI-RS triggering, CSI reporting, …) and new UE measurements with yet unknown reliability compared to SS/PBCH blocks.

Considering the above and that NR Rel-15 has embedded capability for “dormant cell” functionality using BWPs, the following is proposed.

Observation 1: Enhancements to Rel-15 NR SCell activation procedure are not further considered.

NR Rel-15 provides a flexible framework to control the UE behavior through configuration of BWPs and of different values for parameters per BWP. It can also avoid one of the main structural problems of SCell activation in that it is reactive (gNB needs to wait for UE to first receive/process signaling for SCell activation and then perform RF warm up, sync, CSI measurement, CSI report, … before being able to schedule the UE) instead of proactive (gNB has all necessary information available and UE is ready for scheduling at any time).

The discussion in RAN1 on using BWPs to functionally obtain a “dormant” state for a SCell has focused on using L1 signaling to switch BWPs for one or more SCells. However, L1-based BWP switching is an optional UE feature and so is configuration of multiple BWPs to a UE. Therefore, “dormant cell” operation should be enabled at least by RRC signaling. This is already possible in Rel-15 since a BWP can be configured with or without search space sets. Moreover, configuration by higher layers (including MAC CE) enables adjustments to other UE operations such as periodic CSI-RS measurements and CSI reporting (for example, they can be made more infrequent when the SCell is in “dormant” state).

Observation 2: NR Rel-15 allows for SCell “dormancy” based on RRC signaling.

The next issue is whether, for UEs supporting L1-based BWP switching, it is beneficial to switch SCell(s) in and out of a “dormant” state based on L1 signaling instead of RRC signaling or MAC signaling. Although a UE can process L1 signaling a few msec faster than RRC signaling and ~0.5 msec faster than a MAC CE, there are additional components determining a delay for scheduling a UE on an SCell that is switch out of a “dormant” state, including:

- BWP switching delay – 3 msec

- CSI measurement and reporting delays – UE needs to switch BWP and then obtain CSI from P/SP-CSI-RS or be triggered an A-CSI-RS reception by DCI format 0_1 (UE can then report CSI in a PUSCH) – enhancements may be beneficial but are also likely to require material specification support.

- Delay due to the UE being outside DRX Active Time when traffic arrives.

- Delay associated with use of WUS for power savings on the scheduling cell (UE cannot be reached for a number of DRX cycles after indication to skip PDCCH monitoring).

Considering the above delay components, the relative latency reduction from using L1 signaling over RRC signaling for switching the active BWP for a UE on a SCell is not material. Potential improvements are:

- Configure same physical BWP for “dormant” state and “non-dormant” state to avoid BWP switching delay – this can be by gNB implementation but it needs to be clarified that there is no BWP switching delay if only the BWP index changes (the physical BWP does not change).

- Allow A-CSI-RS to be triggered by the same DCI format that triggers the active BWP switching (currently only DCI format 0_1).

The next issue is how the L1 signaling, from a scheduling cell, for BWP switching on a scheduled is done. One approach is to use a DCI format 0_1 (to also trigger A-CSI-RS measurement and report in a corresponding PUSCH). Some fields will need to be reinterpreted for the UE to determine the new functionality of DCI format 0_1. Also, DCI format 0_1 should be enabled to simultaneously trigger BWP switching and A-CSI-RS for multiple cells; otherwise, overhead and delay increase (as the UE may not expect to receive multiple DCI format 0_1 for the scheduling cell at a same PDCCH monitoring occasions and there may also be PDCCH blocking). It is unclear (and probably not possible) whether DCI format 0_1 can schedule a PUSCH while also providing the above functionalities. Otherwise, a new DCI format (size matched to an existing one) needs to be introduced and differentiated (e.g. by an RNTI or by a setting of a field value) from a Rel-15 unicast DCI format. Also, for CSI reporting, either additional delay is incurred or additional mechanisms need to be introduced.

Another approach is to link BWPs for the scheduling cell with BWPs for a group of SCells. This can offer rough granularity as not all SCells in the group of SCells may need to be activated at a given time. Further, relative to Rel-15, all delays other than the RRC delay remain. Therefore, this approach does not offer material functional (CSI-RS triggering/reporting for SCells) or latency gains over the Rel-15 NR capability for RRC-based BWP switching that can also offer flexible granularity for applicable SCells.

Observation 3: An explicit L1 approach that combines triggering an SCell out of “dormant” state, A-CSI-RS triggering on the SCell, CSI report for the SCell, and flexibility in indicating corresponding SCells is preferable in terms of latency and flexibility over implicit L1 approaches.

Rel-15 NR allows for MAC CE based adaptation of CSI measurement/reporting for SCells while configuration of monitoring periodicity for search space sets is by RRC signaling. Introducing a scaling for the monitoring periodicity of search space sets in MAC CE can enable all functionalities targeted by L1 signaling with much less complexity and probably more flexibility at the expense of an immaterial increase (~0.5 msec) in an expected time required for activating/deactivating “dormant” UE operation on SCells. Further, unlike explicit L1-based signaling, HARQ-ACK feedback is provided and this practically prevents costly errors.

Observation 4: Enhancing Rel-15 MAC CE is a simpler and potentially more flexible option than introducing new L1 signaling for activating/deactivating “dormant” UE operation on an SCell.

Proposal: RAN1 should first conclude on a total expected delay for activating/deactivating “dormant” SCells by L1/MAC/RCC and then compare explicit L1 signaling to Rel-15 RRC signaling or to potential enhancements of Rel-15 MAC CE signaling.

# Conclusions

This contribution considered potential mechanisms for reductions in a scheduling latency on a SCell and proposes the following.

Proposal: RAN1 should first conclude on a total expected delay for activating/deactivating “dormant” SCells by L1/MAC/RCC and then compare explicit L1 signaling to Rel-15 RRC signaling or to potential enhancements of Rel-15 MAC CE signaling.

In addition, the following observations are made.

Observation 1: Enhancements to Rel-15 NR SCell activation procedure are not further considered.

Observation 2: NR Rel-15 allows for SCell “dormancy” based on RRC signaling.

Observation 3: An explicit L1 approach that combines triggering an SCell out of “dormant” state, A-CSI-RS triggering on the SCell, CSI report for the SCell, and flexibility in indicating corresponding SCells is preferable in terms of latency and flexibility over implicit L1 approaches.

Observation 4: Enhancing Rel-15 MAC CE is a simpler and potentially more flexible option than introducing new L1 signaling for activating/deactivating “dormant” UE operation on an SCell.

# References:

- TS38.133 v15.6.0, “NR; Requirements for support of radio resource management”