3GPP TSG RAN WG1 Meeting #98   	    	           R1-1908550

Prague, CZ, August 26th – 30th, 2019

Agenda Item:	7.2.9.3

Source: 	LG Electronics

Title: 	Discussion on MIMO layer/number of antenna adaptation

Document for:	Discussion and decision

- Introduction

In RAN#84 meeting, it was decided to include UE adaptation on the maximum number of MIMO layers in the Rel-16 NR UE power saving work item as follows [1]. More specifically, it was decided to support such an adaptation by supporting different MIMO layer configuration for different BWPs while details are to be decided further. In this contribution, we discuss further aspects to support adaptive MIMO layer configuration and suggest some details.

- Specify the power saving techniques of UE adaptation to the maximum number of MIMO layers [RAN1, RAN2, RAN4] - Specify configuration of a different MIMO layer configuration of the initial/default BWP compared with other BWPs of a Serving Cell. [RAN2, RAN4] - Discuss whether to also extend this to define per-BWP MIMO layer configuration [RAN1, RAN2] - Evaluate if switching and interruption times for UE dynamic adaptation to the maximum number of MIMO layers are needed and which case assuming a relationship between the number of RF ports and the MIMO layer configuration [RAN4] NOTE: Switching on/off the RF is part of the evaluation

- Discussion

Based on the contributions in last RAN1 meetings, there were two approaches suggested for MIMO/antenna adaptation for UE power saving:

- Adapting schedulable maximum number of DL MIMO layers [2][3][4][5]

- Adapting number of UE’s RX antennas [2][6]

Since it was already decided to support MIMO/antenna adaptation for UE power saving by support different MIMO layer configurations for different BWPs, approach 2) above is understood as out of the scope as of now. Furthermore, it is not a conventional way of 3GPP specifications to mandate UE’s exact number of RX antenna ports and it has been UE’s implementation choice as long as the implementation satisfies DL reception performance requirements. Therefore, we only consider the approach 1) above, that is, dynamic adaptation of schedulable maximum number of MIMO layers for UE power saving.

In the UE’s power saving perspective, reducing maximum number of DL MIMO layers may not directly guarantee the UE can reduce its number of active RX antenna ports or how many RX antenna ports it can deactivate. This is because reducing number of RX antenna ports usually leads to RX performance degradation. Therefore, without knowing network’s expectation to the UE’s RX performance for DL reception, UE may not reduce its number of RX antenna ports. Similarly, without knowing what the UE’s DL reception performance will be after adapting the maximum number of DL MIMO layers, network may not be able to decide whether/how it should adapt the UE’s maximum number of DL MIMO layers.

Observation 1

Reducing maximum number of DL MIMO layers may not guarantee the UE can reduce its number of active RX antenna ports or how many RX antenna ports it can deactivate since the UE doesn’t know what the network’s target performance for DL reception is.

Observation 2

Network may not be able to decide whether/how it should adapt the UE’s maximum number of DL MIMO layers, without knowing what the UE’s DL reception performance will be after adapting the maximum number of MIMO layers.

It is natural to keep it up to UE implementation how many RX antenna ports to be activated or deactivated for the sake of power saving according to the network’s adaptation of MIMO configuration for the UE. Therefore, network’s control on the UE’s maximum DL MIMO layers may not be based on any assumption on the UE’s number of active RX antenna ports. In that sense, exploiting CSI feedback from the UE will be a simple and direct way of achieving aligned knowledge between network and UE on the DL RX performance according to the different potential maximum number of DL MIMO layers. There can be various ways of reusing existing CSI feedback frameworks for this purpose. One way is network directly triggering CSI feedback for a candidate MIMO configuration based on the existing aperiodic CSI feedback framework.

Proposal 1

Support UE’s CSI feedback based on candidate number(s) of maximum DL MIMO layers which is(are) different from that configured for the current activated BWP, for the purpose of dynamic adaptation of the UE’s DL MIMO configuration.

As quoted in the introduction, it is still for further discussion on whether different MIMO layer configurations can be configured per each configured BWP in a cell or only between initial/default BWP and other BWPs in a cell. In our view, there is no clear necessity to restrict support of power saving via MIMO layer adaptation to a specific BWP(s), especially for the initial/default BWP which may be characterized by a common usage among many UEs operating in a same serving cell. Therefore, if to support dynamic MIMO layer adaptation based on BWP adaptation, it is desirable to support maximum number of DL MIMO layers configurable for each BWP configured for the UE for a serving cell.

Proposal 2

Support independent configuration of maximum number of DL MIMO layers for each BWP configured to the UE for a serving cell.

- Conclusion

In this contribution, we discuss further aspects to support adaptive MIMO layer configuration and suggest some details. Observations and proposals of this contributions are as follows.

Observation 1

Reducing maximum number of DL MIMO layers may not guarantee the UE can reduce its number of active RX antenna ports or how many RX antenna ports it can deactivate since the UE doesn’t know what the network’s target performance for DL reception is.

Observation 2

Network may not be able to decide whether/how it should adapt the UE’s maximum number of DL MIMO layers, without knowing what the UE’s DL reception performance will be after adapting the maximum number of MIMO layers.

Proposal 1

Support UE’s CSI feedback based on candidate number(s) of maximum DL MIMO layers which is(are) different from that configured for the current activated BWP, for the purpose of dynamic adaptation of the UE’s DL MIMO configuration.

Proposal 2

Support independent configuration of maximum number of DL MIMO layers for each BWP configured to the UE for a serving cell.

- Reference

- [1]	RP-191607, “New WID: UE Power Saving in NR,” CATT, CAICT

- [2]	R1-1906176, “Discussion on UE adaptation in antenna domain,” vivo

- [3] 	R1-1906550, “Recommendations on UE Adaptation,” MediaTek

- [4]	R1-1907327, “Discussion on MIMO layers,” Ericsson

- [5]	R1-1906354, “Power saving scheme with UE adaptation to maximum MIMO layer,” CATT

- [6]	R1-1906591, “UE dynamic adaptation to the maximum number of receive antennas,” Huawei, HiSilicon