3GPP TSG RAN WG1 Meeting #98		R1-1908574
 Prague, Czech Republic, August 26th – 30th, 2019

Source:	CATT

Title:	Summary of UE and gNB measurements for NR Positioning

Agenda Item:	7.1.10.3

Document for:	Discussion and Decision

Introduction

This document provides a summary of the issues, proposals, and suggested solutions for AI 7.1.10.3 related to UE and gNB measurements for NR Positioning based on the contributions submitted under this AI [3-17]. It covers the following aspects on to UE and gNB measurements for NR Positioning:

- UE measurements for NR positioning

  - UE RSTD measurements

  - UE Rx-Tx time difference measurements for NR positioning

  - DL PRS-RSRP measurements for NR positioning

  - Additional UE measurements

  - Other issues related to UE measurements

- gNB measurements for NR positioning

  - UL RTOA measurements for NR positioning

  - gNB Rx-Tx time difference measurements for NR positioning

  - UL SRS RSRP measurements for NR positioning

  - UL angle of arrival measurements for NR positioning

  - Additional gNB measurements for NR positioning

- UE and gNB measurement reporting

  - Beam indication in reporting

  - Quality indication in reporting

  - Multi-path measurements

  - Other issues on UE and gNB Measurement reporting

- Mapping between positioning techniques, the DL/UL PRS and the measurements

- E-CID downlink measurements

- UE and gNB measurement configuration

- Other important issues related to UE/gNB measurements

Please note of the following:

- The YELLOW highlights are proposals and issues for discussion during this meeting

- The BLUE highlights are offline consensus/conclusion based on offline discussion or comments

- The GREYed sections are issues that have been discussed or resolved, and no further discussion is expected during this meeting.

UE measurements for NR positioning

According to the WID [1], the following UE measurements for NR positioning need to be specified for serving, reference, and neighboring cells

  - DL RSTD (reference signal time difference) measurements for NR positioning

  - DL RSRP (reference signal received power) measurements for NR positioning

  - UE RX-TX time difference measurements for NR positioning

## UE RSTD measurements

Summary

Most of the contributions have discussed the UE RSTD measurements [3-17]. The discussion and proposals focus on the following issues:

- Issue 1:  The following agreement was made in RAN1#97:

  - Select one (or more) of the following options for the definition of the RSTD:

    - Option 1: RSTD is defined as the time difference with respect to the subframe timings associated with the different TPs;

    - Option 2: RSTD is defined as the time difference with respect to the timings associated with the DL PRS resource sets of different TPs

    - Option 3: RSTD is defined as the time difference with respect to the timings associated with the DL PRS resources of different TPs

    - Note: Definition of the RSTD applies at least to the RSTD measurements obtained from DL PRS. It does not preclude the RSTD obtained from other DL RS.

Most companies have expressed their views on above options. Given that lengthy discussion was conducted on this issue in previous meetings, and the decision on this issue may have impact on the definition of other UE/gNB measurements, it is important for us to make the decision on this issue in this meeting.

The following table summarises the options supported by the companies based on the contributions:

Table 1 Summary of the RSTD definition

| Opinions | Companies |  |
| --- | --- | --- |
|  | Yes | No |
| Option 1 | Huawei, HiSilicon, MTK, CATT, Samsung, Qualcomm, Futurewei |  |
| Option 2 | ZTE, OPPO, Samsung, LGE, Intel |  |
| Option 3 | vivo, Nokia, OPPO, Samsung, LGE, Sony, CMCC, Intel, Futurewei |  |

- Issue 2: How the reported RSTD measurement is determined when receiver diversity is used by UE:

  - based on the first detected path among the detected paths from all receiver branches (CATT)

  - The behavior for Rx diversity in DL timing based measurements is left up to the UE implementation, as long as the requirements of accuracy of the timing of the first path are met (Huawei)

  - Use of RX diversity for received timing estimation is left up to UE implementation and is not specified by measurement definition (Intel)

- Issue 3: One company made a proposal to consider the transmitting time offset between different detected beams in RSTD measurement (ZTE).

  - “The relative timing difference between the ng-RAN neighbour cell j and the NG-RAN reference cell i, defined as TPRSresourceRxj – TPRSresourcexi －Tbeamoffset, where TPRSresourceRxj is the time when the UE receives the start of one PRS resource from ng-RAN cell j; TPRSresourceRxi is the time when the UE receives the corresponding start of PRS resource from NG-RAN cell i that is closest in time to the PRS resource received from NG-RAN cell j. The reference point for the observed PRS resource time difference shall be the antenna connector of the UE. Tbeamoffset is the transmitting time offset between different detected beams.”

- Issue 4: One company made the proposal to support RSTD measurements with mixed E-UTRAN and NR node (MTK)

- Issue 5: One company made the proposal to include RRC IDLE and RRC INACTIVE mode as applicable states for RSTD measurement (Nokia)

- Issue 6: One company made the proposal that DL RSTD (reference signal time difference) measurements can be based on existing CSI-RS/TRS for NR positioning as a baseline and on the new PRS when necessary (Samsung)

Company’s views

| Company |  | Comments |
| --- | --- | --- |
| Huawei/Hisi | We would like to draw companies’ attention that all timing related measurements (including RSTD, UE Rx – Tx time difference, UL-RTOA, gNB Rx – Tx time difference) would have to face the same discussion in issue 1 and issue 2. It is not a RSTD specific issues as the FL mentioned. Meanwhile, for RSTD, it is the timing between two cells or two beams from two TPs (as proposed by some companies). The problem of beam-pair specific RSTD definition is that the reporting quantity would increase quadratically with the number of beams per TP, which increase overhead that is unnecessary, yet can be easily overcome with TP-pair specific RSTD and multi-path reporting that only scales linearly with the number of beams per TP. For issue 2, we are OK if the first path among all receiver branches is specified; however, we should make sure that how to combine all receiver branches is up to UE implementation, which means it should not limit the UE behavior to only estimating the first path per receiver branch and finding the first one. For example, UE may combine coherently or non-coherently the channel estimates across receiver branches. For Issue 4, we think it should be discussed at an early stage. It may have some impact on PRS mapping pattern if LTE and NR cells are deployed on the same frequency layer. |

Proposal

- In RAN1#98, decide which of the following option(s) is (are) are used for the definition of the RSTD:

  - Option 1: RSTD is defined as the time difference with respect to the subframe timings associated with the different TPs;

    - Supported by:

  - Option 2: RSTD is defined as the time difference with respect to the timings associated with the DL PRS resource sets of different TPs:

    - Supported by:

  - Option 3: RSTD is defined as the time difference with respect to the timings associated with the DL PRS resources of different TPs:

    - Supported by:

  - Regardless which option is selected, UE may use multiple DL PRS resources to determine the timing of the first path of the cell. UE should support reporting the PRS resource ID(s) used for determining the timings of the cells in RSTD measurements.

Proposal

- Select one of the following options to determine the reported RSTD measurement when receiver diversity is used by UE:

  - Option 1: based on the first detected path among the detected paths from all receiver branches

  - Option 2: up to the UE implementation, as long as the requirements of accuracy of the timing of the first path are met

  - Option 3: up to UE implementation and is not specified by measurement definition

Issues for further discussion

- whether to support RSTD measurements for RRC IDLE and RRC INACTIVE mode in Rel-16

- whether RSTD measurement is supported with mixed E-UTRAN and NR node in Rel-16

- whether DL RSTD (reference signal time difference) measurements can be based on existing CSI-RS/TRS for NR positioning as a baseline

## UE Rx-Tx time difference measurements for NR positioning

Summary

Most contributions have discussed the UE Tx time difference measurements (e.g., [2-14]. The discussion and proposals focus on the following issues:

- Issue 1: Whether the UE Rx-Tx time difference should be defined with respect to the frame timing associated with the cells, or with the PRS resource sets, or with the PRS resources:

  - Option 1: UE Rx-Tx time difference is defined with respect to the frame timing associated with the TP;

  - Option 2: UE Rx-Tx time difference is defined with respect to the frame timing associated with the DL PRS resource set;

  - Option 3: UE Rx-Tx time difference is defined with respect to the frame timing associated with the DL PRS resource(s).

- Issue 2:  Definition of the reference point(s) for UE Rx-Tx time difference measurements.

  - Number of the reference point(s)

    - Most companies propose two reference points for UE Rx-Tx time difference measurement: one for the Rx time and the other for Tx time, although some companies only propose one reference point for UE Rx-Tx time difference measurement

  - For FR1, it looks the common view is to define the reference point of Rx time as the Rx antenna connector of UE, and the reference point of Tx time as the Tx antenna connector of UE

  - Note: For FR2, RAN1 is still waiting for the response from RAN4 on the definition of the reference points for timing related measurements.

- Issue 3: When receiver diversity is not used by UE to obtain the UE Rx-Tx time measurement, it looks the common view is that the UE Rx-Tx time difference measurement is determined  based on the first detected path. However, when receiver diversity is used there are different views on how the UE Rx-Tx time difference measurement is determined

  - Based on the first detected path among the detected paths from all receiver branches (CATT)

  - The behavior for Rx diversity in DL timing based measurements is left up to the UE implementation, as long as the requirements of accuracy of the timing of the first path are met (Huawei)

  - Use of RX diversity for received timing estimation is left up to UE implementation and is not specified by measurement definition (Intel)

  - If receiver diversity is in use by the UE, the reported UE Rx-Tx time difference value shall not be lower than the corresponding UE Rx-Tx time difference any of the individual receiver branches (Futurewei)

- Issue 4: Whether inter-frequency UE Rx-Tx time difference measurements should be supported.

  - Support  both intra-frequency and inter-frequency UE Rx-Tx time difference measurements

    - CATT, Qualcomm, Intel

- Issue 5: The number of the antenna port(s) for UE Rx-Tx time difference measurements should be the same as the number of the antenna port(s) for the transmission of the DL PRS.

- Issue 6: One company made the proposal that for the UE receiving timing measurement, both CSI-RS/TRS and new DL PRS can be used (Samsung)

- Issue 7: One company made the proposal that Two methods are supported by NR for UE Rx-Tx measurements  (Intel)

    - Method 1: UE Rx-Tx measurements are obtained through ranging with serving (or reference) cell/TRP for RTT estimation and RSTD measurements with respect to serving (or reference) cell/TRP

      - Note: Method 1 assumes synchronized network (TX timing offset among TRPs/gNBs is zero or TRP’s/gNB’s timing offsets with respect to serving(reference) cell/TRP are indicated by network)

    - Method 2: UE Rx-Tx measurements are obtained through ranging with multiple cells/TRPs (including serving(reference) and neighbor cells/TRPs)

Note: Method 2 assumes that network measures gNB Rx-Tx time difference for UEs in serving and neighboring cells. Measured gNB Rx-Tx time differences are reported to UEs or LMF

Comments

| Company |  | Comments |
| --- | --- | --- |
| Huawei/Hisi | For Issue 4, we are not clear what inter-frequency RSTD means; it needs more clarification from the proponents. Does it mean that two cells have different frequencies in DL, yet sharing the same frequency for UL? Not that UE should not be expected to transmit SRS outside its UL bandwidth, but UE may receive PRS outside its DL bandwidth. Also we would also like to mention that it should be clarified if multi-RTT should also be supported for asynchronous cells, where the DL timing may be subject to the neighbouring cell, which is not aligned with the DL/UL timing of the serving cell, including SFN/subframe numbering. This is also applicable to the gNB measurement, where the subframe timing of SRS should be according the serving cell, while the PRS timing is based on the neighbouring cell. |

Proposal

- Select one (or more) of the following options for the definition of the UE Rx-Tx time difference

  - Option 1: UE Rx-Tx time difference is defined with respect to the frame timing associated with the TP

  - Option 2: UE Rx-Tx time difference is defined with respect to timing associated with one DL PRS resource set

  - Option 3: UE Rx-Tx time difference is defined with respect to timing associated with one or more DL PRS resources in one DL PRS resource set

Proposal

- For FR1, the reference point for Rx time in the definition of the UE Rx-Tx time difference is the Rx antenna connector of the UE. The reference point for Tx time in the definition of the UE Rx-Tx time difference is the Tx antenna connector of the UE

Proposal

- Select one of the following options to determine the reported UE Rx-Tx time difference measurement when receiver diversity is used by UE:

  - Option 1: based on the first detected path among the detected paths from all receiver branches

  - Option 2: up to the UE implementation, as long as the requirements of accuracy of the timing of the first path are met

  - Option 3: up to UE implementation and is not specified by measurement definition

Proposal

  - Support  both intra-frequency and inter-frequency UE Rx-Tx time difference measurements

Issues for further discussion

- Whether to support UE receiving timing measurement based on CSI-RS/TRS

- Whether to support UE Rx-Tx measurements are obtained through ranging with multiple cells/TRPs (including serving(reference) and neighbor cells/TRPs)

- The number of the antenna port(s) for UE Rx-Tx time difference measurements if number of the antenna port(s) for the transmission of the DL PRS can be 2.

## DL PRS-RSRP measurements for NR positioning

Summary

- Issue 1:  RAN1 has not decided whether to support 2 ports DL PRS. If it is decided to support 2 ports, the number of ports for DL PRS-RSRP measurement needs to be decided. There are following two proposals:

- (MTK)If 2 port transmission of a DL PRS is agreed, then UE may support PRS-RSRP measurement for 2 ports. If UE measures PRS-RSRP from 2 ports and report one PRS-RSRP, UE should report the RSRP with larger value

- (Qualcomm)There is no need to define a DL PRS-RSRP for 2 ports per PRS resource

Comments

| Company |  | Comments |
| --- | --- | --- |

Issues for further discussion

- If 2 port transmission of a DL PRS is agreed, further discuss whether to define DL PRS-RSRP measurement with 2 ports.

## Additional UE measurements

Summary

- Issue 1: One company made the following proposal to support that the additional UE measurements

      - (Intel)For NR positioning techniques based on DL timing estimation, the following additional measurements based on DL PRS signals are supported

- DL PRS SINR

- DL PRS RSRPFAP

DL PRS RSRPFAP can be replaced by K-Factor indication

      - (Intel)NR Positioning supports DL-AoD measurements based on both DL PRS RSRP and DL PRS RSRPFAP

        - DL PRS Resource ID with max of DL PRS RSRP value

        - DL PRS Resource ID with max of DL PRS RSRPFAP value

      - (Intel)Define DL PRS and UL PRS SINR measurements (i.e. for Rel-16 DL PRS and UL PRS (PRS))

Comments

| Company |  | Comments |
| --- | --- | --- |
| Huawei/Hisi | We agree with Intel that RSRP_FAP is beneficial for finding the AoD of the first path, which is likely targeting UE. |

Issues for discussion

- whether to introduce DL and UL PRS SINR measurements for NR positioning in Rel-16

- whether to introduce DL PRS RSRPFAP measurements for NR positioning in Rel-16

## Other issues related to UE measurements

Summary

The following is a list of the other issues related to UE measurements

- (HW) The reference for RSTD for both network configuration and UE reselection should be a cell/TP, as defined in LTE

- (Sony) NR Positioning supports DL-AoD measurements can be based on DL PRS RSRP and DL PRS RSTD

- (Qualcomm) With respect to TA commands during a positioning session, any accuracy requirements are valid under the condition that no changes to the uplink transmission timing are applied during the measurement period.

Company’s views

| Company |  | Comments |
| --- | --- | --- |

Issues for discussion

- Whether the reference for RSTD for both network configuration and UE reselection should be a cell/TP, as defined in LTE

- Whether to define DL-AoD measurements based on DL PRS RSRP and DL PRS RSTD

- Whether to consider the impact of TA commands during a positioning session on the accuracy requirements in RAN1’s specification.

gNB measurements for NR positioning

According to the WID [1], the following gNB measurements gNB measurements based on UL reference signals applicable for NR positioning need to be specified by RAN1:

  - UL RTOA (relative time of arrival) measurements for NR positioning

  - UL Angle of Arrival (AoA) measurements (including Azimuth and Zenith Angles) for NR positioning

  - UL RSRP (reference signal received power) measurements for NR positioning

  - gNB RX-TX time difference measurements for NR positioning

Note: For simplicity of the discussion, the “transmission measurement function” is referred to as “gNB” in this document [2].

## UL RTOA measurements for NR positioning

Summary

A number of contributions have discussed the UL RTOA measurements [3-17] on the following issues:

- Issue 1: Whether the UL RTOA should be defined with respect to the timing associated with the UE, or with the UL PRS resource sets, or with the UL PRS resources:

  - UL RTOA is defined with respect to the subframe timing associated with the UE;

  - UL RTOA is defined with respect to the subframe timing containing the UL PRS resource set;

  - UL RTOA is defined with respect to the subframe timing containing the UL PRS resource.

- Issue 2: How to define the reference points for UL RTOA measurements.

  - It looks the common view is that for FR1, the reference point for UL RTOA is the Rx antenna connector of gNB

- Issue 3: How the UL RTOA measurement is determined when receiver diversity is used by gNB to obtain the measurement

  - based on the first detected path among the detected paths from all receiver branches

  - up to the gNB implementation

Comments

| Company |  | Comments |
| --- | --- | --- |

Proposal

- Select one (or more) of the following options for the definition of the UL RTOA

  - Option 1: UL RTOA is defined with the respect to the frame [or subframe] timing associated with the UE

  - Option 2: UL RTOA is defined with respect to the subframe timing containing the UL SRS resource set;

  - Option 3: UL RTOA is defined with respect to the subframe timing containing the UL SRS resource(s).

  - Notes:

Regardless which option is selected, gNB may use multiple UL SRS resources to determine the timing of the first path of the UE, and gNB may report the UL SRS resource ID(s) used for determining the timing of the cell.

Proposal

  - For FR1, the reference point for UL RTOA is the Rx antenna connector of gNB

Proposal

- When receiver diversity is used by gNB to obtain the UL RTOA measurement, the reported measurement is determined by one of the following options:

  - Option 1: the first detected path among the detected paths from all receiver branches

  - Option 2: up to the gNB implementation, as long as the requirements of accuracy of the timing of the first path are met

  - Option 3: up to gNB implementation and is not specified by measurement definition

## gNB Rx-Tx time difference measurements for NR positioning

Summary

A number of contributions have discussed the gNB Rx-Tx time difference measurements [3-17]. Different views and proposals are presented on the following issues:

- Issue 1: Whether the gNB Rx-Tx time difference should be defined with respect to the timing associated with the UE, or with the UL PRS resource sets, or with the UL PRS resources:

  - Option 1: gNB Rx-Tx time difference is defined with respect to the frame timing associated with the UE;

  - Option 2: gNB Rx-Tx time difference is defined with respect to the frame timing containing the UL PRS resource set;

  - Option 3: gNB Rx-Tx time difference is defined with respect to the frame timing associated containing the UL PRS resource.

- Issue 2: there is a need to define the reference points for gNB Rx-Tx time difference measurements.  For gNB Rx-Tx time difference measurements, there are two reference points: the reference point for Rx time is at the Rx antenna connector; and the reference point for Tx time is at the Tx antenna connector

- Issue 3: When receiver diversity is used by the gNB for gNB Rx-Tx time difference measurement, different receiver branches may have different Rx timing. The following proposals were made on how the gNB Rx-Tx time difference measurement is determined in this case.

  - based on the first detected path among the detected paths from all receiver branches

  - up to gNB implementation

  - If receiver diversity is in use by the gNB, the reported gNB Rx-Tx time difference value should  not be lower than the corresponding gNB Rx-Tx time difference any of the individual receiver branches. No RAN1 specifications on this is needed. (Futurewei)

Comments

| Company |  | Comments |
| --- | --- | --- |

Proposal

- Select one (or more) of the following options for the definition of the gNB Rx-Tx time difference

  - Option 1: gNB Rx-Tx time difference is defined with respect to the frame timing associated with the UE;

  - Option 2: gNB Rx-Tx time difference is defined with respect to the frame timing containing the UL SRS resource set;

  - Option 3: gNB Rx-Tx time difference is defined with respect to the frame timing associated containing the UL SRS resource.

Proposal

- For FR1, the reference point for the Rx time of the gNB Rx-Tx time difference is the Rx antenna connector of the gNB, and the reference point for Tx time of the gNB Rx-Tx time difference is the Tx antenna connector of the gNB.

Proposal

- When receiver diversity is used by gNB to obtain the gNB Rx-Tx time difference measurement, select one of the following options to determine the reported measurement:

  - Option 1: the first detected path among the detected paths from all receiver branches

  - Option 2: up to the gNB implementation, as long as the requirements of accuracy of the timing of the first path are met

  - Option 3: up to gNB implementation and is not specified by measurement definition

## UL SRS RSRP measurements for NR positioning

Summary

In RAN1#97, the following agreements were made on DL PRS RSRP measurements:

Agreement: DL PRS-RSRP is defined as the linear average over the power contributions (in [W]) of the resource elements of the antenna port that carry a DL PRS resource configured for RSRP measurements within the considered measurement frequency bandwidth - FFS: DL PRS-RSRP for 2 ports, if 2 port transmission of a DL PRS is agreed. Agreement: - For FR1, the reference point for the DL PRS-RSRP is the Rx antenna connector of the UE; - For FR2, DL PRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch Agreement: When receiver diversity is in use by the UE to obtain the DL PRS-RSRP measurement, the reported DL PRS-RSRP value shall not be lower than the corresponding DL PRS-RSRP of any of the individual receiver branches

Also, the SRS-RSRP  was introduced for CLI as follows:

Agreement (AH1901) The following CLI measurements are supported: - SRS-RSRP: - Linear average of the power contributions of the SRS to be measured over the configured resource elements within the considered measurement frequency bandwidth in the time resources in the configured measurement occasions

In RAN1#98, a number of contributions have discussed the definition of the UL RSRP measurements [3-17], covering the following issues:

- Issue 1: One company proposes to discuss what is the usefulness of UL RSRP as a base station measurement for positioning purposes, e.g., is there a plan for the gNBs to forward their RSRP measurements to the location server (Qualcomm)

- Issue 2: the definition of UL-PRS-RSRP. It seems the common view is to define the UL PRS RSRP in a similar way as DL PRS RSRP:

  - UL PRS reference signal received power (UL-PRS-RSRP)  is defined as the linear average over the power contributions (in [W]) of the resource elements of the antenna port(s) that carry the UL PRS configured for RSRP measurements within the considered measurement frequency bandwidth;

  - The UL PRS resource elements for UL RSRP measurements in time domain are within the configured UL PRS occasions;

- Issue 3: How to define the reference points for UL PRS-RSRP measurements. It looks the common view is that

  - For FR1, the reference point for UL PRS-RSRP is the Rx antenna connector of gNB

  - For FR2, UL PRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch

- Issue 4: receiver diversity. It looks the common view is that

  - When receiver diversity is used by the gNB to get UL PRS-RSRP measurement, the reported UL RSRP value shall not be lower than the corresponding UL RSRP of any of the individual receiver branches

Comments

| Company |  | Comments |
| --- | --- | --- |

Proposal

- UL SRS-RSRP for NR positioning is defined as the linear average over the power contributions (in [W]) of the resource elements of the antenna port that carry the UL SRS resource configured for RSRP measurements within the considered measurement frequency bandwidth in the configured measurement occasions

  - FFS: UL SRS-RSRP for 2 ports, if 2 port transmission of the SRS is agreed.

Proposal

- For FR1, the reference point for the UL SRS-RSRP is the Rx antenna connector of the gNB.

- For FR2, UL SRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch.

- When receiver diversity is in use by the UE to obtain the UL SRS-RSRP measurement, the reported UL SRS-RSRP value shall not be lower than the corresponding UL SRS -RSRP of any of the individual receiver branches

## UL angle of arrival measurements for NR positioning

Summary

A number of contributions have discussed the UL angle of arrival measurements [3-17]. Different views are presented on the following issues:

- Issue 1: What is (are) the reference coordinate system(s) for UL angle of arrival measurements?

  - It looks there is a common view that

    - AoA can be defined as the estimated angle of a user in the horizontal plane with respect to the geographical North, positive in a counter-clockwise direction;

    - ZoA can be defined as the estimated angle of a user with respect to the zenith;

  - In addition, there are proposals that UL angle of arrival measurements can also be defined with respect to other reference coordinate system, e.g.,

    - A rotated coordinate systems which are adapted to the orientation of the antenna arrays in special deployments (e.g., horizontal indoor antenna array) for reporting angle measurements and corresponding quality metrics (Nokia)

    - For AoA and ZoA, the reference direction is with respect to the Local Coordinate System (LCS) defined by the translation of the GCS to LCS using the set of angles  (bearing angle),  (downtilt angle),  (slant angle) (Qualcomm)

- Issue 2: Should the angle of arrival be defined in terms of the estimated angle of a user with respect to a reference direction, or the estimated angle of a beam from a user with respect to a reference direction?

  - Option 1: the estimated angle of a user with respect to a reference direction

  - Option 2: the estimated angle of a beam with respect to a reference direction

- Issue 3: One company made a proposal that NR supports UL AoA measurements for the first arrival path of channel impulse response (Intel)

Comments

| Company |  | Comments |
| --- | --- | --- |
| Huawei/Hisi | We are OK for the angle estimate in LCS, as long as the measurement can be specified accurately in the specification. |

Proposal

- Select one (or more options) for the definition of the UL angle of arrival measurements

  - Option 1: UL angle of arrival measurements is defined in terms of the estimated angle of a user with respect to a reference direction

  - Option 2: UL angle of arrival measurements is defined in terms of the estimated angle of a UL PRS resource with respect to a reference direction

  - Option 3: UL angle of arrival measurements is defined in terms of the estimated angle of a UL PRS resource set with respect to a reference direction

Proposal

- AoA (ϕ) and ZoA (θ) define the estimated angles of a user with respect to a configurable reference direction which are determined at the TRP antenna for an UL channel corresponding to this UE.

- The reference directions can at last be defined as

  - For AoA, the geographical North, positive in a counter-clockwise direction.

  - For ZoA, the zenith/vertical with θ=0 pointing to the zenith and θ=900 pointing to the horizon.

- FFS: additional reference directions

  - For AoA and ZoA, the reference direction is with respect to the Local Coordinate System (LCS) defined in TS 38.901. The translation of the GCS to LCS uses the set of angles  (bearing angle),  (downtilt angle),  (slant angle)

Issues for further discussion

- Whether UL AoA measurements should be measured based on the first arrival path of channel impulse response

## Other issues on gNB measurements for NR positioning

Summary

The following proposals are made on additional gNB measurements for NR positioning

- (Nokia) Proposal 9: Introduce a TOA measurement at the gNB for a reference signal from another gNB. FFS the exact RS to be used.

- (Nokia) Proposal 10: Include RS from another gNB in the measurement definition of RTOA when specified. This measurement can be configured as part of multi-cell RTT. FFS the exact RS to be included.

- (Samsung) UL RTOA (relative time of arrival) measurements can be based on SRS and PRACH

- (Samsung) UL AoA measurement for zenith angle should be based on SRS and PRACH

- (Samsung) UL RSRP (reference signal received power) measurements should be based on SRS and PRACH

- (Samsung) For the gNB receiving timing measurement, both SRS and PRACH can be used

- (Intel) For NR positioning techniques based on UL timing estimation, the following measurements are supported for UL SRS (PRS) signals

    - UL SRS (PRS) SINR

    - UL SRS (PRS) RSRPFAP

    - Can be replaced by K-Factor indication

    - (Intel)Define DL PRS and UL PRS SINR measurements (i.e. for Rel-16 DL PRS and UL PRS (PRS))

    - (Ericsson)Define gNB Rx-Tx time difference, timing advance, AoA and UL RTOA as gNB measurements for NR positioning.

Comments

| Company |  | Comments |
| --- | --- | --- |

Issues for further discussion

- whether to introduce a TOA measurement at the gNB for a reference signal from another gNB in Rel-16

- whether to include RS from another gNB in the measurement definition of RTOA when specified in Rel-16

- whether to introduce the following measurements for NR positioning in Rel-16

  - UL PRS(PRS) SINR

  - UL PRS(PRS) RSRPFAP

    - Can be replaced by K-Factor indication

- Whether to support UL RTOA (relative time of arrival) measurements based on PRACH

- Whether to support UL AoA measurement for zenith angle based on SRS and PRACH

- Whether to support UL RSRP (reference signal received power) measurements based on PRACH

- Whether to support gNB receiving timing measurement based on PRACH

- Whether to introduce the following measurements for NR positioning

  - UL SRS (PRS) SINR

  - UL SRS (PRS) RSRPFAP

UE and gNB Measurement reporting

## Beam indication in measurement reporting

Summary

For DL RSTD measurements, it was agreed in last meeting that “The UE may use different DL PRS Resource ID(s) (with the condition that the multiple DL PRS Resource IDs belong to a single DL PRS Resource set) or a different DL PRS Resource set for determining the reference for the RSTD measurement, and if it chooses to do so, it should report the DL PRS Resource ID(s) and/or the information on the DL PRS Resource set used to determine the reference.”

Many contributions have proposed the support of reporting the RSTD and other UE/gNB measurements with the associated beam information [3-17]:

- (HW) For DL timing based measurements, the UE can report the information related to receive beam/panel for deriving the timing of a cell.

- (vivo) Support UE measurements report for each configured DL PRS resource for NR DL-TDOA based positioning.

- (vivo) Support UE to report PRS resource ID together with the corresponding RSTD report.

- (vivo) Combining RSTD measurements is supported. FFS how an RSTD report is derived based on multiple measurements.

- (vivo) UE should report RSRP measurements from multiple TX beams of a certain cell for NR DL-AOD based positioning.

- (vivo) Support reporting RSTD and RSRP measurements simultaneously for a certain beam.

- (MTK) For high-resolution DL-AoD based positioning, an UE may select a subset of all measured PRS resource sets, and report only RSRPs measured from the selected PRS resource sets. UE may not need to report RSRPs measured from non-selected PRS resource sets. A method for UE to select PRS resource sets is given as follows:

- The network configures the parameters T,kmax, kmin to an UE

- For an UE, let m be the number of cells with measured RSRP greater than T

  - If m≥kmax, then UE selects best (maximum power) kmax cells

  - If m<kmax, then UE selects best kmin cells

FFS: values of T,kmax, kmin

- (MTK) For high-resolution DL-AoD based positioning, an UE may select a subset from all measured PRS resources of a PRS resource set, and only report RSRP measured from the selected PRS resources. UE may not need to report RSRPs measured from non-selected PRS resources. A method for UE to select PRS resources is given as follows:

- The network configures a parameter k to the UE

- For a PRS resource set, the UE selects k PRS resources such that the RSRPs measured from these k PRS resources are larger than that measured from the other PRS resources

FFS: value range of k

- (MTK) For DL-AoD based positioning, it is sufficient to use the following configurations:

- UE can be configured to do RSRP measurement for at most 8 cells

- For each cell, UE can be configured to do RSRP measurement for at most 8 beams

- (LGE) At least for AoD-based positioning, NR positioning should support reporting of a PRS resource index showing the minimum propagation time among multiple PRS resource(s).

  - If more than two PRS resources show the same minimum propagation time, signal strength can be considered to report a single PRS resource among those selected PRS resources.

- (LGE) The network can indicate the UE to report information on a PRS resource set and its containing PRS resource which is chosen by the UE as a reference for RSTD calculation.

- (Samsung) The decision criterion of the UE should be defined and reported if UE chooses to use a DL PRS resource(s)/ resource set different from the one indicated by the network to facilitate the positioning.

- (Samsung) The resource/resource set ID(s) to be reported by the UE should depend on how such IDs are defined and if both IDs are defined locally, cell ID should also be reported..

- (Samsung) Subset RSRP measurement reporting should be supported and the reporting granularity can be different from RSTD measurement reporting.

- (CATT) The reported UE measurements (e.g., DL PRS-RSTD, DL PRS RSRP, UE Rx-Tx Time difference) should be associated with both DL PRS Resource ID and DL PRS Resource Set ID. (Sony) Proposal 4: The UE reports the measurement (e.g. RSTD, RSRP) and the associated DL PRS Resource ID(s) (e.g beam information).

- (CATT) The reported gNB measurements (e.g., UL RTOA, UL SRS RSRP, gNB Rx-Tx Time difference, and UL AOA/AoZ) should be associated with both UL SRS Resource ID and the UE ID.

- (Qualcomm) Based on agreement, it should be possible for a UE to report which PRS resource set, or PRS resource(s) from one PRS resource set from each TRP is used to derive the reported RSTD values.

- (Intel) NR Positioning supports DL-AoD measurements based on both DL PRS RSRP and DL PRS RSRPFAP

  - DL PRS Resource ID with max of DL PRS RSRP value

  - DL PRS Resource ID with max of DL PRS RSRPFAP value

- (Intel)UL-AoA is measured and reported for each configured UL PRS Resource (UE Tx Beam)

- (Intel)UL-AoA is measured and reported per gNB Rx beam (DL PRS Resource ID)

- (Ericsson) The network configures values number of base stations (N), the number of paths (P) and number of strongest beams (Q) for the measurements to be performed and reported by the UE

- (Futurewei)Angle of arrival measurements on a per beam basis is supported.

Company’s views

| Company |  | Comments |
| --- | --- | --- |
| LGE | In 3GPP channel model, the channel delay profile except for the signal strength of each signal path is not different depending on TX beam-width and/or beam-direction, but it is difficult to be guaranteed in reality. For OTDOA, therefore, we need to address which PRS resource among multiple PRS resource(s) associated with a TP is selected/reported by the UE. For RSTD calculation, the UE should select a PRS resource corresponding to the propagation time of the first arrival signal path which is the minimum among multiple PRS resources. |

Proposal

- In UE measurement reporting, UE should support reporting the following information together with the UE measurements:

  - For DL PRS RSTD, the information should at least allow the network to uniquely identify the TPs whose the DL PRS Resources or DL PRS Resource Sets are used in deriving  the DL PRS RSTD measurement. The information may include:

    - the DL PRS Resource IDs of the DL PRS Resources, which are used in deriving the DL PRS RSTD measurements, including the RSTD time reference, and/or

    - the DL PRS Resource Set IDs of the DL PRS Resource Sets, which are used in deriving the DL PRS RSTD measurements, including the RSTD time reference, and/or

    - the cell IDs of the TPs, whose the DL PRS Resources or DL PRS Resource Sets are used in deriving the DL PRS RSTD measurements, including the RSTD time reference;

  - For UE Tx-Rx time difference, the information should at least allow the network to uniquely identify the TPs whose the DL PRS Resources or DL PRS Resource Sets are used in deriving  the UE Tx-Rx time difference measurement. The information may include following

    - the DL PRS Resource IDs of the DL PRS Resources, which are used in deriving the UE Tx-Rx time difference measurements

    - the DL PRS Resource Set IDs of the DL PRS Resource Sets, which are used in deriving the Tx-Rx time difference measurements

    - the cell IDs of the TPs, whose the DL PRS Resources or DL PRS Resource Sets are used in deriving the Tx-Rx time difference measurements

  - For DL PRS-RSRP, the information should allow the network to uniquely identify the DL PRS Resources that are used in deriving  the DL PRS-RSRP measurement. The information include:

    - the DL PRS Resource IDs of the DL PRS Resources, which are used in deriving the DL PRS-RSRP measurements

    - the DL PRS Resource Set IDs of the DL PRS Resource Sets, which are used in deriving the DL PRS-RSRP measurements

    - the cell IDs of the TPs, whose the DL PRS Resources or DL PRS Resource Sets are used in deriving the DL PRS-RSRP measurements.

Proposal

- UE should support simultaneously or separately reporting DL PRS RSTD, UE Tx-Rx time difference and DL PRS RSRP, which are measured from the same or different  DL PRS Resources of the same or different DL PRS Resources Sets and TPs

  - FFS: the maximum number of DL PRS RSTD measurements per measurement report

  - FFS: the maximum number of UE Tx-Rx time difference measurements per measurement report

  - FFS: the maximum number of DL PRS RSRP measurements per measurement report

  - FFS: the maximum number of DL PRS RSRP measurements per TP in a measurement report

Proposal

- Decide which of the following options should be used to DL PRS RSRP measurements

  - Option 1: UE needs to measure and report DL PRS RSRP measurement for each configured DL PRS Resource;

  - Option 2: UE is allowed to select reporting DL PRS RSRP measurements for a subset of the configured DL PRS Resources based on the configuration provided the network

    - FFS: the configuration provided the network (e.g., the selection threshold, the maximum/minimum number of selected DL PRS resource sets, etc.) for the selection

Proposal

- Decide which of the following options should be used to UL SRS RSRP measurements

  - Option 1: gNB needs to measure and report UL SRS RSRP measurement for each configured UL SRS Resource;

  - Option 2: gNB is allowed to select reporting UL SRS RSRP measurements for a subset of the configured UL SRS Resources

Issues for further discussion

- For AoD-based positioning, whether the UE should support reporting of the PRS resource index associated with the first detected path

- For DL PRS RSTD measurement, whether there is need to define the decision criterion for the UE to choose a DL PRS resource(s)/ resource set as RSTD reference different from the one indicated by the network.

## Quality indication in reporting

Summary

The following agreement was made in RAN1#97 for reporting the measurement quality related to the following UE/gNB measurements:

- Support reporting separate metric(s) corresponding to quality of each of the following measurements

- RSTD

- UE Rx-Tx time difference

- UL-RTOA

- gNB Rx-Tx time difference

- UL angle of arrival, including the azimuth of arrival (AoA) and the zenith of arrival (ZoA)

- FFS: The details of these quality metrics

In this meeting, many contributions have discussed how the measurement quality metrics should be defined:

- (Huawei) Support in NR modeling the quality of timing measurements as Gaussian distribution in addition to the mechanism as LTE OTDOA measurement quality.

- (Huawei) Support reporting the gNB antenna orientation and modeling angle quality as Gaussian distribution defined in the LCS of gNB antenna array. FFS how to model the Gaussian variable.

- (Huawei) The gNB can report SRS-RSRP along with the measurement and measurement quality of UL-RTOA and AOA/ZOA.

- (Nokia) The reporting resolution of quality metrics for UE Rx-Tx time difference, gNB Rx-Tx time difference, RTOA, and RSTD should be of the same granularities.

- (Nokia) Study different options like a Gaussian PDF or a Kent PDF for defining a quality metric for two-dimensional angle measurements and define the quality metric to be used for reporting.

- (MTK) For high-resolution DL-AoD based positioning, an UE may report RSRP ratios or differential RSRPs derived from measured PRS resources of a PRS resource set. A method for UE to report RSRP ratios or differential RSRPs is given as follows:

  - Suppose pk is the RSRP measured from PRS resource with ID k, k = 1,2,…,N

  - Suppose pm is a maximum RSRP, i.e.,  pm≥pk  for k = 1,2,…,N

  - Then UE reports the PRS resource ID m, but UE does not report pm

  - For PRS resource with ID j, where j≠m, UE reports j and xj to the network, where xj=pj/pm or xj=10logpm-10logpj

FFS: value range and quantization level of RSRP ratios or differential RSRPs

- (MTK) Reuse the OTDOA-MeasQuality defined in LPP as the quality metric for RSTD measurement in NR, and add one field “error-scalingFactor” to  OTDOA-MeasQuality. If error-scalingFactor is set to k, then error-Resolution is scaled by 1/k, i.e., error-Resolution = error-Resolution*(1/k). The value of error-scalingFactor may be 1,2,4,8.

- (MTK) The OTDOA-MeasQuality in proposal 9 can also be used as the quality metric for UE Rx-Tx measurement.

- (CATT) Define the quality indication of DL PRS-RSTD and UE Rx-Tx time difference measurements as follows. It is up to the implementation of the UE on how to determine the quality indication

  - error-Resolution ([0.1, 0.2, 0.5, 1, 2, 5, 10, 20]meter): specifies the resolution used in error-Value field

  - error-Value ([0 ~ 63]): specifies the target device′s best estimate of the uncertainty of the RSTD and UE Rx-Tx time difference measurements

  - error-NumSamples ([0 ~7]): specifies how many measurements have been used by the target device to determine this TDOA MeasQuality

- (CATT) Define the quality indication of UL-RTOA and gNB Rx-Tx time difference measurements as follows. It is up to the implementation of the UE on how to determine the quality indication

  - error-Resolution ([0.1, 0.2, 0.5, 1, 2, 5, 10, 20]meter): specifies the resolution used in error-Value field

  - error-Value ([0 ~ 63]): specifies the target device′s best estimate of the uncertainty of the RSTD and UE Rx-Tx time difference measurements

  - error-NumSamples ([0 ~7]): specifies how many measurements have been used by the target device to determine this TDOA MeasQuality

- (Qualcomm) Propose a configurable logarithm scale of the measurement error of the timing and angle measurrements which is configured with the following three parameters:

  - Minimum error (MIN_VAL)

  - Maximum error (MAX_VAL)

  - Number of bits (NUM_BITS)

  - An example of such a function is the K-factor function Y = C*(1+x)^K, which can be configured as follows:

    - Kmax = 2^NUM_BITS - 1

    - C = MIN_VAL

    - x = 10.^((log10( MAX_VAL ) - log10( MIN_VAL ))/Kmax) – 1

- (Intel)

  - Metrics for quality of measurements can be define per Resource or Resource Set level

  - Support SINR and RSRP measurements as a baseline single-shot indicators of measurement quality applicable for aperiodic reporting

  - Support metric of measurement quality composed of the following components:

    - Resolution

    - Standard Deviation (STD)

    - Number of samples used for standard estimation deviation

  - Maximum number of samples to be used for standard estimation deviation is controlled and configured by network

  - Standard deviation is measured over X = 90% of samples excluding Y = 10% of potential outliers

- (Ericsson) A quality measure of UE Rx-Tx measurement should be reported by the UE along with the UE Rx-Tx measurements.

- (Ericsson) Standard deviation of UE Rx-Tx measurements in terms of Ts should be reported

Comments

| Company |  | Comments |
| --- | --- | --- |
| LGE | Measurement quality information could be used for more accurate estimate UE’s location by applying different weight factor with respect to each measurement depending on the absolute quality information. However, signalling overhead is not negligible to provide the absolute measurement quality for all measurements. For this reason, we could consider a way to lower signalling overhead keeping the positioning accuracy such as reporting of the measurements that exceed a certain measurement quality. |

Proposal

- Support the same definition of the quality metrics for timing related UE and gNB measurements, including DL PRS RSTD, UE Rx-Tx time difference, UL RTOA, and gNB Rx-Tx time difference

Proposal

- The quality metrics of the timing related measurements include the following fields

  - error-Resolution: specifies the resolution used in the error-Value field

  - error-Value(Standard deviation): specifies the target device′s best estimate of the uncertainty of the measurement

  - error-NumSamples: specifies how many measurements used by UE to determine the MeasQuality

  - error- error-scalingFactor: the scalingFactor for error-Resolution

  - FFS: details on above field

Proposal

- Select one of the following options for reporting the quality measure as the quality metrics of UL angle of arrival measurements

  - modeling angle quality as Gaussian distribution defined in the LCS of gNB antenna array

    - FFS: details of Gaussian distribution variable

  - modeling angle quality as Kent distribution defined in the LCS of gNB antenna array

    - FFS: details of Kent distribution variable

  - report SRS-RSRP along with the AOA/ZOA measurements

Proposal

- UE should support reporting DL PRS RSRP along with the DL PRS RSTD and UE Rx-Tx time difference measurements;

- gNB should support reporting UL SRS RSRP along with the UL RTOA, gNB UE Rx-Tx time difference measurements and AOA/ZOA measurements

Issues for further discussion

- whether to support modeling the quality of timing measurements as Gaussian distribution in addition to the mechanism as LTE OTDOA measurement quality.

- whether to support reporting RSRP ratios or differential RSRPs derived from measured PRS resources of a PRS resource set. A method for UE to report RSRP ratios or differential RSRPs is given as follows:

  - Suppose pk is the RSRP measured from PRS resource with ID k, k = 1,2,…,N

  - Suppose pm is a maximum RSRP, i.e.,  pm≥pk  for k = 1,2,…,N

  - Then UE reports the PRS resource ID m, but UE does not report pm

  - For PRS resource with ID j, where j≠m, UE reports j and xj to the network, where xj=pj/pm or xj=10logpm-10logpj

FFS: value range and quantization level of RSRP ratios or differential RSRPs

- whether to support configurable logarithm scale of the measurement error of the timing and angle measurements which is configured with the following three parameters:

  - Minimum error (MIN_VAL)

  - Maximum error (MAX_VAL)

  - Number of bits (NUM_BITS)

  - An example of such a function is the K-factor function Y = C*(1+x)^K, which can be configured as follows:

    - Kmax = 2^NUM_BITS - 1

    - C = MIN_VAL

    - x = 10.^((log10( MAX_VAL ) - log10( MIN_VAL ))/Kmax) – 1

  - whether to support SINR measurements as measurement quality

## Multi-path measurements

Summary

A number of companies have discussed the measurement reporting related to multi-path measurements:

- (Huawei) Support reporting multi-path measurements with jointly reporting

  - Timing and angle of each path, up to 3 paths at gNB

  - Timing and power of each path under multiple beams, up to 3 paths at UE.

- (vivo) Support reusing measurements like ‘additional path’ for NR DL-TDOA based positioning.

- (Ericsson) UE should be configured to measure and report the position of strongest peak along with the detected first peak. The RSTD measurement report should indicate which of the reported peaks is the strongest peak.

- (Ericsson) UE reports RSTD between neighbour and reference cells, and additional paths for neighbour and reference cells if detected.

- (Ericsson) The maximum value for the number of cells in an RSTD measurement N , number of measured paths per cell P and number of reported paths per cell Q are a the UE capability .

- (Ericsson) Value of P should be decided based on number of peaks required for positioning method such as, single base station based positioning, fingerprinting, multipath based positioning.

- (Ericsson) Network should be able to configure finer RSTD measurements in CIR around any peak.

- (Ericsson) Dynamic range of peaks in the power delay profile should be reported along with the RSTD measurements on these peaks

- (Ericsson) Include the UE Rx – Tx time difference relative to the first reported peak of the reference cell in the DL PRS RSTD measurement report

Comments

| Company |  | Comments |
| --- | --- | --- |
| Huawei/Hisi | In our view, multi-path reporting can well manage the functionality of beam-specific/beam-pair specific timing based measurement. Since each path may show a different power under different beams (both Tx and Rx), and delay of each respective path is supposedly to be identical under different beams. |

Proposal

- UE should support reporting multi-path measurements

- Select one or more of the following options for supporting UE multi-path measurements

  - Option 1: Jointly reporting Timing and power of each path under multiple beams, up to 3 paths at UE

  - Option 2: Support reusing measurements like ‘additional path’ for NR DL-TDOA based positioning.

  - Option 3: UE should be configured to measure and report the position of strongest peak along with the detected first peak. The RSTD measurement report should indicate which of the reported peaks is the strongest peak.

  - Option 4: UE reports RSTD between neighbour and reference cells, and additional paths for neighbour and reference cells if detected.

  - Option 5: The maximum value for the number of cells in an RSTD measurement N , number of measured paths per cell P and number of reported paths per cell Q are the UE capability .

    - Value of P should be decided based on number of peaks required for positioning method such as, single base station based positioning, fingerprinting, multipath based positioning.

  - Option 6: Network should be able to configure finer RSTD measurements in CIR around any peak.

  - Option 7: Dynamic range of peaks in the power delay profile should be reported along with the RSTD measurements on these peaks

  - Option 8: Include the UE Rx – Tx time difference relative to the first reported peak of the reference cell in the DL PRS RSTD measurement report

Proposal

- gNB should support reporting multi-path measurements;

- Select one or more of the following options for reporting multi-path measurements

  - Option 1: Jointly reporting angle of each path under multiple beams, up to 3 paths at gNB

Issues for further discussion

## Other issues on UE measurement reporting

Summary

Additional proposals made on other important issues on UE measurement reporting:

- Issue 1: (Qualcomm) Timing measurements are reported using [16] bits with configurable precision, dependent on Tc = 0.509 nsec and a higher layer configured step T:

  - The step size T may be configured using a step size factor u as follows:  T = Tc*2^u, where “u” is either 1,2,3,4,5,6.

- Issue 2: (Qualcomm) Azimuth Angle measurements are reported with a range [0,360] with one of the following options:

- Option 1: using 10 bits and a step of 0.5 degrees

- Option 2: using 11 bits and a step of 0.25 degrees

Elevation Angle measurements are reported with a range [0,180] with one of the following options:

- Option 1: using 9 bits and a step of 0.5 degrees

- Option 2: using 10 bits and a step of 0.25 degrees

- Issue 3: (Qualcomm) NR supports the following signaling related to each individual “UE/TRP Rx-Tx/RTOA/RSTD” measurement:

  - Time-stamp information during which each reported "UE/TRP Rx-Tx/RTOA/RSTD" is valid.

  - Study whether this will be the frame number (SFN), and/or slot index, and/or finer granularity in a frame of the serving cell.

Comments

| Company |  | Comments |
| --- | --- | --- |

Issues for further discussion

- The resolution and the range for all UE and gNB measurements

- The time-stamp information included in UE and gNB measurement reports

Mapping between positioning techniques, the DL/UL PRS and the measurements

Summary

The mapping between positioning technique and the essential RSs/measurements is discussed in some contributions with the following proposals:

(Huawei) Support the mapping between each positioning technique and the essential measurements as follows.

- FFS other measurements to enhance the performance.

- FFS reference signals to obtain the measurements.

| Positioning techniques | UE measurements |  | gNB measurement |  |
| --- | --- | --- | --- | --- |
|  | Serving cell | Neighbouring cell | Served UE | Neighbouring UE |
| DL-TDOA | RSTD | RSTD |  |  |
| DL-AoD | PRS-RSRP | PRS-RSRP | AoA, ZoA | AoA, ZoA |
| UL-TDOA |  |  | UL-RTOA | UL-RTOA |
| UL-AoA |  |  | AoA, ZoA | AoA, ZoA |
| E-CID | UE Rx – Tx time difference, RSRP/RSRQ | RSRP/RSRQ | gNB Rx – Tx time difference, AoA, ZoA |  |
| Multi-RTT | UE Rx – Tx time difference | UE Rx – Tx time difference | gNB Rx – Tx time difference | gNB Rx – Tx time difference |

  - (Intel) Agree on mapping b/w NR positioning techniques, reference signals and measurements as provided in Table 1.

Table 1: Mapping of NR positioning techniques to reference signals and UE/gNB measurements

| NR Positioning Techniques | DL Reference Signals | UE Measurements | UL Reference Signals | gNB Measurements |
| --- | --- | --- | --- | --- |
| DL-TDOA | Rel.16 DL PRS | DL PRS-RSTD | N/A | N/A |
| DL-AoD | Rel.16 DL PRS | DL PRS-RSRP | N/A | N/A |
| UL-TDOA | N/A | N/A | Rel.16 UL PRS | UL PRS-RTOA |
| UL-AoA | N/A | N/A | Rel.16 UL PRS | UL PRS-AoA (Azimuth, Zenith) |
| Multi-cell RTT | Rel.16 DL PRS | Rx-Tx time difference | Rel.16 UL PRS | Rx-Tx time difference |
| E-CID | Rel.15 SSB Rel.15 CSI-RS for BM | Rx-Tx time difference | Rel.15 PRACH Rel.15 UL PRS | Rx-Tx time difference PRACH/SRS – AoA SRS-RSRP, SRS-SINR |

Note: Rel.16 UL PRS designates UL PRS signals with possible enhancements targeted in Rel.16 as a part of NR positioning

Note: Rel.16 DL PRS designate DL PRS signals introduced in Rel.16 for NR positioning

  - Comments

| Company |  | Comments |
| --- | --- | --- |
| Huawei/Hisi | Since the table is supposed to be used for RAN2 stage-2 specification, we think that it is more important to include whether the measurement is for serving cell only or is also applicable to neighbouring cells than to include the reference signals used. |

Proposal

- Decide if there is a need to define the mapping of NR positioning techniques to reference signals and UE/gNB measurements from RAN1’s perspective. If yes, using the following mapping table as the starting point for the discussion of the mapping table.

Table 1: Mapping of NR positioning techniques to reference signals and UE/gNB measurements

| NR Positioning Techniques | DL Reference Signals | UE Measurements | UL Reference Signals | gNB Measurements |
| --- | --- | --- | --- | --- |
| DL-TDOA | Rel.16 DL PRS | DL PRS-RSTD | N/A | N/A |
| DL-AoD | Rel.16 DL PRS | DL PRS-RSRP | N/A | N/A |
| UL-TDOA | N/A | N/A | Rel.16 UL SRS | UL SRS -RTOA |
| UL-AoA | N/A | N/A | Rel.16 UL SRS | UL SRS -AoA (Azimuth, Zenith) |
| Multi-cell RTT | Rel.16 DL PRS | UE Rx-Tx time difference | Rel.16 UL SRS | gNB Rx-Tx time difference |
| E-CID | Rel.15 DL RS (SSB, CSI-RS, etc.) of the serving cell | UE Rx-Tx time difference and RRM measurements (e.g., RSRP, RSRQ) only measured from the RS of the serving cell | Rel.15 UL SRS | gNB Rx-Tx time difference and UL AoA measured only by the serving cell |

Notes:

- Rel.16 UL SRS refers to UL SRS signals defined in Rel.16 for NR positioning

- Rel.16 DL PRS refers to DL PRS signals defined in Rel.16 for NR positioning

E-CID downlink measurements

Summary

The NR positioning WID includes the following objective [1]

  - Support E-CID downlink measurements based on at least RRM measurements defined in NR Rel. 15

  - Identify whether and which Rel-15 NR reference signals can be used for different NR positioning techniques

Submitted Proposals:

- (vivo) For E-CID positioning, support UE measurements report based on one or more beams from each positioning cell.

- (Ericsson) Support UE reporting of the Rel. 15 NR measurements for positioning

  - SS-RSRP, SS-RSRQ and SS-SINR of serving and intra-frequency neighbor cells

  - SS-RSRP, SS-RSRQ and SS-SINR of inter-frequency cells

  - SS-RSRPB of intra-frequency cells (only FR2)

  - CSI-RSRP, CSI-RSRQ and CSI-SINR of intra- and inter-frequency cells

- (Ericsson) Send an LS to RAN2 to inform about the agreed reporting of Rel 15 UE NR measurements

- (Intel)NR E-CID is supported based on Rel-15 reference signal framework

- (Intel)Additional measurement for NR E-CID support are introduced

  - UL-AoA based on Rel-15 UL PRS/PRACH

  - DL-AoD based on Rel-15 SS-RSRP

  - UE/gNB Tx-Rx time difference

  - Rel-15 SS-SINR (to replace RSRQ)

- (Intel)Use of Rel-15 reference signals for all NR Positioning techniques (except of NR E-CID support) is left up to UE implementation

- (Intel)Network provides assistance signaling to UE in order to facilitate the use of Rel-15 reference signals for the benefit of NR positioning (e.g. quasi-collocation information)

- (Intel)NR positioning measurement requirements/test cases do not assume that Rel-15 reference signals are used by UE

- (Futurewei) Support of E-CID positioning in NR is based on Rel-15 reference signals. Use of other Rel-16 reference signals are left up to UE implementation

Comments

| Company |  | Comments |
| --- | --- | --- |

Proposal

- Rel-15 NR E-CID is supported based on Rel-15 reference signal framework

  - Support UE reporting of the following existing measurements for E-CID positioning

    - RSRP (SS-RSRP, CSI-RSRP) of serving and intra-frequency neighbor cells

    - Either RSRQ (SS-RSRQ, CSI-RSRQ) of serving and intra-frequency neighbor cells, or SINR (SS-SINR, CSI-SINR) of serving and intra-frequency neighbor cells

    - SS-RSRPB of intra-frequency cells (only FR2)

  - FFS: Support UE reporting of the following additional measurements for E-CID positioning

    - UE Rx-Tx time difference

  - FFS: Support gNB reporting of the following measurements for E-CID positioning

    - Time advance (TADV)

      - Type1: TADV = (gNB Rx–Tx time difference) + (UE Rx-Tx time difference),

    - gNB Rx-Tx time difference

    - UL-AoA

    - DL-AoD

  - Rel. 15 NR measurements in one measurement report for E-CID positioning can be obtained from one or more beams from the serving cell

- It is left up to UE implementation on whether and which Rel-15 NR reference signals are used for different Rel-16 NR positioning techniques (except of NR E-CID support). Rel-16 NR positioning measurement requirements/test cases do not assume that Rel-15 reference signals are used by UE.

- FFS: Send an LS to RAN2 to inform RAN1’s progress on Rel.15 E-CID.

Issues for further discussion

  - Whether to provide assistance signaling to UE in order to facilitate the use of Rel-15 reference signals for the benefit of NR E-CID positioning (e.g. quasi-collocation information)

UE and gNB Measurement Configuration

Summary

A number of contributions have discussed the UE measurement configurations with the following proposals [3-15]:

- (MTK) For high-resolution DL-AoD based positioning, an UE may select a subset of all measured PRS resource sets, and report only RSRPs measured from the selected PRS resource sets. UE may not need to report RSRPs measured from non-selected PRS resource sets. A method for UE to select PRS resource sets is given as follows:

- The network configures the parameters T,kmax, kmin to an UE

- For an UE, let m be the number of cells with measured RSRP greater than T

  - If m≥kmax, then UE selects best (maximum power) kmax cells

  - If m<kmax, then UE selects best kmin cells

FFS: values of T,kmax, kmin

- (MTK) For high-resolution DL-AoD based positioning, an UE may select a subset from all measured PRS resources of a PRS resource set, and only report RSRP measured from the selected PRS resources. UE may not need to report RSRPs measured from non-selected PRS resources. A method for UE to select PRS resources is given as follows:

- The network configures a parameter k to the UE

- For a PRS resource set, the UE selects k PRS resources such that the RSRPs measured from these k PRS resources are larger than that measured from the other PRS resources

FFS: value range of k

- (MTK) For high-resolution DL-AoD based positioning, an UE may report RSRP ratios or differential RSRPs derived from measured PRS resources of a PRS resource set. A method for UE to report RSRP ratios or differential RSRPs is given as follows:

  - Suppose pk is the RSRP measured from PRS resource with ID k, k = 1,2,…,N

  - Suppose pm is a maximum RSRP, i.e.,  pm≥pk  for k = 1,2,…,N

  - Then UE reports the PRS resource ID m, but UE does not report pm

  - For PRS resource with ID j, where j≠m, UE reports j and xj to the network, where xj=pj/pm or xj=10logpm-10logpj

FFS: value range and quantization level of RSRP ratios or differential RSRPs

- (Sony) Support to limit the positioning measurement report (i.e. based on the selected gNBs).

- (Samsung) The configuration of UL PRS should be exchanged between serving gNB and neighboring gNBs via X2 interface so that the neighboring gNBs know when and where exactly to measure UL PRS.

- (CATT) It needs FFS on whether the DL PRS configuration for UE measurements can be configured to be the same as the DL PRS configuration for UE transmission for a cell. (Intel)NR supports signaling to configure UE with DL PRS Measurement Report (DPMR) settings

- (Intel)For DL PRS RSRP and SINR measurements,

  - UE is configured with a list of DL PRS Resource Set IDs and DL PRS Resource IDs for measurement and reporting of measurement results

- (Intel)For DL PRS RSTD measurements,

  - UE is configured whether to provide single or multiple (M) RSTD measurements per DL PRS Resource Set (i.e. whether RSTD measurement and report should be done per configured DL PRS Resource ID or per DL PRS Resource Set)

  - If a UE is configured with a list of DL PRS Resource IDs for each configured DL PRS Resource Set ID,

    - UE is expected to use indicated DL PRS Resources for RSTD measurements and reporting

  - Otherwise, if there is no configuration of DL PRS Resource IDs provided by network,

    - UE is expected to autonomously select one or M DL PRS Resources for RSTD measurements and report their IDs jointly with RSTD measurements

- (Intel)DL PRS Measurement Report Configuration contains at least the following information

  - Reference DL PRS Resource ID(s) / Resource Set

  - List of DL PRS Resource Set IDs

  - List of DL PRS Resource IDs for each DL PRS Resource Set ID

  - DL PRS Report Type (e.g. periodic, semi-persistent, aperiodic)

  - DL PRS Measurement Set (e.g. RSTD, RSRP, SINR, RSRPFAP, etc.)

  - Periodicity of DL PRS Measurement Report (for periodic and semi-persistent reporting)

  - Type of RSTD report

    - per DL PRS Resource or per DL PRS Resource Set

  - Type of UE Rx-Tx time difference report

    - per DL PRS Resource or per DL PRS Resource Set

  - multi-cell ranging or serving cell ranging + RSTD with neighbor cells

- (Intel)UE is not expected to be configured with number of DL PRS Resources /Resource Sets that exceed UE capabilities in terms of DL PRS measurements and reporting

- (Ericsson) The network configures values number of base stations (N), the number of paths (P) and number of strongest beams (Q) for the measurements to be performed and reported by the UE

- (Ericsson) The maximum value for the number of cells in an RSTD measurement N , number of measured paths per cell P and number of reported paths per cell Q are a the UE capability .

- (Ericsson) Value of P should be decided based on number of peaks required for positioning method such as, single base station based positioning, fingerprinting, multipath based positioning.

- (Ericsson) Network should be able to configure finer RSTD measurements in CIR around any peak.

- (Ericsson) The measurement rates and measurement reporting rates should be configured by the network.

- (Ericsson) Requirements on the measurement precision (RSTD precision) should be set by the network based on the required time of arrival accuracy.

Comments

| Company |  | Comments |
| --- | --- | --- |

Issues for further discussion

Suggest further discuss the details of the configuration and parameters for the UE/gNB measurements based on the decision made on the design of the DL PRS and UL PRS, the definition of the UE and gNB measurements, as well as parameters included in the UE and gNB measurement reports.

Other important issues related to UE/gNB measurements

Summary

- (Ericsson)RAN1 to inform RAN4 about the benefits of using interpolation in connection with TOA estimation.

Comments

| Company |  | Comments |
| --- | --- | --- |

Issues for further discussion

- Whether RAN1 needs to inform RAN4 about the benefits of using interpolation in connection with TOA estimation.

References

- RP-190752 	New WID: NR Positioning Support 	Intel Corporation, Ericsson

- R1-1907869	LS on Reference Point for Timing Related Measurements	RAN1

- R1-1908116	NR positioning measurements	Huawei, HiSilicon

- R1-1908176	Discussion on UE and gNB measurements for NR positioning	vivo

- R1-1908309	Remaining issues on measurements for NR positioning	ZTE Corporation

- R1-1908348	Measurements for NR Positioning	Nokia, Nokia Shanghai Bell

- R1-1908358	Discussion on Measurements for NR Positioning	OPPO

- R1-1908406	Measurement and reporting design for NR Positioning	MediaTek Inc.

- R1-1908511	Discussion on UE and gNB measurements for NR Positioning	Samsung

- R1-1908573	UE and gNB measurements for NR Positioning	CATT

- R1-1908661	Physical layer measurements for NR positioning	Intel Corporation

- R1-1908705	Discussion on UE and gNB measurements for NR Positioning	LG Electronics

- R1-1908790	UE measurements for NR Positioning	Sony

- R1-1908874	Discussion on UE and gNB measurements	CMCC

- R1-1909280	UE and gNB measurements for NR Positioning	Qualcomm Incorporated

- R1-1909426	UE and gNB measurements for NR positioning	Ericsson

- R1-1909545 	Measurements for NR Positioning 	FUTUREWEI

| Definition | The relative timing difference between the neighbouring cell j and the reference cell i, defined as TSubframeRxj – TSubframeRxi, where TSubframeRxj is the time when the UE receives the start of one subframe from cell j, TSubframeRxi is the time when the UE receives the corresponding start of one subframe from cell i that is closest in time to the subframe received from cell j. For intra-frequency RSTD measurements, if the measurement gap is not configured, UE is not expected to measure the PRS resource(s) outside of the active downlink bandwidth part or with a different numerology from that of the active downlink bandwidth part. For NR frequency range 1, the reference point for the RSTD shall be the antenna connector of the UE. For NR frequency range 2, TBD |
| --- | --- |
| Applicable for | RRC_CONNECTED intra-frequency RRC_CONNECTED inter-frequency |