3GPP TSG RAN WG1 #98			R1-1908601

Prague, CZ, August 26th – 30th, 2019

Source:	CATT

Title:	Considerations on Type II CSI enhancement

Agenda Item:	7.2.8.1

Document for:	Discussion and Decision

Introduction

DFT-based compression scheme has been agreed as Type II rank 1~4 codebook in Rel-16, where frequency domain (FD) compression is employed to remove the redundancy among all the subbands. In this contribution, we focus on the unresolved issues of Type II CSI.

FD basis subset selection

In RAN1#97 meeting, the following agreement on parameter setting for SD/FD basis were achieved [1]:

Agreement

For further details on the agreed UCI parameters in Table 1 of R1-1905629:

- RI ({1,…, RIMAX}∈1,…,RIMAX) and KNZ,TOTKNZ,TOT (the total number of non-zero coefficients summed across all the layers, where KNZ,TOTKNZ,TOT {1,2,…, 2K0}KNZ,TOT∈1,2,…,2K0 are reported in UCI part 1

  - FFS: If the total number of non-zero coefficients are jointly encoded with M’ (if supported) or independently encoded

- For RI=3-4, bitmaps, each with size-2LMi (i=0,1,…, RI-1, where i denotes the i-th layer) are reported in UCI part 2

  - FFS: If alt 3-4 is supported, size-2LMi2LMi-1 (i=0,1,…, RI-1i=0,1,…,RI-1, where i i denotes the ii-th layer) are reported in UCI part 2

- The following FD basis subset selection scheme is supported:

  - For N3≤19N3≤19, one-step free selection (cf. Alt5.1 in RAN1#96bis) is used

  - For N3>19, IntS is window-based and fully parameterized with MinitialMinitial, indicating that the intermediate set consists of FD bases mod(Minitial + n, N3), n=0,1, …,  modMinitial+n,N3, n=0,1,⋯N3'-1

    - The value N3'=αM where  is higher-layer configured from two possible values

      - FFS (to be finalized in RAN1#98 Prague): the supported parameter combinations for (L, p, β, )L,p,β,α

  - The 2nd step subset selection is indicated by an X2-bit combinatorial indicator (for each layer) in UCI part 2

From the above agreement, both one-step selection and two-step selection are supported for FD basis subset selection. The two-step selection is used for larger N3 value to reduce CSI overhead. Two parameters, i.e. α and Minitial, are therefore introduced to indicate the length and the initial FD basis of the intermediate set. In this section, details on these two parameters are discussed individually.

Configuration of α

In RAN1#97 meeting, further agreement on α was achieved [1]:

Agreement

In RAN1#98, finalize the values of  based on the following aspects

- Candidate values for  to be down selected/evaluated: at least {1.5, 2, 2.5}

  - The set of values is to be finalized via offline email discussion prior to RAN1#98

- Configuration of :

  - Whether it is independent of other FD compression parameters, or dependent on at least one of the other FD compression parameters, i.e. p (=y0, and/or v0 for RI=3-4), L, β, and/or R

  - Whether  is rank-specific or rank-common

  - Note: This is to be discussed along with the supported parameter combinations for (L, p, β, ) L,p,β,α

Based on the codebook structure, the value of p determines the number of FD basis Mi used for each layer. With a larger p value, more FD basis could be used for subband compression, which may result in a larger intermediate set. The value of β defines the number of NZ coefficients K0. If K0 is larger, more FD basis would be allowed and used for subband compression. Since α indicates the length of the intermediate set, it is possible to be related to p or β. In the following, we evaluate the distribution of the length of intermediate set N3' with different values of p and β. The N3' is determined as a union of the FD basis for each layer. And the FD basis for each layer is selected freely without any windowing operation. Thus the optimum N3' value is obtained. Assume the number of subbands is Nsb=13 and R=2. As given in Table 1, three configurations of p values are considered. The other evaluation assumptions of the simulation are given in Table AI in the Appendix.

Table 1:  Configurations of p for evaluation

|  | config1 | config2 | config3 |
| --- | --- | --- | --- |
| rank=1 | 12 | 14 | 14 |
| rank=2 | 12, 12 | 14, 14 | 14, 14 |
| rank=3 | 14, 14,14 | 14, 14,14 | 18, 18,18 |
| rank=4 | 14,14,14,14 | 14,14,14,14 | 18,18,18,18 |

Fig.1 shows the distribution of N3' with different configurations of p values. The other codebook parameters are kept the same where β=12 and L=4. According to Fig.1, the same p value results in the same N3', which is irrespective to these three codebook configurations. Taking rank=3 for example, with the same p=14, the distributions of N3' for config1 and config2 almost coincide. On the other hand, we compare the cases with different p values. For rank=3, the resulted N3' for config1 is 17 (@80% probability) and the value is 11 for config3. This means  N3' increases as p increasing. But N3' is not doubled from p=18 to p=14. Therefore, for optimation, smaller value of α could be configured for larger p value. However, such optimization increases the specification complexity and seems not essential. In our opinion, α should not be related to p values.

Figure 1: N3' distribution with different p values

Observation-1: The same p value results in the same N3', and N3' increases as p increasing.

Fig.2 shows the distribution of N3' with different configurations of β values. The other codebook parameters are kept the same where config2 is used for p values and L=4. According to Fig.2, for β=14, β=12  and β=34, the corresponding distributions of N3' almost coincide with each other. In this way, α should be independent to β values to maintain the same length of the intermediate FD basis set.

Figure 2: N3' distribution with different β values

Observation-2: For β=14, β=12  and β=34, the corresponding distributions of N3' almost coincide with each other.

Proposal-1: Configuration of  does NOT depend on p and β.

For α configuration, another issue is whether  is rank-specific or rank-common. Fig.3 shows the N3' distribution for config2 in Table 1. From Fig.3, we can observe that the N3' difference is obvious for different rank values. For example, at 50% probability, we have N3'=8 for rank=1 and N3'=10 for rank=4. Correspondingly, as M=4, α should be 2 and 2.5 for rank=1 and rank=4, respectively. Such result is quite reasonable, since the total number of FD basis required becomes linear to the rank values, i.e. i∙M, i=1,2,3,4. The larger number of FD basis needed would result in a larger intermediate FD basis subset. Further, we compare the system performance for the rank-common and rank-specific cases. For the rank-common case, α=2 is used for rank=1~4. For rank-specific case, α=2 and α=2.5 are adopted for rank=1~2 and rank=3~4, respectively. The performance of free selection of FD basis without window restriction is given as a baseline. From Fig.4, the rank-specific case achieves both cell edge gain and cell average gain against the rank-common case. Therefore, α should be rank-specific to adjust N3' for different rank values.

Figure 3: N3' distribution for different rank values

Figure 4: Performance comparison for rank-common and rank-specific cases

Proposal-2:  should be rank-specific.

After the email discussion, the following offline agreement has been made for the candidate α values.

Offline agreement:

In RAN1#98, finalize the values of  via down selection from {1.5, 2, 2.5}

         FFS: =3 as an additional candidate

In order to decide which  to be chosen, the results of system-level simulation are given. In the simulation, Minitial is chosen by using brute force method, i.e., each FD basis of the intermediate set could be regarded as an initial point. Then, the optimal Minitial is selected. Note that the intermediate set must include the first FD basis corresponding to the strongest linear combination coefficient after cycle shift for FD basis. Let the cell average UPT and cell edge UPT of  be the performance baseline. Fig.5~Fig.6 show the performance comparison for config1 ~ config2 in Table 1 with different , respectively.

Figure 5: Relative performance comparisonMinitial for config1 with different 

Figure 6: Relative performance comparisonMinitial for config2 with different 

As shown in Fig.5, the performance of different  values is almost same for config1, while =1.5 requires less feedback overhead compared with the other values. Hence, =1.5 is preferred.  In Fig. 6, the UPT of cell average or cell-edge of =2.5 for config2 achieves the best one.  Compared with =1.5, =2.5 only requires additional 4 bits to choose FD basis for each layer when config2 is adopted.  =2.5 should be another configured value.

Observation-3: From feedback overhead perspective, =1.5 requires least overhead with a little performance loss. From performance perspective, =2.5 achieves the best performance with a little overhead increase.

Proposal-3: {1.5, 2.5} are preferred considering the tradeoff between performance and overhead.

Minitial Minitial reporting

In addition to the length of the intermediate FD basis set, the initial value of FD basis, Minitial,was agreed to be reported in UCI part 2 according to the following agreement [1]:

Agreement

On SCI (RI>1) and FD basis subset selection indicator, support Alt B described in the following table.

- FFS: details on bitwidth and possible values for Minitial Minitial reporting in UCI part 2

- FFS: whether the possible value(s) for Minitial Minitial can depend on configured FD compression parameters

- Up to the editor to capture this agreement

|  | Alt B |
| --- | --- |
| SCI for RI>1 | Alt3.4: Per-layer SCI, where SCIi SCIiis a –bit (i=0,1,…(RI - 1)). The location (index) of the strongest LC coefficient for layer i before index remapping is (li*,mi*), SCIi=li*, and mi* is not reported |
| Index remapping | For layer i, the index mimi of each nonzero LC coefficient cli,mi is remapped with respect to mi* to mi such that mi*=0. The FD basis index kmiassociated to each nonzero LC coefficient cli,mi is remapped with respect to kmi* to kmi such that kmi*=0. The sets cli,mi≠cli*,0 and kmi≠0 are reported. Informative note (for the purpose of reference procedure): The index li,mi of nonzero LC coefficients is remapped as (li,mi)→(li,mi-mi*modMi). The codebook index associated with nonzero LC coefficient index li,mi is remapped as kmi→kmi-kmi*modN3. |
| Combinatorial indicator for N3 ≤ 19N3≤19 | log2N3-1Mi-1bits |
| Combinatorial indicator for N3>19N3 > 19 | log2N3'-1Mi-1 bits |
| Minitial | Reported in UCI part 2, details on bitwidth and possible values are FFS |

Firstly, to verify whether MinitialMinitial depends on configured FD compression parameters, such as compression parameter p, the number of beam L and the NZC parameter β, the Minitial distributions for different configurations are given. Without loss generality, α is set to 2 for rank=1~4. The number of subbands is still Nsb=13 and R=2. Therefore, there are total 26 candidate initial points due to N3 = NsbR = 26.

Figure 7: Minitial distributions for rank=1~4 with different configurations of p

Fig. 7 shows the Minitial distributions for rank=1~4 with different configurations of p values. For the Minitial statistics,  L and β are set to 4 and 0.5 respectively. Note that Minitial =27 denotes the first FD basis corresponding to strongest compression coefficients after using cycle shift for FD basis in order to be convenient for statistics. For the same window size, the Minitial distributions are similar as shown in the four subfigures of Fig. 7. This indicates that same rule that selecting Minitial values could be applied to the configurations which have the same window size. Minitial

Figure 8: Minitial distributions for rank=1~4 with different L for config1

Figure 9: Minitial distributions for rank=1~4 with different L for config2

Figure 10: Minitial distributions for rank=1~4 with different L for config3

Fig. 8 ~ Fig. 10 show the Minitial distributions for rank=1~4 with different L for config1 ~ config3 respectively.  We could see that the Minitial distributions are almost same when rank=1 with different configurations. It is also be seen that the distribution difference for rank=2~4 with different L for different configurations is only about 10%, which could be neglected. Therefore, Minitial Minitial does not depend on L.

Since the compression coefficients are calculated by using the selected FD basis, the Minitial distribution has been determined before configuring NZC parameter β.  Therefore, β values should not have impact on Minitial distribution. For simplicity to verify the assumption, Fig. 11 shows the Minitial distributions for rank=1~4 with different β when config2 is adopted, and the number of beam L=4.

Figure 11: Minitial distributions for rank=1~4 with different β for config2

As shown in Fig. 11, the Minitial distributions are similar for rank=1~4 with different β. Therefore, Minitial does not depend on β.

Observation-4: Minitial does not depend on L and β. If the length of the intermediate set N3’ remains invariable, the Minitial distributions are similar.

In subsection 2.1, the Minitial is chosen using a brute force way to obtain the best initial point of window. Although it could achieve optimal performance, the computation complexity is too high to afford for UE. Hence, to reduce computation complexity and feedback overhead, we propose option 2 and option 3 to report Minitial as follows.

- Option 1：Brute force

- Option 2：Five fixed Minitial

- Minitial = N3 - N3’ + 2

- Minitial = N3 – 0.75N3’ + 1

- Minitial = N3 – 0.5N3’ + 1

- Minitial = N3 – 0.25N3’ + 1

- Minitial = 1

- Option 3：One fixed Minitial

- Minitial = N3 – 0.5N3’+ 1

Option 2 means that five candidate points are regarded as initial points of the window. In this way, computation complexity of UE could be reduced with less performance loss. But it still needs a bitwidth to report Minitial. To check how much performance is degraded, the option 3 that Minitial is set to a fixed point is proposed. Obviously, option 3 consumes less overhead and has less computation complexity to obtain the initial point of window, compared with the other options. For simplicity, let the first FD basis be almost located in the middle point of the window as calculated in option 3. Config2 is adopted to evaluate the performance of these options. In Fig. 12, performance comparisons with different options to report Minitial are given. In the simulation, the FD compression parameters L and β are set to 4 and 0.5, respectively.

Figure 12:  Performance comparisons for config2 with different options to report Minitial.

As shown in Fig. 12, the cell average UPT and cell edge UPT of option2 and option 3 are similar to that of option 1, and their performance is approach to that of free selection FD basis without window restriction.  Both option 2 and option 3 could be considered as the possible Minitial values in order to reduce computation complexity of UE.

Observation-5:  The performance of the proposed option 2 and option 3 approaches to that of option 1.

Proposal-4: The possible Minitial value should depend on the window size N3’. Minitial could be chosen from either option 2 or option 3.

- Option 2：Five fixed Minitial

- Minitial = N3 - N3’ + 2

- Minitial = N3 – 0.75N3’ + 1

- Minitial = N3 – 0.5N3’ + 1

- Minitial = N3 – 0.25N3’ + 1

- Minitial = 1

- Option 3：One fixed Minitial

- Minitial = N3 – 0.5N3’+ 1

CSI omission

In NR Rel-15, Type II CSI omission is performed in a non-transparent way, where CSI omission is known both to gNB and UE. The non-transparent CSI omission is beneficial for gNB to adjust the resource allocation for the subsequent CSI reporting. For example, if the PUSCH resource is allocated for rank=1 CSI reporting and the real CSI reporting is rank=2, CSI has to be omitted for this instance. In the following, PUSCH resource allocation would refer to the payload of rank=2 CSI to avoid CSI omission. Therefore, non-transparent CSI omission should also be supported for Rel-16 Type II CSI omission.

Proposal-5:Non-transparent CSI omission should be supported for Rel-16 Type II CSI.

Based on the Type II CSI structure, a straightforward way for CSI omission is to discard the NZ coefficients to meet the capacity of PUSCH. In this section, we give an analysis on such omission scheme. Since the PMI payload mainly comes from the bitmap of location indication and NZ coefficients, only these two parts are considered. Assume the number of SD basis L=4, the number of subbands is Nsb=13, R=2, and β=14. In addition, 6 bits quantization is used for each NZ coefficient. CSI omission is discussed for the following three configurations. As illustrated by Table 2~4, if PUSCH is allocated according to rank=1 CSI payload, CSI omission would occur for rank=2~4. There are cases (rank=4 of config-1, rank=2 of config-2 and rank=2/4 of config-3) that only 2 NZ coefficients are possible to be reported. Further, for the cases rank=3/4 of config-2, the CSI payload would never meet the PUSCH capacity even if all the NZ coefficients are omitted. This is due to the bitmap payload increasing for higher rank. In this way, it is not feasible to omit the NZ coefficients only.

Observation-6: Due to the bitmap payload increasing for higher rank, it is not feasible to omit the NZ coefficients only.

Table 2:  CSI omission for configuration-1

| Config-1 | p value | Bitmap payload (bits) | Before omission |  | After omission |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | #NZC | NZC payload (bits) | #NZC | NZC payload (bits) |
| rank=1 | 12 | 56 | 14 | 84 | 14 | 84 |
| rank=2 | 12, 12 | 112 | 28 | 168 | 4 | 24 |
| rank=3 | 14, 14,14 | 96 | 28 | 168 | 7 | 42 |
| rank=4 | 14,14,14,14 | 128 | 28 | 168 | 2 | 12 |

Table 3:  CSI omission for configuration-2

| Config-2 | p value | Bitmap payload (bits) | Before omission |  | After omission |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | #NZC | NZC payload (bits) | #NZC | NZC payload (bits) |
| rank=1 | 14 | 32 | 8 | 48 | 8 | 48 |
| rank=2 | 14, 14 | 64 | 16 | 96 | 2 | 12 |
| rank=3 | 14, 14,14 | 96 | 16 | 96 | - | - |
| rank=4 | 14,14,14,14 | 128 | 16 | 96 | - | - |

Table 4:  CSI omission for configuration-3

| Config-3 | p value | Bitmap payload (bits) | Before omission |  | After omission |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | #NZC | NZC payload (bits) | #NZC | NZC payload (bits) |
| rank=1 | 14 | 32 | 8 | 48 | 8 | 48 |
| rank=2 | 14, 14 | 64 | 16 | 96 | 2 | 12 |
| rank=3 | 18, 18,18 | 48 | 16 | 96 | 5 | 30 |
| rank=4 | 18,18,18,18 | 64 | 16 | 96 | 2 | 12 |

From the above analysis, omission should also be applied to the bitmap. This means gNB and UE have a common understanding on the omitted bitmap size and the omitted number of NZ coefficients. In addition, the correspondence between bitmap and the NZ coefficients has to be maintained. We propose the following scheme accordingly. The main principle is to guarantee the correspondence between bitmap and the NZ coefficients, with the adequate CSI payload known to both gNB and UE.

- Step-1: Determine the number of FD basis, Ms, needed to be omitted. Ms may depend on the predefined omission granularity (i.e. half of Mi or Mi) or be decided by UE and reported to gNB.

- Step-2: Determine the number of NZ coefficients, Ks, needed to be omitted. Ks could be calculated according to the size of the omitted bitmap and the allocated PUSCH capacity, or could depend on the predefined omission granularity (i.e.2K0RI).

- Step-3: Associate bitmap and the NZ coefficients. Define those NZ coefficients, which correspond to the omitted Ms FD basis, as candidate omission coefficients. Denote the number of candidate omission coefficients as Kc.

  - If Kc≤Ks, besides Kc candidate omission coefficients, additional Ks-Kc NZ coefficients should be omitted. Then the values of the corresponding locations of the bitmap should be revised.

  - If Kc>Ks, only Ks coefficients among the candidate omission coefficients are omitted. The other Kc-Ks candidate omission coefficients, called padding coefficients, are reported and finally ignored by gNB. These padding coefficients aims to maintain the CSI payload thus gNB could decode correctly.

For illustration, taking the case rank=4 of config-2 in Table 3 for example. Assume the omission granularity for FD basis is Mi=4, which means at least one layer is omitted during CSI omission. For FD basis omission, since the PUSCH capacity is allocated for rank=1, three layers have to be omitted (i.e. Ms=12). Then, the payload of bitmap becomes 32bits. For NZ coefficients omission, compared with rank=1, the additional 48bits NZ coefficients should be omitted (i.e. Ks=8). Fig.13(a) shows the case Kc>Ks, where the omitted three bitmaps (layers 2~4) correspond to 11 candidate NZ coefficients (i.e. Kc=11). In this way, 3 NZ coefficients among the candidate omission coefficients are remained as padding coefficients. Another case Kc≤Ks is depicted in Fig.13 (b), where the omitted three bitmaps (layers 2~4) correspond to 7 candidate omission coefficients (i.e. Kc=7). Besides these candidate omission coefficients, 1 more NZ coefficient in layer 1 has to be omitted.

(a) Kc>Ks

(b) Kc≤Ks

Figure 13: CSI omission

Proposal-6: For CSI omission, support the following scheme:

- Step-1: Determine the number of FD basis, Ms, needed to be omitted. Ms may depend on the predefined omission granularity (i.e. half of Mi or Mi) or be decided by UE and reported to gNB.

- Step-2: Determine the number of NZ coefficients, Ks, needed to be omitted. Ks could be calculated according to the size of the omitted bitmap and the allocated PUSCH capacity, or could depend on the predefined omission granularity (i.e.2K0RI).

- Step-3: Associate bitmap and NZ coefficients. Define those NZ coefficients, which correspond to the omitted Ms FD basis, as candidate omission coefficients. Denote the number of candidate omission coefficients as Kc.

  - If Kc≤Ks, besides Kc candidate omission coefficients, additional Ks-Kc NZ coefficients should be omitted. Then the values of the corresponding locations of the bitmap should be revised.

  - If Kc>Ks, only Ks coefficients among the candidate omission coefficients are omitted. The other Kc-Ks candidate omission coefficients, called padding coefficients, are reported and finally ignored by gNB. These padding coefficients aims to maintain the CSI payload that gNB could decode correctly.

CBSR

According to the following agreement [2], CBSR is supported for the DFT-based compression Type II codebook. In this section, our views on CBSR design will be provided.

Agreement

For RAN1 NR-AH 1901:

- Identify the remaining details required to finalize Type II rank 1-2 compression, e.g. range of values and configuration for each DFT-based compression parameter, CBSR utilization, detailed UCI design (such as reporting of coefficients associated with strongest beam/polarization)

- Companies are encouraged to evaluate the options A, B, C, D, and E (“other schemes”) summarized in Table 3 of R1-1813002 for potential support for Type II rank 1-2 overhead reduction

In NR Rel-15, RI restriction and beam restriction are supported for Type II CSI. The aim of beam restriction is to control the combined beam direction to avoid interference with other cells. For Rel-15 Type II CSI, the beam direction of Type II CSI is determined by the linear combination of multiple DFT beams, which is related to DFT vectors and the corresponding coefficients. The total N1O1N2O2 DFT beams are divided into O1O2 beam groups, where each group comprises N1N2 beams. Then through restricting the maximum WB amplitude associated with each DFT beam in the selected beam groups, the beam direction could be controlled.

Following the same principle, both RI restriction and beam restriction should be supported for the DFT-based compression scheme. According to the structure of the codebook, the beam direction is related to the SD basis, the NZ coefficients and the FD basis. However, how the FD basis would influence the beam direction is quite complicated, which makes it difficult to determine the restricted FD basis correctly for gNB implementation.  Since the benefit of CBSR on FD basis is not clear, it should not be supported in Rel-16. On the other hand, the SD basis restriction could reuse the grouping scheme adopted in Rel-15, where several adjacent SD basis comprise one group. The compression amplitude coefficient set restriction could be achieved by restricting the maximum value of the amplitude coefficient within the set. Take the following codebook structure into account,

W=W1W2WfH
=B0B1…BL-100B0B1…BL-1∙pdiff0,0q(0,0)pdiff0,1q(0,1)…pdiff0,M-1q(0,M-1)⋮⋮…⋮pdiffL-1,0q(L-1,0)pdiffL-1,1q(L-1,1)…pdiffL-1,M-1q(L-1,M-1)prefpdiffL,0q(L,0)prefpdiffL,1q(L,1)…prefpdiffL,M-1q(L,M-1)⋮⋮…⋮prefpdiff2L-1,0q(2L-1,0)prefpdiff2L-1,1q(2L-1,1)…prefpdiff2L-1,M-1q(2L-1,M-1)∙fk0fk1…fkM-1H

where pdiff(i,j) and qi,j denote the differential amplitude coefficient and the phase coefficient of SD basis i and FD basis j. Reference amplitude coefficient is denoted as pref. For each beam, the weighting factor corresponds to the linear combination M FD basis instead of a single combining coefficient as in Rel-15.

If restriction selects B1 only, the amplitude coefficients set pdiff(1,0),pdiff(1,1),…,pdiff(1,M-1) for one polarization and the amplitude coefficients set pdiff(L+1,0),pdiff(L+1,1),…,pdiff(L+1,M-1) for another polarization would be restricted. Note that, for the second polarization, the overall amplitude is determined by the product of the reference amplitude  pref and the differential amplitude. The beam direction is actually controlled by the overall amplitude, which should be taking into account for CBSR. Assume the CBSR rule is the maximum value of the amplitude coefficients, we’ll have

maxj=0,1,…,M-1pdiff(1,j) ≤Amax

maxj=0,1,…,M-1prefpdiff(L+1,j) ≤Amax

where Amax represents the maximum allowed value of the amplitude coefficient.

Proposal-7: CBSR is applied only to the SD basis. The maximum value of the associated overall amplitude (i.e. the product of reference amplitude and differential amplitude) is restricted.

Conclusions

In this contribution we discussed the unresolved issues of Type II CSI including FD basis subset selection, CSI omission and CBSR. Based on the analysis and simulation results, our observations and proposals are summarized below.

Observations:

Observation-1: The same p value results in the same N3', and N3' increases as p increasing.

Observation-2: For β=14, β=12  and β=34, the corresponding distributions of N3' almost coincide with each other.

Observation-3: From feedback overhead perspective, =1.5 requires least overhead with a little performance loss. From performance perspective, =2.5 achieves the best performance with a little overhead increase.

Observation-4: Minitial does not depend on L and β. If the length of the intermediate set N3’ remains invariable, the Minitial distributions are similar.

Observation-5:  The performance of the proposed option 2 and option 3 approaches to that of option 1.

Observation-6: Due to the bitmap payload increasing for higher rank, it is not feasible to omit the NZ coefficients only.

Proposals:

Proposal-1: Configuration of  does NOT depend on p and β.

Proposal-2:  should be rank-specific.

Proposal-3: {1.5, 2.5} are preferred considering the tradeoff between performance and overhead.

Proposal-4: The possible Minitial value should depend on the window size N3’. Minitial could be chosen from either option 2 or option 3.

- Option 2：Five fixed Minitial

- Minitial = N3 - N3’ + 2

- Minitial = N3 – 0.75N3’ + 1

- Minitial = N3 – 0.5N3’ + 1

- Minitial = N3 – 0.25N3’ + 1

- Minitial = 1

- Option 3：One fixed Minitial

- Minitial = N3 – 0.5N3’+ 1

Proposal-5:Non-transparent CSI omission should be supported for Rel-16 Type II CSI.

Proposal-6: For CSI omission, support the following scheme:

- Step-1: Determine the number of FD basis, Ms, needed to be omitted. Ms may depend on the predefined omission granularity (i.e. half of Mi or Mi) or be decided by UE and reported to gNB.

- Step-2: Determine the number of NZ coefficients, Ks, needed to be omitted. Ks could be calculated according to the size of the omitted bitmap and the allocated PUSCH capacity, or could depend on the predefined omission granularity (i.e.2K0RI).

- Step-3: Associate bitmap and NZ coefficients. Define those NZ coefficients, which correspond to the omitted Ms FD basis, as candidate omission coefficients. Denote the number of candidate omission coefficients as Kc.

  - If Kc≤Ks, besides Kc candidate omission coefficients, additional Ks-Kc NZ coefficients should be omitted. Then the values of the corresponding locations of the bitmap should be revised.

  - If Kc>Ks, only Ks coefficients among the candidate omission coefficients are omitted. The other Kc-Ks candidate omission coefficients, called padding coefficients, are reported and finally ignored by gNB. These padding coefficients aims to maintain the CSI payload that gNB could decode correctly.

Proposal-7: CBSR is applied only to the SD basis. The maximum value of the associated overall amplitude (i.e. the product of reference amplitude and differential amplitude) is restricted.

References

- Chairman's Notes RAN1#97, Reno, USA, 13th – 17th May 2019

- Chairman's Notes RAN1#95, Spokane, USA, 12th - 16th November 2018

Appendix

Table AI: Evaluation assumptions

| Parameters | Values |
| --- | --- |
| Duplex mode | FDD |
| Scenario | Dense Urban (Macro) |
| Carrier frequency | 4GHz |
| Channel model | According to the TR 38.901 |
| BS Tx power | 41dBm |
| BS antenna configuration | 32 ports: (8,8,2,1,1,2,8), (dH, dV) = (0.5, 0.8)λ |
| BS antenna height | 25m |
| UE antenna configurations | 4RX: (1,2,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| UE receiver noise figure | 9 dB |
| Modulation | Up to 256QAM |
| Coding on PDSCH | LDPC Max code-block size=8448bits |
| Number of RBs | 52 RBs for 15 kHz SCS |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes |
| UE distribution | 80% Indoor, 3km/h, 20% Outdoor, 30km/h |
| UE receiver type | MMSE and IRC |
| Feedback assumption | Realistic |
| Channel estimation | Realistic |
| MIMO mode | SU-MIMO with rank adaption |
| CSI feedback period | 5ms |
| Feedback delay | 4ms |