3GPP TSG RAN WG1 Meeting #97																R1-1908613

Prague, CZ, August 26th – 30th, 2019

Source: 	Intel Corporation

Title:	UE-group wake-up signal for eMTC

Agenda item:	6.2.1.1

Document for:	Discussion and Decision

# Introduction

In RAN#80 a new work item (WI) on Rel-16 MTC enhancements for LTE was agreed. One of the WI objectives is to improve DL transmission efficiency and/or UE power consumption [1]. In particular, specify support for UE-group wake-up signal (WUS).

In the previous RAN1#96bis meeting, the following agreements regarding UE-group WUS were made [2]:

Agreement Support the following options for WUS resource configuration - Up to 2 orthogonal resources may be configured in time domain. - Up to 2 orthogonal resources may be configured in frequency domain - FFS: both options can be configured simultaneously to have up to 4 orthogonal WUS resources Maximum total number of WUS resources is no larger than 4 including the legacy WUS resource For further discussion Different WUS sequences associated with the same PO are used for UE groups regardless of whether the UE groups are in the same or different WUS resources. Working Assumption - UE assumes the transmit power for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence. - Maximum WUS duration for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence Agreement Each UE monitors up to X WUS sequences. - Value of X will be selected between 2 and 3 - X=3 can only be supported if a common WUS for a subset of UE groups for service-based grouping is supported

In this contribution, we express our views on remaining details of UE-group WUS design for eMTC devices in Rel-16.

# Discussion

There are two factors which should be taken into account while choosing among WUS multiplexing options: network (NW) flexibility and UE complexity.

From the network flexibility perspective, simultaneous configuration of up to 2 orthogonal WUS resources per each dimension (time and frequency), which results in up to 4 orthogonal WUS resources total, seems to be the most preferable one. It potentially allows the NW to do TDM between the WUS resources, e.g., to provide additional power boosting to the WUS. On other hand, it does not preclude FDM in deployments where the boosting is not needed, and the NW just prefers to increase the number of available WUS resources to serve a larger number of UE groups.

Proposal 1:

Support simultaneous configuration of up to 2 orthogonal WUS resources per each dimension (time and frequency), which results in up to 4 orthogonal WUS resources total.

  - This accounts the legacy WUS resource

From the UE complexity perspective, the number of WUS sequences the UE monitors within the configured WUS resources should be minimized. Therefore, the support of UE grouping where UEs may belong to multiple UE-groups (more than 2) simultaneously and which may result in additional common WUS to monitor for is not preferred. Accordingly, the UE should monitor only up to 2 WUS sequences (UE-group and the common one) within the configured resource.

Proposal 2:

Each UE monitors up to 2 WUS sequences within the configured WUS resource.

An illustration of WUS multiplexing concept described above can be found in Figure 1.

Figure 1. Multiplexing of WUS resources in Rel-16.

The use of different WUS sequences even in the orthogonal WUS resources is not preferred because this potentially reduces the multiplexing capacity of each WUS resource. In other words, a smaller number of WUS sequences becomes available for SS-CDM per resource in this case. The concern regarding PAPR increase in the time domain due to repeated structure of WUS in the frequency domain across the NB can be addressed by BS transmitter implementation. For example, by applying different quasi-random phase terms to FDM-ed WUS resources. This solution will not impact the UE performance because, according to the agreement from RAN1#96 [3], the UE is required to monitor WUS(s) in only one WUS (time/frequency) resource location, i.e., no cross-resource processing is expected. Moreover, some BS Tx handling of repeated WUS may be required anyway in case of waking up all served UEs using the same common WUS signal, e.g., when the legacy WUS is configured as the common WUS.

Proposal 3:

The same WUS sequences can be reused for UE groups associated with orthogonal WUS resources corresponding to the same PO.

# Conclusions

In this contribution, we discussed remaining considerations on UE-group WUS for eMTC devices in Rel-16. Our proposals are summarized as below:

Proposal 1:

Support simultaneous configuration of up to 2 orthogonal WUS resources per each dimension (time and frequency), which results in up to 4 orthogonal WUS resources total.

  - This accounts the legacy WUS resource

Proposal 2:

Each UE monitors up to 2 WUS sequences within the configured WUS resource.

Proposal 3:

The same WUS sequences can be reused for UE groups associated with orthogonal WUS resources corresponding to the same PO.

# Reference

RP-181450, “Rel-16 MTC enhancements for LTE”, Ericsson, 3GPP TSG RAN meeting #80, La Jolla, USA, June 2018.

RAN1 Chairman’s Notes. 3GPP TSG RAN WG1 Meeting #97, Reno, USA, 13th – 17th May, 2019.

RAN1 Chairman’s Notes. 3GPP TSG RAN WG1 Meeting #96, Athens, Greece, February 25th – March 1st, 2019.