3GPP TSG RAN WG1 Meeting #98		R1-1908616

Prague, CZ, August 26th – 30th, 2019

Source:	Intel Corporation

Title:	Quality report in Msg3 and connected mode for eMTC

Agenda item:	6.2.1.5

Document for:	Discussion/Decision

# Introduction

At RAN #82 meeting, the WID for Rel-16 MTC enhancements was updated [1]. The objective for improving DL transmission efficiency and/or UE power consumption was updated to include the following:

- Specify quality report in MSG3 at least for EDT [RAN1, RAN2]

- Specify aperiodic quality report in connected mode using same quality definition as in Msg3 [RAN1, RAN2, RAN4]

In RAN1 #97 meeting, the following agreements were made regarding the quality report in Msg3 [2]:

Agreement - Specify DL quality information with up to 8 bits for DL quality report in at least - Msg3 report for EDT case - Msg3 report for connected mode - Fixed range of DL quality information is defined for DL quality report with up to 8 bits Agreement For both DL quality report in Msg3 in IDLE mode and DL quality report in connected mode: - Confirm the following working assumption: - For DL quality report in CE mode A (PRACH CE level 0, 1), the pre-defined maximum aggregation level is fixed to 24. - For DL quality report in CE mode B (PRACH CE level 2, 3), the pre-defined maximum aggregation level is fixed to 24. Agreement Confirm the following working assumption: - For DL quality report in Msg3 for IDLE mode UEs, the narrowband(s) for downlink quality measurement includes at least the narrowband(s) on which MPDCCH of RAR is monitored. Agreement For DL quality report in connected mode, the mechanism for triggering is based on one or more of the following: - DCI (e.g. UL grant, PDCCH order) - MAC CE For further discussion - Specify DL quality information with the following size for DL quality report in Msg3 for non-EDT case - Up to 8 bits if there is enough room in the scheduled TBS, otherwise 2 bits - Fixed range of DL quality information is defined for DL quality report with up to 8 bits - Adaptive range of DL quality information is defined for DL quality report with up to 2 bits - FFS details Agreement For IDLE mode DL quality report in Msg3 in CE mode A (PRACH CE level 0, 1) with up to 8 bits, if the repetition number in DL quality information equals to 1, select one between the following alternatives in RAN1#98 meeting: - Alternative 1: Repetition number =1 is reported with aggregation level - Alternative 2: Repetition number =1 is reported assuming aggregation level of 24 Agreement For IDLE mode DL quality report in Msg3 in CE mode A (PRACH CE level 0, 1) with up to 8 bits, if frequency hopping for MPDCCH is enabled, select one between the following alternatives in RAN1#98 meeting: - Alternative 1: Only wideband DL quality is reported. - Alternative 2: DL quality on a preferred narrowband and position of the preferred narrowband are reported in addition to wideband DL quality. - The preferred narrowband is selected within the set of narrowband(s) on which wideband DL quality is measured.

In this contribution, we present our views on the designs to support DL quality report in Msg3, as well as the quality report in connected mode using the same quality definition as in Msg3.

# DL quality report in Msg3 in Idle mode

In this section, we discuss about the designs for the support of DL quality report in Msg3 in Idle mode.

## UE capability

The support of DL quality report in Msg3 should be UE capability. However, for initial access, the UE has not reported its capability yet during the random access procedure. Therefore, whether or how to indicate the UE capability should be studied. One option is to configure dedicated PRACH resources for UE supporting DL quality report in Msg3. However, considering the need of PRACH partitioning for EDT and the existence of legacy eMTC UEs, this option may impact the system capacity, and thus is not preferred.

On the other hand, the UL grant and Msg3 can be designed to enable the scheduling of both Msg3 without DL quality report and Msg3 with DL quality report, as elaborated in Section 2.2. It can be up to UE to select which one to follow depending on its capability. With this method, there would be no need of UE capability indication for support of DL quality report in Msg3. eNB can perform blind decoding of Msg3 to determine whether the DL quality report is carried by the UE or not.

Observation 1:

Indication of UE capability for support of DL quality report in Msg3 via PRACH partitioning would impact system capacity.

Proposal 1:

It is UE capability whether the DL quality report in Msg3 for Idle mode is supported.

  - The design of UL grant in RAR and Msg3 should enable UE to choose whether DL quality report is transmitted in Msg3 or not, which requires no report of UE capability.

## DL quality message contents and format

Among the two alternatives of the DL quality report when the number of repetitions is equal to 1, Alternative 1 with the reported aggregation level could provide more detailed DL quality information. In addition, considering that the supported number of MPDCCH repetitions is {1, 2, 4, 8, 16, 32, 64, 128, 256} and the supported aggregation levels is {1, 2, 4, 8, 16, 12, 24}, both Alternative 1 and Alternative 2 require 4 bits for the DL quality report indication, if joint encoding of repetition number and aggregation level for Alternative 1 is used. Therefore, it is preferred to adopt Alternative 1 if the repetition number in DL quality report equals to 1, which is able to provide more detailed information with the same required number of bits for the indication.

Proposal 2:

For IDLE mode DL quality report in Msg3 in CE mode A with up to 8 bits, if the repetition number in DL quality information equals to 1,

  - Only aggregation level is reported with pre-defined repetition number equals to 1 (Alternative 1).

Apply joint encoding of the repetition number and aggregation level for IDLE Mode DL quality report in Msg3 in CE mode A (see Table 1 for details).

Table 1. Reported number of MPDCCH repetitions and aggregation level for providing DL quality information in Msg3.

| # | Repetition Number | Aggregation Level | CE mode |
| --- | --- | --- | --- |
| 0 | 2 | 24 | A, B |
| 1 | 4 | 24 | A, B |
| 2 | 8 | 24 | A, B |
| 3 | 16 | 24 | A, B |
| 4 | 32 | 24 | A, B |
| 5 | 64 | 24 | A, B |
| 6 | 128 | 24 | A, B |
| 7 | 256 | 24 | A, B |
| 8 | 1 | 24 | A |
| 9 | 1 | 1 | A |
| 10 | 1 | 2 | A |
| 11 | 1 | 4 | A |
| 12 | 1 | 8 | A |
| 13 | 1 | 16 | A |
| 14 | 1 | 12 | A |
| 15 | reserved |  |  |

When frequency hopping for MPDCCH is enabled there is an alternative to report DL quality on a preferred NB and position of the preferred NB in addition to wideband DL quality for IDLE mode DL quality report in Msg3 in CE mode A with up to 8 bits. Recall that the frequency hopping occurs between 2 or 4 NBs configured by higher layer signaling. Thus, 2 bits would be sufficient to indicate the position of the preferred narrowband. Therefore, if frequency hopping is configured, the position of the preferred NB carried in the DL quality report can be indicated. The remaining 2 bits may be used as a delta to the number of repetitions/aggregation level reported in the wideband DL quality part to carry additional DL quality on the preferred NB.

Proposal 3:

- For IDLE mode DL quality report in Msg3 in CE mode A (PRACH CE level 0, 1) with up to 8 bits, if frequency hopping for MPDCCH is enabled:

  - DL quality on a preferred narrowband and position of the preferred narrowband are reported in addition to wideband DL quality.

As elaborated above, to carry the DL quality report in Msg3, it is expected that a larger TBS is needed for Msg3 with DL quality report. Recall that it is not preferred to indicate UE capability via further PRACH partitioning as discussed in Section 2.1. For the cases where eNB is not aware of UE capability for the support of DL quality report, eNB can always schedule a larger TBS, considering that a larger TB is expected for Msg3 with DL quality report as discussed above. For UEs not supporting the DL quality report, padding bits would be added to the Msg3. Depending on the number of bits needed for DL quality report, the efficiency of UL transmission may be impacted for UEs not supporting the DL quality report. To improve the performance of UL transmission for UEs not capable of DL quality report, similar to the EDT design, the UL grant can be used to indicate multiple TBS values, where a certain TBS value is for Msg3 without DL quality report and another TBS value is for Msg3 with DL quality report. It is up to UE to choose which TBS value to be used depending on its capability for support of DL quality report in Msg3. Specifically, the UL grant in RAR would indicate a TBS assuming no DL quality report, which would be used for UEs not capable of DL quality report in Msg3. A larger TBS value associated to the indicated TBS can be predefined, which would be used for UEs capable of DL quality report. With this option, the eNB would blindly detect whether the DL quality report is carried in the Msg3.

Proposal 4:

Consider support scheduling of multiple TBS values similar as EDT design, and let UE choose the TBS value depending on its capability, where

  - The indicated TBS is for Msg3 without DL quality report,

  - And a larger TBS associated to the indicated value can be predefined, and be used for Msg3 with DL quality report.

## Triggering method for the DL quality report

In the previous meeting, there was a proposal to support triggering of DL quality report in Msg3 by RAR [3]. Specifically, the CSI request field in UL grant carried in RAR can be reused for this purpose. However, this approach is limited to CE mode A only, because the CSI request field is not available in the UL grant carried in RAR for CE mode B. It is preferred to have a common design for CE mode A and CE mode B. With this approach, RAR needs to be updated for CE mode B to include the trigger. Considering the backward compatibility, where the triggering is not applicable to the UEs which are not capable of the DL quality report in Msg3, two RARs need to be transmitted for CE mode B, one for UEs capable of DL quality report in Msg3 while the other for UEs without such capability. This would increase DL overhead, which is not preferred. Therefore, it is proposed that the RAR-based triggering for DL quality report is not supported. As discussed in Section 2.2, actually the triggering is not needed.

Proposal 5:

For DL quality reporting in Msg3 in IDLE mode, RAR-based triggering is not supported.

## NB(s) to be measured

It was confirmed in the last meeting that at least the narrowband(s) on which MPDCCH of RAR is monitored for DL quality report in Msg3 for IDLE mode UEs. Monitoring other NB(s) would require additional power consumption from the UE which is not preferred. Therefore, it is proposed not to support additional NB(s) for DL quality report other than the NB(s) on which MPDCCH of RAR is monitored.

Proposal 6:

No additional NBs for DL quality report are supported.

# DL quality report in connected mode

As the DL quality report in Msg3 in IDLE mode has been already supported, we can extend the design to support the DL quality report in Msg3 in connected mode. In addition, in connected mode, the aperiodic DL quality report can be carried in PUSCH as well. For the aperiodic report in connected mode using the same quality definition as in Msg3, it should be UE capability to support this feature

Regarding the triggering method, for the DL quality report in Msg3 in connected mode, different from IDLE mode where the reserved bit ‘R’ in RAR is used to differentiate the EDT and non-EDT cases, the reserved bit ‘R’ in RAR is available in connected mode and can be used as the trigger. For the aperiodic DL quality report in PUSCH, the CSI request field in UL grant can be reused. Note that this is applicable to CE mode A only. There is no support of CSI measurement and thus no CSI request field in UL grant for CE mode B. If the aperiodic DL quality report is supported in PUSCH in CE mode B, the UL grant in CE mode B can be extended to include the 1-bit trigger. Note that different from IDLE mode, eNB knows the UE capability in connected mode, and thus the extension in DCI for CE mode B would not result in increased DL overhead as only the new DCI would be transmitted.

When the aperiodic report is triggered, UE needs to decide which metric to be used for the DL quality report, the legacy CQI or the new DL quality report using the same definition as in Msg3. As one option, once this feature is enabled, UE will always report the DL quality report in PUSCH using the same quality definition as in Msg3 instead of legacy CQI. Alternatively, the DCI can be extended to include an additional bit to indicate whether the legacy CQI or the new DL quality report using the same definition as in Msg3 should be reported.

Proposal 7:

It is UE capability to support DL quality report in connected mode using the same quality definition as in Msg3.

Support the DL quality report in Msg3 and PUSCH in connected mode.

For triggering method of the DL quality report in connected mode:

  - The reserved bit ‘R’ is used as the trigger for DL quality report in Msg3.

  - The CSI request field in UL grant is used as the trigger for DL quality report in PUSCH.

# Conclusion

In this contribution, we discuss the design for the support of DL quality report in Msg3 and in connected mode for eMTC. Based on the discussions, we make the following observations and proposals:

Observation 1:

Indication of UE capability for support of DL quality report in Msg3 via PRACH partitioning would impact system capacity.

Proposal 1:

It is UE capability whether the DL quality report in Msg3 for Idle mode is supported.

  - The design of UL grant in RAR and Msg3 should enable UE to choose whether DL quality report is transmitted in Msg3 or not, which requires no report of UE capability.

Proposal 2:

For IDLE mode DL quality report in Msg3 in CE mode A with up to 8 bits, if the repetition number in DL quality information equals to 1,

  - Only aggregation level is reported with pre-defined repetition number equals to 1 (Alternative 1).

Apply joint encoding of the repetition number and aggregation level for IDLE Mode DL quality report in Msg3 in CE mode A (see Table 1 for details).

Proposal 3:

- For IDLE mode DL quality report in Msg3 in CE mode A (PRACH CE level 0, 1) with up to 8 bits, if frequency hopping for MPDCCH is enabled:

  - DL quality on a preferred narrowband and position of the preferred narrowband are reported in addition to wideband DL quality.

Proposal 4:

Consider support scheduling of multiple TBS values similar as EDT design, and let UE choose the TBS value depending on its capability, where

  - The indicated TBS is for Msg3 without DL quality report,

  - And a larger TBS associated to the indicated value can be predefined, and be used for Msg3 with DL quality report.

Proposal 5:

For DL quality reporting in Msg3 in Idle mode, RAR-based triggering is not supported.

Proposal 6:

No additional NBs for DL quality report are supported.

Proposal 7:

It is UE capability to support DL quality report in connected mode using the same quality definition as in Msg3.

Support the DL quality report in Msg3 and PUSCH in connected mode.

For triggering method of the DL quality report in connected mode:

  - The reserved bit ‘R’ is used as the trigger for DL quality report in Msg3.

  - The CSI request field in UL grant is used as the trigger for DL quality report in PUSCH.

# References

- RP-182891, “Revised WID: Additional MTC enhancements for LTE”, Qualcomm, 3GPP TSG RAN meeting #82, Sorrento, Italy, December 10th – 13th, 2018.

- RAN1 Chairman’s notes, 3GPP TSG RAN WG1 meeting #97, Reno, USA, 13th –17th May 2019.

- R1-1906895, “Discussion on quality report in Msg3 and connected mode”, Samsung, Reno, USA, 13th –17th May 2019.