3GPP TSG RAN WG1 #98	R1-1908630

August 26 – August 30, 2019

Prague, Czech

Agenda item:	7.2.3.2

Source: 	Intel Corporation

Title: 	RACH Enhancements for NR IAB

Document for:	Discussion and decision

# Introduction

RAN1#96bis has made the following agreements on RACH enhancements for NR IAB.

- The periodicity of a backhaul RACH configuration in frames takes the form x_iab = x * λ where:

  - x is the periodicity of an existing RACH configuration,

  - λ is a scaling factor taking values in {1, 2, 4, 8, 16, 32, 64} subject to the constraint x_iab ≤ 64.

- The frame containing backhaul ROs is identified by (nSFN mod x_iab) = ((y + Δy) mod x_iab) where Δy denotes a time offset in frames taking values in the range from 0 to x_iab – 1.

- The subframe (slot) number for a RO of a backhaul RACH configuration is identified by (Sn + Δs,) mod L, where:

  - Sn is the subframe (slot) number of an existing RACH configuration,

  - Δs denotes a time offset in subframes (slots) taking values in the range from 0 to L – 1, where L is the number of subframes (slots) in a frame.

NOTE: The usage of the terms subframe or slot is meant to align to the terminology used in the existing RACH configuration tables in TS 38.211, e.g. subframe in Tables 6.3.3.2-2 and 6.3.3.2-3, and slot in Table 6.3.3.2-4.

- The validity of ROs for backhaul RACH configurations is regulated by the rules defined in Rel-15 for existing RACH configurations.

- Partial overlap of ROs between RACH configurations used in two adjacent links (upstream towards the parent and downstream towards the children from an IAB node perspective) is allowed.

RAN1#97 has further made the following agreements.

- The introduction of scaling factor for PRACH configuration period also scales the SSB-RACH association period and association pattern period accordingly.

- Assuming the scaling factor to be λ, for IAB nodes table 8.1-1 of 38.213 gets modified to the following:

Mapping between scaled PRACH Configuration Period (x * λ) and SSB to PRACH Occasion Association Period

| Scaled PRACH configuration period (ms) | Association period |
| --- | --- |
| 10 | {1, 2, 4, 8, 16, 32, 64} |
| 20 | {1, 2, 4, 8, 16, 32} |
| 40 | {1, 2, 4, 8, 16} |
| 80 | {1, 2, 4, 8} |
| 160 | {1, 2, 4} |
| 320 | {1, 2} |
| 640 | {1} |

- The definition of the SSB-RACH association pattern period for IAB nodes should be updated as follows: An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH blocks repeats at most every 640 msec.

- The signalling design for the IAB specific backhaul RACH configurations is up to RAN2. RAN1 has the following recommendations:

  - x is the periodicity of an existing RACH configuration,

  - The ability to configure the IAB specific backhaul RACH configuration additionally to the Rel 15 RACH configuration for access UEs is supported.

      - If the IAB specific RACH configuration is not provided, RAN1 assumes that IAB node will use the configured Rel-15 RACH configuration for IAB node initial access.

  - Capability to configure IAB specific backhaul RACH configurations need to be provided for both CBRA & CFRA.

In this contribution, based on those discussion and agreements, we further elaborate on RACH procedures for IAB.

# RACH Configurations for IAB Nodes

It has been agreed that new RACH configuration specific to IAB nodes are derived with extension of existing Rel-15 RACH configuration obtained by scaling factor and offset (frame-based or slot/subframe-based), and the signalling design for the IAB specific backhaul RACH configurations is up to RAN2.

There can be different alternatives for the IAB specific RACH configuration based on existing Rel-15 PRACH configuration tables, which will generate different parameter lists:

- Alt 1: The scaling factor λ (if any), frame-based offset Δy (if any), slot/subframe-based offset Δs (if any), are separately signaled in addition to the PRACH configuration index (0-255). In this alternative, new information elements on the scaling factor and offsets are signaled in RACH configuration. The network can coordinate and choose the scaling factor, the offsets values and the PRACH configuration index for each of the IAB nodes to minimizing time overlapping on PRACH resources among adjacent hops.

- Alt 2: Pre-define the scaling factor and the offset values for entries of the PRACH configuration tables, so that given a PRACH configuration index, an IAB node can find out the corresponding scaling factor and offsets. Rel-15 PRACH configuration is reused. No additional parameters are needed for IAB RACH configuration.

Alt 1 allows flexibly applying scaling factors and slot/subframe offset values for entries of the PRACH configuration table with the cost of additional signalling overhead. Alt 2 does not introduce additional signalling, with the cost of extra effort in defining the scaling factors and slot/subframe offset values for entries of the PRACH configuration table, but it has less flexibility of applying different combinations of scaling factor, slot/subframe offsets and PRACH configuration index.

Proposal 1: Consider the following two alternatives on RACH configuration for an IAB node:

- Alt 1: Explicitly signal the following information to an IAB node:

- PRACH configuration index (0-255);

- scaling factor λ (if any);

- time offset in frames Δy (if any);

- time offset in subframes/slots Δs (if any).

- Alt 2: Pre-define scaling factor and time offsets for each PRACH configuration index. Signal PRACH configuration index as in Rel-15. No explicit signalling on scaling factor and slot/subframe offsets.

# Conclusion

In this contribution, we discussed RACH procedure in IAB networks. It is summarized by the following proposals.

Proposal 1: Consider the following two alternatives on RACH configuration for an IAB node:

- Alt 1: Explicitly signal the following information to an IAB node:

- PRACH configuration index (0-255);

- scaling factor λ (if any);

- time offset in frames Δy (if any);

- time offset in subframes/slots Δs (if any).

- Alt 2: Pre-define scaling factor and time offsets for each PRACH configuration index. Signal PRACH configuration index as in Rel-15. No explicit signalling on scaling factor and slot/subframe offsets.