3GPP TSG RAN WG1 Meeting #98																	R1-1908634

Prague, Czech Republic, August 26th – 30th, 2019

Source:	Intel Corporation

Title:	NR V2X Sidelink Communication Under gNB Control

Agenda item:	7.2.4.2.1

Document for:	Discussion and Decision

Introduction

In the SI phase, it was agreed that NR sidelink may be scheduled by NR Uu in dynamic manner (DCI based), type 1 configured grant manner (RRC based), and type 2 configured grant manner (RRC+DCI based). Also, sidelink Mode-1 operation in NR can be controlled by LTE Uu in a form similar to type 1 configured grant under restriction of no additional LTE Uu functionality/procedure.

The following agreements were made by RAN1 WG with respect to support of Mode-1 for NR V2X.

RAN1# 96bis - A dynamic grant provides resources for one or multiple sidelink transmissions of a single TB. - A configured grant (type-1, type-2) provides a set of resources in a periodic manner for multiple sidelink transmissions. - UE decides which TB to transmit in each of the occasions indicated by a given configured grant. - FFS: whether different transmissions of a TB can take place across multiple configured grants. - Other restrictions on what can be transmitted in a given configured grant (e.g., based on QoS, destination UE, etc.) are up to RAN2. RAN1# 97 - Sidelink HARQ ACK/NACK report from transmitter UE to gNB is supported with details FFS. - Note: this reverts the following agreement from RAN1#96: - Sidelink HARQ ACK/NACK report from UE to gNB is not supported in Rel-16. - SR/BSR report to gNB for the purpose of requesting resources for HARQ retransmission is not supported. - NR sidelink does not support performing different transmissions of a TB using different configured grants. - For mode 1: - A dynamic grant by the gNB provides resources for transmission of PSCCH and PSSCH.

In this contribution, we discuss NR V2X Mode-1 sidelink resource allocation aspects. Our views on other NR-V2X design aspects are summarized in companion contributions [2]-[8].

NR Mode-1

A fundamental assumption for design of Mode-1 should be that receivers are agnostic to mode of operation used for sidelink transmissions. Under this assumption, Mode-1 functionality is mostly confined to design of DCI and RRC signaling for the dynamic and configured scheduling together with assistance signaling from UEs. While the design of sidelink transmission procedures should be commonly developed in a framework of sidelink physical layer procedures.

UE Assistance Information for Scheduling

In order to manage sidelink resources, it was agreed to support UE assistance signalling on UE geographic information and V2X traffic related information (message size, periodicity, time offset). The details are now up to RAN2 to define.

A UE should also be able to provide sidelink physical layer measurements which are critical to assists gNB scheduling decisions and resource allocation for sidelink transmissions. The following information on sidelink can be considered for sharing b/w UE and gNB:

Resource occupation

  - Congestion metric (e.g. CBR if agreed)

  - Detected collision in scheduled sidelink transmissions

Sensing information

  - For Mode-1 & Mode-2 multiplexing

Power control & CSI related measurements

  - Time-frequency selective SL-RSRP/SL-RSSI

  - L3 filtered SL-RSRP in case of unicast

  - CQI and RI for unicast

  - At least the following information is reported to gNB in Mode-1 using L2 signalling mechanism

    - L3 filtered SL-RSRP measured on unicast links

    - Sidelink CQI/RI on unicast links

    - Sidelink congestion metrics (e.g. CBR)

    - FFS details of sensing related measurements

Dynamic Scheduling

Dynamic scheduling of sidelink transmissions involves monitoring of a specific DCI format that grants sidelink transmission. The content of the DCI should allow control of most of the transmission parameters, such as time-frequency resource allocation, power, etc.

DCI monitoring

It is natural to size-match the new DCI with one of the other formats to be monitored for Uu operation in order to keep the same DCI size budget and blind decoding budget. When the DCI is size-matched, a new SL-RNTI for monitoring of this DCI is desirable. Moreover, it should be possible to size match DL/UL scheduling DCI to the SL scheduling DCI as well if the resulting payload size is large. Another alternative is to enable configurable size for DCI scheduling sidelink transmissions.

  - SL scheduling DCI is size-matched to one of configured formats for Uu operation

    - It should be possible to size match DL/UL scheduling DCI to the SL scheduling DCI as well

  - New RNTI (e.g. SL-RNTI) is introduced for monitoring of the DCI for dynamic Mode-1 scheduling of sidelink transmission

This new DCI may have its own parameters for monitoring (CORESET, search space, periodicity, number of candidates per aggregation level, etc.).

Timing information

One important role of the DCI is also to carry timing of sidelink transmission with respect to the instance of DCI reception. In LTE, this timing was a fixed value, while in NR it should at least be variable because of potentially different subcarrier spacing between Uu carrier and SL carrier. Moreover, the timing should capture different gaps between DL occasion for monitoring and SL occasion for transmission due to different UL-DL configurations.

A preferred option is to place timing offset between DCI and SL transmission into DCI content similar as it was done in NR uplink. However, the timing value K2 was encoded together with SLIV for PUSCH. Since time domain resource allocation for SL within a slot is expected to be designed differently, it may not be possible to follow same approach. Thus, the sidelink K2 may be explicitly signalled as a DCI field.

Note, that K2 should follow minimum preparation time restriction wherein N2/Tproc,2 should suit well since PSSCH preparation and SL grant decoding should take similar time as PUSCH preparation + UL grant decoding.

  - SL scheduling DCI carries signalling of a time gap between DCI reception time instance and corresponding SL transmission time instance

    - The UE should not expect the time gap is smaller than the minimum PSCCH/PSSCH preparation time

MCS signalling

Another important aspect is whether MCS is selected by a UE or may be signalled by gNB. It is natural to support at least the case of UE-autonomous MCS assignment since in many cases it is hard to know actual channel quality at gNB and adapt transmission parameters. If gNB-based control of MCS is deemed necessary, it may be done using semi-static UE-specific configuration, or be a part of congestion control restrictive settings.

  - DCI scheduling sidelink does not convey MCS for sidelink transmission

Power control

It was already agreed that the pathloss to gNB may be used for setting sidelink transmit power. This mechanism may be common for Mode-1 and Mode-2. However, Mode-1 also has possibility to control sidelink transmit power more dynamically by using TPC-like signalling (e.g. max power on/off). Alternative mechanism is to switch between different OLPC loops, which may be dependent on QoS, etc.

  - Introduce means of dynamic change of TX power per transmission based on gNB scheduling decision

    - FFS between

      - Switching OLPC loops

      - TPC commands

HARQ related information

For groupcast/unicast HARQ operation it being discussed whether to introduce HARQ process ID, NDI, RV signalling in SCI. With HARQ-ACK feedback forwarding to gNB it may be useful in some cases to control NDI and HARQ process ID at gNB.

  - When sidelink HARQ feedback forwarding is enabled, HARQ ID and NDI are conveyed in SL DCI

Configured Scheduling

Type 1 configured scheduling relies purely on RRC configuration to provide transmission parameters and resources. First of all to enable this type of scheduling, the parameters being carried by DCI need to be conveyed in a UE-specific RRC message. In addition, timing parameters and periodicity of sidelink occasions which follow these transmission parameters should be also signalled.

Type 2 configured scheduling should reuse most of the past considerations by applying DCI-based activation/deactivation of SL semi-persistent transmissions, while RRC may convey at least periodicity of such a process. The DCI should be distinguished from other formats by another SPS-SL-RNTI.

  - New RNTI (e.g. SPS-SL-RNTI) is introduced for monitoring of the DCI for Type 2 configured Mode-1 scheduling of sidelink transmission

It is important to be able to provide more than one such a configuration (both Type 1 and Type 2), so that quasi-periodic traffic and different services may be efficiently multiplexed. For example, a UE may have unicast, groupcast, broadcast services enabled each of them having different QoS attributes and therefore transmission parameters. RAN2 already agreed to support multiple active configured sidelink grants for V2X:

RAN2 agreement:

1: 	Multiple active configured sidelink grants should be supported in NR sidelink.

Specific to Type 2, it may be of high importance to inform gNB about successful reception of activation/deactivation DCI for SL configured scheduling since there is no other way for gNB to know about successful reception besides waiting for another request for resources from a UE in case it missed the DCI. RAN2 already discussed this aspect and agreed on the following:

RAN2 agreement:

2: 	A confirmation for activation/deactivation of SL configured grant type-2 is needed. Details are FFS.

For that purpose, same mechanism as for SPS/CG in a form of MAC confirmation may be reused. However, PUSCH then needs to be granted separately, if not already scheduled. Therefore, potentially a mechanism of HARQ-ACK transmission to gNB may suit better since PUCCH resource may be allocated by the grant scheduling SL transmission. The mechanism for HARQ-ACK based confirmation may largely reuse NR Uu for DL scheduling.

  - Confirmation of Type 2 activation/release of a configured sidelink grant is based on HARQ-ACK in PUCCH/UCI

Sidelink HARQ Support in Mode-1

In general, HARQ-ACK bits related to SL should be multiplexed identically to feedbacks in response to PDSCH. In that sense, all types of codebooks and physical channels (UCI on PUCCH and UCI on PUSCH) should be supported as for DL.

In both cases, dynamic and semi-static codebooks, PUCCH resource and timing to HARQ feedback should be explicitly indicated in DCI.

- HARQ feedback timing value, in slots, should be carefully considered. First, it should be counted from the slot of associated PSSCH transmission, not the slot of DCI reception, due to causality between PSSCH and HARQ feedback. Second, the value range may need to be extended beyond current 15 slots since SL HARQ RTT may be typically larger especially with N = 4 and sparse resource pools.

- PUCCH resource indicator should be completely reused, pointing to particular PUCCH resource in the slot indicated by the timing value.

- DAI should also be signaled for the purpose of dynamic codebook selection as it is currently done in DCI scheduling PDSCH.

Note, that for proper scheduling, both K and K' (namely PSSCH-to-PSFCH and PSFCH-to-PSSCH processing/preparation times as discussed in [6]) should be known to gNB. Therefore, these values need to be fixed in specification, potentially subject to some other system parameters (e.g. SCS) and UE capability.

  - For the purpose of HARQ feedback forwarding, the mechanism of HARQ-ACK bit multiplexing into PUCCH and PUSCH is fully reused from HARQ-ACK in response to PDSCH

  - At least for dynamic sidelink scheduling:

    - PUCCH resource indication is signalled in DCI scheduling sidelink

    - Timing offset between associated PSSCH transmission and UCI carrying the associated HARQ feedback is signalled in DCI scheduling sidelink

    - DAI in case of dynamic HARQ-ACK codebook

The potential behavior is clear for single-shot transmissions and ACK/NACK indications. However, when blind retransmissions or potential retransmissions are also provided, there may be different approaches. For example, HARQ feedback may be either forwarded after the last TTI, or after each TTI, or after an intermediate number of TTIs.

In our understanding, single SL grant should schedule only single HARQ-ACK transmission towards gNB in order to smoothly reuse the Rel.15 UCI procedures. Moreover, it should cover most of the cases.

In NACK-only cases, the UE should report ACK if does not detect NACK, except the cases when it was not able to detect PSFCH.

  - At least for dynamic sidelink scheduling, single SL DCI conveys resources for single UCI with HARQ-ACK even when more than one TTI is scheduled

  - In NACK-only cases, the UE should report ACK if it does not detect NACK

    - FFS the cases when UE was not able to detect PSFCH due to collisions

Similar to DL SPS, a semi-static HARQ codebook can be used to reserve HARQ-ACK bit for configured grant cases. In that case, the PUCCH resource may be semi-statically configured for a UE. Therefore, a UE is then expected to transmit HARQ according to each configured grant occasion.

  - Semi-static HARQ codebook is used for sidelink HARQ-ACK feedback forwarding in case of configured SL grants

Mode-1 and Mode-2 Multiplexing

Intra-UE

It was agreed to support simultaneous Mode-1 and Mode-2 operation from single UE perspective. While from RX perspective the mode should be transparent, from TX perspective additional handling may be needed. For example, when Mode-1 and Mode-2 transmissions are going to collide, at least QoS and services type of the transmissions should be taken into account for the purpose of prioritization. However, when it happens that both Mode-1 and Mode-2 have same QoS of the traffic, a tie-breaking rule would be to prioritize Mode-1 transmission.

Note, the collisions may even be avoided so that Mode-2 sensing procedure excludes known Mode-1 transmission opportunities if they have lower associated priority. Alternatively, when Mode-1 collides with Mode-2, depending on QoS the mode-2 resource may be reselected.

  - QoS attributes (i.e. priorities) are used to prioritize between Mode-1 and Mode-2 if a UE is configured to perform both Mode-1 and Mode-2

  - If a UE is configured to perform both Mode-1 and Mode-2 at the same time and QoS attributes are equal, the Mode-1 transmissions are prioritized over Mode-2 sidelink transmissions

    - FFS Mode-2 reselection triggering in case of collisions

Inter-UE

Sharing of sidelink resources in gNB-controlled (Mode-1) and UE-autonomous (Mode-2) resource allocation modes is one of the practical issues due to fundamentally different approaches used for sidelink resource allocation. The seamless coexistence between these two modes was one of the opens for LTE V2X communication that was addressed at some extent in LTE R15, where possibility of sensing and reporting was agreed for Mode-3 UEs. Lack of sidelink radio-environment/measurement information may results in sub-optimal scheduling decision for gNB controlled sidelink resource allocation.

There are two basic mechanisms that may facilitate resource sharing between modes:

Sensing information reporting

  - As in Rel.15 LTE V2X, a mechanism to collect and report sensing information to gNB may be introduced for seamless mode co-existence. This would have impact of Mode-2 sensing procedure itself, i.e. the Mode-2 sensing information should be feasible to report with manageable overhead and taking into account latency.

Resource reservation by a UE and/or by a gNB

  - Mode-1 UEs may transmit resource reservation signaling on dedicated sidelink control channel resources and reserve sidelink resources in a shared sidelink resource pool

  - gNB may send reservation/preemption signal using broadcast/groupcast DCI, as illustrated in Figure 1.

Figure 1: Broadcasting of information on gNB-controlled scheduled transmissions

  - At least the following information needs to be reported to gNB for Mode-1 and Mode-2 resource sharing

    - Congestion metrics

    - Sensing related measurements, FFS details

LTE eNB-based control of NR Sidelink

Based on RAN1 WG agreements made so far we do not foresee any additional physical layer enhancements at LTE side to control NR sidelink. Most of the work is expected in RAN2 side where RRC signalling and UE assistance signalling forwarding is being specified.

Conclusions

In this contribution, network-controlled sidelink communication mode was analyzed for both NR Uu and LTE Uu cases. Based on discussion and analysis we have the following proposals:

  - At least the following information is reported to gNB in Mode-1 using L2 signalling mechanism

    - L3 filtered SL-RSRP measured on unicast links

    - Sidelink CQI/RI on unicast links

    - Sidelink congestion metrics (e.g. CBR)

    - FFS details of sensing related measurements

  - SL scheduling DCI is size-matched to one of configured formats for Uu operation

    - It should be possible to size match DL/UL scheduling DCI to the SL scheduling DCI as well

  - New RNTI (e.g. SL-RNTI) is introduced for monitoring of the DCI for dynamic Mode-1 scheduling of sidelink transmission

  - SL scheduling DCI carries signalling of a time gap between DCI reception time instance and corresponding SL transmission time instance

    - The UE should not expect the time gap is smaller than the minimum PSCCH/PSSCH preparation time

  - DCI scheduling sidelink does not convey MCS for sidelink transmission

  - Introduce means of dynamic change of TX power per transmission based on gNB scheduling decision

    - FFS between

      - Switching OLPC loops

      - TPC commands

  - When sidelink HARQ feedback forwarding is enabled, HARQ ID and NDI are conveyed in SL DCI

  - New RNTI (e.g. SPS-SL-RNTI) is introduced for monitoring of the DCI for Type 2 configured Mode-1 scheduling of sidelink transmission

  - Confirmation of Type 2 activation/release of a configured sidelink grant is based on HARQ-ACK in PUCCH/UCI

  - For the purpose of HARQ feedback forwarding, the mechanism of HARQ-ACK bit multiplexing into PUCCH and PUSCH is fully reused from HARQ-ACK in response to PDSCH

  - At least for dynamic sidelink scheduling:

    - PUCCH resource indication is signalled in DCI scheduling sidelink

    - Timing offset between associated PSSCH transmission and UCI carrying the associated HARQ feedback is signalled in DCI scheduling sidelink

    - DAI in case of dynamic HARQ-ACK codebook

  - At least for dynamic sidelink scheduling, single SL DCI conveys resources for single UCI with HARQ-ACK even when more than one TTI is scheduled

  - In NACK-only cases, the UE should report ACK if it does not detect NACK

    - FFS the cases when UE was not able to detect PSFCH due to collisions

  - Semi-static HARQ codebook is used for sidelink HARQ-ACK feedback forwarding in case of configured SL grants

  - QoS attributes (i.e. priorities) are used to prioritize between Mode-1 and Mode-2 if a UE is configured to perform both Mode-1 and Mode-2

  - If a UE is configured to perform both Mode-1 and Mode-2 at the same time and QoS attributes are equal, the Mode-1 transmissions are prioritized over Mode-2 sidelink transmissions

    - FFS Mode-2 reselection triggering in case of collisions

  - At least the following information needs to be reported to gNB for Mode-1 and Mode-2 resource sharing

    - Congestion metrics

    - Sensing related measurements, FFS details

References

- RP-190766, “New WID on 5G V2X with NR sidelink”, LGE, Huawei, Shenzhen, China, March, 2019

- R1-1908633, “Physical structure for NR V2X”, Intel Corporation, Prague, Czech Republic, August, 2019

- R1-1908635, “UE-autonomous resource allocation for NR V2X sidelink communication”, Intel Corporation, Prague, Czech Republic, August, 2019

- R1-1908636, “NR V2X sidelink synchronization” , Intel Corporation, Prague, Czech Republic, August, 2019

- R1-1908637, “In-device coexistence solutions for NR V2X sidelink” , Intel Corporation, Prague, Czech Republic, August, 2019

- R1-1908638, “NR V2X sidelink physical layer procedures”, Intel Corporation, Prague, Czech Republic, August, 2019

- R1-1908639, “QoS and congestion control for NR V2X”, Intel Corporation, Prague, Czech Republic, August, 2019

- R1-1908640, “LTE sidelink communication under gNB control”, Intel Corporation, Prague, Czech Republic, August, 2019