3GPP TSG RAN WG1 Meeting #98																R1-1908660

Prague, Czech Republic, August 26th – 30th, 2019

Source:	Intel Corporation

Title:	UL Reference Signals for NR Positioning

Agenda item:	7.2.10.2

Document for:	Discussion and Decision

Introduction

Work item on NR Positioning support was approved in [1]. The WID has the following objectives:

Define new DL positioning reference signals applicable at least for DL-TDOA, DL-AoD, RTT

Define UL SRS with possible enhancements for positioning which is applicable at least for RTT, UL-TDOA, UL-AoA

In this contribution, we continue discussion on UL PRS design based on UL SRS and follow agreements made by RAN1 at the previous meeting (provided in Annex A). Our views on other NR Positioning aspects are provided in our companion contributions [3]-[5].

# UL Reference Signals for NR Positioning

UL SRS was agreed to be used as a starting point for NR Positioning in UL and DL+UL positioning techniques. It was designed for UL channel sounding and beam management. SRS Resources can be combined in SRS Resource Sets. Each SRS Resource can be configured with 1, 2, or 4 antenna ports, transmitted in 1, 2 or 4 consecutive symbols at the end of slot with a configurable signal bandwidth. NR SRS is represented by Zadoff-Chu sequences (if sequence length > 36) and has comb structure. The NR SRS Resource Set can be configured as periodic or on-demand (semi-persistent or aperiodic). Sequence generation is a function of nSRSID, slot and symbol number. The NR SRS supports group or sequence hopping as well as frequency hopping.

Following options can be a potential enhancement for SRS design in order to improve UL PRS capability:

Staggered pattern configuration for UL SRS

Number of symbols and subcarrier density for NR UL SRS

Mechanism to improve UE multiplexing capacity for positioning

## On Positioning Usage for UL SRS

One of the topics that was discussed by RAN1 WG is whether to introduce “positioning” usage field for UL SRS. In our view, it is premature given that no additional enhancements were agreed so far for UL PRS. Given that UL SRS enhancements (that may be introduced) can be also applicable to other use cases, we are not convinced at this stage that this field is really needed. In addition, we assume that UL PRS will be defined in LPP and therefore the usage of configured signal can be automatically determined.

## Periodic, Semi-persistent, Aperiodic UL SRS (PRS) Configuration

At the previous meeting, the following agreement was reached by RAN1 WG with respect to SRS configuration for positioning.

Support the following configurations of SRS for positioning - Semi-persistent configuration - Periodic configuration - Aperiodic configuration

In our view, from physical layer perspective signaling details to support periodic, semi-persistent, and aperiodic UL SRS (PRS) transmissions can be reused.

## UL SRS (PRS) Resource Mapping Pattern

RAN1 agreed to extend number on configurable comb- size N and number of symbols M per UL PRS Resource. The RE Offset in frequency domain was also agreed, however its values were not fixed. In order to complete discussion on RE Offset pattern for UL PRS, in the next table we propose several RE Offset patterns that are recommended to be supported by NR Positioning, an example of some patterns is illustrated in Figure 1.

Table 1: Resource Element Patterns for UL SRS (PRS) Transmission

|  |  | Comb Size – N |  |  |
| --- | --- | --- | --- | --- |
|  |  | 2 | 4 | 6 |
| Number of symbols per DL PRS Resource M | 1 | 0 | 0 | 0 |
|  | 2 | {0, 1} | {0, 2} | {0, 3} |
|  | 4 | {0, 1, 0, 1} | {0, 2, 1, 3} | NA |
|  | 8 | {0, 1, 0, 1, 0, 1, 0, 1} | {0, 2, 1, 3, 0, 2, 1, 3} | NA |
|  | 12 | {0, 1, 0, 1, 0, 1, 0, 1, 0, 1, 0, 1} | {0, 2, 1, 3, 0, 2, 1, 3, 0, 2, 1, 3} | {0, 3, 1, 4, 2, 5, 0, 3, 1, 4, 2, 5} |

Figure 1: RE Patterns for UL SRS Resources

  - Resource element offset in frequency domain depends on number of symbols and comb-size N configured per resource and varies depending on symbol index of UL SRS (PRS) Resources

  - For UL SRS (PRS), adopt resource element patterns provided by Table 1

## Remaining Aspects of Resource Configuration for UL SRS (PRS)

One of the mechanisms to increase UL positioning capacity is to increase frequency division multiplexing order from system perspective. The use of comb-structure is also beneficial to increase the link budget and as a result is capable improve UL based NR positioning performance.

On previous RAN1 meeting, it was agreed to extend the set of comb size to following {2, 4, 8}. To answer on the question of extending current set of values with additional comb-sizes, we can refer to the Figure 2, where the performance of Comb-8 and Comb-12 resource mapping patterns are compared. We have not observed  significant difference in performance and thus it is recommended to follow the current agreement (evaluation assumptions can be found in Annex-B).

Figure 2: UL-TDOA Performance Analysis in UMi for different comb structures

On previous RAN1 meeting, it was agreed to extend the number of symbols allocated for SRS resource {1, 2, 4, 8, 12} and other values such as {3,6,14} are currently FFS. It is very easy to predict, that there is no significant difference between 3 and 4, 6 and 8, 14 and 12 consecutive symbols allocated for SRS transmission. Thus we prefer to follow current agreement for SRS transmission.

  - Follow current agreement on number of symbols and comb-size per UL SRS (PRS) Resource

  - Use NR Rel-15 SRS sequence generation mechanism for UL PRS generation

## Bandwidth operation for UL SRS (PRS)

The amount of bandwidth allocated for UL SRS transmission as a positioning reference signal is directly proportional to the positioning measurement accuracy obtained in that bandwidth, from this perspective it is beneficial to have ability to configure the UL BWP for SRS transmission which is potentially wider than currently active BWP. Thus, we propose to provide configuration mechanism for UL BWP dedicated for UL PRS (SRS) transmission for positioning use case.

  - Enable mechanism for separate configuration of BWP for UL SRS transmission for positioning use case.

## CDM for UL SRS (PRS)

In order to further improve capacity / amount of positioned users, a CDM in time can be used for UL SRS (PRS) transmission. For instance, orthogonal cover codes in time can be applied in assumption of disabled frequency hopping. In this case the SRS sequence generation procedure can be presented by the following formula:

r(pi)l,k=r(pi)l,kwt(l)

r(pi)l,k – SRS sequence calculated according to section 6.4.1.4.2 in TS 38.211 [1];

wt – time spreading sequence / orthogonal cover code. Spreading sequences can be orthogonal and based on Hadamard matrices, DFT based matrices or pseudo orthogonal based of randomly generated sequences.

  - Further consider to introduce time domain code multiplexing for UL SRS (PRS) in order to increase the UE multiplexing capacity for NR positioning in UL

Benefits of using time domain CDM on top of the enhancements proposed in the previous sections are shown in Figure 3, where comb-4 TD-OCC physical structure of SRS is compared with SRS Comb-4 TDM physical structure limited number of resources for SRS transmission were used, evaluation assumptions can be found in Annex-B.

Figure 3: Use of orthogonal cover codes for UL SRS (PRS) transmission

# Conclusion

In this contribution, we have provided our views on UL reference signal design for NR positioning. In summary, we have following proposals:

  - Resource element offset in frequency domain depends on number of symbols and comb-size N configured per resource and varies depending on symbol index of UL SRS (PRS) Resources

  - For UL SRS (PRS), adopt resource element patterns provided by Table 1

  - Follow current agreement on number of symbols and comb-size per UL SRS (PRS) Resource

  - Use NR Rel-15 SRS sequence generation mechanism for UL PRS generation

  - Enable mechanism for separate configuration of BWP for UL SRS transmission for positioning use case.

  - Consider to introduce time domain code multiplexing for UL SRS (PRS) in order to increase the UE multiplexing capacity for NR positioning in UL

# References

- RP-190752, “New WID: NR Positioning Support”, Intel Corporation, Ericsson

- TS38.211, V15.6.0, “NR: Physical channels and modulation”

- R1-1908659, “DL reference signals for NR positioning”, Intel Corporation, Prague, Czech Republic, August, 2019

- R1-1908661, “Physical layer measurements for NR positioning”, Intel Corporation, Prague, Czech Republic, August, 2019

- R1-1908662, “NR positioning physical layer procedures”, Intel Corporation, Prague, Czech Republic, August, 2019

# Annex-A: RAN1#96bis Agreements on UL PRS

In this section, we provide list of agreements made by RAN1 with respect to UL PRS design at the previous meeting.

Agreement: Support the following configurations of SRS for positioning - Semi-persistent configuration - Periodic configuration - Aperiodic configuration Agreement: For positioning, regarding the number of SRS symbols per resource, decide by RAN1#97 whether to increase the set of allowable number of SRS symbols per resource compared to the NR Rel-15 allowable set {1,2,4}. - If it is not decided to increase the number of SRS symbols per resource by RAN1#97, increasing the set of allowable number of SRS symbols per resource compared to the NR Rel-15 allowable set {1,2,4} will not be considered further after RAN1#97. Agreement: For positioning, regarding the possible SRS symbol locations per slot decide by RAN1#97 whether the SRS can be configured only in the last N symbols of a slot with N>6 - If the above decision is not made by RAN1#97, configuring SRS only in the last N symbols of a slot with N>6 will not be discussed further after RAN1#97. Agreement: Select one or both of the following options to support staggered SRS transmissions for UL SRS positioning - Staggered patterns (a collection of SRS symbols from the same antenna port with different offsets for at least some symbols) in a single SRS resource - Configuration of the same antenna port for the transmission of SRS symbols in different SRS resources in a SRS resource set.

RAN1#97 Agreements

Agreement: SRS transmissions for positioning are realized with staggered patterns (a collection of SRS symbols from the same antenna port with different offsets for at least some symbols) in a single SRS resource - FFS: construction of the pattern inside the SRS resource structure Agreement: For positioning, the number of consecutive OFDM symbols in an SRS resource is configurable with one of the values in the set {1, 2, 4, 8, 12} - FFS: Other values including 3,6,14 - Note: Values of 1, 2 and 4 within an SRS resource can already be configured in Rel-15 Agreement: For positioning, starting positions in the time domain for the SRS resource can be anywhere in the slot, i.e. an offset loffset range of {0,1,…,13}. Agreement: For positioning, with regard to UL Beam management/alignment towards serving and neighboring cells, at least the following (option 2 from prior agreement) can be used: - UE Tx beam-sweeping on UL SRS transmissions across multiple UL SRS Resources - FFS: other options Agreement: For positioning, the SRS comb size set is extended from {2,4} to {2,4,8} - FFS: Additional comb sizes: 1, 6, 12 - Note: For the comb sizes of 6 and 12, the number of PRBs may be restricted if currently defined sequences are to be used - FFS: Maximum number of cyclic shifts for the different comb sizes (cyclic shifts for comb sizes of 2 and 4 already exist in Rel-15)

# Annex-B: Evaluation Assumptions

Table 2. Table of system level evaluation assumptions.

| Extended Frequency Mapping Pattern for UL SRS |  |
| --- | --- |
| Parameter | Value |
| Deployment scenario, Bandwidth | Open Office, 5/100 MHz |
| Muting assumption | Fixed number of resources for UL PRS transmission |
| Number of used symbols for UL SRS | 1 |
| Frequency mapping pattern | Comb-8, Comb-12 |
| TD-OCC for UL SRS |  |
| Parameter | Value |
| Deployment scenario, Bandwidth | UMa, 5/100 MHz |
| Muting assumption | Fixed number of resources for UL PRS transmission |
| Number of used symbols for UL SRS | 1 for no TD-OCC case, 4 for TD-OCC case |
| Frequency mapping pattern | Comb-4 |