3GPP TSG RAN WG1 #98	R1-1908671

Prague, Czech, August 26th – 30th, 2019

Source:	OPPO

Title:	Inter UE Tx prioritization and multiplexing

Agenda Item:	7.2.6.5

Document for:	Discussion and Decision

Introduction

In RAN1 #97, some working assumption and agreements were achieved in the following [1]:

Agreements:

- Support at least group common DCI for cancelation indication

  - FFS whether or not to additionally support UE-specific DCI for cancelation indication

Conclusion:

To down-select from the following options for enhanced power control

- Option 1: Indication of open-loop parameter sets by DCI

  - For DG-PUSCH, an open-loop parameter set indicated to the UE by scheduling DCI without using SRI is applied to the scheduled transmission

  - FFS At least for single active CG-PUSCH, an open-loop parameter set is indicated to the UE by a UE-specific field in group common DCI

    - FFS for the case of multiple active CG-PUSCH

  - FFS For a UE, the open-loop parameter sets for DG-PUSCH and CG-PUSCH may be same or different

- Option 2: Indication of TPC with increased range by DCI

  - For DG-PUSCH, a TPC with increased range is indicated to the UE by the TPC field in scheduling DCI

  - FFS At least for single active CG-PUSCH (and potentially also for DG-PUSCH), a TPC with increased range is indicated to the UE by a UE-specific TPC field in group common DCI

    - FFS for the case of multiple active CG-PUSCH

  - At least for DG-PUSCH, for a UE, the number of TPC entries (4 or 8) and power adjustment value for each entry is higher layer configured

  - FFS For a UE, the TPC configuration for DG-PUSCH and CG-PUSCH may be same or different

- Option 3:

  - For DG-PUSCH, use either the solution from option 1 or option 2 for DG-PUSCH as above

    - To down-select from option 1 and 2

  - FFS At lea`st for single active CG-PUSCH, UE derives the transmissions power based on the time/frequency resource indicated by a group common DCI

  - If a CG-PUSCH transmission overlaps with the indicated time/frequency resource, UE use one open-loop parameter set with higher power for the transmission

  - If a CG-PUSCH transmission does NOT overlap with the indicated time/frequency resource, UE use another open-loop parameter set with lower power for the transmission

  - FFS for the case of multiple active CG-PUSCH

  - Note: some companies have concern that this was not captured in the TR as one potential solutions

In this contribution, we focus on UL preemption and power control.

Remaining issues on UL preemption indication

Whether to support stop with resume

Resuming the transmission afterwards is an efficient way but additional efforts are needed, e.g. ensuring the phase continuity of the discontinuous transmission. So Stop with resume is not supported.

Proposal 1: Stop with resume is not supported due to phase discontinuity issue.

What physical channels or signals can be preempted

For PUCCH, resource allocation is not small due to middle or large UCI payload considering CA, CBG, HARQ-ACK multiplexing and CSI. And Collision between PUCCH and URLLC cannot be avoided. So PUCCH needs to be preempted.

For PRACH, initial access procedure is highest priority. Similarly, SSB cannot be preempted in downlink. In addition, PRACH resource is not large, e.g. small number of PRBs in the BWP boundary is enough, and collision between PRACH and URLLC can be avoided. So PRACH cannot be preempted.

For SRS, small number of symbols, e.g. 1-2 symbol is configured for SRS and delay due to collision coordination with SRS is very small. So SRS cannot be preempted.

Proposal 2: PUSCH and PUCCH needs to be preempted but PRACH and SRS cannot be preempted.

Group common DCI design for UL PI

DL PI can be a baseline for UL PI to reduce specification effort; however there are some difference among DL preemption procedure and UL preemption procedure:

- UL PI needs to be transmitted before preemption and there is a gap for UL PI decoding and stop transmission.

- The search space period for UL PI is very short to transmit UL PI promptly. If preemption window equals to search space period for UL PI, then preemption window will be very small and PDCCH overhead will increase significantly.

- Bandwidth for uplink transmission resource may be smaller than DL due to power limitation or small packet for uplink.

- Reliability requirement on UL PI is higher, e.g. BLER = 10e-5, to ensure URLLC transmission reliability.

- UL PI needs to be monitored frequently to stop eMBB transmission promptly, which increases eMBB UE complexity significantly.

For above problems, some modification based on DL PI is needed to support UL PI.

  - DCI contents

Preemption window

UL PI is transmitted before preemption and taking processing time into account, the gap between UL PI and preemption window needs to be defined, as shown in Figure 1. The gap between UL PI and preemption window is determined by PDCCH decoding time and stop transmission time, which is usually smaller than N2.

Figure 1 timing line for DCI with UL PI

In addition, if preemption window equals to search space period for UL PI, then preemption window will be very small and PDCCH overhead will increase significantly. Take Figure 2 as an example, when the length of preemption window keeps the same as search space periodicity of UL PI, then multiple UL PIs is needed. When the length of preemption window is larger than the search space periodicity of UL PI, then only one UL PI is enough. So it is better to decouple the length of preemption window and the search space periodicity of UL PI and it is suggested that the length of preemption window is larger than the search space periodicity of UL PI.

Figure 2 preemption window

Considering system flexibility, the gap between UL PI and preemption window and the length of preemption window can be configured by RRC.

Proposal 3: Considering system flexibility, the gap between UL PI and preemption window and the length of preemption window can be configured by RRC.

Due to preemption window and the search space periodicity is different, the resource indicated by multiple UL PIs may be overlapped and indication result may be different, as shown in Figure 3. It is reasonable to follow latest UL PI due to the latest UL PI is determined by the latest scheduling. And there should be a limitation for UL PI. For example, 1st PI indicates a resource which is not used by URLLC, but 2nd PI indicates the same resource which is used by URLLC, the eMBB UE should consider that the resource is used for URLLC. The situation that 1st PI indicates a resource used for URLLC and 2nd PI indicates the same resource not used for URLLC is not allowed, shows as Figure 3.

Figure 3 Overlapped preemption resources

Proposal 4: The resource indicated by PI maybe overlapped and the overlapped resource is determined by the latest UL PI.

Time/Frequency resource granularity

In DL PI, frequency resource granularity is coarse, e.g. 1/2 BWP or BWP due to large bandwidth transmission for URLLC is assumed. However, for uplink transmission, narrow bandwidth transmission is common due to uplink power limitation and small data transmission for uplink. So the frequency resource granularity for UL PI needs to be finer.  The value of frequency resource granularity can be multiple of RBG, which is better to match URLLC resource allocation and reduce excessive preemption. Time resource granularity is similar as that of DL PI.

Proposal 5: The value of frequency resource granularity needs to be finer and can be multiple of RBG.

Similar as DL PI, frequency resource granularity and time resource granularity can be configured by RRC.  However, the bit-length for preemption resource indication needs to be studied due to 14 can only be decomposed by 2*7 and 1*14 which may not suitable for frequency partition.

Proposal 6: Frequency resource granularity and time resource granularity can be configured by RRC and bit-length for UL PI needs to be studied.

  - Signaling mechanism

Detection mechanism

eMBB complexity increases significantly due to frequent PDCCH monitoring. So some simplified detection mechanism needs to be considered

- Conditional detection

If eMBB UE always monitor UL PI, it would make higher effort and calculation resource. However, transmission stop only occurs when eMBB transmits uplink data. So eMBB UE detects UL PI only before eMBB UE will transmit uplink data. As shown in Figure 4, UE detects UL PI in the T1, which is from UL grant to PUSCH and does not detect UL PI before UL grant and after PUSCH. Furthermore, detection time of UL PI can be determined by the gap between UL PI and preemption window, and then detection occasion will decrease further, e.g. only PI#2 and PI#3 during T2 in Figure 4.

Figure 4 Timeline for UL PI detection

Proposal 7: eMBB UE detects UL PI only before eMBB UE will transmit uplink data. And the UL PI detection occasion can further decrease if UL PI detection occasion is determined by the gap between UL PI and preemption window.

- Specific resource

To avoid complexity of PDCCH blind decoding and reduce UL PI processing time, specific PDCCH resource could be configured for UL PI. When UL PI detection is needed, eMBB UE only need monitor PDCCH in the pre-defined resource.

Proposal 8: Specific PDCCH resource could be configured for UL PI to avoid complexity of PDCCH blind decoding and reduce UL PI processing time

Validation issue

Group common DCI is resource specific indication and shared among some UEs. So there maybe inconsistence between group common DCI and Scheduling signaling. For example, as shown in Figure 5, PRACH, SRS, URLLC PUSCH and URLLC PUCCH still needs to transmit in overlapping resource.

Figure 5 Validation issue

Proposal 9: Stop transmission is determined not only by UL PI, but also by UL grant and/or channel type.

  - Scenario

Group common DCI is usually shared by multiple UE, aggregation level should be selected based on the requirement of edge UEs. As shown in Figure 6, When group common DCI includes one UL PI set, e.g. 14bit, then AL=8 is needed to meet BLER=10^-5@SINR=-3.42dB, which is 5% geometry[2]. When group common DCI includes multiple UL PI sets, which has similar payload as fallback DCI, then AL=16 is needed.

Figure 6 Performance of group common DCI

However, in terms of PDCCH monitoring capability, non-overlapped CCE number for channel estimation is not larger than 56. Then if group common DCI includes multiple UL PI sets, then non-overlapped CCE number for channel estimation will be deficient and PDCCH monitoring capability needs to enhance for eMBB UE. If group common DCI includes one UL PI set, then non-overlapped CCE number for channel estimation will be used up by UL PI and considering DCI for other functions, then PDCCH monitoring capability still needs to enhance for Embb UE.

Proposal 10: If group common DCI is shared multiple UEs, especially for cell edge UE. The non-overlapped CCEs for channels estimation will be deficient and PDCCH monitoring capability needs to be enhanced for eMBB UE.

One solution to avoid PDCCH monitoring capability enhancement is that group-common DCI with low aggregation level is shared by cell center and cell middle UE and UE specific DCI with UE specific beamforming is used for cell-edge UE. Figure 6 shows performance of UE specific DCI. It could be seen that only aggregation level 4 is enough for fallback DCI to satisfy reliability requirement.

Figure 6 Performance of UE-specific DCI

Proposal 11: To avoid PDCCH monitoring capability enhancement, group-common DCI with low aggregation level is shared by cell center and cell middle UE and UE specific DCI with UE specific beamforming is used for cell-edge UE.

UE specific DCI for UL PI

  - UE specific DCI design

Preemption part usually needs to be retransmitted later, UL grant for transmission of preemption part is necessary. Therefore, UL grant for re-scheduling can be reused as preemption indication. It could avoid additional signaling design and overhead.

Reuse re-transmission grant for indicating UL PI and re-scheduling the preemption part. This grant needs to be transmitted between the scheduling grant and ending of PUSCH transmission, which is out-of-order. So when eMBB UE detects out-of-order UL grant, it would cancel current PUSCH transmission and prepare the re-transmission for the canceled PUSCH according to the detected grant. In this way, there is no additional signal for PUSCH cancelation and re-scheduling.

Proposal 12: Reuse re-transmission grant as UL PI.

  - Co-exist of group common DCI and UE specific DCI

In order to meet the requirement for reliability 99.999% and the overhead limitation for PDCCH, co-exist of group common DCI and UE specific DCI could be considered. For center UE, which require low CCE aggregation could use group common DCI. In this way, group common DCI would use less PDCCH resource, avoid transmitting PDCCH for multiple UE with UE specific DCI in the same time. For edge UE, UE specific DCI is used, and the reliability requirement is satisfied with lower aggregation level.

Proposal 13: Co-exist of group common DCI and UE specific DCI should be supported for UL PI.

Power control solution

  - Comparison among three options

In 97 meeting, three options are summarized and to be down-selected. Then we compare three options in terms of dynamic scheduling and configured grant transmission.

For dynamic scheduling, option 3 applies the same solution as option 1and option 2. So we just compare option1 and option 2. The advantage and disadvantage of option 2 and option 3 is listed in Table 1. From Table 1, it is shown that Option 2 has lower complexity and specification effort than option 1. So option 2 is preferred.

Table 1 comparison between option 2 and option 3 for dynamic scheduling

|  | Advantage | Disadvantage |
| --- | --- | --- |
| Option 1 |  | - Define open-loop parameter set for dynamic indication - New procedure, e.g. close-loop PC with open-loop parameter. |
| Option 2 | - Reuse current procedure and less specification effort | - Extend TPC values |

For configured grant, the advantage and disadvantage of option 1- option 3 is listed in Table 2. From Table 2, it is shown that Option 2 provides accurate power information with lower complexity and specification effort. So option 2 is still preferred.

Table 2 comparison between options 1-3 for configured grant

|  | Advantage | Disadvantage |
| --- | --- | --- |
| Option 1 | - Dynamic power control, which is power saving and avoid strong interference to neighbor cell. | - Define open-loop parameter set for dynamic indication - New procedure, e.g. close-loop PC with open-loop parameter. - Resource to power boost is not clear, especially for multiple CGs |
| Option 2 | - Dynamic power control, which is power saving and avoid strong interference to neighbor cell. - Reuse current procedure and less specification effort | - Resource to power boost is not clear, especially for multiple CGs |
| Option 3 | - Information of power boost resource is provided, but detail is not clear. | - Semi-static power control, which is high power assumption and leads strong interference to neighbor cell. - New power control procedure |

Proposal 14: Option 2 is preferred due to it provides accurate power information with lower complexity and specification effort.

  - Remaining issues on power control

For multiple configured grants, configured grant resource needs to be indicated, otherwise, unwanted power boost will occur. Figure 7 shows an example of unwanted power boost for configured grant transmission. UE A receives group common DCI for power boost, which is due to inter UE collision in configured grant resource 1. However, UE A transmits data in configured grant resource 2 with power boost. So Time and frequency resource information should be included in group common DCI.

Figure 7 Examples of group common DCI issues

Figure 8 is an example of Group common DCI, which includes multiple CG specific and UE specific field for TPC.  Each UE and each configured grant could be configured by its own power control parameter.

Figure 8 Group common DCI content

Proposal 15: Reuse Rel-15 Group common DCI for TPC command with CG specific field further.

Conclusions

In this contribution, we show our views on UL inter UE Tx prioritization and multiplexing with following observations and proposals:

Proposal 1: Stop with resume is not supported due to phase discontinuity issue.

Proposal 2: PUSCH and PUCCH needs to be preempted but PRACH and SRS cannot be preempted.

Proposal 3: Considering system flexibility, the gap between UL PI and preemption window and the length of preemption window can be configured by RRC.

Proposal 4: The resource indicated by PI maybe overlapped and the overlapped resource is determined by the latest UL PI.

Proposal 5: The value of frequency resource granularity needs to be finer and can be multiple of RBG.

Proposal 6: Frequency resource granularity and time resource granularity can be configured by RRC and bit-length for UL PI needs to be studied.

Proposal 7: eMBB UE detects UL PI only before eMBB UE will transmit uplink data. And the UL PI detection occasion can further decrease if UL PI detection occasion is determined by the gap between UL PI and preemption window.

Proposal 8: Specific PDCCH resource could be configured for UL PI to avoid complexity of PDCCH blind decoding and reduce UL PI processing time

Proposal 9: Stop transmission is determined not only by UL PI, but also by UL grant and/or channel type.

Proposal 10: If group common DCI is shared multiple UEs, especially for cell edge UE. The non-overlapped CCEs for channels estimation will be deficient and PDCCH monitoring capability needs to be enhanced for eMBB UE.

Proposal 11: To avoid PDCCH monitoring capability enhancement, group-common DCI with low aggregation level is shared by cell center and cell middle UE and UE specific DCI with UE specific beamforming is used for cell-edge UE.

Proposal 12: Reuse re-transmission grant as UL PI.

Proposal 13: Co-exist of group common DCI and UE specific DCI should be supported for UL PI.

Proposal 14: Option 2 is preferred due to it provides accurate power information with lower complexity and specification effort.

Proposal 15: Reuse Rel-15 Group common DCI for TPC command with CG specific field further.

References

- RAN1 97Chairman notes

- R1-1900288 Geometry Results for URLLC evaluation OPPO

Appendix

Table A-1 Simulation assumption for PDCCH performance

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 4GHz |
| Channel model | TDL-C (delay spread: 300ns) as in 38.901 |
| UE speed | 3 km/h |
| MIMO | 4x2 Precoder cycling for Group common DCI, Eigen BF for UE specific DCI |
| System bandwidth | 40 MHz |
| Sub-carrier spacing | 15kHz |
| Channel estimation | MMSE |
| Receiver type | ML(sequence), List32 SC (DCI) |
| Aggregation level | 4/8/16 |
| Payload | 14,42 bits(+24bit CRC, Polar) |