3GPP TSG RAN WG1 Meeting #98			 		R1-1908691

Prague, CZ, August 26 – 30, 2019

Agenda Item:	7.2.1.1

Source: 	LG Electronics

Title: 	Discussion on Channel Structure for 2-step RACH

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss on channel structure for 2-step RACH.

- Subcarrier spacing for msgA preamble and msgA PUSCH

- DMRS configuration type for msgA PUSCH

- DMRS port and sequence

- PUSCH scrambling sequence

- msgA PUSCH configuration

- Supported MCS(s) and time-frequency resource size(s) of PUSCH in msgA etc

- RO mapping for 2-step RACH

- Discussion

Subcarrier spacing for msgA PRACH preamble and msgA PUSCH

In NR system, BWP is defined by frequency location, bandwidth, subcarrier spacing and cyclic prefix. Within a BWP, control channel (i.e., PDCCH, PUCCH), shared channel (i.e., PDSCH, PUSCH) and related signal (i.e., DMRS, CSI-RS, SRS) should have same numerology. Also, RACH related configuration is involved in BWP-UplinkCommon, and RACH can be configured per BWP. That is, the subcarrier spacing for PRACH preamble can be separately configured, but the subcarrier spacing for msg3 PUSCH for 4-step RACH procedure should follow that of UL BWP.

In the same way, the subcarrier spacing of msgA RACH preamble for 2-step RACH procedure can be separately configured within a BWP. But, the subcarrier spacing of msgA PUSCH should follow the configured subcarrier spacing for UL BWP. The subcarrier spacing between msgA preamble and msgA PUSCH can be same or different.

When different subcarrier spacing is configured for msgA preamble in a UL BWP, the PUSCH resource for msgA needs to be configured with timing gap from RACH occasion for subcarrier spacing switching. On the other hand, if same subcarrier spacing is configured for msgA preamble in a UL BWP, PUSCH resource for msgA can be configured without timing gap between RACH occasion and PUSCH occasion. These two cases are up to network configuration.

Proposal 1:

- The subcarrier spacing of msgA RACH preamble for 2-step RACH procedure can be separately configured within a BWP. But, the subcarrier spacing of msgA PUSCH should follow the configured subcarrier spacing for UL BWP.

- If same subcarrier spacing is configured for msgA preamble in a UL BWP, PUSCH resource for msgA can be configured without timing gap between RACH occasion and PUSCH occasion. These two cases are up to network configuration.

DMRS configuration type for msgA PUSCH

In NR system, two types of DMRS are supported which are DMRS configuration type 1 and type 2. Since type 1 DMRS is a mandatory feature, it was agreed for type 1 DMRS to be applied for msg3 PUSCH in 4-step RACH procedure. Even in a certain UL BWP, type 1 DMRS is used for msg3 transmission. Similarly, in 2-step RACH, only type 1 DMRS which is a mandatory feature should be applied for msgA PUSCH since RACH configuration for 2-step RACH procedure can be configured via BWP-UplinkCommon which could be applied for UEs.

Proposal 2:

- In 2-step RACH, only type 1 DMRS which is a mandatory feature should be applied for msgA PUSCH.

PUSCH DMRS port/sequence

For type 1 DMRS, if one OFDM symbol is used, maximum 4 antenna ports can be assigned. These antenna ports are composed by frequency resource (i.e., two 2-comb type resources) and cyclic shift value (i.e. 0 and pi). In 4-step RACH, single antenna port is applied for msg3 transmission. But, for 2-step RACH, all antenna ports could be used for PUSCH resource efficiency enhancement. Also, gNB can configure the number of antenna ports for msgA PUSCH DMRS. Consideration points to decide the number of antenna port are network coverage and geometry. If the network coverage is wider and geometry is not good to support multiple UE in a PUSCH occasion, it is good to assign small number of antenna port (e.g., 1 or 2). On the other hand, when 2-step RACH is operated within small coverage and in good geometry, gNB can allow large number of antenna ports (e.g., 2 or 4) in a PUSCH occasion.

In addition, if two antenna ports are configured, it is better to use antenna port 0 and 1 (or 2 and 3) which have same frequency resource and difference cyclic shift value, since cyclic shift can be distinguish the antenna ports even in case where OFDM symbol reception timing different is quite large (e.g., FFT size / 4).

Proposal 3:

- gNB can configure the number of antenna ports for msgA PUSCH DMRS.

  - The maximum number of antenna ports is four.

  - When the value of configured antenna ports is two, antenna port 0 and 1 are used.

  - When the value of configured antenna ports is one, antenna port 0 is used.

For PUSCH resource efficiency enhancement, we can apply multiple DMRS sequences per antenna ports. In NR system, for the case of CP-OFDM, PN sequence is applied as DMRS sequence. Also, for UL MU-MIMO, two different seed values can be configured via RRC signal, and one of values are designated via DCI. For example, for spatially separated UEs, these two different DMRS sequence can be applied even though same antenna port are assigned for the UEs. Similarly, for msgA PUSCH, multiple DMRS sequences can be applied when CP-OFDM is used. In this case, these seed valued can be configured via SIB1 and/or RACH-ConfigCommon, and values can be designated depending on RAPID.

On the other hand, for the case of DFT-s-OFDM, low-PAPR sequence (a.k.a, Zadoff-chu sequence) is applied, and one sequence is assigned as DMRS sequence.

Proposal 4:

- In case of CP-OFDM, maximum two different initial values are configured via higher layer signal (e.g., SIB1 and/or RACH-ConfigCommon).

  - Sequence is designated depending on the RAPID.

- For case of DFT-s-OFDM, one root index of ZC-sequence is configured via higher layer signal.

PUSCH scrambling sequence

As described in TS38.211, initialization value cinit for scrambling sequence is composed by nRNTI and nID. For msgA PUSCH in 2-step RACH case, we need to decide which values are applied for nRNTI and nID.

In case of 4-step RACH, TC-RNTI/C-RNTI as nRNTI is applied for msg3 PUSCH. On the other hand, for msgA PUSCH in 2-step RACH case, TC-RNTI/C-RNTI could not be applicable for UE in RRC_IDLE and RRC_INACTIVE. So, for these cases, it can be considered to apply other RNTI (i.e. RA-RNTI) as nRNTI instead of TC-RNTI/C-RNTI. And, for UE in RRC_CONNECTED, C-RNTI can be applied as nRNTI.

In case of 4-step RACH, NIDcell as nID is applied for msg3 PUSCH. However, for msgA PUSCH in 2-step RACH case, multiple RAPIDs can be mapped to single PUSCH occasion. In this case, inter-layer interference between PUSCH data RE could be increased. So, in order to reduce the interference, we may consider to apply multiple scrambling sequences generated by different values of nID instead of NIDcell which is applied for UL MU-MIMO. For msgA PUSCH, RAPID can be applied as nID.

Proposal 5:

- For msgA PUSCH data scrambling sequence, RA-RNTI and RAPID are applied as seed value of initialization.

  - For UE in RRC_IDLE and RRC_INACTIVE, RA-RNTI is applied as nRNTI. For UE in RRC_CONNECTED, C-RNTI is applied as nRNTI.

  - RAPID can be applied as nID.

msgA PUSCH configuration

Similar with RACH configuration, msgA PUSCH occasion configuration needs to be included in SIB1 and BWP-UplinkCommon. In the msgA PUSCH configuration, at least two sets of parameters (i.e., time domain configuration and frequency domain configuration) could be defined. For a PUSCH occasion configuration, we may consider at least these parameters:

- Time domain configuration

  - Number of OFDM symbol for single PUSCH occasion

  - Whether timing gap is necessity or not

  - Starting OFDM symbol index within a slot

- Frequency domain configuration

  - Number of PRBs for single PUSCH occasion

  - Starting PRB with a BWP

  - Contiguous or non-contiguous allocation

  - Null PRB is necessity or not

On top of the single PUSCH occasion configuration, it is required to define configuration for multiple PUSCH occasions. Before discussing detail parameters, we may need to decide whether only contiguous PUSCH occasion assignment is applied for msgA PUSCH. But, as a starting point for discussion, if contiguous PUSCH occasion assignment in time and frequency domain is considered, we may consider following parameters:

- Time domain configuration

  - Number of PUSCH occasion in a slot

  - Slot indication within a duration (e.g., 10ms, 20ms, …, 80ms) for PUSCH occasion

- Frequency domain configuration

  - Number of PUSCH occasion within a BWP

Proposal 6:

- For a PUSCH occasion configuration, at least these parameters are necessity:

  - Time domain configuration

    - Number of OFDM symbol for single PUSCH occasion

    - Whether timing gap is necessity or not

    - Starting OFDM symbol index within a slot

  - Frequency domain configuration

    - Number of PRBs for single PUSCH occasion

    - Starting PRB with a BWP

    - Contiguous or non-contiguous allocation

    - Null PRB is necessity or not

- For multiple PUSCH occasions, it is needed to decide whether only contiguous PUSCH occasion assignment is applied for msgA PUSCH. If contiguous PUSCH occasion assignment in time and frequency domain is supported, at least following parameters are necessity:

  - Time domain configuration

    - Number of PUSCH occasion in a slot

    - Slot indication within a duration (e.g., 10ms, 20ms, …, 80ms) for PUSCH occasion

  - Frequency domain configuration

    - Number of PUSCH occasion within a BWP

Supported MCS(s) and time-frequency resource size(s) of PUSCH in msgA etc

In 4-step RACH, MCS for msg3 is assigned by UL grant in RAR message. So, depending on UE channel state, gNB can designate MCS from low index to high index. Also, time/frequency resource for PUSCH can be assigned depending on the selected MCS level and required coverage. On the other hand, for 2-step RACH, it is hard to allow flexible MCS selection. If UE selects MCS level for UL transmission depending on DL measurement result, it could not applicable for UL transmission because not only channel state but also interference level are quite different between DL channel and UL channel. Furthermore, depending on the MCS level, the required amount of resource of PUSCH for msgA is changed. So, if several MCS levels are allowed, many type of PUSCH resource are defined and pre-assigned. It is not good in terms of resource utilization. Therefore, for PUSCH in msgA, it is desired to use very limited number of MCS level (e.g. one or two MCS levels). For example, only QPSK for CP-OFDM is applied for PUSCH in msgA, and two types of coding rate are used.

Proposal 7:

- For PUSCH in msgA, use very limited number of MCS level (e.g. one or two MCS levels).

  - E.g., only QPSK for CP-OFDM is applied for PUSCH in msgA, and two types of coding rate are used.

As discussed, if multiple MCS levels are allowed for PUSCH transmission, multiple types of PUSCH resource are defined depending on MCS level. Consequently, if PUSCH resource is corresponded with RAPID, the RAPID can be also associated with MCS level. So, if UE decide suitable MCS level for PUSCH transmission, UE may select RAPID which is associated with the MCS level.

Furthermore, if multiple sets of DMRS frequency resource is allowed, it can be defined that each DMRS frequency resource is associated with MCS level. For example, if two different PUSCH resources (e.g., one is larger frequency resource for lower MCS level, and another is smaller frequency resource for higher MCS level) are assumed, two different frequency resource sets are designated for each PUSCH resources. In figure 2, an example of PUSCH resource and DMRS RE designation depending on MCS level is depicted.

Figure 2. An Example of PUSCH resource and DMRS RE designation depending on MCS level

Proposal 8:

- If multiple MCS levels are allowed for PUSCH transmission, multiple types of PUSCH resource are defined depending on MCS level.

  - RAPID can be associated with MCS level.

- If multiple sets of DMRS frequency resource is allowed, it can be defined that each DMRS frequency resource is associated with MCS level.

RO mapping for 2-step RACH

One possible discussion point is whether RO is shared between 2-step RACH and 4-step RACH. If RO sharing is allowed, contention-based PRACH preamble for 2-step RACH can be configured among the remaining PRACH preambles in the configured RO for 4-step RACH, hence the purpose of PRACH transmission (i.e., 2-step RACH or 4-step RACH) can be identified to gNB. Also, in RO sharing case, PUSCH in msgA can be assigned at the PUSCH slot following RACH slot.

If RO sharing is not allowed, RO is configured for 2-step RACH. For RO configuration for 2-step RACH, we can consider two alternatives. (1) First alternative is to reuse RACH configuration table for 4-step RACH. This configuration table is designed based on the assumption that most of OFDM symbols within RACH slot or within second half RACH slot are used as ROs. Hence, for this case, it is possible that RO and PUSCH can be multiplexed at different slots. In figure 1 (a), slot level time multiplexing is depicted as an example of multiplexing of RACH and PUSCH of msgA.

(2) Second alternative is to allow new configuration that OFDM symbols within a first half RACH slot are used as RO. The OFDM symbols following RO can be assigned for PUSCH of msgA. In figure 1 (b), symbol level time multiplexing is depicted as an example of multiplexing of RACH and PUSCH of msgA.

(a) Slot level time multiplexing

(b) Symbol level multiplexing within a slot

Figure 1. Example of multiplexing between RACH and PUSCH of msgA

Proposal 9:

- For RO configuration for 2-step RACH,

  - For the case of shared RO for 2-step and 4-step RACH, reuse RACH configuration table for 4-step RACH

  - For the case of separate ROs for 2-step and 4-step RACH, allow new configuration that OFDM symbols within a first half RACH slot are used as RO

Conclusion

In this contribution, we discuss on channel structure for two-step RACH. We proposed as follows:

Subcarrier spacing for msgA PRACH preamble and msgA PUSCH

Proposal 1:

- The subcarrier spacing of msgA RACH preamble for 2-step RACH procedure can be separately configured within a BWP. But, the subcarrier spacing of msgA PUSCH should follow the configured subcarrier spacing for UL BWP.

- If same subcarrier spacing is configured for msgA preamble in a UL BWP, PUSCH resource for msgA can be configured without timing gap between RACH occasion and PUSCH occasion. These two cases are up to network configuration.

DMRS configuration type for msgA PUSCH

Proposal 2:

- In 2-step RACH, only type 1 DMRS which is a mandatory feature should be applied for msgA PUSCH.

PUSCH DMRS port/sequence

Proposal 3:

- gNB can configure the number of antenna ports for msgA PUSCH DMRS.

  - The maximum number of antenna ports is four.

  - When the value of configured antenna ports is two, antenna port 0 and 1 are used.

  - When the value of configured antenna ports is one, antenna port 0 is used.

Proposal 4:

- In case of CP-OFDM, maximum two different initial values are configured via higher layer signal (e.g., SIB1 and/or RACH-ConfigCommon).

  - Sequence is designated depending on the RAPID.

- For case of DFT-s-OFDM, one root index of ZC-sequence is configured via higher layer signal.

PUSCH scrambling sequence

Proposal 5:

- For msgA PUSCH data scrambling sequence, RA-RNTI and RAPID are applied as seed value of initialization.

  - For UE in RRC_IDLE and RRC_INACTIVE, RA-RNTI is applied as nRNTI. For UE in RRC_CONNECTED, C-RNTI is applied as nRNTI.

  - RAPID can be applied as nID.

msgA PUSCH configuration

Proposal 6:

- For a PUSCH occasion configuration, at least these parameters are necessity:

  - Time domain configuration

    - Number of OFDM symbol for single PUSCH occasion

    - Whether timing gap is necessity or not

    - Starting OFDM symbol index within a slot

  - Frequency domain configuration

    - Number of PRBs for single PUSCH occasion

    - Starting PRB with a BWP

    - Contiguous or non-contiguous allocation

    - Null PRB is necessity or not

- For multiple PUSCH occasions, it is needed to decide whether only contiguous PUSCH occasion assignment is applied for msgA PUSCH. If contiguous PUSCH occasion assignment in time and frequency domain is supported, at least following parameters are necessity:

  - Time domain configuration

    - Number of PUSCH occasion in a slot

    - Slot indication within a duration (e.g., 10ms, 20ms, …, 80ms) for PUSCH occasion

  - Frequency domain configuration

    - Number of PUSCH occasion within a BWP

Supported MCS(s) and time-frequency resource size(s) of PUSCH in msgA etc

Proposal 7:

- For PUSCH in msgA, use very limited number of MCS level (e.g. one or two MCS levels).

  - E.g., only QPSK for CP-OFDM is applied for PUSCH in msgA, and two types of coding rate are used.

Proposal 8:

- If multiple MCS levels are allowed for PUSCH transmission, multiple types of PUSCH resource are defined depending on MCS level.

  - RAPID can be associated with MCS level.

- If multiple sets of DMRS frequency resource is allowed, it can be defined that each DMRS frequency resource is associated with MCS level.

RO mapping for 2-step RACH

Proposal 9:

- For RO configuration for 2-step RACH,

  - For the case of shared RO for 2-step and 4-step RACH, reuse RACH configuration table for 4-step RACH

  - For the case of separate ROs for 2-step and 4-step RACH, allow new configuration that OFDM symbols within a first half RACH slot are used as RO

Reference

[1] RAN1#97 meeting, Chairman’s note

- Annex: RAN1 agreement

RAN1#97 [1] - The following parameters are defined per msgA PUSCH configuration: - Common parameters for both option 1 (separate configuration) and option 2 (relative location), at least include: - MCS and/or TBS (to be further decided) - Number of FDMed POs - POs (including guard band or guard period, if exist) under the same msgA PUSCH configurations are consecutive in frequency domain - Number of PRBs per PO - Number of DMRS symbols/ports/sequences (if support) per PO - FFS whether or not support repetitions for msgA PUSCH - FFS bandwidth of PRB-level guard band or duration of guard time - FFS PUSCH mapping type - Parameters specific to option 1, at least include: - Periodicity (msgA PUSCH configuration period) - FFS value range - Offset(s) (e.g., symbol, slot, subframe, etc.) - Time domain resource allocation, details FFS, e.g., in a slot for msgA PUSCH: starting symbol, number of symbols per PO, number of time-domain POs, etc. - Frequency starting point - Parameters specific to option 2, at least include: - Single time offset (combination of slot-level and symbol-level indication) with respect to a reference point - FFS, e.g., each PRACH slot (e.g., start or end of the PRACH slot), etc. - Number of symbols per PO - FFS explicit or implicit indication - Single frequency offset with respect to FFS (the start of the first RO in frequency or the end of the last RO in frequency) - FFS: Number of TDMed POs - Support multiple msgA PUSCH configurations for a UE - FFS the maximum number of configurations - FFS which parameters, if any, are common for all configurations - FFS indication of different msgA PUSCH configurations, e.g. by different ROs, by different preamble groups, or by UCI - FFS whether or not resources for different msgA PUSCHs can be overlapped in time-frequency, and if so, any spec impact - FFS whether the frequency resource of msgA PUSCH should be limited to the bandwidth of PRACH - FFS validation rule of msgA PUSCH - The c_init for msgA PUSCH scrambling is at least derived based on a RNTI, preamble index, and/or n_ID (which can be cell ID or configurable, to be FFS). - FFS details of the RNTI - FFS the inclusion of DMRS index. Proposals: - For the definition of PRU, support both DMRS ports and (working assumption) DMRS sequences at least for CP-OFDM - FFS how to support multiple sequences for DFT-s-OFDM - FFS the maximum number of sequences - Confirm the working assumption that both one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit (PRU) are supported - Configurable number of preambles (including one or multiple) mapped to one PRU, explicitly or implicitly - FFS 1-to-multiple mapping Supported by: HW, HiSi, LGE, E///, QC, vivo, ZTE, Sanechips, Intel, SONY, Panasonic Objected by: SS Discuss till RAN1#98