3GPP TSG RAN WG1 Meeting #98   		 		R1-1908692

Prague, Czech, May 13th – 17th, 2019

Agenda Item:	7.2.1.2

Source: 	LG Electronics

Title: 	Discussion on 2-step RACH Procedure

Document for:	Discussion and Decision

- Introduction

In this document, we will mainly discuss about the details of the power control, beam direction between for msgA preamble and msgA PUSCH and fall-back procedure respectively.

- Discussion

Tx spatial filter for msgA PUSCH

In RAN1#96bis meeting, there was a discussion regarding MsgA Tx beam selection, and following three options are identified [1]:

- For MsgA Tx beam selection further study at least the following options:

- Option 1: The MsgA PRACH and MsgA PUSCH use the same Tx spatial filter (beam).

- Option 2: The MsgA PRACH and MsgA PUSCH use same or different Tx spatial filter (beam) up to UE implementation.

  - No spec impact expected.

  - Note: in 4-step RACH it is up to UE implementation to decide the beams for Msg1 and Msg3.

- Option 3: The MsgA PRACH and MsgA PUSCH use same or different Tx spatial filter (beam) under network control/assistance.

In case of option 1 that same Tx spatial filter which is used for msgA preamble transmission is applied for msgA PUSCH transmission, additional UE processing complexity for Tx beam re-selection would not be required. On the other hand, when SS/PBCHs or CSI-RSs for measurement are transmitted between msgA preamble and msgA PUSCH, UE may or may not operate Tx beam re-selection based on the selected SS/PBCH. If time varying of wireless channel is fast, the previously selected Tx beam might be no longer valid beam direction for msgA PUSCH transmission. In this case, it could be better for UE to operate Tx beam reselection for msgA PUSCH transmission. This operation is up to UE implementation. According to this reason, option 2 seems more reasonable.

Observation 1:

- If SSB/CSI-RS exists between msgA preamble and msgA PUSCH, the UEs can estimate channel by using them, and then the information obtained from channel estimation can be used for msgA PUSCH transmission.

  - It is helpful to UE to adjust their TX spatial filter depending on channel condition.

Proposal 1:

- Regarding beam selection between msgA preamble and msgA PUSCH, allowing UEs to adjust their Tx spatial filter by themselves (option2) should be considered.

Additionally, since gNB expects to decode msgA PUSCH through resources that are associated with detected preamble/RO which is also related with SSB index, the SSB index which is used for msgA PUSCH should be same with msgA preamble even though different Tx spatial filter beams are applied. That is, the SSB index that UEs have used previously to transmit preamble should not be changed.

Observation 2:

- Since gNB expects to decode msgA PUSCH through resources that are associated with detected preamble/RO which is also related with SSB index, the SSB index which is used for msgA PUSCH should be same with msgA preamble even though different Tx spatial filter beams are applied.

Proposal 2:

- In case of 2-step RACH, the selected SSB index for msgA preamble should be sustained for msgA PUSCH transmission.

Power control for msgA preamble

In RAN1#96-bis meeting [1], following two options were agreed for 2-step RACH preamble power control.

- Option 1: Power control parameters can be separately configured for 2-step and 4-step RACH.

  - If a power control parameter is not configured for 2-step RACH, the corresponding power control parameter of 4-step RACH is used instead for 2-step.

- Option 2: The corresponding power control parameter of 2-step RACH preamble follows that of 4-step RACH preamble.

In order to select one option among above two options, it is necessity to verify which option can provide a configuration flexibility considering on both cases which are shared RO and separated RO.

If option 2 (i.e., same configuration of preamble power control for both 2-step and 4-step RACH) is adopted, it can be expected that the target detection probability of preamble is same for msgA preamble and msg1 preamble. Also, option 2 has a benefit that signalling overhead for configuring power control is not increased.

On the other hand, in case of option 1 (i.e. separate configuration of preamble power control for 2-step and 4-step RACH), there is a benefit that network can independently configure power control parameter for 2-step and 4-step RACH. Especially, in case of separate RO, it would be allowed for more efficient resource utilization that the target detection probability of preamble for 2-step RACH is better than 4-step RACH. If option 1 is adopted, network can configure the parameters of power control for 2-step RACH separately. Also, when network wants to set that target detection probability of preamble for both 2-step and 4-step RACH, it is possible that same value of power control parameter is configured by using option 1 which is a super set of option 2. According to this reason, it seems better to adopt option 1 (i.e. separate configuration) in network flexibility perspective.

Observation 3:

- In case of separate RO, it would be allowed for more efficient resource utilization that the target detection probability of preamble for 2-step RACH is better than 4-step RACH. If option 1 (i.e. separate configuration) is adopted, network can configure the parameters of power control for 2-step RACH separately.

There are two ways to support separate configuration, the one thing is introducing additional new parameter which is composed of whole value. But, the configuring additional value within the entire range might cause signalling overhead for gNB. The other way is using offset value which is relative to msg1 for msgA preamble. In that case, the signalling overhead can be reduced, and the transmission power for msgA preamble can be written as follows:

PPRACH_2step=PPRACH,target,4-step+ΔPRACH,offset,2-step+PL

Observation 4:

- For separate Preamble Received Target Power (P-RTP), configuring additional value within the entire range could be large overhead for configuring power control information element.

- For 2-step RACH, offset type of indicator for additional P-RTP has a benefit to reduce signaling overhead.

Next, we will discuss about ramping step size of msgA preamble. If same step size and P-RTP are applied for msgA preamble comparing with Msg1, the detection probability of preamble might be equal even though more power and time/frequency resource are spent for msgA. In that case, not only UE power consumption should be increased but also overall time for RACH procedure could be increased when gNB does not success to detect PRACH preamble in msgA. As a result, this is not only unfair 2-step UE, but also it causes inefficient usage of resources. To resolve this problem, separated configuration of step size should be considered for 2-step RACH.

Observation 5:

- When the value of step size and Preamble Received Target Power (P-RTP) for msg1 is applied for msgA preamble, target detection probability of preamble might be equal. So, it causes extending overall time and inefficient usage of resources such as time/frequency resources and power for retransmission.

If separated configuration of step size is applied for 2-step RACH, there are many ways to configure it. Here are some simple examples. The first thing is introducing additional new parameter to configure step size of msgA preamble. It can be configured by either the parameter which is composed of whole value or offset value which is relative to msg1 for msgA preamble. The next way is reusing existing parameter for msgA preamble. In 4-step RACH, the powerRampingStepHighPriority is already used for prioritized random access. Actually, it can be interpreted that the 2-step RACH has higher precedence than 4-step RACH because the burden of msgA retransmission is bigger than 4-step RACH. In this respect, 2-stetp RACH might be interpreted as prioritized case. In this sense, we can use it for step size of msgA preamble instead.

Proposal 3: For power control of msgA preamble,

- Adopt option 1 (i.e. separate configuration)

- For configuration of preamble received target power (P-RTP), one alternative among followings is selected:

  - Alt 1. Introduce new parameter which is composed of whole range of values which used for msg1

  - Alt 2. Introduce Offset value which is relative to P-RTP for msg1.

- For configuration of step size, one alternative among followings is selected:

  - Alt 1. Introduce new parameter which is composed of whole range of values which used for msg1

  - Alt 2. Introduce Offset value which is relative to msg1 for msgA preamble

Power control for msgA PUSCH

At the previous RAN1#97 meeting, the MsgA PUSCH Tx power in transmission instance i is defined by

PPUSCHi=maxPCMAX,[MsgA]preambleReceivedTargetPower+∆MsgA_PUSCH+10log102μMRBPUSCHi+αPL(i)+∆TFi+∆rampupi

And then, the power ramping component is given by

∆rampup(i)=minmax0,PCMAX-10log102μMRBPUSCHi+[MsgA]preambleReceivedTargetPower+∆MsgA_PUSCH+αPLi+∆TFi,∆rampuprequested

The remaining large issue here is ∆rampuprequested and it will be down select from from the following alternative:

- Alt1: Same ramp up for MsgA PUSCH and MsgA PRACH

  - ∆rampuprequested=PREAMPLE_POWER_RAMPING_COUNTER-1×[MsgA]powerRampingStep

  - FFS: same power ramping counters for 2-step RACH MsgA PRACH and 4-step RACH Msg1.

- Alt 2: Separate ramp up for MsgA PUSCH and MsgA PRACH, with different counters

  - ∆rampuprequested=MSGAPUSCH_POWER_RAMPING_COUNTER-1×PUSCHpowerRampingStep

- Alt3: Separate ramp up for MsgA PUSCH and MsgA PRACH, with the same counter

  - ∆rampuprequested=PREAMBLE_POWER_RAMPING_COUNTER-1×PUSCHpowerRampingStep

In this chapter, we will deal with both ramping counter and ramping step respectively. In case of ramping counter, in general, the number of preamble transmission is restricted by preambleTransMax. It cannot be controlled in an ordinary way under one restriction. Actually, since msgA preamble is always accompanied with PUSCH, so there are no reason to control both of them separately. However, if one to one relationship is broken, in that case, using individual counter might be proper. That is, if msgA is always accompanied by related PUSCH, using one counter should be considered.

In ramping step case, msgA preamble and msgA PUSCH might have different detection or collision probability. For example, when multiple UEs transmit preamble at that time and PUSCH resources might be individually allocated, using same step size between msgA preamble and msgA PUSCH can cause the waste of ramping power for PUSCH. That is, although random access is failed due to fail to detect msgA preamble, it seems that there is no reason for ramping PUSCH with same step size. In that case, if the step size is larger, the waste of power becomes bigger. So, configuring independent ramping step size is reasonable for preamble/PUSCH in msgA. In summary, option 3 should be applied for 2-step RACH according to reasons described above.

Observation 6:

- Considering two components (ramping counter/step size) for msgA PUSCH retransmission, followings are observed:

  - For ramping counter: since msgA preamble is always accompanied with PUSCH, there is no reason to control them individually under one restriction (such as maximum the number of transmission (preambleTransMax)).

  - For ramping step size: there might be a case that msgA preamble and msgA PUSCH have different detection or collision probability.

    - Although random access is failed due to fail to detect msgA preamble, it seems that there is no reason for ramping PUSCH with same step size. In that case, if the step size is larger, the waste of power becomes bigger.

Proposal 4:

- For msgA power control, adopt alt.3 which is separate ramp up for MsgA PUSCH and MsgA PRACH, with the same counter

If same P-RPT and/or ramping step size are set for 2-step RACH as 4-step RACH to provide f fairness between 2-step and 4-step RACH, it also imposes unfairness on 2-step RACH UEs because more resources are required to once transmission. So, we introduce the parameter (msgATransMin) to resolve this problem.

The singularity of this parameter is that the value is always be higher than 1 and lower than the max number of msgA transmission. That is, UEs transmit msgA using same value such as P-RPT and/or ramping step size compared with 4-step RACH until below/equal to the number. And after that, UEs retransmit msgA using different value of them as shown figure 1.

Figure 1. Introducing indicator to indicate UEs to boost their transmission power using different ramping step size

Proposal 5:

- For 2-step RACH,

  - In addition to separated configuration for 2-step RACH (i.e., P-TRP, ramping step, etc), the network configures additional parameter that plays a role as indicating that UEs boost their transmission power of msgA.

  - 2-step RACH UE follows parameters for 4-step RACH until the corresponding time indicator. After that, they apply configured parameters for 2-step RACH to their retransmission of msgA.

Fall back to 4-step RACH

In this chapter, we will deal with maximum the number of transmission of msgA. Basically, in 4-step RACH, the number of retransmission is restricted by preambleTransMax as shown in figure 2. Likewise, the maximum number of transmission for msgA will be inevitably considered. If the same value is applied for 2-step RACH and the number is large, it can be very inefficient in terms of resource usage since msgA preamble and PUSCH are transmitted simultaneously until the number of transmission reaches the maximum number of retransmission. Therefore, separate configuration of the maximum number of transmission should be applied for 2-step RACH.

Proposal 6:

- Separately configure the maximum number of transmission for 2-step RACH.

To provide fairness between 2-step and 4-step RACH, the total retry count should be same. When put together, if network configures the value which is smaller than the maximum number msg1 transmission, UEs don’t need to transmit whole of msgA anymore after the maximum number of msgA transmission. That is, UEs retransmit preamble only as shown figure 2 to satisfy the total recount. We think that the ramping counter should be sustained to improve detection probability in that case.

Figure 2. Example of fallback to 4-step RACH when number of msgA transmission is exceeded over msgATransMax.

Observation 7:

- To provide fairness between 2-step and 4-step RACH, the total retry count for random access should be same. Here, retry count means the number of RA attempts itself.

Proposal 7:

- The number of msgA transmission should not be larger than the maximum number of msg1 transmission to provide fairness between 2-step and 4-step RACH.

- Fall-back to 4-step RACH if number of msgA transmission is exceeded over msgATransMax.

Conclusion

In this contribution, we overview issues related with about the details of msgA such as beam selection and power control with regard to preamble/PUSCH respectively. As a conclusion of the discussion, we summarize our views as follows:

Tx spatial filter for msgA PUSCH

Observation 1:

- If SSB/CSI-RS exists between msgA preamble and msgA PUSCH, the UEs can estimate channel by using them, and then the information obtained from channel estimation can be used for msgA PUSCH transmission.

  - It is helpful to UE to adjust their TX spatial filter depending on channel condition.

Proposal 1:

- Regarding beam selection between msgA preamble and msgA PUSCH, allowing UEs to adjust their Tx spatial filter by themselves (option2) should be considered.

Observation 2:

- Since gNB expects to decode msgA PUSCH through resources that are associated with detected preamble/RO which is also related with SSB index, the SSB index which is used for msgA PUSCH should be same with msgA preamble even though different Tx spatial filter beams are applied.

Proposal 2:

- In case of 2-step RACH, the selected SSB index for msgA preamble should be sustained for msgA PUSCH transmission.

Power control for msgA preamble

Observation 3:

- In case of separate RO, it would be allowed for more efficient resource utilization that the target detection probability of preamble for 2-step RACH is better than 4-step RACH. If option 1 (i.e. separate configuration) is adopted, network can configure the parameters of power control for 2-step RACH separately.

Observation 4:

- For separate Preamble Received Target Power (P-RTP), configuring additional value within the entire range could be large overhead for configuring power control information element.

- For 2-step RACH, offset type of indicator for additional P-RTP has a benefit to reduce signaling overhead.

Observation 5:

- When the value of step size and Preamble Received Target Power (P-RTP) for msg1 is applied for msgA preamble, target detection probability of preamble might be equal. So, it causes extending overall time and inefficient usage of resources such as time/frequency resources and power for retransmission.

Proposal 3: For power control of msgA preamble,

- Adopt option 1 (i.e. separate configuration)

- For configuration of preamble received target power (P-RTP), one alternative among followings is selected:

  - Alt 1. Introduce new parameter which is composed of whole range of values which used for msg1

  - Alt 2. Introduce Offset value which is relative to P-RTP for msg1.

- For configuration of step size, one alternative among followings is selected:

  - Alt 1. Introduce new parameter which is composed of whole range of values which used for msg1

  - Alt 2. Introduce Offset value which is relative to msg1 for msgA preamble

  - Alt 3.Reuse the existing parameter (i.e., powerRampingStepHighPriority)

Power control for msgA PUSCH

Observation 6:

- Considering two components (ramping counter/step size) for msgA PUSCH retransmission, followings are observed:

  - For ramping counter: since msgA preamble is always accompanied with PUSCH, there is no reason to control them individually under one restriction (such as maximum the number of transmission (preambleTransMax)).

  - For ramping step size: there might be a case that msgA preamble and msgA PUSCH have different detection or collision probability.

    - Although random access is failed due to fail to detect msgA preamble, it seems that there is no reason for ramping PUSCH with same step size. In that case, if the step size is larger, the waste of power becomes bigger.

Proposal 4:

- For msgA power control, adopt alt.3 which is separate ramp up for MsgA PUSCH and MsgA PRACH, with the same counter

Proposal 5:

- For 2-step RACH,

  - In addition to separated configuration for 2-step RACH (i.e., P-TRP, ramping step, etc), the network configures additional parameter that plays a role as indicating that UEs boost their transmission power of msgA.

  - 2-step RACH UE follows parameters for 4-step RACH until the corresponding time indicator. After that, they apply configured parameters for 2-step RACH to their retransmission of msgA.

Fall back to 4-step RACH

Proposal 6:

- Separately configure the maximum number of transmission for 2-step RACH.

Observation 7:

- To provide fairness between 2-step and 4-step RACH, the total retry count for random access should be same. Here, retry count means the number of RA attempts itself.

Proposal 7:

- The number of msgA transmission should not be larger than the maximum number of msg1 transmission to provide fairness between 2-step and 4-step RACH.

- Fall-back to 4-step RACH if number of msgA transmission is exceeded over msgATransMax.

Reference

- RAN1#96-bis meeting Chairman’s note

- RAN1#97 meeting Chairman’s note