3GPP TSG RAN WG1 #98		R1-1908732

Prague, CZ, August 26th – 30th, 2019

Agenda Item:	7.2.4.2.2

Source:	Lenovo, Motorola Mobility

Title:	Discussion on resource allocation for NR sidelink Mode 2

Document for:	Discussion

Introduction

During the RAN1#97, regarding resource allocation in Mode 2, the related agreements and conclusion were made as follows:

Agreements:

- NR V2X Mode-2 supports resource reservation for feedback-based PSSCH retransmissions by signaling associated with a prior transmission of the same TB

  - FFS impact on subsequent sensing and resource selection procedures

  - At least from the transmitter perspective of this TB, usage of HARQ feedback for release of unused resource(s) is supported

    - No additional signaling is defined for the purpose of release of unused resources by the transmitting UE

    - FFS the behavior of the receiver UE(s) of this TB and other UEs

Conclusion:

- RAN1 to discuss further the following

  - Maximum number of blind retransmissions supported for one TB

  - Maximum number of reserved blind retransmission

  - Maximum number of HARQ feedback-based retransmissions supported for one TB

  - Maximum number of reserved HARQ feedback-based retransmission

Agreements:

- RAN1 to further select between the following options of sidelink resource reservation for blind retransmissions:

  - Option 1: A transmission can reserve resources for none, one, or more than one blind retransmission

  - Option 2: A transmission can reserve resource for none or one blind retransmission

Agreements:

- Resource selection window is defined as a time interval where a UE selects sidelink resources for transmission

  - The resource selection window starts T1 ≥ 0 after a resource (re-)selection trigger and is bounded by at least a remaining packet delay budget

  - FFS T1 value, whether it is measured in slots, symbols, ms, etc.

  - FFS other conditions

Agreements:

- Support a sub-channel as the minimum granularity in frequency domain for the sensing for PSSCH resource selection

  - No additional sensing for other channels

In this contribution, we provide our views on sensing and resource selection/reservation mechanism in Mode 2 for NR sidelink.

Discussion

Sensing mechanism

In LTE V2X, long term sensing mechanism is adopted for periodic traffic with fixed packet size. Tx UE can identify occupied/reserved sidelink resources by decoding SCI. This sensing mechanism can be reused in NR V2X for both periodic and aperiodic traffic. The Tx UE with periodic traffic or aperiodic traffic can perform long term sensing to select resource in shared pool for both periodic traffic and aperiodic traffic. However, whether long term sensing can avoid the collision between periodic/aperiodic and aperiodic transmission effectively that may be depended on the number of reported candidate resources (i.e. in LTE long term sensing). For example, if the reported candidate resource is greater than or equal to 20% of total resources, which is used in LTE long term sensing, the random selection step may help to avoid these collisions. Otherwise, additional enhancements should be considered.

Proposal 1: Reuse LTE-V2X long term sensing mechanism in NR V2X

In NR V2X, it will support more delay sensitive traffics. The resource selection window will be shorter than LTE V2X for these kind of traffics. So less candidate resources are available. It causes the random selection step cannot avoid the collision effectively. In that way, additional sensing mechanism is needed, for example short term sensing. Short term sensing can be performed within the resource selection window which is after the packet generation, as shown in Figure 1, Tx UE performs resource selection based on long term sensing result, and then the short term sensing is used to confirm whether the selected resource has potential collision with higher priority transmission or other aperiodic traffics.

Proposal 2: If random selection step cannot avoid the collision effectively, considering short term sensing mechanism in NR V2X.

Figure 1, long term sensing and short term sensing window

Resource selection mechanism

Based on above discussed sensing mechanism, for long term sensing, if random selection step is used to avoid the collision, it needs enough candidate resources (e.g., 20% of). But for the delay sensitive packet, the resource selection window size is shorter and the reported candidate resources may be not enough for a certain PSSCH-RSRP threshold. If excessively increasing PSSCH-RSRP threshold, the reported candidate resource may have low SINR. From system perspective, it will reduce the coverage of sidelink transmission and increase collision probability among multiple Tx UEs. So the reporting ratio (e.g., 20% in LTE long term sensing) and the maximum number of increasing PSSCH-RSRP threshold (e.g., in LTE long term sensing) can be reconsidered for the traffic with lower latency requirement in NR V2X.

Proposal 3: For long term sensing, reconsidering reporting ratio and maximum number of increasing PSSCH-RSRP threshold for the traffic with lower latency requirement in NR V2X

Otherwise, if random selection step cannot avoid the collision effectively, short term sensing mechanism can be considered for NR V2X. A UE with shorter delay budget and/or with higher priority traffic can be allowed to reserve or pre-empte resource in resource pool. The UE performs a long term sensing to avoid collision for a certain extent, and further send a reservation/pre-emption indication prior to its selected resource. This indicator can be detected by the other sensing UE for collision avoidance, especially, for candidate resource limitation case.

Proposal 4: Reservation/pre-emption indication needs to be designed if short term sensing is introduced.

Resource reservation for sidelink transmission

Reservation of resources for blind retransmission(s)

In previous meeting, Look-ahead reservation and Chain reservation methods were discussed.

For Chain reservation, each transmission can explicitly indicate another transmission of the same TB. The indication information can include the location of time/frequency resource for another transmission. In order to optimize the payload size of indication information (e.g., SCI size), LTE V2X indication mechanism can be reused. That is only explicitly indicate the time gap between two transmissions and restricting the frequency resource location is same for two transmissions. Further, if every two adjacent transmissions for one TB have same/fixed time gap, Rx/sensing UE can infer the resources of all transmissions of this TB based on time gap and order of current transmission in all transmissions of this TB. Here, the order of current transmission is similar to the ‘Retransmission index’ in LTE, it is used to indicate the number of this transmission of a certain TB.

For Look-ahead reservation, each transmission needs to explicitly indicate multiple transmissions of one TB. For example, indicating multiple time gap values between current transmission and the others transmissions or a Time resource pattern (similar to D2D). The potential payload size of SCI may be larger than Chain reservation indication method.

So, considering the potential payload size of SCI, Chain reservation is better than Look-ahead reservation.

Proposal 5: Support Chain reservation and indication method in NR V2X.

Reservation of resources for feedback-based HARQ retransmission(s)

In previous meeting, RAN1 has agreed that NR V2X Mode-2 supports resource reservation for feedback-based PSSCH retransmission by signaling associated with a prior transmission of the same TB.

Reservation of resource for feedback-based HARQ transmission(s) can provide retransmission resource for Tx UE timely when NACK is received from Rx UE in unicast/groupcast case. If there is no reserved resource for retransmission, Tx UE has to request a retransmission resource from network (in mode1) or autonomously select a resource (in mode 2) for retransmission. Both of them may increase the delay of retransmission. Reserving the retransmission resource can reduce this delay.

In other side, the probability of successful decoding for each transmission is at least 90%, so the reserved resource for subsequent transmission(s) of same TB may be not used in most instances. So RAN1 needs to discuss the enhancement mechanisms to improve utilization efficiency of reserved resource. Some conditional resource reservation mechanism also can be considered, e.g. based on channel status, CBR etc.

Based on above discussion, RAN1 needs to discuss enhancement mechanisms to improve utilization efficiency of reserved resource and conditional resource reservation mechanism from the following aspects:

Option 1: Tx UE uses the re-transmission resource for new TB transmission

Option 2: Rx UE uses the re-transmission resource

Option 3: The others UEs, except Tx UE and Rx UE, use the re-transmission resource

Option 4: Conditionally reserve re-transmission resource (e.g. based on channel status, CBR etc.)

Proposal 6: Discuss enhancement mechanisms to improve utilization efficiency of reserved resource and conditional resource reservation mechanism.

Conclusion

In this contribution, we focus on sensing and resource selection mechanism in NR V2X Mode 2 and present our views. We have the following proposals:

Proposal 1: Reuse LTE-V2X long term sensing mechanism in NR V2X

Proposal 2: If random selection step cannot avoid the collision effectively, considering short term sensing mechanism in NR V2X.

Proposal 3: For long term sensing, reconsidering reporting ratio and maximum number of increasing PSSCH-RSRP threshold for the traffic with lower latency requirement in NR V2X

Proposal 4: Reservation/pre-emption indication needs to be designed if short term sensing is introduced.

Proposal 5: Support Chain reservation and indication method in NR V2X.

Proposal 6: Discuss enhancement mechanisms to improve utilization efficiency of reserved resource and conditional resource reservation mechanism.

- References

- RP-190766, New WID on 5G V2X with NR sidelink