3GPP TSG RAN WG1 #98		R1-1908760

Prague, CZ, August 26th – 30th, 2019

Agenda Item:	6.2.1.9

Source: 	Sony

Title:	Overhead reduction in signalling of neighbour cell RSS parameters

Document for:	Discussion / decision

# Introduction

In RAN1#97, we made the following working assumption and agreement:

Working Assumption

Restrict the RSS locations by reducing the number of possible RSS locations

Agreement

Downselect and/or combine from the following in RAN1#98 for overhead reduction of the RSS time offset & RSS frequency location:

- Method 1: Reduce the number of possible RSS time offsets and RSS frequency locations

- Method 2: The RSS time offset & RSS frequency location is a function of Cell ID, i.e. no signaling bits required

- Method 3: The restricted RSS locations are dependent upon the Cell ID

- Method 4: The RSS time offset & RSS frequency location is carrier specific

- Method 5: The RSS frequency location of neighbor cells are within a frequency block relative to the RSS frequency location of the serving cell, e.g. the frequency block can be a narrowband

Other methods are not precluded

For further discussion

Introduce an indicator to indicate whether the RSS locations are from a restricted smaller set of possible RSS locations or from the full set of possible RSS locations for each neighbour cell. The exact signalling method is up to RAN2.

For further discussion

Study the impact of the UE assuming that the number of CRS ports is carrier specific.

This contribution provides further considerations on reducing overhead in signaling neighbor cell RSS parameters.

# Discussions

In RAN1#97 we made a working assumption to restrict the number of possible RSS locations (i.e. RSS frequency location and time offset).  This is preferred to reducing the resolution of the RSS locations since a reduction in resolution would lead to the UE having to perform extensive searches of the neighbour’s RSS location.  Hence, we would like to confirm the working assumption:

Proposal 1: Confirm the following working assumption:

- Restrict the RSS locations by reducing the number of possible RSS locations

The methods identified to restrict the RSS locations are:

- Method 1: Reduce the number of possible RSS time offsets and RSS frequency locations

- Method 2: The RSS time offset & RSS frequency location is a function of Cell ID, i.e. no signaling bits required

- Method 3: The restricted RSS locations are dependent upon the Cell ID

- Method 4: The RSS time offset & RSS frequency location is carrier specific

- Method 5: The RSS frequency location of neighbor cells are within a frequency block relative to the RSS frequency location of the serving cell, e.g. the frequency block can be a narrowband

One of the concerns in restricting the RSS locations raised by some companies is that the flexibility in allocating the RSS is impacted [1], [2]. Since all the 5 identified methods do not lead to any blind searches from the UE, then we will select the method that has the least impact on flexibility.

Observation 1: Select the RSS locations restriction scheme that has the least impact on the flexibility of configuring the RSS locations.

Method 2 & Method 4 do not require any signaling of the RSS location and minimal configuration (e.g. indication that restriction is applied).  Method 2 does not offer any flexibility in allocating the RSS locations since they are fixed with the Cell ID.  Method 4 has flexibility in selecting one allocation of the RSS locations but that allocation is fixed for all neighbor cells.  Hence these two methods have the least flexibility.

Method 5 has more flexibility compared to Method 2 & Method 4 but the RSS location is restricted within the narrowband of a serving cell, which effectively means that RSS locations are constrained within 3 locations of a selected narrowband.  The rationale of Method 5 is that the UE does not need to retune to another narrowband to measure a neighbor cell’s RSS.  However, this can be accomplished by more flexible methods such as Method 1 and Method 3.

In Method 1, the number of RSS frequency locations can be reduced to 4 locations if 2 bits are used to indicate RSS location for each neighbor cell, whereas if any location could have been indicated, 7 bits would have been required to allow indication of one of up to 99 possible locations.  These 4 locations can be uniformly spread out across the system bandwidth.  Similarly, the RSS time offset can be reduced to 4 offsets which give a total of 16 possible different RSS locations that can be configured.  This offers some flexibility for the network.

In Method 3, the restricted RSS frequency locations are a function of the Cell ID, for example, for each Cell ID there are 2 or 4 possible RSS frequency locations and 2 or 4 possible RSS time offsets.  In addition to Cell ID, the system bandwidth can also be used to determine these RSS locations.  This will offer a lot more possible RSS locations than can be configured compared to Method 1.  However, we have a slight preference in using Method 1 since it is a simpler solution.

Proposal 2: In reducing the signaling of the RSS locations (frequency locations & time offsets) restrict the number of possible RSS frequency locations and number of RSS time offsets, i.e.:

- The number of RSS frequency locations is reduced to 4 locations

- The number of RSS time offsets is reduced to 4 offsets

Proposal 3: The restricted RSS frequency locations are a function of the system bandwidth.

The flexibility in scheduling the RSS locations (frequency locations & time offsets) can be restored fully for one or more of the neighbor cells.  A 1 bit indicator can be introduced for each neighbor cell where this bit would indicate whether the RSS locations are from a restricted set or from the full set of possible RSS locations.  For example a restricted set of RSS locations uses 4 bits, where 2 bits are for the frequency location and 2 bits are for the time offset.  This 1 bit indicator would then tell the UE whether 4 bits are used to indicate the RSS location or the full 12 bits are used (i.e. 7 bits for frequency location and 5 bits for time offset) [3].  We expect that in the majority of the cases, the network would use the restricted set thereby giving a net saving in overall signalling bits.  The exact signalling method is up to RAN2.

Proposal 4: Introduce an indicator to indicate whether the RSS locations are from a restricted smaller set of possible RSS locations or from the full set of possible RSS locations for each neighbour cell.  The exact signalling method is up to RAN2.

# Conclusion

In this contribution, we discuss some remaining details in reducing the overhead in signalling the neighbour cells’ RSS paramters.  We observe the following:

Observation 1: Select the RSS locations restriction scheme that has the least impact on the flexibility of configuring the RSS locations.

Therefore, we propose the following:

Proposal 1: Confirm the following working assumption:

- Restrict the RSS locations by reducing the number of possible RSS locations

Proposal 2: In reducing the signaling of the RSS locations (frequency locations & time offsets) restrict the number of possible RSS frequency locations and number of RSS time offsets, i.e.:

- The number of RSS frequency locations is reduced to 4 locations

- The number of RSS time offsets is reduced to 4 offsets

Proposal 3: The restricted RSS frequency locations are a function of the system bandwidth.

Proposal 4: Introduce an indicator to indicate whether the RSS locations are from a restricted smaller set of possible RSS locations or from the full set of possible RSS locations for each neighbour cell.  The exact signalling method is up to RAN2.

# References

[1] R1-1906503, “Use of RSS for measurement improvement,” ZTE, RAN1#97

[2] R1-1906709, “Use of RSS for measurement improvements,” Nokia, Nokia Shanghai Bell, RAN1#97

[3] R1-1906847, “Overhead reduction in signalling of neighbour cell RSS parameters,” Sony, RAN1#97