3GPP TSG RAN WG1 Meeting #98	R1-1908761

Prague, CZ, August 26th – 30th, 2019

Agenda Item:	6.2.2.1

Source: 	Sony

Title:	UE-group wake-up signal for NB-IoT

Document for:	Discussion and Decision

# Introduction

In RAN1 meeting #97 [1], the following related agreements and working assumptions on UE-group wake up signal for NB-IoT were made:

Agreement

- If the group WUS resource is configured to be shared by Rel-15 WUS and Rel-16 WUS, the common WUS sequence for all the group WUS UEs in the same WUS resource can be configured to be the Rel-15 WUS sequence or a Rel-16 WUS sequence.

- Each UE monitors up to X WUS sequences.

- Value of X will be selected between 2 and 3

  - X=3 can only be supported if a common WUS for a subset of UE groups for service-based grouping is supported

Working Assumption

- At least for the group WUS in the same WUS resource, legacy WUS with phase shifts is selected as group WUS sequence design according to wgroup(m’) = w(m’) exp(j2πgm/G)

  - FFS: Details of g and G

- The UE may assume that the transmit power for Rel-16 WUS sequence is the same as that of the Rel-15 WUS sequence.

- The maximum WUS duration for the Rel-16 WUS sequence is the same as that of the Rel-15 WUS sequence.

In this contribution, we discuss some aspects on the operation of UE-group wake-up signal for NB-IoT (NWUS).

# Discussion

Grouping mechanism of UEs within a NWUS will allow only a group of UEs to be woken up by the NWUS for a paging occasion (PO). The UE grouping operation will reduce the number of UEs having to wake up unnecessarily to detect NPDCCH and decode PDSCH. Thus, it reduces UE power consumption for such UEs.

# Multiplexing of NWUS

Apart from specific UE-group(s) NWUS for rel-16 and a UE-group NWUS rel-15 signals (legacy), it has been agreed to support common-group NWUS signal. A common-group NWUS signal is a NWUS signal targeting to all UEs that has been assigned to monitor a WUS resource. In RAN1 #97, it was agreed if the group WUS resource is configured to be shared by Rel-15 NWUS and Rel-16 NWUS signals, the common WUS sequence for all the group WUS UEs in the same WUS resource can be configured to be the Rel-15 WUS sequence or a Rel-16 WUS sequence. A UE rel.16 can be configured to detect either its own NWUS sequence or common NWUS sequence while a UE rel.15 can only be configured its own NWUS sequence. In case an eNB configures the common NWUS sequence is the Rel-15 WUS sequence and transmitted in the shared WUS resource, then the UE rel.16 will always wake-up although the eNB intends to only wake-up UE rel.15. This is basically increasing false wake-up to the UE rel.16 and result in higher power consumption. In another scenario, in case eNB configures the common NWUS sequence is the Rel-16 WUS sequence and transmitted in the shared WUS resource, then the UE rel.15 cannot be woken-up as it cannot detect Rel-16 WUS sequence (blocking).

Observation 1: The usage of Rel-15 WUS sequence or a Rel-16 WUS sequence as the common WUS sequence in the shared WUS resource by rel-15 NWUS and rel-16 NWUS can create false wake-up and blocking issues.

In a legacy LTE system, the probability of UEs having the same paging frame as a function of the number of paging frames is given in Table 1. While a reduction of the occasions of many UEs being paged at the same time can be achieved by allocating many PFs, this solution might not be the optimal one as more resources are being used to carry paging information.

Table 1: Number of PF versus Probability UEs with same PF

| N | UE_ID mod N | % UEs have same paging frame |
| --- | --- | --- |
| 1 | 0 | 100% |
| 2 | 0,1 | 50% |
| 4 | 0,1,2,3 | 25% |
| 8 | 0,1,2,…,7 | 12.5% |
| 16 | 0,1,2,…,15 | 6.75% |

In scenarios where the number of paging frames is limited or can be configured to be a small number, introducing large number of UE-groups would be clearly beneficial. The advantage is that the network can have more flexibility and the number of UEs being paged unnecessarily can be further reduced. In terms of the maximum supported number of UE-groups, we propose to maximize the benefit of the UE-group mechanism. In order to limit the probability of overhearing to below 15% we propose the maximum number of UE groups supported is at least 6.

Proposal 3: The maximum number of UE groups supported is at least 6 (i.e. to maximize the benefit of UE-group mechanism, such as reducing false wake-up rate).

# UE NWUS Detection Operation

The common NWUS is supported in Rel-16. In such case, basically there is a dedicated NWUS to wake up all UEs assigned to the corresponding paging occasion. Common NWUS is in addition to the individual UE group NWUS. The benefit of having common NWUS is that the eNB only needs to transmit the common NWUS when eNB needs to wake up all UEs. However, from the UE point of view, this could increase UE power consumption and complexity as the UE needs to detect both common NWUS and group specific NWUS when listening to the channel for potential WUS. The design of NWUS signal in Rel-16 should consider the UE power consumption and complexity aspects. A design constraint is, for example, the UE does not have to detect twice even when no NWUS exist on the channel.

Observation 2: On the supporting of both common NWUS and group NWUS, the NWUS sequence design shall consider UE power consumption and complexity.

NWUS grouping may support the operation where the eNB pages only one group, several groups (sub-set groups), or all groups. In RAN1 meeting #96, we concluded to further study the additionally support the feature of waking up a subset of all WUS UE groups. The information on a subset of WUS groups should be carried in group ID indication embedded in NWUS for the case of NWUS with SS-CDM. NWUS with SS-CDM also indicate that a NWUS resource can be assigned for multiple groups. The UE needs to identify the group ID information in order to decide whether to continue with paging reception or go to sleep. The UE-group ID can be included in form of bitmap operation as illustrated in Table 2. Here, an example is given when the supported number of groups is 4. It can be observed that the table size is significantly bigger in proportional to the maximum number of UE groups.

Table 2: UE-group ID with bitmap operation

| No. | Bitmap | UE-Group ID being paged |
| --- | --- | --- |
| 1 | 1111 | 1,2,3,4 |
| 2 | 1110 | 2,3,4 |
| 3 | 1101 | 1,3,4 |
| 4 | 1100 | 1,2,4 |
| 5 | 1011 | 1,2,3 |
| 6 | 1010 | 3,4 |
| 7 | 1001 | 2,4 |
| 8 | 1000 | 2,3 |
| 9 | 0111 | 1,4 |
| 10 | 0110 | 1,3 |
| 11 | 0101 | 1,2 |
| 12 | 0100 | 4 |
| 13 | 0011 | 3 |
| 14 | 0010 | 2 |
| 15 | 0001 | 1 |

It can also be observed that most of the possible operation/combination is to page subset of all WUS UE groups. The number of operations/combinations will be significantly less in case MWUS with single-seq CDM for some of the groups is not supported. Furthermore, the number of supported groups can be higher if it does not require to waking up a subset of all WUS UE groups. On the other hand, it may have some potential issues if group MWUS is only designed to page a group and all groups. In RAN1 #97, it has been agreed that each UE monitors up to X WUS sequences and X is either 2 or 3. Here, basically the UE needs to monitor a NWUS sequence for its own group, common group, and a service-based group. Having one sequence represents subset of all WUS UE groups may not be sub-optimal. In order to keep the power consumption low, we propose X=2.

Proposal 4: Each UE monitors up to 2 WUS sequences.

In RAN1 #97, we discussed the transmit power and duration of rel.16 WUS signal. We consider it is important to keep the same parameters on transmit power and maximum WUS duration for both WUS rel-15 and WUS rel-16.

Proposal 5: Confirm these working assumptions:

- UE assumes the transmit power for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence

- Maximum WUS duration for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence

Regarding on how the UEs are grouped, configurable weighting factor for UE distribution per time/freq WUS resource was discussed in RAN1 #96bis meeting. The intention was to reduce false wake-up rate by allowing dynamic UE distribution per WUS resource. This would be beneficial for reducing the UE power consumption. We consider this as a way to assign/re-assign the UE group based on paging. According to the progress in the other working group, it is feasible for the MME to provide assistance information including paging probability information to the eNB [3]. Further detailed on defining the grouping is described in our companion contribution [4]. In our view, the method to allocate the UE in certain group is beyond RAN1 scope.

# 3	Conclusions

In this contribution, the following observations and proposals are made:

Observation 1: The usage of Rel-15 WUS sequence or a Rel-16 WUS sequence as the common WUS sequence in the shared WUS resource by rel-15 NWUS and rel-16 NWUS can create false wake-up and blocking issues.

Observation 2: On the supporting of both common NWUS and group NWUS, the NWUS sequence design shall consider UE power consumption and complexity.

Proposal 1: The maximum number of UE groups supported is at least 6 (i.e. to maximize the benefit of UE-group mechanism, such as reducing false-paging alarm rate).

Proposal 2: Each UE monitors up to 2 WUS sequences.

Proposal 3: Confirm these working assumptions:

- UE assumes the transmit power for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence

- Maximum WUS duration for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence

# References

- RAN1 #96bis Chairman notes, Xian, China April 2019

- R2-1905261 LS on assistance indication for WUS grouping, Huawei, April 2019.

- S2-1908147, Reply LS on assistance indication for WUS grouping

- R2-1909910 On supporting UE-group wake-up signal for MTC, Sony, RAN2#107, Prague, August 2019.