3GPP TSG RAN WG1 Meeting #98                                                              	R1-1908776

Prague, CZ, August 26th-30th, 2019

Source:	Sony

Title:	Consideration on delay-tolerant HARQ for NTN

Agenda Item:	7.2.5.4

Document for: 	Discussion

# Introduction

In RAN#80, a new SI “Solutions for NR to support non-terrestrial networks (NTN)” was approved[1]. The SID was further revised to limit the scope in Release 16 to key issues and solutions associated with transparent GEO and LEO satellite based non-terrestrial access networks (moving beam on earth)[2]. The new study item has objectives related to the physical layer as below:

Physical layer Consolidation of potential impacts as initially identified in TR 38.811 and identification of related solutions if needed [RAN1]: - Physical layer control procedures (e.g. CSI feedback, power control) - Uplink Timing advance/RACH procedure including PRACH sequence/format/message - Making retransmission mechanisms at the physical layer more delay-tolerant as appropriate. This may also include capability to deactivate the HARQ mechanisms. Performance assessment of NR in selected deployment scenarios (LEO based satellite access, GEO based satellite access) through link level (Radio link) and system level (cell) simulations [RAN1]

In this contribution, we share our considerations on more delay-tolerant re-transmission mechanisms which may include the capability to deactivate the HARQ mechanisms.

# HARQ disabling

2.1 Dynamic HARQ disabling

Based on the analysis in [3], the round-trip delay (RTD) of LEO satellites at 600 km is 28.408ms and 544.751ms for GEO satellites at 35786km. These RTDs exceed the maximum conventional HARQ timers (after which an ACK is received) or the maximum possible number of HARQ processes, especially for GEO satellites. As HARQ process is a very time-critical mechanism, mechanisms to deactivate the HARQ shall be considered in RAN1.

In RAN1#97 meeting [4], the following agreements were achieved:

Agreement: Network disabling of HARQ via RRC configuration should be supported. - FFS: Dynamic disabling of HARQ by gNB.

In RAN2#106 meeting [5], the following agreements were achieved:

Agreement: - If HARQ feedback is disabled, blind HARQ (re)transmissions are still possible to improve robustness. What blind HARQ retransmissions mean will be captured in email discussion. - Even if HARQ feedback is disabled, the HARQ processes are still configured. 4: Enabling / disabling of HARQ feedback is a network decision

Compared to RRC configured HARQ disabling, dynamic HARQ disabling via DCI is more flexible. In NR, various activation/deactivation commands in PDSCH are used for semi-persistent CSI-RS/SRS/CSI report resource configuration and beam configuration. The activation/deactivation commands are applied after the HARQ-ACK corresponding to the PDSCH carrying the commands transmitted. If HARQ is disabled for those PDSCH carrying the activation/deactivation command, it will cause continuous error if the PDSCH carrying the commands is not detected or is decoded in error. The HARQ feedback shall be enabled to ensure the PDSCH carrying important signaling e.g., RRC signaling, activation/deactivation command is decoded correctly and disabled to improve the efficiency of traffic data transmission. Therefore, dynamic HARQ disabling shall be supported in NTN.

To support dynamic HARQ disabling, adding a new bit in DCI to indicate the HARQ disabling is the most direct way, however, it will increase the signaling overhead of DCI. Another method is reusing the existing fields in DCI to indicate HARQ disabling. The following methods can be considered:

- One option is field of HARQ process ID, when UE is informed with one special HARQ ID, UE will know that this PDSCH transmission is without ACK/NACK feedback. As RAN2 agreed, blind HARQ (re)transmissions are still possible to improve robustness if HARQ feedback is disabled. Using one specific HARQ process ID, e.g., HARQ process ID 0 to indicate HARQ disabling may impact the blind HARQ (re)transmission as the UE cannot distinguish multiple TBs with same HARQ process number. For example, if two TBs #1, #2 are ACK/NACK disabled, the gNB needs to set the HARQ process ID for both as 0, then UE should not perform soft-combining of the TBs even though they have the same HARQ process number.

- Another option is field of PDSCH-to-HARQ_feedback timing indicator which is used to indicate the time gap between PDSCH transmission and the reception of ACK/NACK for the PDSCH represented as K1. A specific state of this field, e.g., all ‘0’ can be defined to indicate the HARQ disabling. When UE is informed with all ‘0’ for field of PDSCH-to-HARQ_feedback timing indicator, UE will know this PDSCH transmission is without ACK/NACK feedback and perform soft-combining for each TB based on the HARQ process number if repetition is supported. The values of K1 corresponding to the states other than all’0’ should be modified to accommodate the long RTD in NTN.

Observation 1: If HARQ is disabled for the PDSCH carrying semi-persistent resource activation/deactivation command, it will cause continuous error if the PDSCH carrying the commands is not detected or is decoded in error.

Proposal 1: Dynamic HARQ disabling via DCI signaling should be supported. The following options should be studied:

- Option 1: Additional field in DCI to inidcate HARQ disabling

- Option 2: Reuse existing DCI field to indicate HARQ disabling

  - Option 2_1: Specific HARQ process ID to indicate HARQ disabling

  - Option 2_2: Specific PDSCH-to-HARQ_feedback timing indicator state to indicate HARQ disabling

2.2 Beam switching timeline when HARQ is disabled

NTN is a beam-based network as illustrated in Fig. 1. In Option a, one satellite cell includes multiple satellite beams while in Option b, there is one PCI per satellite beam. In the case of one satellite cell including multiple satellite beams (Option a), UE should track the serving beam due to the relative movement between UE and satellite and perform Layer-1 beam switching based on the beam measurement results and/or satellite ephemeris.

Fig. 1 Options for PCI and SSB mapping into satellite beams [6]

According to the current specification, each UE can be configured with multiple beams for physical channels, e.g., PxCCH, PDSCH, MAC CE is used to active/deactivate one or part of beam(s) from the multiple beams, and a HARQ-ACK based timeline is defined to ensure beam alignment between gNB and UE. Take PDCCH beam activation as an example: as Fig. 2(a) shows, if the UE receives a MAC CE activation command for one of the RRC configured beams represented as TCI #2, the UE applies the activation command in the first slot 3 msec after a slot where the UE transmits HARQ-ACK information for the PDSCH providing the activation command.

As discussed in section 2.1, HARQ-ACK shall be supported for PDSCH carrying important signaling like beam activation/deactivation, however, assuming mechanisms like lower MCS, repetition are adopted to maintain the reliability when HARQ is disabled, the following beam activation/deactivation timeline can be considered:

- When HARQ-ACK is disabled for the PDSCH carrying the MAC CE activation/deactivation command, if the UE received a MAC CE of beam activation/deactivation command for one of the beams for PDCCH/PUCCH at slot n, the UE applies the activation/deactivation command at slot n+m as Fig.2(b) shows, wherein the slot number [m] can be configured by RRC.

- When HARQ-ACK is disabled for the PDSCH carrying the MAC CE activation/deactivation command, if the UE received a MAC CE of beam activation/deactivation command for part of the beams for PDSCH at slot n, the UE applies the indicated mapping between beams and codepoints of the DCI field at slot n+m, wherein the slot number [m] can be configured by RRC.

Similar methods can be considered for semi-persistent resource activation/deactivation of ZP-CSI-RS, NZP CSI-RS, CSI-IM, CSI reporting and SRS in NR.

gNB

UE

TCI#2 activation

ACK feedback

TCI #2 apply

3∙Nslotsubframe,μ+1

Fig. 2(a) HARQ-ACK based beam switching timeline

gNB

UE

TCI#2 activation

TCI #2 apply

Slot number m

Fig. 2(b) HARQ-ACK less based beam switching timeline

Proposal 2: When HARQ-ACK is disabled for the PDSCH carrying the MAC CE of beam activation/deactivation command:

- If the UE received a MAC CE of beam activation/deactivation command for PUCCH/PDCCH at slot n, the UE applies the activation/deactivation command at slot n+m, FFS the value of slot number [m].

- If the UE received a MAC CE of beam activation/deactivation command for PDSCH at slot n, the UE applies the indicated mapping between beams and the codepoints of the DCI field at slot n+m, FFS the value of slot number [m].

# HARQ Process Number

Another issue in NTN is the number of HARQ processes, and following agreement was achieved in the last meeting [4].

Agreement: Evaluate impact of Satellite RTT when HARQ is enabled and potential solutions if needed - At least the following aspects should be considered if the number of HARQ processes is > 16: - DCI size - HARQ soft buffer size

In current spec., 4-bit HARQ process field in DCI is used to indicate the HARQ process ID. To minimize the spec. impact, we think the field size of HARQ process shall remain unchanged even though the HARQ process number is increased.

Regarding the HARQ soft buffer size, different UEs may have different capabilities with different HARQ soft buffer sizes. The gNB can configure different numbers of HARQ processes based on UE capability. Based on RAN2’s agreements, blind HARQ (re)transmissions are still possible to improve robustness even if HARQ is disabled, and the HARQ processes are still configured. If the repetition number is 4 for blind HARQ (re)transmission, the UE still needs to store all the received data in the soft buffer for a specific HARQ process. Therefore, we propose to support more than 16 HARQ process number in NTN for high capability UEs with 4-bit HARQ process field in DCI remain unchanged, and methods of increasing HARQ process number implicitly, e.g., synchronous HARQ scheme can be considered for high capability UEs.

Proposal 3: The 4-bit HARQ process field in DCI should remain unchanged.

Proposal 4: Support more than 16 HARQ process number in NTN for high capability UEs.

# Summary

In this contribution, we provide considerations on more delay-tolerant HARQ for NTN. Following is our observations and proposals.

Observation 1: If HARQ is disabled for the PDSCH carrying semi-persistent resource activation/deactivation command, it will cause continuous error if the PDSCH carrying the commands is not detected or is decoded in error.

Proposal 1: Dynamic HARQ disabling via DCI signaling should be supported. The following options should be studied:

- Option 1: Additional field in DCI to inidcate HARQ disabling

- Option 2: Reuse existing DCI field to indicate HARQ disabling

  - Option 2_1: Specific HARQ process ID to indicate HARQ disabling

  - Option 2_2: Specific PDSCH-to-HARQ_feedback timing indicator state to indicate HARQ disabling

Proposal 2: When HARQ-ACK is disabled for the PDSCH which carrying the MAC CE of beam activation/deactivation command:

- If the UE received a MAC CE of beam activation/deactivation command for PUCCH/PDCCH at slot n, the UE applies the activation/deactivation command at slot n+m, FFS the value of slot number [m].

- If the UE received a MAC CE of beam activation/deactivation command for PDSCH at slot n, the UE applies the indicated mapping between beams and the codepoints of the DCI field at slot n+m, FFS the value of slot number [m].

Proposal 3: The 4-bit HARQ process field in DCI should remain unchanged.

Proposal 4: Support more than 16 HARQ process number in NTN for high capability UEs.

# References

- 3GPP, RAN #80, RP-180664, “NR-NTN: Preliminary solutions for NR to support non-terrestrial networks”.

- 3GPP, RAN #82, RP-182880, “Study on solutions for NR to support non-terrestrial networks (NTN)”.

- 3GPP, TR 38.811, “Study on New Radio (NR) to support non terrestrial networks”.

- 3GPP, RAN1#97, Chairman’s Notes

- 3GPP, RAN2#106, R2-1908105, “Report from session on Legacy LTE, Rel-15 LTE, and NR NTN SI, NR power saving SI”.

- 3GPP, TR 38.821 Solutions for NR to support non-terrestrial networks (NTN), Figure 7.3.1-1