3GPP TSG RAN WG1 Meeting #98	R1-1908790

Prague, CZ, August 26th – 30th, 2019

Agenda Item:	7.2.10.3

Source: 	Sony

Title:	Considerations on UE measurements for NR Positioning

Document for:	Discussion / Decision

# Introduction

A new work item “NR positioning support” was approved In RAN plenary #83 [1] . During study item phase, NR positioning techniques (e.g. DL-TDOA, DL-AoD) were presented and evaluated. The preliminary results can be found in [2] .

In RAN1 meeting #97, The following agreements were made [3]:

Agreement:

- Select one (or more) of the following options for the definition of the RSTD

  - Option 1: RSTD is defined as the time difference with respect to the subframe timings associated with the different TPs;

  - Option 2: RSTD is defined as the time difference with respect to the timings associated with the DL PRS resource sets of different TPs

  - Option 3: RSTD is defined as the time difference with respect to the timings associated with the DL PRS resources of different TPs

  - Note: Definition of the RSTD applies at least to the RSTD measurements obtained from DL PRS. It does not preclude the RSTD obtained from other DL RS

- DL PRS-RSRP is defined as the linear average over the power contributions (in [W]) of the resource elements of the antenna port that carry a DL PRS resource configured for RSRP measurements within the considered measurement frequency bandwidth

  - FFS: DL PRS-RSRP for 2 ports, if 2 port transmission of a DL PRS is agreed.

- When receiver diversity is in use by the UE to obtain the DL PRS-RSRP measurement, the reported DL PRS-RSRP value shall not be lower than the corresponding DL PRS-RSRP of any of the individual receiver branches

- Support inter-frequency DL PRS-RSRP measurements for NR positioning in Rel-16 when the UE is in RRC_CONNECTED state.

- UE can be indicated to report the DL PRS Resource ID(s) or the DL PRS Resource set ID(s) associated with the DL PRS Resource ID(s) or the DL PRS Resource set(s), which are used in determining the UE measurements (DL RSTD, UE Tx-Rx time difference, DL PRS-RSRP) in UE measurement reporting.

- Support reporting separate metric(s) corresponding to quality of each of the following measurements

  - RSTD

  - UE Rx-Tx time difference

  - UL-RTOA

  - gNB Rx-Tx time difference

  - UL angle of arrival, including the azimuth of arrival (AoA) and the zenith of arrival (ZoA)

- FFS: The details of these quality metrics

# In this contribution, we discuss our view on UE positioning measurements for downlink-based positioning solutions in NR.

# Discussions

An illustration of a downlink-based positioning using multi-lateration in shown in Figure 1. Here, a UE needs to receive multiple beam-forming signals from gNBs/TRPs for the multi-lateration and then perform positioning measurement. The measurement can be a timing-based measurement, such as RSTD measurement that is commonly used in observed time different of arrival (OTDOA) or it can also be a received power-based measurement, such as RSRP measurement. In this example, each gNB transmits positioning reference signal (PRS) using a certain beam direction that is typically required in NR with FR2.

Figure 1: Illustration of OTDOA with beam-forming signal from three gNBs.

In order to ensure the UE can perform positioning measurement efficiently, the configuration on PRS multiplexing should be introduced within preconfigured PRS resources, which can accommodate the PRS transmitted from multiple beams and from multiple gNBs. Furthermore, a set of PRS resources (e.g. multiple resource elements carrying PRS) can represent PRS angle of departure (AoD) or transmit beam of PRS from a gNB. This is illustrated in Figure 2. Here, a gNB is illustrated with possible 4 different beam directions and it translates to 4 different PRS resources.

Figure 2: Illustration of PRS multiplexing supporting multiple beams

During the reception of a configured set of PRS resources, the UE performs measurement for each gNB. Furthermore, the measurement for each gNB can results in multiple measurements in case the gNB has been configured with multiple set of PRS resources where each PRS resources represent certain beam direction (AoD). The UE can perform timing measurements (RSTD) and/or received power measurements (RSRP) for each set of PRS resources. RSTD measurement has been used in LTE and the definition is defined in [4]. In LTE, the RSTD measurement is associated to the timing of PRS signal from a base-station. However, the RSTD measurement in NR is associated with the measurement based on the received PRS from a base-station and in a specific PRS resource (i.e. beam). Thus, we consider to support Option 3 as agreed in RAN1 meeting 97.

Proposal 1: Support Option 3: RSTD is defined as the time difference with respect to the timings associated with the DL PRS resources of different TPs

It has been agreed that a UE can be indicated to report the DL PRS Resource ID(s) or the DL PRS Resource set ID(s) associated with the DL PRS Resource ID(s) or the DL PRS Resource set(s). As one of the forms of the indication to the UE, we consider it can be based on the selected gNBs. For example, a UE can report the measurement report of selected gNBs. The selection can be based on the measurement quality, for example best N measurement reports from a set of gNBs. This operation would be beneficial in order to improve the efficiency of positioning measurement report.

Proposal 2: Support to limit the positioning measurement report (i.e. based on the selected gNBs).

Referring to the configured PRS multiplexing and beam operation as shown in Figure 2, the UE can perform these set of measurements as shown in Figure 3. The measurement can be an RSTD measurement and/or RSRP measurement. In this example, if the UE is located between gNB1 - beam 1 and gNB 2 – beam 3 then the UE is expected to obtain the best measurements based on those beams. Assuming the UE has known knowledge of the PRS configuration, the UE can identify the beam index in the form of resource ID of that measurement. Here, basically the UE has obtained the angle of departure (AoD) information of each gNB.

Meas (gNB1,b4)

Meas (gNB1,b3)

Meas (gNB1,b2)

Meas (gNB1,b1)

Meas (gNB1,b4)

Meas (gNB2,b3)

Meas (gNB2,b2)

Meas (gNB2,b1)

Figure 3: UE measurements

Proposal 3: NR Positioning supports DL-AoD measurements based can be based on DL PRS RSRP and DL PRS RSTD

Furthermore, and related to this example, the UE reports the measurement based on (gNB1, b1) and (gNB2, b3). Essentially, the measurement from the UE at least contain a pair of RSTD or RSRP measurement and the associated DL PRS resource ID from each gNB.

Proposal 4: The UE reports the measurement (e.g. RSTD, RSRP) and the associated DL PRS Resource ID(s) (e.g beam information).

# Conclusion

In this contribution, we discussed our view on downlink-based positioning particularly on the UE measurement aspects. We have made the following proposals:

Proposal 1: Support Option 3: RSTD is defined as the time difference with respect to the timings associated with the DL PRS resources of different TPs

Proposal 2: Support to limit the positioning measurement report (i.e. based on the selected gNBs).

Proposal 3: NR Positioning supports DL-AoD measurements based can be based on DL PRS RSRP and DL PRS RSTD

Proposal 4: The UE reports the measurement (e.g. RSTD, RSRP) and the associated DL PRS Resource ID(s) (e.g beam information).

# References

- RP-190752, “New WID: NR Positioning Support”

- TR38.855,” Study on NR positioning support”

- Chairman notes RAN1 Meeting #97

- TS36.214