3GPP TSG RAN WG1 Meeting #98	R1-1908791

Prague, CZ, August 26th – 30th, 2019

Agenda Item:	7.2.10.4

Source: 	Sony

Title:	On physical-layer procedures to support UE measurements

Document for:	Discussion / Decision

# Introduction

A new work item “NR positioning support” was approved In RAN plenary #83 [1] . One of the objectives are [1] :

If necessary, define physical-layer procedures to support UE/gNB measurements for NR positioning [RAN1]

# During study item phase, NR positioning techniques (e.g. DL-TDOA, DL-AoD, UL-TDOA, UL-AoA) were presented and evaluated. The preliminary results can be found in [2] .

# The following were some agreements made during in RAN1 meeting #97 [3] :

Agreement:

For intra-frequency measurements, when measurement gaps are configured, the UE is expected to measure the DL PRS resource outside the configured (active and inactive) BWPs (Option 2 in the related Agreement from RAN1 #96-Bis).

  - Note: The UE has flexibility to measure only on a part of the DL PRS configured bandwidth as long as measurement requirements are met.

For positioning purposes, to assist UE to perform Rx beamforming, the following options are supported (options are from previous related agreement in RAN1#96bis):

  - Option 1: The DL PRS can be configured to be QCLed Type D with a DL Reference Signal from a serving or neighboring cell.

  - Discuss further which DL reference signal can be used (e.g., SSB, CSI-RS, DL-PRS).

  - Option 2: The UE may perform a Rx beam sweeping on DL PRS resources which are transmitted with the same downlink spatial domain transmission filter.

  - Option 3: The UE may use a fixed Rx beam to receive DL PRS resources which are transmitted with different downlink spatial domain transmission filter.

  - FFS: Whether this option can be achieved by Option 1

  - FFS on specification impacts.

For positioning purposes, with regards to the UL SRS transmission power, support at least the following in addition to the existing Rel-15 behaviour:

  - Support configuring a DL reference signal of a neighbouring cell to be used as DL path loss reference for the purpose of SRS power control.

  - FFS: Which reference signals can be used (e.g. a CSI-RS, or a SSB, or DL PRS).

  - Study further the UE configuration signalling and procedure.

  - FFS on a fallback procedure if the UE is not able to obtain the pathloss reference.

  - FFS on number of measurements for pathloss.

For positioning purposes, for UL Beam management/alignment towards serving and neighbouring cells, the following is supported (in addition to UE TX beam sweeping):

  - Configuration of a spatial relation between a reference DL RS from serving or neighbouring cells and the target SRS.

  - Reference DL RS that can be used include at least SSB. FFS on CSI-RS, DL-PRS.

  - A fixed Tx beam for UL SRS transmissions across multiple UL SRS Resources, for both FR1 and FR2.

  - FFS how the fixed beam is selected.

  - FFS on specification impacts.

  - Note: In Rel-16, UE is not expected to transmit multiple SRS resources with different spatial relations in the same OFDM symbol (Rel-15 behaviour)

# In this contribution, we discuss our view on physical-layer procedures to support UE measurements, particularly related to the above recent agreements.

# Discussions

## DL-based positioning

In RAN1 meeting 96bis, it was agreed to support DL PRS with the same numerologies (CP length and sub-carrier spacing) as for data transmissions in Rel-15. It implies a possibility of multiplexing DL PRS with other channels (e.g. PDSCH). Furthermore, it also exposes the case where different gNB may use different numerology (SCS, CP). Those possible situations do not exist in LTE positioning. In order to facilitate an efficient DL PRS reception at the UE side, the design of DL-PRS shall consider the possibility of a UE to receive DL-PRS from multiple gNBs within the certain time/frequency resource. It can be called as within a positioning occasion. Furthermore, in order to facilitate an efficient DL-PRS multiplexing, the DL-PRS from multiple gNBs can be grouped together within a positioning occasion with the same numerology. This can be supported by introducing NR positioning bandwidth part. Within an NR positioning bandwidth part, it may contain one or multiple DL PRS resource set.

Proposal 1: Support NR positioning bandwidth part, in which it may contain multiple DL PRS resource set with the same numerology.

We consider the NR positioning bandwidth part is typically for cell-specific DL-PRS. Hence, it may not always be the same as the legacy UE DL bandwidth parts. The NR positioning BWP may also be located in separate carrier frequency than the on-going transmission (e.g. control / data channel) between gNB and UE. In case the UE needs or instructed to perform positioning measurement based on that allocated NR positioning BWP, a positioning measurement gap is needed. The positioning measurement gap information/configuration shall be informed to the UE.

Proposal 2: Support positioning measurement gap to allow the UE to perform positioning measurement operation.

It has been agreed that The DL PRS can be configured to be QCLed Type D with a DL Reference Signal from a serving or neighboring cell. However, it is for further study on the DL reference signal (e.g., SSB, CSI-RS, DL-PRS) that can be used for the QCLed type D operation. In principle, we consider the aforementioned reference signals can be used for the QCL configuration. In our companion contribution [4] , we discuss on cell-specific DL-PRS and UE-specific DL-PRS. The cell-specific DL-PRS can be configured to be QCLed type D with SSB. The UE-specific DL-PRS can be configured to be QCLed type D with SSB, cell-specific DL-PRS. In addition, the serving cell can be configured to have UE-specific DL-PRS can be configured to be QCLed type D with CSI-RS.

Proposal 3: The DL PRS can be configured to be QCLed Type D with a DL Reference Signal from a serving cell (SSB, CSI-RS, DL-PRS) or selected neighboring cell (SSB, DL-PRS).

## UL-based positioning

The UE measures RSRP from each surrounding gNBs within a predefined interval and duration. Moreover, this measurement assists the UE to collect information on the best gNB transmit beam. Based on the downlink measurement in NR rel.15, the UE is basically has collected information related to the surrounding cells, such as channel quality and beam direction of each gNBs. Furthermore, this information can be used by the UE with minimum effort for the uplink-based positioning, particularly for beam coordination purpose. The only additional effort is the UE transmits the measurement report in preparation for configuring the UE SRS transmission for positioning purpose. The UE can transmit the measurement report either to LS or the serving cell that has been configured to support a similar LS function.

In practice, the UE can be allocated to transmit SRS without the UE downlink measurement reports. However, the SRS may not be optimally transmitted by the UE. The UL-SRS may not be received by sufficient number of gNBs or best gNBs. The UE can perform blind beam-sweeping operation with the objective the UL-SRS can be received by multiple gNBs. However, this may also increase the interference to other UEs.

Proposal 4: The beam sweeping operation of UL SRS can be determined based on the UE reports on selected downlink measurement information.

The beam coordination for uplink-based positioning can be facilitated based on the downlink measurement operation at the UE. NR downlink measurement mechanism has been established/defined in NR rel.15. It is typically performed by the UE to measure mainly the channel quality (e.g. RSRP) of serving gNB. The RSRP measurement of neighbor cell is only performed if the RSRP measurement of serving cell is deteriorated. For positioning purpose, first the UE needs to perform measurement based on the received downlink reference signals from multiple gNBs. Secondly, this measurement may affect the uplink SRS configuration, such as SRS transmit power. In case the UE needs to reach at least 3 gNBs, the pathloss estimation that may affect transmit power can be based on the selected gNB where the measured RSRP is as the third-largest according to the list of measured RSRP values from the neighbor gNBs.

Proposal 5: The pathloss estimation for the purpose of UL SRS power control is determined based on the selected gNBs.

## Supporting On-demand positioning

The initial target of NR positioning for both regulatory and commercial requirements are described in [2] . The report also mentioned that it is expected that no single solution would fit all of the requirements for various use-cases and scenarios. In downlink-based positioning (e.g. DL-TDOA), an accurate positioning result typically requires wide bandwidth and/or long reference signal (e.g. PRS) transmission. Long PRS transmission may, however, increase the latency. Yet, some use-cases require high accuracy and short latency. In practice, we consider the majority of use-cases would not require high accuracy and only require reasonable latency. Nevertheless, the solution should cover both the more demanding use cases with very high accuracy requirement (e.g. sub 1m), as well as the uses cases with less demanding requirements

Observation 1: NR positioning is expected to support demanding requirements (high accuracy and low latency) for some UEs and relaxed requirements for typical applications in most UEs.

Based on the above observation, an adaptive/flexible approach to positioning techniques may be used to address the multiple requirements. A flexible approach can include the combination of RAT dependent and RAT independent techniques which is not the scope of this work item [1] . Another consideration is the ability to flexibly configure OTDOA positioning with a goal of achieving the positioning requirements. It has been agreed that the flexibility can be achieved by the possibility of allocating on-demand NR DL PRS resource allocation. By doing so, the UE may receive more reference signals in order to obtain an improved positioning measurement for higher positioning accuracy. A location server is assumed to know the required positioning accuracy. It can trigger (e.g. via LPPa protocol) the selected gNBs to allocate on demand resource allocation for NR DL PRS transmission.

Proposal 6: UE-specific DL PRS resources can be provided to the UE(s) in addition to the DL PRS resources that is periodically transmitted by the gNB.

Another aspect which we would like to propose is on supporting the combined downlink and uplink-based positioning (and vice-versa) to improve the positioning accuracy. In this case, the uplink-based positioning (UL-TDOA) can be operated in conjunction with the downlink-based positioning (DL-TDOA). We can expect downlink PRS signal is periodically transmitted by the gNBs. Here, the SRS resources for UL-TDOA can be allocated after the DL-PRS resources. The time-gap between the different uplink and downlink resources shall be relatively small so that the obtained measurements can be utilized to a single positioning estimate. This is illustrated in Figure 1. Two positioning measurement results represents the two directions and can be obtained and utilized by the location server for positioning estimation. Furthermore, the UE needs to receive the configuration of both positioning configurations including the timing relationship (i.e T-gap) between downlink and uplink positioning reference signals.

T-gap

Resources for UL pos

Resources for DL pos

Figure 1:Illustration of DL-PRS and UL-SRS to support DL/UL positioning

Proposal 7: On supporting DL & UL based positioning measurements, a timing relationship between DL and UL positioning reference signals shall be provided.

# Conclusion

In this contribution, we discussed our view on some aspects related to physical-layer procedures to support UE measurements. We have made the following observation and proposals:

Proposal 1: Support NR positioning bandwidth part, in which it may contain multiple DL PRS resource set with the same numerology.

Proposal 2: Support positioning measurement gap to allow the UE to perform positioning measurement operation.

Proposal 3: The DL PRS can be configured to be QCLed Type D with a DL Reference Signal from a serving cell (SSB, CSI-RS, DL-PRS) or selected neighboring cell (SSB, DL-PRS).

Proposal 4: The beam sweeping operation of UL SRS can be determined based on the UE reports on selected downlink measurement information.

Proposal 5: The pathloss estimation for the purpose of UL SRS power control is determined based on the selected gNBs.

Proposal 6: UE-specific DL PRS resources can be provided to the UE(s) in addition to the DL PRS resources that is periodically transmitted by the gNB.

Proposal 7: On supporting DL & UL based positioning measurements, a timing relationship between DL and UL positioning reference signals shall be provided.

# References

- RP-190752, “New WID: NR Positioning Support”

- TR38.855,” Study on NR positioning support”

- Chairman notes RAN1 Meeting #97

- R1-1908788, DL reference signals for NR positioning, Sony, Prague, CZ, August 2019