3GPP TSG RAN WG1 #98		R1-1908803

Prague, CZ, August 26th – 30th, 2019

Source:	Panasonic

Title: 	Discussion on physical layer structure for sidelink in NR V2X

Agenda Item:	7.2.4.1

Item for:	Discussion, Decision

Introduction

This contribution is modification of R1-1906402. It is the discussion on physical layer structure for sidelink in NR V2X, especially on two step SCI design.

Discussion

Relation between SL BWP and Uu BWP

In our view SL BWP and Uu BWP should be independently configured in a carrier of licensed band and the linkage should be decoupled to reduce the design complexity especially considering dynamic BWP switching for Uu operation. Following Rel.15 NR framework, the baseline is only one activated BWP in certain slot in a carrier so activated SL BWP and activated Uu BWP are TDMed for a UE. The merit of such approach is simpler design but latency is increased due to BWP switching depending on the configuration. Simultaneous activated Uu BWP and SL BWP may reduce the latency for sidelink but it will introduce many other issues like half duplex, intra-band emission, PAPR and so on. In the end the system performance may be even worse considering these new problems.

Proposal 1:  Baseline is activated SL BWP and activated Uu BWP are TDMed in a carrier for a UE. FFS FDMed SL BWP and Uu BWP are simultaneously activated in a carrier.

Proposal 2:  SL BWP and Uu BWP are independently (pre)configured in a carrier of licensed band and not necessary to restrict SL BWP is always within Uu BWP. The network can configure two BWPs which do not cause or minimize BWP switching delay. UE is allowed to implement constant RF reception/transmission bandwidth over two BWPs.

In FDD, following LTE V2X principle, SL BWP is configured in uplink carrier considering many potential collisions with DL traffics. In addition, if SL BWP is configured/activated in DL carrier while DL BWP is not activated, it may require UE to simultaneously transmit SL channels and UL channels, which increases UE’s implementation complexity and PAPR. To deactivate reception of DL BWP will also impact Uu performance considering DL will transmit many important system information like MIB, SIB and so on. Instead, the impact caused by deactivating UL transmissions is little less.

For TDD, at least SL BWP is configured to align with UL symbols. Whether SL BWP can be aligned with flexible symbols or DL symbols needs to be further discussed. One issue is flexible symbols may be indicated as DL symbols and it have collision with SL transmission in SL BWP. If SL BWP is aligned with all symbols in a slot, DL reception of Uu may be impacted due to half duplex and the handling is complicated. If SL BWP is only aligned with UL symbols in slot, DL symbols are not impacted. For mode 1, gNB could indicates usable symbols for SL. Then UL and flexible symbols could be used for SL. For mode 2, only UL symbols are used for SL to avoid half-duplex issue and usable symbols for SL should be (pre-)configured.

Proposal 3:  In FDD, SL BWP is configured in uplink carrier. In TDD, SL BWP is configured to align with UL symbols and flexible symbols for mode 1 and UL symbols for mode 2.

Time domain granularity of resource pool configuration

For both dedicated SL carrier and licensed carrier, a slot is used for granularity of resource pool configuration. For licensed carrier in mode 2, as discussed in above, usable symbols in a slot would be additionally (pre-)configured. Tx UE can use SL usable symbols in a slot which configured for a resource pool.

Proposal 4: For both dedicated SL carrier and licensed carrier, a slot is used for granularity of resource pool configuration.

Proposal 5: For licensed carrier in mode 2, usable symbols in a slot are additionally (pre-)configured.

Separation of resource pool for unicast/groupcast/broadcast

One (pre-)configured resource pool used for any combination of unicast, groupcast, and broadcast has better resource utilization considering dynamic traffic changes of different data type. Even for broadcast, (pre-)configured PSFCH resources can be avoided to allocate PSCCH/PSSCH. For sensing perspective, when unicast/groupcast share the resource pool with broadcast traffic, it is required that PSCCH in a resource pool should be able to be decoded by all UEs sharing the resource pool. Therefore, CRC of PSCCH of unicast/groupcast should not be scrambled by UE ID/group ID as proposed in [4].

Proposal 6: One (pre-)configured resource pool used for any combination of unicast, groupcast, and broadcast.

PSCCH and PSSCH multiplexing

RAN4 discussed transient times [1] and sent a LS to RAN1[2]. WI for NR V2X [3] was agreed and following points were clarified, which are related to PSCCH/PSSCH multiplexing.

-  Multi-rank PSSCH transmission is supported up to two antenna ports.

- NR sidelink design starts with frequencies in FR1, and NR sidelink in FR2 is supported by applying the design for FR1 and PT-RS to the numerologies agreed for FR2. No FR2 specific optimization is supported in this WI except PT-RS. No beam management is supported in this work.

- 2Rx antennas as well as 4Rx antennas are supported. The full range of speeds defined in SA1 (TS 22.186) needs to be supported in FR1.

In previous our contribution R1-1902201, we see the need of option 1B from following points.

- Flexibility of rank usage between PSSCH and PSCCH,

- Power boosting for PSCCH

- Reduced blind decoding,

- Reduced Tx dynamic range in the same PA operation point (total signal dynamic range is increased as the symbol containing PSCCH has different PA setting),

- Reduced reception buffer,

- No collision between PSCCH and PSSCH among different Tx UEs,

- No separate resource usage indication of PSSCH in the symbol containing PSCCH,

- Simplified sensing/resource selection, simplified channel estimation.

On the other hand, as based on WID Rel.16 will not have specific optimization on FR2, several motivations to support option 1B were relieved. We interpret this means Rel.16 is not required to support very wideband operation of sidelink bandwidth. To focus on only one multiplexing scheme in Rel.16 can reduce the standardization effort for NR sidelink design. Therefore, we would propose to focus on option 3 in Rel.16 assuming no support of very wideband operation. As we expect future option 1B usage would be different frequency band, no forward compatibility preparation would be required.

We propose following usage of option 3.

- In order to keep RB configuration or total transmit power constant, within a transmission including the symbol containing both PSCCH and PSSCH, PSD is constant over all REs.

- PSCCH is rank 1 transmission. As described in [1][2], to transmit only from single antennas connector requires the change of the power. Therefore, rank 1 with 2 port transmission as Tx diversity should be used. For the number of antennas, 2 and 4 antennas are supported according to WI[3]. In case of two antennas, each antenna is mapped to one port. In case of four antennas, the antennas are virtualized to 2 ports. If more antennas are supported, it can be realized in a same way based on implementation.

As the summary, we propose following.

Proposal 7: To support option 3 in Rel.16.

Proposal 8: PSCCH is rank 1 with two ports transmission (Tx diversity) or 1 port with virtualization.

Proposal 9: For PSSCH, based on different rank situation or number of antennas as well as agreements in WID, we propose following,

- Rank 1 is same implementation as PSCCH rank 1.

- Rank 2 based transmission is based on 2 ports.

- Four Tx antennas are virtualized to 2 ports based on different implementation.

DMRS pattern for PSSCH

For Frequency domain, following is working assumption.

Working assumption: - Rel-15 PDSCH DMRS Configuration type 1 and/or type 2 are reused for frequency-domain pattern of PSSCH DMRS. - FFS whether to support either one or both types - FFS details on multiplexing of different ports for PSSCH DMRS

In our view, both PDSCH DMRS Configuration type 1 and type 2 could be supported and which type is used is (pre-)configured for a resource pool.

For time domain, the set of time domain DMRS configuration is (pre-)configured. Tx UE can choose the one of DMRS configuration and 1st SCI indicates it. In case of one DMRS configuration is (pre) configured, SCI doesn’t need to indicate DMRS configuration.

SCI Design

There are basically three alternatives on SCI design to support different cast traffics: two step SCI, single SCI and two SCIs with separate BD times. The detailed comparison among three alternatives are shown in Table 1. In our view, two step SCI can have better coverage especially for broadcast traffic while other two have some issues on coverage.

In case of two step SCI, the first SCI of two stage SCI is designed as PSCCH and PSSCH multiplexing option 3. LTE V2X-like resource mapping with reduced time duration with PSSCH can be used for PSCCH as starting point. PSCCH can be located in certain PRBs of PSSCH in each sub-channel from Rx UE point of view, like LTE V2X. CORESET may be needed to define location of PSCCH in order to reuse NR Uu specification as much as possible. The CORESET configuration including the symbol length could be (pre-) configured depending on the coverage requirement. From Tx UE point of view, PSSCH avoids the mapping to the REs where the scheduling PSCCH is located. The REs, other than used for PSCCH in the CORESET, can be used for PSSCH.

For second SCI of two stage SCI it is transmitted like UCI encoded within PUSCH. More specifically, the mapping of HARQ-ACK in NR Uu can be used for PSCCH. The first SCI is designed to carry information related to demodulation and common information for broadcast/groupcast/unicast.  The second SCI is designed to carry the information specific to group or unicast. It has merit on reducing the size on first SCI. More details on field information are shown in table 2 and table 3 as an example,

Table 1 Comparison among three alternatives on SCI design

|  | Two step SCI | Single SCI (Padding to larger size) | Two SCIs with separate BD sizes (one is for broadcast and another is for unicast/groupcast) |
| --- | --- | --- | --- |
| Coverage | Similar as LTE V2X (assuming same SCS) | Bad due to large size | For SCI for broadcast, similar as LTE V2X; for SCI for unicast/groupcast, bad coverage due to large size |
| Commonality | Common design for all casts | Common design for all casts | Differentiated design for different cast |
| Spec Complexity | A little larger | Small | Middle |

Table 2 First SCI of two step SCI

| 1st SCI (Taking SCI format 1 in LTE as reference) | Size of 1st SCI |
| --- | --- |
| Priority | Same as LTE |
| Resource reservation | Same as LTE |
| Frequency resource location of initial transmission and retransmission | Same as LTE |
| Time gap between initial transmission and retransmission | Same as LTE |
| Modulation and coding scheme | Same as LTE |
| Retransmission index | Same as LTE |
| Transmission format | Same as LTE |
| Reserved information bits | Same as LTE |
| Time domain DMRS pattern | 0, 1, or 2 bits |
| Joint encoding on indicating "whether to receive 2nd SCI or not" and "what is 2nd SCI size" | 3 or 4 bits Unicast and groupcast are distinguished by Destination ID/Source ID in 2nd SCI |
| CRC | 24 bits as used in NR PDCCH |

Table 3 Second SCI of two step SCI

| 2nd SCI (Taking DCI format 0_1 in NR as reference) | Size of 2nd SCI | Comment on 2nd SCI |
| --- | --- | --- |
| HARQ ID | 2-4 bits | Depending on supported number of HARQ process |
| RV | 0 bits | Fixed rule is possible |
| NDI | 1 bit |  |
| Precoding | 0 bit | Assuming PSSCH and 2nd SCI share the same DMRS |
| Antenna port | 0 bit | Fixed rule |
| CBGTI | 0-2 bits | No support |
| PTRS-DMRS association | 0 bit | Pre-configuration |
| DMRS sequence initialization | 0 bit | Fixed rule |
| Destination ID | 8 bits | Less than L2 ID by hash function |
| Source ID | 8 bits | Less than L2 ID by hash function |
| CSI request | 1 bit | Independent resource allocation between CSI request and CSI report |
| CRC | 11 bits | Assuming total size of 2nd SCI is larger than 20 bits |

Proposal 10: Two-step SCI is supported for NR sidelink

PSFCH format

In RAN1#98, a sequence-based PSFCH format with one symbol (not including AGC training period) is agreed. Currently we don’t see the need to support multiple PSFCH formats since the coverage for unicast/groupcast could be closer area than broadcast. For FDM between PSCCH/PSSCH and a PSFCH, in system perspective, we see only FDM with PSSCH is the possibility to avoid PSCCH is not receive by half-duplex issue [5]. If the symbol containing PSFCH is also used for PSSCH, the symbol should not carry DMRS of PSSCH in order not to skip DMRS reception.

Proposal 11: only a sequence-based PSFCH format with one symbol is supported.

CSI request procedure

Our thinking on CSI request procedure is shown in Figure 1, where there are 5 steps:

Step 1) From UE A, PSCCH/PSSCH are transmitted and CSI-RS is confined within PSSCH. In PSCCH, 1 bit CSI request is indicated in SCI.

Step 2) UE B measures CSI from CSI-RS transmitted by UE A and selects the resource of PSCCH/PSSCH for CSI report.

Step 3) From UE B, PSCCH/PSSCH are transmitted with CSI report.

Step 4) UE A does link adaptation and resource selection of PSCCH/PSSCH.

Step 5) UE A transmits PSCCH/PSSCH.

Here the resource selection among step 1, step 3 and step 5 is independent.  Based on above we propose

Proposal 12: Resource selection of PSCCH/PSSCH is independent between CSI request and CSI report.

Figure 1 CSI request procedure

Conclusion

In this contribution we discussed physical layer structure for sidelink in NR V2X and followings are proposed,

Proposal 1:  Baseline is activated SL BWP and activated Uu BWP are TDMed in a carrier. FFS FDMed SL BWP and Uu BWP are simultaneously activated in a carrier.

Proposal 2:  SL BWP and Uu BWP are independently (pre)configured in a carrier of licensed band and not necessary to restrict SL BWP is always within Uu BWP. The network can configure two BWPs which do not cause or minimize BWP switching delay. UE is allowed to implement constant RF reception/transmission bandwidth over two BWPs.

Proposal 3:  In FDD, SL BWP is configured in uplink carrier while in TDD, SL BWP is configured to align with UL symbols.

Proposal 4: For both dedicated SL carrier and licensed carrier, a slot is used for granularity of resource pool configuration.

Proposal 5: For licensed carrier in mode 2, usable symbols in a slot are additionally (pre-)configured.

Proposal 6: One (pre-)configured resource pool used for any combination of unicast, groupcast, and broadcast.

Proposal 7: To support option 3 in Rel.16.

Proposal 8: PSCCH is rank 1 with two ports transmission (Tx diversity) or 1 port with virtualization.

Proposal 9: For PSSCH, based on different rank situation or number of antennas as well as agreements in WID, we propose following,

- Rank 1 is same implementation as PSCCH rank 1.

- Rank 2 based transmission is based on 2 ports.

Four Tx antennas are virtualized to 2 ports based on different implementation.

Proposal 10: Two-step SCI is supported for NR sidelink

Proposal 11: only a sequence-based PSFCH format with one symbol is supported.

Proposal 12: Resource selection of PSSCH/PSCCH is independent between CSI request and CSI report.

Reference

[1] R4-1902511, WF on NR V2X UE RF parameters and transient period, LG Electronics

[2] R4-1902514, Reply LS on NR UE transient period in sidelink channels, 3GPP RAN WG4

[3] RP-190766, New WID on 5G V2X with NR sidelink, LG Electronics, Huawei

[4] R1-1908804, Discussion on sidelink resource allocation in mode 2 for NR V2X, Panasonic

[5] R1-1908805, Discussion on physical layer procedures for sidelink in NR V2X, Panasonic