3GPP TSG RAN WG1 #98	R1-1908891

Prague, CZ, August 26th – 30th, 2019

Agenda item:	7.2.4.6 QoS management for sidelink

Source:	ITRI

Title:	QoS management for LTE and NR congestion control

Document for:	Discussion & Decision

Introduction

In RAN1#97 meeting [1], the QoS management for sidelink was discussed, and the following agreements were made:

Agreement:

- LTE V2X sidelink congestion control is the starting point for defining NR sidelink congestion control.

Agreement:

- Higher-layer reporting of CBR to the gNB is supported for RRC_CONNECTED UEs.

As a result, the NR and LTE sidelink congestion control for the QoS management should be studied to handle the related issues.

LTE and NR sidelink congestion control

Channel Busy Ratio (CBR) is a time-dependent value between zero and one representing the fraction of time that a single channel is busy with transmissions [2], and the ratio is as follows [3]:

| Definition | Channel busy ratio (CBR) measured in subframe n is defined as follows: - For PSSCH, the portion of sub-channels in the resource pool whose S-RSSI measured by the UE exceed a (pre-)configured threshold sensed over subframes [n-100, n-1]; - For PSCCH, in a pool (pre)configured such that PSCCH may be transmitted with its corresponding PSSCH in non-adjacent resource blocks, the portion of the resources of the PSCCH pool whose S-RSSI measured by the UE exceed a (pre-)configured threshold sensed over subframes [n-100, n-1], assuming that the PSCCH pool is composed of resources with a size of two consecutive PRB pairs in the frequency domain. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

As a result, the CBR is the sensing results within 100 sub-frames, and the measurement method differs in view of PSSCH or PSCCH. While we adopt the LTE V2X sidelink congestion control as the starting point for defining NR congestion control, the CBR is of the first priority on discussion.

While the higher-layer reporting of CBR to the gNB is supported for RRC_CONNECTED UEs, the UEs in RRC_IDLE and RRC_INACTIVE state should also be considered. Without active links available for UL reporting, it is not available for IDLE and INACTIVE UEs to perform the higher-layer CBR reporting. Besides, Event V1 and V2 is triggered after the sensed CBR is above or below the threshold. As a result, the corresponding behavior in NR should also be considered.

- Proposal 1: CBR reporting for NR IDLE and INACTIVE UEs should be considered, along with the V1and V2 event.

Since the congestion control is the starting point for NR QoS control, and the LTE congestion control as the baseline, it is necessary to check the LTE congestion control mechanism. The LTE congestion control mechanism includes several components, such as CBR and channel occupancy ratio (CR). In addition, decentralized congestion control (DCC) specified in [2] also plays an important role in the mechanism. DCC is especially useful in case of LTE mode 4 since the DCC provides reactive/adaptive mechanism algorithm and a CBR threshold for UEs to work in a distributed manner. The DCC_ACC is located in the access layer, and it includes the transmit power control, transmit rate control, and transmit data rate control for the traffic. With the three transmit parameters, the bandwidth, latency, and sustained traffic of the given sidelink carrier can be guaranteed, which is beneficial for the CBR applicability over the NR system. The ETSI DCC defines the CBR threshold based on the given parameters to achieve the desired QoS metrics, so it is advantageous to directly link the required QoS metrics to the CBR determination.

- Proposal 2: The method and parameters of CBR determination helps achieve the desired QoS requirement.

Conclusion

In this contribution, the following proposals are provided:

- Proposal 1: CBR reporting for NR IDLE and INACTIVE UEs should be considered, along with the V1and V2 event.

- Proposal 2: The method and parameters of CBR determination helps achieve the desired QoS requirement.

Reference

- Draft Minutes Report RAN1 #97 v0.3.0

- ETSI TS 102 687 V1.2.1, Apr. 2018

- TS 36.214 V15.3.0, Sept. 2018.

- TR 22.886 V16.0.0, Jun. 2018.