3GPP TSG RAN WG1 #98	R1-1908892

Prague, CZ, August 26th – 30th, 2019

Source:	ITRI

Title:	The discussions on NR-Uu controlling LTE sidelink

Agenda Item:	7.2.4.7

Document for:	Discussion and Decision

Introduction

In RAN1#97 meeting [1], the following agreements were made for the support of NR Uu controlling LTE sidelink.

Agreements: - RRC-based activation/deactivation is not supported - DCI-based activation/deactivation is supported - Support of LTE PC5 scheduling by NR Uu (mode 3-like ) is based on UE capability - NR DCI provides the fields of DCI 5A in LTE-V that are related to SPS scheduling - The size of DCI for activation/deactivation is one of the DCI size(s) that will be defined for NR Uu scheduling NR V2V - FFS whether the DCI format is the same as one of the DCI formats that will be defined for NR Uu scheduling NR V2V - Activation/deactivation applies to the first LTE subframe after Z+X ms after receiving the DCI - Z is the same timing offset in current LTE V2X specs - X>0. FFS value(s) of X, and if one or multiple values of X are possible

As shown in the agreements, RRC-based and DCI-based activation/release for LTE PC5 scheduling by NR Uu (mode 3-like) have been discussed and some agreements have been made. However, there are topics in this subject required to be discussed. In this contribution, we provide our views on this topic.

- The NR-Uu controlling to LTE sidelink

In RAN1#97 meeting [1], it has been agreed that DCI-based activation/release is supported for LTE PC5 scheduling (mode 3-like), while the DCI format has not been determined yet. In RAN1#ah1901, it has been agreed that NR Uu, similarly to LTE Uu, provides configurations to LTE mode-4 sidelink through RRC or SIB signaling. From RAN1 perspective, signaling should be similar to LTE. Signaling details should be RAN2’s decisions. Hence, we urge that the details of signaling for both schemes should be discussed.

The NR-Uu controlling to LTE sidelink mode-3

It has been agreed that NR DCI provides the fields of DCI 5A in LTE-V that are related to SPS scheduling. Currently, the DCI format 5A including the information of carrier indicator, Lowest index of the subchannel allocation, frequency resource location, Time gap between initial transmission and retransmission, etc. which is shown in Table I.

Table I: DCI Format 5A

Currently, we may define the DCI format to be the same as one of the DCI formats that will be defined for NR Uu scheduling NR V2V. The other option is to consider a modified DCI formats to support. The former approach has advantages in lower complexity and less standard impact. However, considering the current DCI formats of NR-Uu may not be efficiently enough to support. The latter approach may also be applicable. Otherwise, it is required to consider some piggyback approaches. We urge to discussion this issue in the meeting.

- We recommend to discuss a more suitable DCI formats of NR-Uu or considering some piggyback approaches.

The NR-Uu controlling to LTE sidelink mode-4

In RAN1#AH1901, it has been agreed that NR Uu, similarly to LTE Uu, provides configurations to LTE mode-4 sidelink through RRC or SIB signaling. From RAN1 perspective, signaling should be similar to LTE in terms of UE-specific or cell-specific. Signaling details should be up to RAN2. Hence, we urge that the details of signaling for RRC-based schemes should be discussed in RAN2’s meeting.

- The details of RRC-based signalling should be discussed in RAN2’s meeting.

Conclusion

In this paper, we suggest the use case should be considered when RAN1 makes the decision on two schemes on the table. The proposals are listed as follows.

- We recommend to discuss a more suitable DCI formats of NR-Uu or considering some piggyback approaches.

- The details of RRC-based signalling should be discussed in RAN2’s meeting.

[1] Chairman’s note, RAN1#97, Reno, US, May 2019.