3GPP TSG-RAN WG1 Meeting #98                                	R1-1908929

Prague, Czech Republic, 26th – 30th August 2019

Agenda Item:	7.2.4.2.2

Source:	Asia Pacific Telecom

Title:	Discussion on sidelink resource allocation mode 2

Document for:	Discussion and decision

# Introduction

At RAN1#96 meeting, the following agreements [1] related to NR mode 2 sidelink resource allocation were reached:

- Agreements:

- Blind retransmissions of a TB are supported for SL by NR-V2X

  - Details are for the WI phase

- Agreements:

- NR V2X Mode-2 supports reservation of sidelink resources at least for blind retransmission of a TB

  - Whether reservation is supported for initial transmission of a TB is to be discussed in the WI phase

  - Whether reservation is supported for potential retransmissions based on HARQ feedback is for the WI phase

Agreements:

- Mode-2 sensing procedure utilizes the following sidelink measurement

  - L1 SL-RSRP based on sidelink DMRS when the corresponding SCI is decoded

    - FFS whether/which measurement is used if the corresponding SCI is not decoded e.g. SL-RSRP after blind DMRS detection, SL-RSSI

At RAN1#96 bis meeting, the following agreements [2] related to NR mode 2 sidelink resource allocation were reached:

Agreements:

- NR V2X supports an initial transmission of a TB without reservation, based on sensing and resource selection procedure

- NR V2X supports reservation of a sidelink resource for an initial transmission of a TB at least by an SCI associated with a different TB, based on sensing and resource selection procedure

  - This functionality can be enabled/disabled by (pre-)configuration

- FFS Standalone PSCCH transmissions for resource reservations are supported in NR V2X

At RAN1#97 meeting, the following agreements [3] related to NR mode 2 sidelink resource allocation were reached:

Agreements:

- Support a sub-channel as the minimum granularity in frequency domain for the sensing for PSSCH resource selection

  - No additional sensing for other channels

Agreements:

- NR V2X Mode-2 supports resource reservation for feedback-based PSSCH retransmissions by signaling associated with a prior transmission of the same TB

  - FFS impact on subsequent sensing and resource selection procedures

  - At least from the transmitter perspective of this TB, usage of HARQ feedback for release of unused resource(s) is supported

    - No additional signaling is defined for the purpose of release of unused resources by the transmitting UE

    - FFS the behavior of the receiver UE(s) of this TB and other UEs

In this contribution, we discuss and provide our views on the sensing procedure for NR V2X sidelink mode-2 resource.

# Mode 2 resource allocation

## Sensing & resource selection

Sidelink sensing is the procedure where the UE identifies available sidelink resources for transmission. In the last meeting, RAN1 has agreed to support the sensing and resource (re)-selection procedures which include at least decoding of sidelink control channel transmissions, sidelink measurements, and detection of sidelink transmissions. Sensing can be used to avoid potential collisions not only for out-of-coverage scenario but also for in-coverage scenario, if gNB configures UE to perform mode 2 resource allocation in the resource pool. Sidelink resource selection is the mechanism where the UE selects resources at least for  PSSCH, and UE will determine which resource to be used for transmission based on the result of the sensing procedure. Besides, UE will select the resource candidates based on received SCI, such as the information field corresponding to resource allocation or resource reservation for PSSCH. Moreover, in the last meeting, it has been agreed that the PSFCH resource is in slot n+a for the corresponding HARQ feedback of a PSSCH transmission with its last symbol in slot n. Hence, UE may also consider the PSFCH resource as reserved resource while performing sensing according to the resource allocation of the corresponding PSSCH.

Proposal 1: Sensing and resource selection procedures of mode 2 should also consider corresponding PSFCH resource of a PSSCH transmission

CSI acquisition for unicast will be supported by NR V2X. We would like to discuss the impact on SL-CSI measurement and report for resource selection and sensing procedure. Regarding resource selection, UE can select a better resource pool or carrier when UE obtained a CSI report which may contain RI/CQI. On the other hand, UE can give a lower priority to resource pool with bad quality for CSI measurement. For sensing procedure, we would like to discuss CSI acquisition mechanism in NR-V2X first. CSI acquisition mechanism has been discussed in the last meeting, and RAN1 concluded is that NR-V2X will strive to reuse the CSI framework of NR Uu. Thus, we assume there are at least CSI resource configuration and CSI report configuration for SL CSI framework. CSI resource configuration is to indicate the resource allocation RS for CSI measurement, and RS for CSI measurement could be CSI-RS or DMRS of PSCCH or PSSCH as discussed last meeting [4]. Although broadcast UEs do not need to monitor those RSs, broadcast UEs may better aware the existing of those RSs to reduce interference and ensure the accuracy of CSI measurement. For example, if NR-V2X introduces Period/Semi-persistent CSI resource transmission and broadcast UEs do not aware these RSs, some CSI resource may be interfered by broadcast UEs, but some CSI resource may not. In consequence, the accuracy of CSI will be affected. Further, RAN plenary decides that CSI report is delivered using PSSCH (including PSSCH containing CSI only). In NR Uu, CSI report can also be transmitted through PUSCH. For example, semi-persistent CSI report can be transmitted on PUSCH based on activation of MAC-CE, and aperiodic CSI report can be transmitted on PUSCH if UE received CSI request indication in UL DCI. Hence, we think the framework of at least semi-persistent and aperiodic CSI report can be supported in NR V2X. However, different from LTE V2X, there are broadcast UEs which may use the same resource on shared carriers, and we need to avoid collision between broadcast transmission and CSI report. For example, if we adopt similar design as NR-Uu to activate semi-persistent CSI report on PUSCH, it is impossible for broadcast UE to decode the MAC-CE of unicast transmission and obtain information for CSI report.

Observation 1: Reusing NR-Uu CSI report and measurement framework for NR-V2X CSI acquisition will introduce some possible resource collision on the shared carrier.

Proposal 2: Suggest consider the impact of P/SP/AP CSI measurement and report procedure on resource selection and sensing procedure for NR-V2X mode 2 resource allocation.

As discussed during previous meetings, whether to support standalone PSCCH transmissions for resource reservations is FFS. From our perspective, standalone PSCCH for resource reservation can be helpful for UE to achieve service requirement. For example, one UE can reserve one or multiple slots for PSSCH transmission for future transmission to fulfill the latency requirement of QOS attributes pertaining to the service. If this feature is not allowed, UE can only reserve resource when there are data to transmit. However, those data may have stringent delay requirement, and reserving resource when performing the first transmission of those data may increase latency. Hence, we propose to support standalone PSCCH transmission for resource reservation. Furthermore, we think if there is standalone PSCCH for CSI request as in PDCCH of Uu interface, the PSSCH resource for CSI report can also be reserved by that standalone PSCCH which triggers CSI report

Proposal 3: Support standalone PSCCH transmissions for resource reservations in NR V2X

## LBT-like sensing

In order to fulfill latency requirement for some advanced services, such as remote driving, some companies propose to apply collision avoidance technique like listen-before-talk (LBT) where a UE performs a clear channel assessment and potentially random back-off before accessing the channel. However, random back-off may increase latency performance especially when the system overload is high. Moreover, even though LBT procedure defined for the unlicensed spectrum may be reused for sidelink carrier, the detail and feasibility are still unclear, and LBT procedure may need further discussion and enhancement on how to utilize reserved resource information and resource allocation information in SCI to assist LBT-like sensing. Considering the overhead of specification work for specifying a new LBT-like sensing for V2X operation, we propose to reuse LTE-like sensing procedure which used for the Rel-14/15 LTE-V2X sensing-based resource selection procedure with some enhancement to reduce latency, such as adaptation of the sensing window length according to service type.

Observation 2: Applicability of LBT-like sensing procedure for V2X sensing needs to be further justified due to potentially larger latency introduced from random back-off at least for licensed band case.

Proposal 4: Suggest use Rel-14/15 LTE V2X sensing procedure as the baseline for sensing procedure of Rel-16 NR-V2X. Enhancement of Rel-16 NR-V2X sensing procedure for advanced service can be achieved by supporting sub-slot or symbol level granularity

## Sensing procedure for V2X

The sensing-based resource selection procedure standardized in Rel-14/15 has several drawbacks to achieve low latency requirement for advance NR-V2X services. Hence, some enhancements to reduce latency for sensing procedure is needed if we attempt to reuse the LTE-like sensing procedure for NR-V2X. Specifically, timing granularity of sensing in LTE V2X is subframe-based, and UE may exclude some subframes when determining possible transmitting resource with in a TX resource pool if the subframes are subframes in which SLSS resource is configured, or if the subframe is subframes configured as downlink subframes or special subframes. Then, UE will only perform sensing from those remaining transmitting resources in the TX resource pool, and UE will group a candidate resource set which satisfies RSRP and/or RSSI condition defined in specification.

However, in NR carrier/BWP, the DL/UL/flexible format can be defined per symbol level, so the rule or granularity of sensing in NR-V2X may be enhanced for compatibility with Uu symbol level format (e.g., symbol level format can be configured/indicated by TDD-UL-DL-ConfigCommon, TDD-UL-DL-dedicated in RRC signaling, or slot format indicator in DCI format 2-0). It is noted that slot format may also be changed when UE switches its active Uu and/or SL BWP to a new Uu and/or SL BWP. Therefore, resource granularity for sensing can be further discussed to be compatible with NR slot format framework. For example, UE may also consider UL symbols within special slots (i.e., flexible slot) or DL slots as candidate resource for sensing as shown in Figure 1 instead of considering only UL slots as in LTE-V2X.

Figure 1. SL and Uu coexist in the shared carrier in NR if only UL symbol can be used for sidelink based on slot format indicator

Proposal 5: RAN1 needs to consider compatibility with NR Uu and QoS requirement when determining resource granularity for sensing in NR-V2X

## Releasing of unused resource

It was agreed that at least from the transmitter perspective of this TB, usage of HARQ feedback for release of unused resource(s) is supported, and no additional signaling is defined for the purpose of release of unused resources by the transmitting UE. When a PSSCH transmission is successful, i.e, NACK is not received for HARQ-NACK only case or ACK is received for HARQ-ACK/NACK case, then there is no need for re-transmissions. Therefore, any resources that have previously been reserved can be released by transmitting UE. Resources that are released can then be reused by other UEs for their transmissions. Therefore, it is important that NACK/ACK transmissions are monitored by other UEs wanting to perform resource selection. It is agreed that sequence-based PSFCH format with one symbol is supported, and the sequence of PSFCH format may reuse sequence of PUCCH format 0. In order for other UEs to decode HARQ feedback on PSFCH resource from receiving UE, resource allocation parameter for PSFCH resource, such as number of symbols, number of subchannels or cyclic shift, should be known by other UEs.  Moreover, in order to increase multiplexing gain on PSFCH resource, we propose that cyclic shift value for sequence-based PSFCH format can be indicated via SCI instead of applying a fixed cyclic shift value for sequence-based PSFCH .

Proposal 6: Support to introduce PSFCH resource parameter indicator in SCI for other UEs to monitor HARQ feedback from receiving UE.

If a NACK is received before resource selection window when other UEs perform resource selection and sensing,  the reserved resources cannot be utilized for the transmission as shown in Figure 2. Other UEs must avoid selecting the reserved resources for their PSSCH transmission. In addition, if other UEs receive neither NACK nor ACK on the corresponding PSFCH resource of a PSSCH transmission and transmitting UE has already reserved resource for retransmitting this PSSCH, other UEs cannot utilize those reserved resources in case that other UEs miss-detect HARQ feedback from receiving UE.

Figure 2. reserved resource cannot be utilized by other UEs if NACK for the corresponding PSSCH is received during sensing window

Proposal 7: Reserved PSSCH resource should not be utilized by other UEs if other UEs detect neither ACK nor NACK from receiving UE.

# Conclusions

In this contribution, we discussed the sensing and resource allocation for mode 2 resource allocation for NR V2X sidelink transmission.   We have the following proposals:

Observation 1: Reusing NR-Uu CSI report and measurement framework for NR-V2X CSI acquisition will introduce some possible resource collision on the shared carrier.

Observation 2: Applicability of LBT-like sensing procedure for V2X sensing needs to be further justified due to potentially larger latency introduced from random back-off at least for licensed band case.

Proposal 1: Sensing and resource selection procedures of mode 2 should also consider a reserved resource for HARQ feedback.

Proposal 2: Suggest consider the impact of P/SP/AP CSI measurement and report procedure on resource selection and sensing procedure for NR-V2X mode 2 resource allocation.

Proposal 3: Support standalone PSCCH transmissions for resource reservations in NR V2X

Proposal 4: Suggest use Rel-14/15 LTE V2X sensing procedure as the baseline for sensing procedure of Rel-16 NR-V2X. Enhancement of Rel-16 NR-V2X sensing procedure for advanced service can be achieved by decreasing time granularity of sensing

Proposal 5: RAN1 needs to consider compatibility with NR Uu and QoS requirement when determining resource granularity for sensing in NR-V2X

Proposal 6: Support to introduce PSFCH resource parameter indicator in SCI for other UEs to monitor HARQ feedback from receiving UE.

Proposal 7: Reserved PSSCH resource should not be utilized by other UEs if other UEs detect neither ACK nor NACK from receiving UE.

# References

- Chairman’s Notes, 3GPP RAN1#96

- Chairman’s Notes, 3GPP RAN1#96 bis

- Chairman’s Notes, 3GPP RAN1#97 bis

- R1-1907682, Feature lead summary for agenda item 7.2.4.5 Physical layer procedures for sidelink, LG