3GPP TSG-RAN1#98		                  		                              R1-1908935

Prague, Czech Republic, Aug. 26th – 30th, 2019

Source: 	Fraunhofer IIS, Fraunhofer HHI

Title:	Enhancements on Type-II CSI reporting

Agenda item:       7.2.8.1

Document for:	Decision

# Introduction

Based on the agreements made in RAN1#97[1], this contribution discusses the following issues

- p setting

- Configuration of α

- Details on Bitwidth and possible value(s) for Minit

- Reporting of M' and KNZ,TOT

In addition to the issues mentioned above, UCI omission and CBSR are also discussed.

# Discussion on remaining details

## p setting

Agreement Alt3C (illustrated in the table below) is supported where the parameter p=v0 for RI=3-4 is higher-layer configured in conjunction with the parameter p=y0 for RI=1-2. - The parameters (y0, v0)(y0,v0) take value from 12,14,14,14,14,18 RI; Layer; L; p. 1; 0; x0; y0. 2; 0. 1. 3; 0; v0. 1. 2. 4; 0. 1. 2. 3. - FFS: Possible down-selection on the FD combination parameters in RAN1#98

In this subsection, the setting of p is discussed for the above three options agreed in RAN1#97.

For the agreed L,p settings, the number of FD components per layer for RI>2 is identical, and given by v0, where the supported values of v0 are 12,14,18. For example, when N3=13, the number of FD components per layer is given by 7,4,2. Hence, setting a small value for v0, for example v0=18, would result in only 2 FD components per layer. Moreover, the feedback overhead for RI=3 and RI=4 is already strongly limited by the maximum number of non-zero coefficients 2K0 that can be reported by the UE. Further reducing the feedback overhead by configuring a small value for the number of FD components when RI>2 results in a significant performance loss.

Observation 1: Configuring a small value for v0, e.g., v0=18 results in a high-performance degradation.

Proposal 1: Support the two combinations 12,14 and 14,14 for (y0,v0).

## Configuration of α

Agreement In RAN1#98, finalize the values of based on the following aspects - Candidate values for to be down selected/evaluated: at least {1.5, 2, 2.5} - The set of values is to be finalized via offline email discussion prior to RAN1#98 - Configuration of : - Whether it is independent of other FD compression parameters, or dependent on at least one of the other FD compression parameters, i.e. p (=y0, and/or v0 for RI=3-4), L, β, and/or R - Whether is rank-specific or rank-common - Note: This is to be discussed along with the supported parameter combinations for (L, p, β, ) L,p,β,α

In this subsection, the dependency of the parameter α on the number of SD components L and transmission rank is discussed.

The impulse response of the channel associated with an SD beam is associated with only one or few channel cluster(s) of the radio channel, where each channel cluster is characterized by few delays. Therefore, when increasing the number of SD beams, the corresponding delay spread of the beam-formed channel impulse responses associated with all SD beams becomes larger, thus requiring a larger window size (N3') for covering the FD components of the precoder. A similar dependency can be observed with respect to the transmission rank. The higher the transmission rank, the larger the window size (N3') required for covering the FD components of the precoder, see also [2] for the dependency of the window sizes on the RI value.

Observation 2: The parameter α should increase with increasing values of L and RI.

Proposal 2: Support the configuration of parameter α

- to be dependent on the transmission rank (RI), e.g., α=1.5 for RI=1,2, and α=2 for RI={3,4}.

- to be dependent on the number of SD beams L, e.g., α=1.5 for L=2, and α=2 for L=4.

## Details on Bitwidth and possible values for Minit

Agreement On SCI (RI>1) and FD basis subset selection indicator, support Alt B described in the following table. - FFS: details on bit width and possible values for Minitial Minitial reporting in UCI part 2 - FFS: whether the possible value(s) for Minitial Minitial can depend on configured FD compression parameters – - Up to the editor to capture this agreement Alt B. SCI for RI>1; Alt3.4: Per-layer SCI, where SCIi SCIiis a –bit (i=0,1,…(RI - 1)). The location (index) of the strongest LC coefficient for layer i before index remapping is (li*,mi*), SCIi=li*, and mi* is not reported. Index remapping; For layer i, the index mimi of each nonzero LC coefficient cli,mi is remapped with respect to mi* to mi such that mi*=0. The FD basis index kmiassociated to each nonzero LC coefficient cli,mi is remapped with respect to kmi* to kmi such that kmi*=0. The sets cli,mi≠cli*,0 and kmi≠0 are reported. Informative note (for the purpose of reference procedure): The index li,mi of nonzero LC coefficients is remapped as (li,mi)→(li,mi-mi*modMi). The codebook index associated with nonzero LC coefficient index li,mi is remapped as kmi→kmi-kmi*modN3.. Combinatorial indicator for N3 ≤ 19N3≤19; log2N3-1Mi-1bits. Combinatorial indicator for N3>19N3 > 19; log2N3'-1Mi-1 bits. Minitial; Reported in UCI part 2, details on bitwidth and possible values are FFS.

In the following, the value range of Minitial for different configurations of N3 and N3' is discussed.

It was agreed in RAN#96 that for N3>19, a log2N3'-1Mi-1 bit indicator is used to indicate the selected FD basis subset per layer. As the window size is given by N3' and a cyclic shift is performed by the UE such that the FD basis index 0 is associated with the strongest coefficient, the FD index 0 must be contained in the N3'-sized window selected by the UE. Only then, Mi-1 FD indices can be selected from N3'-1 indices and the selected FD basis subset can be represented by a log2N3'-1Mi-1 bit indicator. Therefore, when selecting the N3'-sized window, Minit needs to be selected in such a way that the N3'-sized window contains the FD index 0. When doing so, the value range of Minit is represented by N3' values:

|  | modN3-N3'+n+1, N3, n∈0, N3'-1. | (1) |
| --- | --- | --- |

Therefore, a log2N3'-bit indicator is sufficient for the reporting of Minit.

Observation 3: Minit must be selected such that the FD index 0 is contained in the N3'-sized window; the value range of Minit is represented by N3' values, and a log2N3'-bit indicator is sufficient for the reporting of Minit.

Table 1: Performance gain comparison of Minit is fixed versus Minit is reported with respect to Rel.-15 Type-II CSI for different configurations of N3, N3' and M. For fixed Minit, Minit=N3- N3'2+1.

| N3=26 | α=1.5, N3'=6 |  | α=2, N3'=8 |  | α=2.5, N3'=10 |  |
| --- | --- | --- | --- | --- | --- | --- |
| M = 4 | Minit Fixed | Minit reported | Minit Fixed | Minit reported | Minit Fixed | Minit reported |
| Rank-1 | 6.18 | 6.3 | 6.26 | 6.34 | 6.28 | 6.35 |
| Rank-2 | 5.7 | 6 | 5.95 | 6.25 | 5.97 | 6.26 |
| Rank-3 | 5.48 | 5.9 | 5.79 | 6.1 | 5.86 | 6.12 |
| Rank-4 | 4.65 | 5.25 | 5.12 | 5.43 | 5.18 | 5.46 |
| N3=38 | α=1.5, N3'=8 |  | α=2, N3'=10 |  | α=2.5, N3'=13 |  |
| M = 5 | Minit Fixed | Minit reported | Minit Fixed | Minit reported | Minit Fixed | Minit reported |
| Rank-1 | 6.4 | 6.51 | 6.48 | 6.54 | 6.5 | 6.55 |
| Rank-2 | 5.35 | 5.68 | 5.6 | 5.9 | 5.64 | 5.92 |
| Rank-3 | 5.15 | 5.64 | 5.48 | 5.8 | 5.6 | 5.82 |
| Rank-4 | 4.33 | 4.95 | 4.72 | 5.13 | 4.9 | 5.15 |

Table 1 shows the performance gain obtained by the Rel.-16 precoder with respect to Type-II Rel.-15 precoder for different N3' values (parameterized by α={1.5, 2, 2.5}). The performance gain is shown for the two cases: (i) Minit is fixed and reported, and (ii) Minit is reported.

From Table 1, it can be observed that for α≤2, the performance loss of fixed Minit compared to reported Minit is only 0.12%, 0.3%, 0.5% and 0.6% for RI=1, RI=2, RI=3 and RI=4, respectively, for the two different configurations of N3 and M. Since the performance loss is very minor at low transmission ranks, Minit can be fixed and does not need to be reported for RI=1 and RI=2.

Moreover, when reporting Minit, we should also take into account the UE complexity required to calculate the optimal value of Minit. For larger values of N3', the UE has to optimize Minit over a large range of values, requiring a large amount of UE resources. Since the performance loss is very minor at low transmission ranks, and a large amount of UE complexity can be saved, Minit should be fixed at least for RI=1 and RI=2.

Observation 4: Negligible performance loss is observed for RI=1 and RI=2 when Minit is fixed.

Observation 5: For fixed Minit, the UE does not need to optimize Minit over a large range of values; hence a large amount of UE resources can be saved.

Furthermore, as seen from Table 1, when increasing the value of α, the performance loss for fixed Minit compared to reported Minit gradually decreases, irrespective of the RI value. For α>2, the performance loss for fixed Minit compared to reported Minit is only 0.07% for RI=1, and for RI={2,3,4}, the loss is less than 0.3%. As the performance loss is minor for all transmission ranks, there is no need to report the value of Minit when α>2.

Observation 6: No significant performance loss is observed for large values of N3' when fixing Minit.

Proposal 3: Consider fixing Minit for large values of N3' (α>2).

Fig. 1 shows the distribution of selected values of Minit when N3=26, N3'=8 and M=4 for RI=4. Although, the selection of Minit values are spread over the entire value range of Minit, i.e., n∈[0, N3'-1], the indices that lie in the centre of the value range are selected with higher probability than the remaining indices.

Figure 1: Distribution of selected values of Minit for N3=26, N3'=8 and M=4.

Observation 7: The indices that lie in the centre of the value range of Minit are selected with higher probability than the remaining indices.

Observation 8: For the values of Minit that are selected with high probability, the FD index 0 is centred in the N3'-sized window.

Therefore, when reporting Minit, the value range of Minit can be limited to the value range that is selected with high probability. Also, it is observed that for values of Minit that are selected with high probability, the N3'-sized window is centred with respect to FD index 0. Therefore, the value range of Minit can be reduced from N3' to d' values:

|  | Minit=N3- N3'2+q, | (2) |
| --- | --- | --- |

where q∈[-1, 0, 1, 2] is defined by d'=4 values. When selecting q from one of the set [-1, 0, 1, 2], the N3'-sized window is centred with respect to FD basis index 0. Irrespective of the values of N3 and N3', as d'=4, a 2-bit indicator is sufficient to report the selected value of Minit.

Proposal 4: To reduce UE complexity, Minit is fixed for RI=1 and RI=2, and for RI>2,

- Minit is selected from the value range modN3-N3'+n+1, N3, n∈0, N3'-1, and reported using a log2N3'-bit indicator, or

- Minit is selected from the limited value range N3- N3'2+q, where q∈[-1, 0, 1, 2], and reported using a 2-bit indicator.

## Reporting of M' and KNZ,TOT

Agreement For further details on the agreed UCI parameters in Table 1 of R1-1905629: - RI ({1,…, RIMAX}∈1,…,RIMAX) and KNZ,TOTKNZ,TOT (the total number of non-zero coefficients summed across all the layers, where KNZ,TOTKNZ,TOT {1,2,…, 2K0}KNZ,TOT∈1,2,…,2K0 are reported in UCI part 1 - FFS: If the total number of non-zero coefficients are jointly encoded with M’ (if supported) or independently encoded

In the following, we discuss the reporting of M' and the indication of the total number of non-zero combining coefficients across all layers KNZ,TOT in CSI part 1.

As discussed by some companies, for reporting M', the unused code-points used for the indication of the number of non-zero coefficients in CSI part 1 may be used. The total number of non-zero combining coefficients across all RI layers is given by KNZ,TOT. For example, when K0=42, 7 bits are used to indicate the number of non-zero combining coefficients, and out of the 128 code-points, 44 codepoints are unused and can be used for the indication of M'. To further increase the number of unused codepoints for the indication of M', the number of reporting states for KNZ,TOT can be reduced. As the amplitude and phase information of the strongest coefficient is not reported, the total number of non-zero coefficients can be reduced to KNZ,TOT-RI. By doing so, when M' and KNZ,TOT are jointly encoded, RI additional codepoints can be used in addition to the 44 unused codepoints to report the total number of non-zero coefficients.

Observation 9: The total number of non-zero coefficients KNZ,TOT across all RI layers for which the amplitude and phase information is reported in UCI part 2 can be given by KNZ,TOT-RI.

Proposal 5: Support joint encoding of KNZ,TOT and M' and reporting of only KNZ,TOT-RI instead of KNZ,TOT in CSI part 1.

## UCI omission

UCI omission for PUSCH-based resource allocation and CSI reporting was introduced in Rel.-15. It allows a UE to drop some parts of one or more CSI report(s) in the case that the PUSCH resource allocation is not sufficient to carry the entire content of the CSI report(s). In Rel.-15, the dropping is achieved by decomposing the CSI part 2 information associated with the CSI reports into smaller portions, the so-called priority levels, see Table 5.2.3-1 of [3], where priority level 0 has the highest priority, and NREP represents the total number of CSI reports configured to be carried on PUSCH. Each priority level is associated with a part of a CSI report. The UE drops the CSI portions with lower priority until the payload size of the CSI reports fits with the PUSCH allocation. Due to the similarities between the Rel.-15 Type II codebook and the new Rel.-16 codebook, the Rel.-15 priority reporting scheme shown in Fig. 2 can be used as a starting point for the discussions on the Rel.-16 UCI omission procedure.

Similar to Rel.-15, for UCI omission for the Rel.-15 codebook, a joint CSI subgroup that is associated with highest priority level 0 and contains CSI information of the 1:NREP CSI reports can be present in CSI part 2. Also, each CSI report can be partitioned into two CSI subgroups resulting in 2NREP CSI subgroups for NREP CSI reports. The 2NREP CSI subgroups are associated with the lower priority levels 1 to 2NREP, where the last CSI subgroup 2NREP is associated with the lowest priority.

Priority 0: Part 2 joint CSI subgroup for CSI report 1 to NREP. Priority 1: Part 2 CSI subgroup 1 for CSI report 1. Priority 2: Part 2 CSI subgroup 2 for CSI report 1. ⋮. Priority 2NREP-1: Part 2 CSI subgroup 1 for CSI report NREP. Priority 2NREP: Part 2 CSI subgroup 2 for CSI report NREP.

Figure 2: Priority level reporting where the joint CSI subgroup (priority level 0) contains CSI information of all NREP CSI reports.

Fig. 3 shows the information content of CSI part 2 of a CSI report for the Rel.-16 codebook. It contains the bitmaps, the SD and FD basis subset indicators, the SCIs, and the polarization reference amplitudes for the RI layers, the value of Minit (if reported), and the amplitude and phase information of the selected KNZ,TOT combining coefficients associated with the RI layers.

| CSI parameters: - The SD basis subset indicator and the rotation factors - The FD basis subset indicator for the RI layers - SCIs of the RI layers - The polarization reference amplitudes for the RI layers - Value of Minit (if reported) | Bitmaps of the RI layers | Amplitude and phase information of the KNZ,TOT combining coefficients |
| --- | --- | --- |
| Figure. 3: Contents of CSI part 2 of each CSI report. |  |  |

As mentioned by some companies, the joint CSI subgroup that has the highest priority may contain the CSI parameters and the bitmaps shown in Fig. 3 for the RI layers of the precoding matrix for the 1:NREP CSI reports. The remaining 2NREP CSI subgroups only contain the amplitude and phase information of the combining coefficients of the NREP CSI reports. However, such a segmentation of the CSI parameters, bitmaps, and combining coefficients has a serious drawback. In case of omission, if all CSI subgroups containing the combining coefficient information of one or more CSI reports are dropped, then the information contained in the joint CSI subgroup is not useful for the gNB to  reconstruct the precoder(s) associated with the dropped CSI report(s). Hence, there is no need that all the CSI parameters shown in Fig. 3 of the 1:NREP CSI reports are contained in the joint CSI subgroup.

Observation 10: The CSI parameters and bitmaps contained in the joint subgroup that are associated with the dropped CSI reports become redundant in case of omission.

Proposal 6: The joint CSI subgroup (associated with priority level 0) shall contain the SD basis indicators of the 1:NREP CSI reports.

The joint CSI subgroup in Rel.-15 contains in addition to the SD basis indicators and rotation factors the wideband amplitude information of the precoders. In the case that all CSI subgroups associated with one or CSI reports are dropped by the UE, the gNB can still use the wideband information contained in the joint CSI subgroup to perform wideband precoding based on the information of the SD basis indicators and wideband amplitudes. A similar concept can be applied for the Rel.-16 CSI reporting scheme. The gNB can derive a wideband precoding matrix when it has knowledge on the amplitude information of the combining coefficients and the fraction of the bitmap associated with the strongest FD basis index for each of the RI layers. In many cases, the strongest FD basis vector index corresponds to the FD basis index 0 (the index associated with the SCI). Therefore, based on the SCI, the bitmap associated with the FD basis index 0 and amplitude information of the combining coefficients, the gNB may reconstruct a wideband precoding matrix.

Observation 11: Including wideband information in the joint CSI subgroup is beneficial for the Rel.-16 codebook in terms of performance.

Proposal 7: In addition to the SD basis indicator and the rotation factors, the bitmaps and the amplitude information associated with the FD index 0 of all RI layers and SD basis indices of the NREP CSI reports can be included in the joint CSI subgroup of priority level 0.

Segmentation of combining coefficients

In case of omission, the gNB reconstructs the precoding matrix of a CSI report based on the remaining fraction of the combining coefficients contained in the non-dropped CSI subgroup. The performance of the corresponding precoder depends on the partitioning of the combining coefficients contained in a CSI report to the two CSI subgroups. The following discusses two partitioning schemes, where it is assumed that the bitmaps of all RI layers are contained in a single CSI subgroup and the combining coefficients are ordered in the same way as the ordering of the bitmaps.

| Ordering with respect to layer index Bitmap of RI layers – size 2LM×RI |  |  |  |
| --- | --- | --- | --- |
| Segment 1 of size 2LM associated with the first layer | Segment 2 of size 2LM associated with second layer | … | Segment R of size 2LM associated with RI-th layer |
| Figure 4: Segmentation of a bitmap into RI segments, each segment containing 2LM bits. |  |  |  |

In scheme 1, the bitmaps of the RI layers are segmented into RI segments, each of size 2LM, as shown in Fig. 4.  Each segment is associated with all SD and FD basis indices of a single layer. As the combining coefficients are ordered in the same way as bitmaps, the fraction of combining coefficients contained in first CSI subgroup that has a higher priority than the second CSI subgroup is associated with all SD and FD indices of the lower layers. In case of omission, the UE drops at first the CSI subgroup that contains the combining coefficients associated with the higher layers. This kind of dropping reduces the transmission rank of the precoder which may result in significant loss of performance.

| Ordering with respect to FD basis index Bitmap of RI layers of size 2LM×RI |  |  |  |  |
| --- | --- | --- | --- | --- |
| Segment 1 of size 2L×RI associated with the first FD index | … | Segment m of size 2L×RI associated with m-th FD index | … | Segment M of size 2L×RI associated with M-th FD index |
| Figure 5: Segmentation of bitmap into M segments, each segment containing 2L×RI bits. |  |  |  |  |

In scheme 2, the bitmaps of the RI layers are segmented into M segments, each of size 2L×RI, as shown in Fig 5. Each segment is associated with all SD indices of all RI layers and with a single FD basis index. In case of omission, the UE drops the combining coefficients associated with one or more FD basis indices of all RI layers. The gNB reconstructs the precoder based on the non-dropped combining coefficients of all RI layers of some FD basis indices. An advantage of scheme 2 over scheme 1 is that the transmission rank of the precoding matrix remains the same after dropping a fraction of the combining coefficients. It is therefore expected that scheme 2 achieves a better performance than scheme 1.

Observation 12: Segmentation of combining coefficients for the RI layers with respect to FD basis indices is preferred over segmentation of combining coefficients with respect to layer indices.

Proposal 8: Support segmentation of bitmaps and combining coefficients for the RI layers with respect to FD basis indices.

Two further partitioning schemes of each segment from Fig. 5 with respect to layer indices and SD basis indices are shown in Fig. 6 and Fig. 7, respectively. In Fig. 6, each segment is further partitioned into 2L sub-segments, each sub-segment containing RI bits and is associated with an SD basis index and FD basis index of all RI layers. In Fig. 7, each segment is further partitioned into RI sub-segments, where each sub-segment contains 2L bits and is associated with all SD indices, a single FD index and a single layer.

| Ordering of a segment with respect to SD basis index Segment m of size 2L×RI |  |  |
| --- | --- | --- |
| RI bits associated with the first SD basis index | … | RI bits associated with the 2L-th SD basis index |
| Figure 6: Segmentation of 2L×RI bits into 2L sub-segments, each containing RI bits. |  |  |

| Ordering of a segment with respect to layer index Segment m of size 2L×RI |  |  |
| --- | --- | --- |
| 2L bits associated with the first layer | … | 2L bits associated with the RI-th layer |
| Figure 7: Segmentation of 2L×RI bits into RI sub-segments, each containing 2L bits. |  |  |

As a dropping of non-zero combining coefficients associated with an SD index results in a higher performance loss than a dropping of non-zero combining coefficients associated with a layer index, the sub-segmentation shown in Fig. 7 is preferred over the sub-segmentation shown in Fig. 6.

Proposal 9: Support further partitioning of bitmaps and combining coefficients of a segment with respect to layer indices, as shown in Fig. 7.

Partitioning of bitmaps and combining coefficients and distribution to CSI subgroups

When the combining coefficients contained in a CSI subgroup for the above segmentation scheme 2 (see Fig. 5) are dropped, the associated bitmaps can be dropped as well as they are not required by the gNB to interpret the remaining non-dropped combining coefficients. Therefore, the combining coefficients and bitmaps should be grouped together and distributed to the different CSI subgroups.

Observation 13: Including the bitmaps of all RI layers in a single CSI subgroup is not efficient.

For example, the bitmaps and combining coefficients of all RI layers of a CSI report can be segmented into M segments with respect to an increasing FD basis index. Each segment contains the sub-bitmap and the combining coefficients which are associated with all SD indices of all RI layers and a single FD basis index. Based on the bit-widths of the first and second CSI subgroups, the M segments are distributed to the two CSI subgroups, as shown in Fig. 8. The first CSI subgroup contains k1 segments of bitmaps and combining coefficients and the second CSI subgroup contains k2 segments of bitmaps and combining coefficients, where k1+k2=M.

Proposal 10: Partition the bitmaps and combining coefficients for the RI layers into M segments with respect to FD basis indices and distribute them to the two CSI subgroups.

| Bitmaps and amplitude and phase information of the combining coefficients |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Segment 1 associated with the first FD index |  | … |  | Segment m associated with the m-th FD index |  | … |  | Segment M associated with the last FD index |  |  |
| Sub-bitmap 1 containing 2L×RI bits | Amplitude and phase information associated with all non-zero combining coefficients of the sub-bitmap 1 | … | … | Sub-bitmap m containing 2L×RI bits | Amplitude and phase information associated with all non-zero combining coefficients of the sub-bitmap m | … | … | Sub-bitmap M containing 2L×RI bits | Amplitude and phase information associated with all non-zero combining coefficients of the sub-bitmap M |
| First CSI subgroup (k1 segments) Second CSI subgroup (k2 segments) |  |  |  |  |  |  |  |  |  |
| Figure 8: Segmentation of bitmaps and combining coefficients with respect to increasing FD index. |  |  |  |  |  |  |  |  |  |

The last segment of the first CSI subgroup contains a portion of the sub-bitmap, or it contains the complete sub-bitmap and only a portion of the amplitude and phase information of the combining coefficients associated with the sub-bitmap. Note when the last segment contains no information of the combining coefficients, the gNB has to drop the last segment which would result in a waste of resources. To overcome this problem, each segment shown in Fig. 8 can be further partitioned into RI sub-segments, as shown in Fig. 9. Each sub-segment contains the sub-bitmap of length 2L bits that is associated with all SD components, followed by the amplitude and phase information associated with that sub-bitmap.

| Segment m associated with FD index the m |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sub-segment m1 |  | … | Sub-segment mr |  | … | Sub-segment mRI |  |
| Sub-bitmap m1 containing 2L bits | Amplitude and phase information associated with sub-bitmap m1 | … | Sub-bitmap mr containing 2L bits | Amplitude and phase information associated with sub-bitmap mr | … | Sub-bitmap mRI containing 2L bits | Amplitude and phase information associated with sub-bitmap mRI |
| First Portion Second Portion |  |  |  |  |  |  |  |
| Figure 9: Segmentation of m-th segment into mRI sub-segments with respect to the layer index. |  |  |  |  |  |  |  |

Moreover, each sub-segment may be further portioned into 2L sub-segments, as shown in Fig. 10. Bit ‘mr,l’ is followed by the amplitude and phase information if mr,l is ‘1’. If mr,l is ‘0’, bit ‘mr,l+1’ immediately follows bit mr,l. By doing so, the last portion of the first CSI subgroup contains only a single bit associated with sub-segment mr,l or it contains only a fraction of the amplitude and phase information of a single combining coefficient.

| Sub-segment mr |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit mr,1 | Amplitude and phase information of a single combining coefficient | … | Bit mr,l | Amplitude and phase information of a single combining coefficient | … | Bit mr,2L | Amplitude and phase information of a single combining coefficient |
| Figure 10: Segmentation of sub-segment mr into 2L sub-segments. |  |  |  |  |  |  |  |

Observation 14: For a fixed CSI subgroup size, the ordering scheme shown in Fig. 8 - Fig. 10 provides a higher packing density of the amplitude and phase information of the combining coefficients in the first CSI subgroup compared to other ordering/segmentation schemes.

For the proposed segmentation scheme, the ordering/segmenting of the bitmaps and the combining coefficients is performed with respect to an increasing FD index first, then increasing layer index, and then increasing beam index. Such an ordering is beneficial as it does not reduce the transmission rank of the precoder in case of omitting a CSI subgroup (since the combining coefficients of the non-dropped CSI subgroup are associated with all RI layers, all SD basis indices and some of the M FD basis indices).

Proposal 11: Support the following order of segmentation of bitmaps and combining coefficients:

- Segmentation of bitmaps and combining coefficients for the RI layers of a CSI report into M segments with respect to an increasing FD basis index,

Further segmentation of each segment into RI sub-segments with respect to an increasing layer index,

- Further segmentation of each sub-segment with respect to an increasing beam index.

Number of CSI subgroups of a CSI report

In some cases, there is no need to segment the CSI part 2 information of a CSI report into two CSI subgroups. For example, when the number of non-zero coefficients KNZ,TOT is given by a very small value, it is very likely that the UE drops the combining coefficients of the complete CSI report, and hence a grouping of the KNZ,TOT combining coefficients to different CSI subgroups seems to be unnecessary. Therefore, the number of CSI subgroups can be made dependent on the total number of non-zero combining coefficients KNZ,TOT to be reported.

Proposal 12: Consider partitioning the CSI report into two subgroups only when KNZ,TOT is larger than a threshold value.

## Codebook subset restriction

Codebook subset restriction (CBSR) was agreed in RAN1#95 to be supported for the Rel.-16 codebook. As Rel.-16 precoding is performed in both spatial and delay dimensions using DFT codebooks, CBSR could be applied in both dimensions. For the CBSR in the spatial dimension, the Rel.-15 CBSR mechanism can be re-used by restricting the power of a beam. However, as unlike to Rel.-15 there is no concept of wideband amplitude in the Rel.-16 codebook, the sum power over all non-zero combining coefficients associated with a spatial beam can be considered for the restriction. It was also discussed by several companies to apply CBSR for the Rel.-16 codebook in the delay dimension. However, the performance benefits using such a restriction are not clear.

Observation 15: CBSR can be based on a power restriction of the spatial beams.

Proposal 13: Re-use the Rel.-15 CBSR framework for the spatial beams by restricting the sum power of all non-zero combining coefficients associated with a spatial beam.

# Conclusions

Based on the above discussion, we have the following observations and proposals.

Observation 1: Configuring a small value for v0, e.g., v0=18 results in a high-performance degradation.

Proposal 1: Support the two combinations 12,14 and 14,14 for (y0,v0).

Observation 2: The parameter α should increase with increasing values of L and RI.

Proposal 2: Support the configuration of parameter α

- to be dependent on the transmission rank (RI), e.g., α=1.5 for RI=1,2, and α=2 for RI={3,4}.

- to be dependent on the number of SD beams L, e.g., α=1.5 for L=2, and α=2 for L=4.

Observation 3: Minit must be selected such that the FD index 0 is contained in the N3'-sized window; the value range of Minit is represented by N3' values, and a log2N3'-bit indicator is sufficient for the reporting of Minit.

Observation 4: Negligible performance loss is observed for RI=1 and RI=2 when Minit is fixed.

Observation 5: For fixed Minit, the UE does not need to optimize Minit over a large range of values; hence a large amount of UE resources can be saved.

Observation 6: No significant performance loss is observed for large values of N3' when fixing Minit.

Proposal 3: Consider fixing Minit for large values of N3' (α>2).

Observation 7: The indices that lie in the centre of the value range of Minit are selected with higher probability than the remaining indices.

Observation 8: For the values of Minit that are selected with high probability, the FD index 0 is centred in the N3'-sized window.

Proposal 4: To reduce UE complexity, Minit is fixed for RI=1 and RI=2, and for RI>2,

- Minit is selected from the value range modN3-N3'+n+1, N3, n∈0, N3'-1, and reported using a log2N3'-bit indicator, or

- Minit is selected from the limited value range N3- N3'2+q, where q∈[-1, 0, 1, 2], and reported using a 2-bit indicator.

Observation 9: The total number of non-zero coefficients KNZ,TOT across all RI layers for which the amplitude and phase information is reported in UCI part 2 can be given by KNZ,TOT-RI.

Proposal 5: Support joint encoding of KNZ,TOT and M' and reporting of only KNZ,TOT-RI instead of KNZ,TOT in CSI part 1.

Observation 10: The CSI parameters and bitmaps contained in the joint subgroup that are associated with the dropped CSI reports become redundant in case of omission.

Proposal 6: The joint CSI subgroup (associated with priority level 0) shall contain the SD basis indicators of the 1:NREP CSI reports.

Observation 11: Including wideband information in the joint CSI subgroup is beneficial for the Rel.-16 codebook in terms of performance.

Proposal 7: In addition to the SD basis indicator and the rotation factors, the bitmaps and the amplitude information associated with the FD index 0 of all RI layers and SD basis indices of the NREP CSI reports can be included in the joint CSI subgroup of priority level 0.

Observation 12: Segmentation of combining coefficients for the RI layers with respect to FD basis indices is preferred over segmentation of combining coefficients with respect to layer indices.

Proposal 8: Support segmentation of bitmaps and combining coefficients for the RI layers with respect to FD basis indices.

Proposal 9: Support further partitioning of bitmaps and combining coefficients of a segment with respect to layer indices, as shown in Fig. 7.

Observation 13: Including the bitmaps of all RI layers in a single CSI subgroup is not efficient.

Proposal 10: Partition the bitmaps and combining coefficients for the RI layers into M segments with respect to FD basis indices and distribute them to the two CSI subgroups.

Observation 14: For a fixed CSI subgroup size, the ordering scheme shown in Fig. 8 - Fig. 10 provides a higher packing density of the amplitude and phase information of the combining coefficients in the first CSI subgroup compared to other ordering/segmentation schemes.

Proposal 11: Support the following order of segmentation of bitmaps and combining coefficients:

- Segmentation of bitmaps and combining coefficients for the RI layers of a CSI report into M segments with respect to an increasing FD basis index,

Further segmentation of each segment into RI sub-segments with respect to an increasing layer index,

- Further segmentation of each sub-segment with respect to an increasing beam index.

Proposal 12: Consider partitioning the CSI report into two subgroups only when KNZ,TOT is larger than a threshold value.

Observation 15: CBSR can be based on a power restriction of the spatial beams.

Proposal 13: Re-use the Rel.-15 CBSR framework for the spatial beams by restricting the sum power of all non-zero combining coefficients associated with a spatial beam.

# References

Chairman’s Notes, RAN1#97, Reno, USA, May 13th – May 17th, 2019.

R1-1907053, Fraunhofer IIS, Fraunhofer HHI, “Enhancements on Type-II CSI reporting,” Reno, USA, May 13th – 17th, 2019.

3GPP TS 38.214 V15.3.0: “3GPP; TSG RAN; NR; Physical layer procedures for data (Rel.-15),” Sept 2018.

- Table 2: Simulation parameters and setup

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Macro Dense Urban |
| Frequency Range |  | 4GHz (FR1) |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS /275 for 30 kHz SCS |
| Simulation bandwidth |  | 10 MHz,15kHz SCS; 100 MHz, 30 kHz SCS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation |
| MIMO layers |  | Up to 2 layers |
| Overhead |  | DMRS, CSI-RS, PDCCH |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Evaluation Metric |  | Throughput vs CSI feedback overhead (bits) |
| Baseline for performance evaluation |  | Rel-15 Type II Codebook |