3GPP TSG RAN WG1 #98		  R1-1908960

Prague, CZ, August 26th – 30th, 2019

Agenda Item:     7.2.8.4

Source:	Spreadtrum Communications

Title:	Discussion on full TX power for UL transmission

Document for:	Discussion and decision

Introduction

In RAN1 #1901, three different capabilities were agreed to support full TX power UL transmission at least for codebook based UL transmissions for non-coherent and partial/non-coherent capable UEs for power class 3  [1].

Agreements Full TX power UL with multiple power amplifier is supported at least for codebook based UL transmission for non-coherent and partial/non-coherent capable UEs. The support of this feature is indicated by the UE as part of UE capability signalling. For power class 3: - UE capability 1: for the UE to support full Tx power in UL transmission, full rated PAs on each Tx chain is supported with a new UE capability - FFS: detailed power scaling description - Note: Full Tx power means UE delivers total power of 23dBm for PC3 - UE capability 2: for the UE to support full Tx power in UL transmission, no Tx chain is assumed to deliver full power with the new UE capability - FFS: detailed design - UE capability 3: for the UE to support full Tx power in UL transmission, subset of Tx chains with full rated PAs is supported with a new UE capability FFS: Whether all three capabilities will be specified or a subset will be specified FFS: UE capability signalling/reporting details Note: Two or more of the above capabilities could be merged depending on the further details.

In RAN1#96b, the following agreements was achieved [2]

Agreement

Regardless of UE capability 1, 2, or 3, signalling of “UL full power tx capability” is supported for UEs with full power uplink transmission capability

- FFS: For UE capability 1, if any other information is necessary

- For UE capability 2 and UE capability 3, in addition to signalling “UL full power tx capability”, further information on UE capability are signalled if needed

  - FFS: Details such as support of UE capability signaling of supported one or group of TPMI precoder(s) for full power transmission, support different number of SRS ports for resources for codebook, and other UE capability signaling can be introduced

  - FFS: Whether full uplink TX power capability can be explicitly/implicitly derived from the TPMI/TPMI group precoders for full power transmission

UEs with full power uplink transmissions are those Rel-16 UEs which can transmit at full power at least for rank1

The signalling of above information does not imply any specific UE PA architecture implementation.

In RAN1#97, a positive progress about the schemes for UL full power TX for UE capability 2 and UE capability 3 has been achieved [3]:

Working Assumption

Support following scheme for UL full power Tx for UE capability 2 and 3:

- A UE can be configured for one of two modes of full power operation to support ‘Capability 2’ and ‘Capability 3’ subject to UE capability

- A UE can be configured by the network to support full power transmission

- Mode 1: The UE can be configured with one or more SRS resources with same number of SRS ports (according to Rel-15) within an SRS resource set which usage is set to ‘codebook’

  - gNB can configure the UE to use a subset of TPMIs that combine ports in a layer to produce full power transmission.

  - A new codebookSubset is introduced only for the rank value(s) where full power transmission in UL is not achievable includes the TPMI precoders in fullyAndPartialAndNonCoherent defined in Rel-15

    - FFS: At least a subset of the non-antenna selection TPMI precoder(s) is(are) supported

    - FFS: Additional support of antenna selection TPMI precoders

  - Note: as non-coherent UE, it is not capable of maintaining relative phase of antenna ports according to TPMI

- Mode 2: The UE can be configured with one SRS resource or multiple SRS resources with different number of SRS ports within a SRS resource set which usage is set to ‘codebook’

  - UE transmits SRS and PUSCH in same manner, whether antenna virtualization is used or not

  - Rel-15 codebooks and codebook subsets are used

    - Note: Antenna selection precoder can be used to enable full power related PA(s) to produce full power transmission for Capability-3 UE.

  - UL full power Tx is achieved for PUSCH transmission according to indicated SRI and/or TPMI

    - A set of TPMIs that deliver full power can be signalled by the UE in order to support at least  UEcap3, for SRS resource with more than 1 ports,

    - e.g. For SRI indicating SRS resource with 1 port then single layer PUSCH is transmitted with full power in same manner as single port SRS, if SRI indicating SRS resource with multiple ports is signalled based on Rel-15 MIMO behaviour (transmission rank indicator, TPMI indicator, etc) except the power scaling aspects

    - The following cases are not precluded

      - For example, for 4TX on UE side (with 20+20+17+17dBm) virtualized as 2 SRS ports, full uplink power transmission can be enabled by precoder [1 0] or [0 1]

    - FFS: number of SRS resources supported

      - 2

      - 3

    - FFS: for 4 Tx, how many different TPMIs/TPMI groups support full power

  - FFS: any rules for spatial filter update for the SRS resources with different number ports

Note: How to capture the behaviour for ‘Mode 1’ and ‘Mode 2’ in specifications is TBD

Note: For single port, there is no SRI and TPMI

Note: Support of Mode 1, Mode 2 have separate UE capability

In this contribution, we present our opinions on UE capability signaling and power scaling scheme for UE capability 1, 2 and 3. In addition, some discussions on full coherent UE for full power transmission are also provided.

Discussion

- For UE capability 2 and 3

Fortunately, after great effort, there is one working assumption on schemes for UL full power Tx for UE capability 2 and 3. Considering different benefits and drawbacks of mode 1and mode 2, e.g., for mode 1 new codebook subset would be introduced, while SRS overhead would be increased for mode 2, we think the WA should be supported for UE capability 2 and 3 depending on UE capability.

Proposal 1: Confirm the WA from last meeting for capability 2 and 3.

For Mode 1:

For mode 1, at least a subset of the non-antenna selection TPMI precoder(s) should be supported for UE capability 2 to combine ports in a layer to produce full power transmission. On the other hand, indeed, the schemes for UE capability 2 could also be applied for capability 3 with subset of Tx chains with full rated PAs. However, it would result in some performance loss, e.g., for blocking case, or for the inaccurate channel estimation caused by introduced delay factor. Thus, antenna selection TPMI precoder should also be supported at least for UE capability 3.

Proposal 2: For mode 1, the new codebookSubset could include both non-antenna selection and antenna selection TPMI precoder.

Similar to mode 2, a set of TPMIs from the new codebookSubset can be signaled by the UE in order to support at least UE capability 3, for SRS resource with more than 1 port. UL full power TX can be achieved for PUSCH transmission according to indicated SRI and/or TPMI.

Proposal 3: For mode 1, UL full power Tx can be achieved for PUSCH transmission according to indicated SRI and/or TPMI

- A set of TPMIs that deliver full power can be signalled by the UE in order to support at least  UE cap3, for SRS resource with more than 1 ports,

For UE with 2Tx, at least two TPMIs 1210,1201  should be supported for full power transmission. For UE capability 3, UE could report to support 1210or 1201 to inform gNB which port could achieve full power transmission. For capability 2, UE could not report any precoder for full power transmission, then gNB could schedule precoder 1211 to achieve full power transmission.

For UE with 4Tx, UE could also report TPMI to achieve the full power transmission. For example, for the non-coherent transmission capable UE with [20dBm, 17dBm, 20dBm, 17dBm] PAs, instead of supporting 121 0 0 0T for rank 1 transmission, 121 0 1 0T should be selected. Similar to 2 TX, if UE could not report any precoder for full power transmission, then gNB could schedule precoder121 1 1-1T  for rank =1, 121010010-jfor rank =2, and 12101001000001 for rank =3, to achieve full power transmission. Table 1 provides the specific TPMI precoders signaled by UE for different number of Tx and rank to support full power transmission for mode1.

Table 1. TPMI/TPMI group signaled by UE to support full power transmission for mode 1

|  | Rank-1 | Rank-2 | Rank-3 |
| --- | --- | --- | --- |
| TPMI/TPMI group for nonCoherent UE with 2Tx | 1210,1201 | - | - |
| TPMI/TPMI group for nonCoherent or partialAndNonCoherent UE with 4Tx | 121000,120100,120010,120001 121010,1210-10,1210j0,1210-j0 120101,12010-1,12010j,12010-j | 1210000100, 12100000101210000001 1201000010,1201000001,1200100001 | 12100001000010, |

Proposal 4: For mode1, TPMI/TPMI group signaled by UE to support full power transmission is as shown in Table 1.

Regarding to power scaling scheme for mode 1 to support full power transmission, based on the indicated SRI/TPMI and UE TPMI signaling reporting, we have the following proposal:

Proposal 5: For mode 1, if the indicated  SRI/TPMI is the TPMI from UE capability reporting to support full power transmission, then the power scaling factor is always 1, otherwise power scaling factor is equal to the ratio of number of non-zero PUSCH port to the number of SRS port indicated by SRI.

For Mode 2:

Compared with mode 1, mode 2 supports virtualization by configuring SRS resources with different SRS ports in a SRS resource set. For example, if a UE with non-full rated PA structure of [17dBm, 17 dBm, 17 dBm, 17 dBm], in order to support full power transmission for each rank, the UE can be configured with three SRS resources in a set: one SRS resource with one port, one SRS resource with two ports, one SRS resource with four ports. For rank = 1, four Tx chains could be virtualized into one port to support full power transmission by transmitting one 1-port SRS resource. For rank = 2, four Tx chains could be virtualized into two ports to support full power transmission by transmitting one 2-port SRS resource.

For UE with 2Tx, at least two TPMIs 1210,1201  should be supported for full power transmission. For UE capability 3, UE could report to support 1210or 1201 to inform gNB which port could achieve full power transmission. For capability 2, UE could not report any precoder for full power transmission, then gNB could schedule 1-port SRS resource to achieve full power transmission.

For UE with 4Tx, other than TPMI/TPMI group in mode1, we should also consider the TPMIs supported by virtualization. For example, if a UE with non-full rated PA structure of [20dBm,  20dBm, 17 dBm, 17 dBm], then UE could report precoder 1210 to achieve full power transmission for rank =1. Table 2 provides the specific TPMI precoders signaled by UE for different number of Tx and rank to support full power transmission for mode2.

Table 2. Signaled TPMI/TPMI group to support full power transmission for mode 2

|  | Rank-1 | Rank-2 | Rank-3 |
| --- | --- | --- | --- |
| TPMI/TPMI group for nonCoherent UE with 2Tx | 1210,1201 | - | - |
| TPMI/TPMI group of nonCoherent UE with 4Tx | 1210,1201, 121000,120100,120010,120001 | 121001, 1210000100, 1210000010,1210000001 1201000010,1201000001,1200100001 | 12100001000010 |
| TPMI/TPMI group of partialAndNonCoherent UE with 4Tx | 1210,1201, 121000,120100,120010,120001 121010,1210-10,1210j0,1210-j0 120101,12010-1,12010j,12010-j | 121001, 1210000100, 12100000101210000001, 1201000010,1201000001,1200100001 | 12100001000010 |

Proposal 6: TPMI/TPMI group signaled by UE to support full power transmission for mode 2 is as shown in Table 2

Regarding to power scaling scheme for mode 2 to support full power transmission, based on the indicated SRI/TPMI and UE TPMI signaling reporting, we have the following proposal:

Proposal 7: For mode 2, if the indicated  SRI/TPMI is the TPMI from UE capability reporting to support full power transmission, then the power scaling factor is always 1, otherwise power scaling factor is equal to the ratio of number of non-zero PUSCH port to the number of SRS port indicated by SRI.

- For UE capability 1:

In RAN1 #96b, it was agreed that signalling of  “UL full power tx capability” is supported for UEs with full power uplink transmission capacity. For UE capability 1 where full rated PAs on each Tx chain is supported, in our opinion, no additional signaling information is needed. The power scaling factor could be always 1 no matter what the indicated SRI/TPMI is.

Proposal 8: For UE capability 1, no additional information other than signalling of “UL full power tx capability” is needed.

Proposal 9: For UE capability 1, the power scaling factor could be always 1 configured by RRC.

- For full-coherent UE

During previous RAN1 meetings, there are some discussions on whether full TX power UL transmissions should also be supported by full-coherent UE. In general, the function of UL power control could also be used for interference management other than coverage enhancement. In Rel-15, full-coherent UEs could support full power transmission for some TPMIs with no zero PUSCH ports. If gNB wants full TX power UL transmission for full-coherent UEs, it could always schedule some TPMIs. There is no need to introduce additional capability signaling for full-coherent UEs to support full power UL transmission.

Proposal 10: Not need to extend Rel-16 UL full power transmission schemes specified for non-coherent and partial/non-coherent UE to full-coherent UE.

Conclusion

In this contribution, we provide our opinions on UE capability signaling including TPMI/TPMI group consideration, and power scaling schemes. Furthermore, full power transmission for full coherent UE is also discussed. Based on the discussions, we have the following proposals:

Proposal 1: Confirm the WA from last meeting for capability 2 and 3.

Proposal 2: For mode 1, the new codebookSubset could include both non-antenna selection and antenna selection TPMI precoder.

Proposal 3: For mode 1, UL full power Tx can be achieved for PUSCH transmission according to indicated SRI and/or TPMI

- A set of TPMIs that deliver full power can be signalled by the UE in order to support at least  UE cap3, for SRS resource with more than 1 ports,

Proposal 4: For mode1, TPMI/TPMI group signaled by UE to support full power transmission is as shown in Table 1.

Table 1. TPMI/TPMI group signalled by UE to support full power transmission for mode 1

|  | Rank-1 | Rank-2 | Rank-3 |
| --- | --- | --- | --- |
| TPMI/TPMI group for nonCoherent UE with 2Tx | 1210,1201 | - | - |
| TPMI/TPMI group for nonCoherent or partialAndNonCoherent UE with 4Tx | 121000,120100,120010,120001 121010,1210-10,1210j0,1210-j0 120101,12010-1,12010j,12010-j | 1210000100, 12100000101210000001 1201000010,1201000001,1200100001 | 12100001000010, |

Proposal 5: For mode 1, if the indicated  SRI/TPMI is the TPMI from UE capability reporting to support full power transmission, then the power scaling factor is always 1, otherwise power scaling factor is equal to the ratio of number of non-zero PUSCH port to the number of SRS port indicated by SRI.

Proposal 6: TPMI/TPMI group signaled by UE to support full power transmission for mode 2 is as shown in Table 2.

Table 2. Signaled TPMI/TPMI group to support full power transmission for mode 2

|  | Rank-1 | Rank-2 | Rank-3 |
| --- | --- | --- | --- |
| TPMI/TPMI group for nonCoherent UE with 2Tx | 1210,1201 | - | - |
| TPMI/TPMI group of nonCoherent UE with 4Tx | 1210,1201, 121000,120100,120010,120001 | 121001, 1210000100, 1210000010,1210000001 1201000010,1201000001,1200100001 | 12100001000010 |
| TPMI/TPMI group of partialAndNonCoherent UE with 4Tx | 1210,1201, 121000,120100,120010,120001 121010,1210-10,1210j0,1210-j0 120101,12010-1,12010j,12010-j | 121001, 1210000100, 12100000101210000001, 1201000010,1201000001,1200100001 | 12100001000010 |

Proposal 7: For mode 2, if the indicated  SRI/TPMI is the TPMI from UE capability reporting to support full power transmission, then the power scaling factor is always 1, otherwise power scaling factor is equal to the ratio of number of non-zero PUSCH port to the number of SRS port indicated by SRI.

Proposal 8: For UE capability 1, no additional information other than signalling of “UL full power tx capability” is needed.

Proposal 9: For UE capability 1, the power scaling factor could be always 1 configured by RRC.

Proposal 10: Not need to extend Rel-16 UL full power transmission schemes specified for non-coherent and partial/non-coherent UE to full-coherent UE.

Reference

- “RAN1 Chairman's Notes”, RAN1 #AH1901, Taipei, Taiwan, January 21st-25th, 2019.

- “RAN1 Chairman's Notes”, RAN1 #96b, Xi’an, China, April 8th – 12th, 2019.

- “RAN1 Chairman's Notes”, RAN1 #97, Reno, USA, May 13th – 17th, 2019.