3GPP TSG RAN WG1 Meeting #98                              	            R1-1908963

Prague, Czech Rep, 26th – 30th August 2019

Agenda Item:	7.2.2.1.1

Source:	Spreadtrum Communications

Title:	Discussion on DRS in NR-U

Document for:	Discussion and decision

Introduction

DRS in NR-U was defined including the following signals and channels in RAN1#94 [1]:

- SS/PBCH burst set,

- RMSI CORESET(s)+PDSCH(s) associated with SSB/PBCH block(s), and

- CSI-RS.

Agreement: - Inclusion of the CSI-RS and RMSI-CORESET(s)+PDSCH(s) (carrying RMSI) associated with SS/PBCH block(s) in addition to the SS/PBCH burst set in one contiguous burst (tentatively referred to as the NR-U DRS) can be beneficial for - Meeting OCB requirement - Compacting signals in time domain to limit the required number of channel access and for short channel occupancy - Support of stand-alone NR-U deployments - Support of automatic neighbour relations (ANR) functionality in an NR-U deployment - Resolution of PCI confusion in an NR-U deployment - Note: The NR-U DRS (it can be called something else in the future) can include signals and channels that are required for cell acquisition etc. and is not limited only to reference signals - The transmission of additional signals such as OSI and paging within the NR-U DRS is allowed and can be beneficial - Note: This does not imply that RMSI-CORESET+PDSCH and CSI-RS can only be transmitted as part of the NR-U DRS, and does not imply that these are necessarily part of all NR-U DRS transmissions.

This contribution discusses DRS in NR-U for 15kHz and 30kHz, considering 60kHz based SSB/PBCH block is outside the scope of the WI [2]. This contribution is revision of our contribution [R1-1906374] in RAN1#97.

SSB pattern and Type0-PDCCH

As discussed in RAN1#96bis, SSB pattern within a slot is coupled with the starting symbol of Type0-PDCCH. For the new SSB pattern, i.e. the second SSB in a slot starts from symbol #9, it was the common understanding that Type0-PDCCH associated with the second SSB starts from symbol #7, as defined in FR2. But for the legacy SSB pattern, there were different views on whether Type0-PDCCH associated with the second SSB starts from symbol #7 or symbol #6, in case of length-1 CORESET0.

There were 3 alternatives listed in the agreement of RAN1#96bis [3].

Agreement: Select one of the following options in RAN1 #97: - Alt 1: Legacy SSB positions in a slot - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset 0 at least for the first SSB in a slot - Support Type0-PDCCH in symbol (#6, #7) for length-2 coreset 0 and symbol #6 or symbol #7 for length-1 coreset 0 for the second SSB in a slot - FFS: configurable between symbols #6 and #7 for the length-1 coreset 0 for the second SSB in a slot - Alt2: New SSB positions in a slot - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset 0 for the first SSB in a slot - Support Type0-PDCCH in symbol (#7, #8) for length-2 coreset 0 and symbol (#7) for length-1 coreset 0 for the second SSB in a slot - Alt 3: Legacy SSB positions in a slot - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset 0 for the first SSB in a slot - Support Type0-PDCCH in symbol (#7) for coreset 0 for the second SSB in a slot

Half-slot based Type0-PDCCH

Half-slot based Type0-PDCCH, especially Type0-PDCCH associated with the second SSB, is directly impacted by the above three alternatives. Half-slot based Type0-PDCCH for the three alternatives is shown in the following figure.

- Alt1: Length-1 CORESET (two options)

(2) Alt1: Length-2 CORESET

(3) Alt-2: Length-1 CORESET                         (4) Alt-2: Length-2 CORESET

(5) Alt3: length-1 CORESET

Figure 1: Example of the three alternatives for SSB pattern and Type0-PDCCH for half-slot based Type0-PDCCH

Discussion in RAN1#97

In [4], comparison among the three alternatives was listed in different aspects.

In discussion of RAN1#97, the alternatives were down selected to Alt-1 and Alt-2. However, there was no consensus between Alt-1 and Alt-2, and only if there is a majority who believes that enhancement is needed, the topic could be revisited.

Agreement: For SSB positions and type-0 PDCCH monitoring in a slot, only Alternatives 1 and 2 (from the previous agreement in RAN1#96bis) are considered further. Note: As per the previous agreement one of Alternative 1 and 2 is to be selected at RAN1#97 Conclusion: There is no consensus between Alternative 1 and Alternative 2 on the topic of SSB positions and type-0 PDCCH monitoring in a slot. No further online or scheduled offline discussions will be conducted on this topic. If there is consensus on such enhancements, the topic can be revisited in the Rel-16 NR-U work item. Friday: Nokia concern with not down selecting between the two alternatives – may have impact on future work Huawei wondered how the issue could be brought back to discussion, if need be Ad-hoc chair repeated (comment already made during the online session) that if there will be a majority who believes that enhancement is needed and further action must be taken, the topic could be readdressed

Impact of no consensus between Alt-1 and Alt-2

In RAN1#97, the impact of no consensus between Alt-1 and Alt-2 has been discussed. Here, some views are provided.

Impact to Type0-PDCCH

The first impact is Type0-PDCCH. It may be controversial to the following agreement:

Agreement: The Type0-PDCCH monitoring configuration for NR-U should satisfy at least the following properties: - TDM of Type0-PDCCH and SSB similar to existing pattern 1 (already agreed) - Support the monitoring of Type0 PDCCH of the 2nd SSB position in a slot in the gap between 1st and 2nd SSB within the slot - FFS start at symbol #6 of #7 or both - FFS: The Type0-PDCCH candidates associated with an SSB are confined within a slot carrying the associated SSB (with the same QCL assumptions)

Consequently, the half-slot based Type0-PDCCH will not be supported, if we fall back to support the Type0-PDCCH monitoring occasions defined in R15 FR1. As shown in the next section, we can move forward based on the slot based Type0-PDCCH.

Impact to the second SSB in a slot

The second impact is the support of the second SSB in a slot. There could be some options to handle the issue of the support of the second SSB in a slot.

- Option 1: Support the SSB pattern of Case A (15kHz SCS) and Case C (30kHz SCS), i.e. SSB pattern in Alt-1.

- Option 2: Support the SSB pattern of Case B (30kHz SCS);

- Option 3: Do not support the second SSB in a slot.

For Option 1 and 2, there is marginal spec impact. For Option 3, the support of the second SSB in a slot is assumed when we agreed the number of SSB candidate positions in RAN1#97, so the non-support of the second SSB in a slot will be controversial to the agreement on the number of SSB candidate positions. Therefore, we prefer Option 1 and Option 2.

In Option 2, the SSB pattern of Case B is only supported for 30kHz SCS in R15, so it will lead to spec impact if the SSB pattern of Case B is introduced to 15kHz SCS. So, Option 2 may not be acceptable.

For Option 1, it is the legacy SSB pattern, but the half-slot based Type0-PDCCH associated to the second SSB is the argued point which blocks the progress. Hence, as a compromise way only the slot based Type0-PDCCH is supported on the basis of the legacy SSB pattern. It is also understood that the slot base Type0-PDCCH will lead to that gNB can only achieve TXOP at the beginning of a slot instead of a half slot. The more TXOP at gNB is also the reason why we decided to support the half-slot based Type0-PDCCH. But considering the current situation, perhaps we can only accept it.

Proposal 1: The SSB pattern of Case A (15kHz SCS) and Case C (30kHz SCS) as defined in R15 FR1 is supported.

Slot based Type0-PDCCH

Different from half-slot based Type0-PDCCH, it was the common understanding that Type0-PDCCH associated with at least the first SSB starts from symbol #0, for both Alt 1 and Alt 2.

Agreement: Select one of the following options in RAN1 #97: - Alt 1: Legacy SSB positions in a slot - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset 0 at least for the first SSB in a slot - Alt2: New SSB positions in a slot - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset 0 for the first SSB in a slot

Issue of M=1 in the legacy table

To support slot based Type0-PDCCH, M can be set to 1 in the legacy table of Type0-PDCCH monitoring occasions. When M=1 in the legacy table, because there is only one Type0-PDCCH monitoring occasion in a slot, the time span of Type0-PDCCH is twofold of that of SSBs.  On the other hand, the self-contained structure of a “DRS unit” is destroyed. Due to the fact that there could be several slots time gap between an SSB and the associated Type0-PDCCH monitoring occasion, missing TXOP of an SSB in the candidate position will lead to that the associated Type0-PDCCH is useless for UE initial access. The following figure illustrates an example of Type0-PDCCH monitoring occasion when M=1, where the number of the actually transmitted SSBs is 4.

Figure 2: An example of Type0-PDCCH monitoring occasions when M=1 in the legacy table

In our view, when we define the SSB candidate positions, we have beard in mind the self-contained structure of a DRS unit, for example, when we designed the time budget of the number of SSB candidate positions in a 5ms window. Therefore, it is better not to destroy the self-contained structure.

M=1/2 and two SSBs are associated with the two Type0-PDCCH in a slot

As a legacy pattern, two Type0-PDCCH monitoring occasions can be set at the beginning of a slot, and they can be associated to the two SSB in a slot. The only drawback is that it only supports the length-1 CORESET0.

(1) Alt1 & Alt3                                               (2) Alt2

Figure 5: Example of the three alternatives for SSB pattern and Type0-PDCCH for slot based Type0-PDCCH, when “M=1/2 and two SSBs are associated with the two Type0-PDCCH in a slot” and both two SSBs are transmitted

We have the following proposal.

Proposal 2: If the number of search space sets of Type0-PDCCH per slot is two, i.e. M=1/2, the two SSBs in a slot are associated with the two Type0-PDCCH respectively at the beginning of the slot, and duration of CORESET of Type0-PDCCH can be one symbol.

M=1 and two SSBs are associated with the same Type0-PDCCH in a slot

It should be noted that two SSBs are associated with the same Type0-PDCCH in a slot has not been excluded. In NR-U, QCLed SSBs in a SSB burst may be supported to enable beam repetition transmission at gNB. Therefore, two QCLed SSBs can be associated with the same Type0-PDCCH in a slot.

(1) Alt1 & Alt3                                               (2) Alt2

Figure 3: Example of the three alternatives for SSB pattern and Type0-PDCCH for slot based Type0-PDCCH, when “M=1 and two SSBs are associated with the same Type0-PDCCH in a slot”

Furthermore, as a main benefit, length-2 CORESET0 can be supported in this pattern.

(1) Alt1 & Alt3                                               (2) Alt2

Figure 4: Example of the three alternatives for SSB pattern and Type0-PDCCH for slot based Type0-PDCCH, when “M=1 and two SSBs are associated with the same Type0-PDCCH in a slot”

Therefore, the case that two SSBs are associated with the same Type0-PDCCH in a slot can also replace the case of M=1 in legacy table.

Observation 1: The case that two QCLed SSBs can be associated with the same Type0-PDCCH in a slot can also replace the case of M=1 in legacy table.

We have the following proposal.

Proposal 3: If the number of search space sets of Type0-PDCCH per slot is one, i.e. M=1, the two SSBs in a slot is associated with the same Type0-PDCCH at the beginning of the slot, and duration of CORESET of Type0-PDCCH can be one and two symbols.

RMSI PDSCH

It was proposed to discuss the following issues for RMSI PDSCH [5].

Discussion: - Further discuss whether Rate-Match support of RMSI-PDSCH over SSB is warranted for NR-U, RAN1 shall consider the following aspects regarding rate-matching of RMSI PDSCH around SSB in DRS: - Handling of DMRS, in particular for DMRS symbols shared with SSB. - Which SSBs to rate-match around, and how that information is conveyed to the UE - Whether the feature provides sufficient benefits, to consider for NR-U

Assumption of self-contained “DRS unit”

As discussed above, a “DRS unit” can be self-contained with some restriction mentioned in Section 2.2, if Type0-PDCCH is slot based, as shown in the following figure.

Figure 6: Example of “DRS unit” for slot based RMSI PDCCH

RMSI PDSCH rate matching

At cell search

For RMSI PDSCH (scheduled by Type0-PDCCH), R15 does not support RSMI PDSCH rate matching around SSB at cell search.

When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH.

In NR-U, because of the self-contained “DRS unit”, RMSI PDSCH rate matching around SSB at cell search is possible, which is different from R15. Specifically, if RMSI PDSCH rate matching is performed around SSB, UE needs to make sure that the SSB within the current slot is actually transmitted. It is true if the self-contained structure is supported for a “DRS unit”. In cell search procedure, UE should at least detect an SSB, and then monitor the Type0-PDCCH associated to the SSB. If UE decodes the Type0-PDCCH successfully, UE confirms that the SSB(s) within the same slot shall be transmitted. Hence, RMSI PDSCH rate matching around SSB(s) within the same slot is feasible at cell search.

Observation 2: At cell search, RMSI PDSCH rate matching around SSB(s) within the same slot is feasible.

Furthermore, it is needed to discuss rate matching behaviour when UE stay in the initial DL BWP after receiving SIB1. Type0-PDCCH CSS set can be “reconfigured”, including reconfiguration of PCell, and configuration for handover and PSCell/SCell addition etc.

a Type0-PDCCH CSS set configured by pdcch-ConfigSIB1 in MIB or by searchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

Reconfigured by searchSpaceZero in PDCCH-ConfigCommon

After receiving SIB1, Type0-PDCCH CSS set can reconfigured by searchSpaceZero in PDCCH-ConfigCommon:

searchSpaceZero Parameters of the common SearchSpace#0. The values are interpreted like the corresponding bits in MIB pdcch-ConfigSIB1. Even though this field is only configured in the initial BWP (BWP#0) searchSpaceZero can be used in search spaces configured in other DL BWP(s) than the initial DL BWP if the conditions described in TS 38.213 [13], clause 10 are satisfied.

But the reconfiguration is also based on 4 bits like those in pdcch-ConfigSIB1 in MIB, which follow the patterns of Type0-PDCCH described in Section 2. In this case, UE should at least detect an SSB to determine Type0-PDCCH associated to the SSB, and due to the self-contained structure of a “DRS unit”, UE can perform RMSI PDSCH rate matching around the SSB(s) within the same slot, like procedure mentioned in Section 3.2.1, according to the description in Subclause 10.1 of TS 38.213.

If a UE is provided a zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set as described in Subclause 13.

In fact, broadcast PDSCH (besides RMSI PDSCH) scheduled by PDCCH of search space zero can be performed rate matching around SSB(s).

Observation 3: If search space set of broadcast PDCCH is search space zero, broadcast PDSCH rate matching around SSB(s) within the same slot which is scheduled by the broadcast PDCCH is feasible.

Furthermore, for unicast PDSCH, if it is scheduled by PDCCH with CRC scrambled by a C-RNTI in search space zero, UE monitors PDCCH associated with a SSB indicated by TCI state, according to the description in Subclause 10.1 of TS 38.213.

For DCI formats with CRC scrambled by a C-RNTI, the UE monitors corresponding PDCCH candidates only at monitoring occasions associated with a SS/PBCH block, where the SS/PBCH block is determined by the most recent of - a MAC CE activation command indicating a TCI state of the active BWP that includes a CORESET with index 0, as described in [6, TS 38.214], where the TCI-state includes a CSI-RS which is quasi-co-located with the SS/PBCH block, or - a random access procedure that is not initiated by a PDCCH order that triggers a non-contention based random access procedure

Since, as an assumption, PDCCH associated with a SSB and the corresponding SSB are self-contained in a “DRS unit”, UE can perform unicast PDSCH rate matching around SSB(s), if the unicast PDSCH is scheduled by PDCCH of search space zero.

Observation 4: If search space set of unicast PDCCH (with CRC scrambled by a C-RNTI) is search space zero, unicast PDSCH rate matching around SSB(s) within the same slot which is scheduled by the unicast PDCCH is feasible.

Reconfigured by searchSpaceSIB1 in PDCCH-ConfigCommon

In RRC connected state, Type0-PDCCH CSS set can be set to a non-zero value by searchSpaceSIB1 in PDCCH-ConfigCommon, so configuration by SearchSpaceZero is not effective for Type0-PDCCH CSS set.

searchSpaceSIB1 ID of the search space for SIB1 message. In the initial DL BWP of the UE′s PCell, the network sets this field to 0. If the field is absent, the UE does not receive SIB1 in this BWP. (see TS 38.213 [13], clause 10)

It should be noted that, in R15, immediately after receiving SIB1, including RRC idle and RRC connected state, UE can perform PDSCH rate matching around SSB(s) according to the indication of position of SSB(s) by ssb-PositionsInBurst.

When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCH with SPS, the REs corresponding to the configured or dynamically indicated resources in Subclauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.

However, in NR-U, whether a given SSB is transmitted or not depends on the current LBT result, so the information of ssb-PositionsInBurst is not sufficient for rate matching. On the other hand, in this case, UE does not monitor Type0-PDCCH based on the SSB detection, which is different from the UE behavior in Section 3.2.1 or Section 3.2.2. Hence, RMSI PDSCH rate matching around SSB(s) is not so feasible.

Observation 5: If Type0-PDCCH CSS set is reconfigured by searchSpaceSIB1 in PDCCH-ConfigCommon and searchSpaceSIB1 is set to a non-zero value, RMSI PDSCH rate matching around SSB within the same slot is not feasible.

Setting SSB close to the edge of initial DL BWP

As a solution substituting rate matching, setting SSB close to the edge of initial DL BWP [6] can avoid only partial of frequency resource of initial active DL BWP can be scheduled by RMSI PDCCH, as shown in the following figure.

Figure 7: Setting SSB close to the edge of initial DL BWP for slot based RMSI PDCCH

Setting SSB close to the edge of initial DL BWP is up to SS raster. It is general view that the number of candidates of SS raster should be reduced since 20MHz channel in unlicensed band is pre-defined. If the number of candidate of SS raster is reduced, it is not easy to set SSB close to the edge of initial DL BWP.

Moreover, the frequency position of initial DL BWP is the same as that of CORESET0, and the frequency resource of CORESET is globally defined in grid of common resource blocks. Thus, the frequency position of initial DL BWP is related to the grid of common resource blocks. This makes setting SSB close to the edge of initial DL BWP more difficult.

It should be noted that whether to support RMSI PDSCH rate matching around SSB still needs RAN4 feedback.

R1-1907884 [DRAFT] LS to RAN4 on impact of sync raster placement for NR-U in RAN1 Qualcomm Decision: The final LS is approved in R1-1907896 with the main body of the LS changed to the following: “RAN1 understands RAN4 is discussing the sync raster design for NR-U band in 5GHz with the options of placing the sync raster either in approximately the middle of each 20MHz channel, or at approximately the edge of each 20MHz channel. RAN1 evaluated the impact to RAN1 if either of the options are adopted in RAN4. From a RAN1 perspective, in unlicensed bands, where the DL initial BWP is nominally 20 MHz, RAN4 decisions on sync raster points have consequences on RAN1 design. RAN1 has discussed these consequences. - If a SSB is not close to the edge, the number of RBs available to carry SIB1 would be limited although further enhancement to introduce rate matching around the SSBs to address this limitation are being discussed in RAN1 - If a SSB is close to the edge, the number of RBs available to carry SIB1 would be less limited although whether or not further enhancement to introduce rate matching around the SSBs is needed is being discussed in RAN1”

As summary, we have the following proposal.

Proposal 4: If PDSCH rate matching around SSB(s) within the same slot is supported, it can be considered in the following cases.

- The case where the search space set of broadcast PDCCH scheduling the broadcast PDSCH is search space zero.

- The case where the search space set of unicast PDCCH scheduling the unicast PDSCH is search space zero.

PDSCH rate matching according to ssb-PositionsInBurst

As mentioned in Section 3.2.3, in licensed band, the PDSCH including broadcast PDSCH (RMSI/RAR/paging PDSCH) and unicast PDSCH, can be performed rate matching around SSB(s) according to ssb-PostionsInBurst. So, whether to use the parameter ssb-PostionsInBurst for PDSCH rate matching around SSB(s) in NR-U should be discussed.

It should be noted that in licensed band, as a prerequisite, gNB should use the fixed SSB positions in a SS- burst at least in a semi-static time scale. In NR-U, whether gNB should use the fixed SSB positions in DRS transmission window is still open.

The main scenario to be discussed is PDSCH (other than RMSI PDSCH) multiplexing with DRS, which is described in details as follows.

- If gNB achieves TXOP with LBT cat-4 and the corresponding COT is overlapping with DRX transmission window, gNB can multiplex the PDSCH other than RMSI PDSCH and DRS.

In this scenario, there are two options for gNB to send SSB(s), shown as follows.

- Option-1: gNB can choose SSB patterns in an SS-burst flexibly. Under this assumption, gNB can send a subset of the SS-burst in the time overlapping between the COT and DRS window, in order to avoid no TXOP for DRS with the new LBT. But UE cannot derive SSB position(s) according to ssb-PostionsInBurst.

- Option-2: gNB cannot choose SSB patterns in an SS-burst flexibly, and gNB can only shift the SS-burst as a whole. Under this assumption, only if the time overlapping between the COT and DRS can accommodate the SS-burst, gNB will send the SS-burst. UE can derive SSB position(s) according to ssb-PostionsInBurst and the start time of COT, if GC-PDCCH informs COT structure to UE. But the rate matching is still not effective, when GC-PDCCH is not configured, which is a valid case indeed.

In our view, Option-1 may be preferred by gNB vendor due to flexibility and more TXOP for DRS. But if Opiton-1 is accepted, the parameter ssb-PostionsInBurst may not provide the sufficient information on PDSCH rate matching around SSB(s). As mentioned before, broadcast PDSCH can rely on search space zero based scheduling to enable rate matching around SSB(s). Moreover, unicast PDSCH can rely on the dynamic rate matching mechanism by rateMatchPatternGroup1 or rateMatchPatternGroup2 to enable rate matching around SSB(s), as defined in Subclause 5.1.4.1 of TS 38.214. In fact, for unicast PDSCH, the flexible resource allocation is also a way to avoid overlapping with SSB(s).

Observation 6: If gNB can choose SSB patterns in an SS-burst flexibly, the parameter ssb-PostionsInBurst may not provide the sufficient information on PDSCH rate matching around SSB(s).

Handling of RMSI PDSCH DMRS collision

If RMSI PDSCH rate matching around SSB is supported, RMSI PDSCH DMRS may collide with SSB. The following figure shows an example of RMSI PDSCH DMRS collision with SSB.

Figure 8: RMSI PDSCH DMRS for slot based RMSI PDCCH

It should be noted that RMSI PDSCH DMRS collision with SSB seem not present in R15, because rate matching of RMSI PDSCH around SSB is not supported at the initial access stage in R15, and gNB should not allocate RMSI PDSCH frequency resource overlapping with SSB.

It should be noted that setting SSB close to the edge of initial DL BWP can also avoid collision between RMSI PDSCH DMRS and SSB. But this is up to RAN4 discussion on SS raster and resource grid.

Observation 7: Regarding handling of RMSI PDSCH DMRS collision, setting SSB close to the edge of initial DL BWP can be considered.

As another solution, it was mentioned in [7] that the collision issue can be solved by defining channel estimation sharing between RMSI PDCCH DMRS and RMSI PDSCH DMRS. In our view, channel estimation sharing between RMSI PDCCH DMRS and PDSCH DMRS may restrict the precoders for RMSI PDSCCH and PDSCH.

Observation 8: Regarding handling of RMSI PDSCH DMRS collision, channel estimation sharing between RMSI PDCCH DMRS and PDSCH DMRS may not be applicable.

As the third solution, UE may assume “frequency-region level rate matching”, i.e. UE may assume the all symbols overlapping with the frequency region of SSB are not used in resource mapping. The following figure shows the “frequency-region level rate matching” for slot based RMSI PDCCH.

Figure 9: The “frequency-region level rate matching” for slot based RMSI PDCCH

Observation 9: Regarding handling of RMSI PDSCH DMRS collision, “frequency-region level rate matching” can be considered, i.e. UE may assume the all symbols in the RBs overlapping with the frequency region of SSB is not used in resource mapping.

Regarding handling of RMSI PDSCH DMRS collision, we slightly prefer “frequency-region level rate matching”.

Proposal 5: If RMSI PDSCH rate matching around SSB(s) within the same slot is supported, “frequency-region level rate matching” can be supported, i.e. UE may assume the frequency resource in all symbols in the RBs overlapping with SSB is not used in resource mapping.

Conclusion

In the contribution, we have the following proposals.

Proposal 1: The SSB pattern of Case A (15kHz SCS) and Case C (30kHz SCS) as defined in R15 FR1 is supported.

Proposal 2: If the number of search space sets of Type0-PDCCH per slot is two, i.e. M=1/2, the two SSBs in a slot are associated with the two Type0-PDCCH respectively at the beginning of the slot, and duration of CORESET of Type0-PDCCH can be one symbol.

Proposal 3: If the number of search space sets of Type0-PDCCH per slot is one, i.e. M=1, the two SSBs in a slot is associated with the same Type0-PDCCH at the beginning of the slot, and duration of CORESET of Type0-PDCCH can be one and two symbols.

Proposal 4: If PDSCH rate matching around SSB(s) within the same slot is supported, it can be considered in the following cases.

- The case where the search space set of broadcast PDCCH scheduling the broadcast PDSCH is search space zero.

- The case where the search space set of unicast PDCCH scheduling the unicast PDSCH is search space zero.

Proposal 5: If RMSI PDSCH rate matching around SSB(s) within the same slot is supported, “frequency-region level rate matching” can be supported, i.e. UE may assume the frequency resource in all symbols in the RBs overlapping with SSB is not used in resource mapping.

Reference

- 3GPP RAN1, “Chairman’s notes of RAN1 94”, Aug. 20th – 24th, 2018.

- RP-182878, NR-U WID, RAN plenary WG #82.

- 3GPP RAN1, “Chairman’s notes of RAN1 96bis”, April 8th – 12th, 2019.

- R1-1907883, “Feature lead summary#2 on initial access signals and channels for NR-U”, Qualcomm Incorporated, RAN1#97, April 13th – 17th, 2019.

- R1-1905785, “Feature lead summary on initial access signals and channels for NR-U”, Qualcomm Incorporated, RAN1#96bis, April 8th – 12th, 2019.

- R1-1902880, “Initial access signals and channels for NR-U”, Ericsson, RAN1#96, Feb. 25th – Mar. 1st, 2019.

- R1-1902982, “Initial access signals and channels for NR-U”, Qualcomm, RAN1#96, Feb. 25th – Mar. 1st, 2019.

- Appendix

- A.1 CSI-RS within DRS

In NSA deployment, DRS will only contain SSB and CSI-RS, since RMSI may not be sent. CSI-RS can be sent to achieve OCB requirement. In SA deployment, CSI-RS is not so necessary for OCB requirement, but CSI-RS can be used for other purpose.

In R15 NR, there are four types of CSI-RS defined, such as CSI-RS for mobility, CSI-RS for beam management, CSI-RS for CSI acquisition and CSI-RS for tracking. In our view, necessity of CSI-RS for mobility and beam management needs FFS, since

- SSB can be used for RRM measurement and beam management, and

- finer beam may be not necessary in sub-7GHz, and

- beamforming gain is limited in NR-U due to limitation of PSD/EIRP.

Therefore, we only focus on CSI-RS for CSI acquisition and CSI-RS for tracking.

In connected mode, including CSI-RS for CSI acquisition in DRS may be necessary to increase accuracy of CSI knowledge at gNB. CSI-RS for tracking (TRS) may provide more accurate T/F synchronization, since CSI-RS for tracking has wider bandwidth.

From perspective of standardization impact, CSI-RS can be flexibly configured by RRC signalling in connected mode in R15, so additional standardization on top of CSI-RS configuration needs further study. First, LBT impact to CSI-RS configuration, e.g. time domain pattern, should be considered. It should be noted that, in LAA CSI-RS for CSI acquisition can be configured in DRS, and LBT will impact DRS as whole. Second, rate matching around CSI-RS for idle-mode UE should be considered, since DRS is used by connected-mode UE and idle-mode UE at the same time. Third, multiplexing between SSB and CSI-RS for CSI acquisition and channel tracking should be considered.

Observation A1: Standardization impact of CSI-RS for CSI acquisition and channel tracking within DRS should be considered:

- LBT impact to CSI-RS configuration;

- Rate matching around CSI-RS for idle-mode UE.

- Multiplexing between SSB and CSI-RS.