3GPP TSG RAN WG1 Meeting #98                            	              R1-1908965

Prague, Czech Rep, 26th – 30th August 2019

Agenda Item:	7.2.9.1

Source:	Spreadtrum Communications

Title:	Discussion on PDCCH-based power saving channel

Document for:	Discussion and decision

Introduction

In the agreed RAN#83 WID [1], the main RAN1-led objective of UE power saving was to specify PDCCH-based power saving signal/channel.

- Specify the power saving techniques with power saving signal/channel - Specify the PDCCH-based power saving signal/channel triggering UE adaptation in RRC_CONNECTED - Note: this objective shall not duplicate DRX operation and impact to DRX is studied at RAN2 - Note: Any change of PDCCH channel coding and payload interleaver is not in the scope

As the updated WID in RAN#84 [2], the objective of specifying PDCCH-based power saving signal/channel was further extended and refined. It was extended that the procedure of cross-slot scheduling should be specified together with PDCCH-based power saving signal/channel. It was refined that procedures triggering a MAC entity to “wake up” to monitor PDCCH at reception of the PDCCH-based power saving signal/channel for the next occurrence(s) of the drx-onDurationTimer should be specified.

- Specify power saving techniques with PDCCH-based power saving signal/channel triggering UE adaptation in RRC_CONNECTED mode [RAN1, RAN2, RAN4] - Specify procedures triggering a MAC entity to “wake up” to monitor PDCCH at reception of the PDCCH-based power saving signal/channel for the next occurrence(s) of the drx-onDurationTimer [RAN2, RAN1] NOTE: Any change of PDCCH channel coding and payload interleaver is not in the scope - Specify the procedure of cross-slot scheduling power saving techniques [RAN1, RAN4] NOTE: The procedure is in addition to Rel-15 cross-slot scheduling procedure

Categories of power saving signal/channel

WUS and GTS signaling

It was agreed [3] that potential DCI contents in DCI format(s) of power saving channel may include triggering the power saving techniques in time domain, frequency domain and spatial/antenna domain.

Agreements: Potential DCI contents in DCI format(s), to be further investigated: - Power saving technique associated with C-DRX– - Essential for UE function for the C-DRX - Wakeup – - UE is indicated to transition from outside Active Time to Active Time - UE is indicated to stay at Active Time - Go to sleep– - UE is indicated to transition from Active Time to outside Active Time. - UE is indicated to stay outside Active Time - FFS: The time of receiving the wakeup and go-to-sleep indication inside or outside Active Time. - Cross-slot scheduling - Triggering RS transmission - CSI report - Single vs. multi-cell operation - BWP /SCell - BWP & SCell together - BWP and SCell have separated fields - MIMO layer adaptation/number of Anenna adaptation - May further depend on RAN4’s work - Indication of CORESET/search space/candidate of subsequent PDCCH decoding - PDCCH monitoring periodicity - PDCCH skipping (skipping duration)- - PDCCH skipping – UE is indicated to skip number of the PDCCH monitoring occasions and stays in the Active Time Note that - For the bullets in italic, there are concerns that some of which may have dependence on the ongoing SI in RAN2. - For the last two bullets, there are additional concerns that these are deemded by some companies to be not in the scope of the power saving WI approved so far The following candidates may be discussed after RAN2’s SI is completed: - Skipping number of DRX monitoring - SPS activation - DRX configuration

In our view, there are two categories for power saving signal/channel, i.e. power saving signal/channel outside the Active Time namely Wakeup Signaling (WUS), and power saving signal/channel in the Active Time namely Go-To-Sleep (GTS) signaling. The above listed power saving techniques can be triggered by WUS and GTS signaling, which is shown in the following figure.

Table 1: UE behaviors for WUS and GTS signaling in different power saving techniques

|  | WUS | GTS signaling |
| --- | --- | --- |
| Power saving technique associated with C-DRX | Switch to the “normal” DRX operation, i.e. transition from outside Active Time to Active Time by starting the DRX timer(s), e.g. on-duration timer | Switch to the “power saving” DRX operation, i.e. transition from Active Time to outside Active Time by stopping the DRX timer(s), .e.g. on-duration timer. |
| Cross-slot scheduling | Use the “normal” TDRA table, i.e. semi-statically configured TDRA table | Use the subset of the semi-statically configured TDRA table which contains the minimum K0 larger than 0 |
| Single vs. multi-cell operation | Perform the “normal” CA operations | Perform the single-cell operations |
| BWP/SCell operation | Switch to the “normal” BWP/SCell operations | Switch to the “power saving” BWP/SCell operations |
| MIMO layer adaptation/number of Antenna adaptation | Switch to the “normal” number of MIMO layer/antenna | Switch to the reduced number of MIMO layer/ antenna |
| Indication of CORESET/search space/candidate of subsequent PDCCH decoding and PDCCH monitoring periodicity | Switch to the “normal” PDCCH configurations | Switch to the “power saving” PDCCH configurations |
| PDCCH skipping | Switch to the “normal” PDCCH monitoring | skip PDCCH monitoring occasions in a time interval |

From the above table, it can be observed that WUS is usually used to trigger UE switching to the “normal” operations as if any power saving technique is not applied. In this meaning, WUS can be considered as a “reset” button.

It is the first reason to make WUS as simple as a “reset” button that WUS should have low miss detection rate, which is included in the agreement of RAN1 AH 1901. If WUS has smaller DCI size, the corresponding miss detection rate can be reduced.

Agreements: The performance evaluation of the power saving signal/channel should target the miss detection at X% and the false alarm rate at Y% with the following aspects identified for the proposed power saving signal/channel - The target of miss detection X% and the false alarm rate at Y% as baseline for evaluation - For power saving signal/channel for wake-up purpose, X=[0.1] and Y=[1] - For power saving signal/channel for go-to-sleep purpose, X=[1] and Y=[0.1] - Additional X and Y values are not precluded for the proposed power saving signal/channel based on the use cases and scenarios - For any other purpose(s) of power saving signal/channel, companies to report X & Y values

As the second reason, it is better to make the effective time of GTS signaling “short” or “controllable”, e.g. within the current Active Time. If the effective time of GTS signaling is within the current Active Time where GTS signaling is triggering, UE should stop go-to-sleep behavior automatically in the “next” Active Time triggered by WUS. It is more robust, in our view, than using dynamic signaling to switch UE from go-to-sleep behavior to wakeup behavior, which usually works with 1% miss detection rate. In this sense, WUS is a “reset” button for GTS signaling.

As the third reason, PDCCH-based WUS may be evolved to sequence-based WUS to further reduce UE power consumption, especially for WUS detection. For sequence-based WUS, it can only convey a small number of information bits, so making WUS as a “reset” button may be a good choice.

Observation 1: WUS can be considered as a “reset” button for almost all power saving techniques.

On the other hand, it can be observed that GTS signaling is used to trigger UE to adapt to the various “power saving” operations. So, dynamically triggering “power saving” operations needs more parameters than dynamically triggering the “normal” operations, because there is always tradeoff between power saving gain and system performance, e.g. throughput and latency, and flexible adaptions are necessary.

Observation 2: GTS signaling is used to trigger UE adapt to the various “power saving” operations.

Triggered by WUS

According to RAN1 agreement [4], the following power saving techniques can be triggered by WUS.

- Power saving technique associated with C-DRX (WUS)

- Cross-slot scheduling

- Triggering RS transmission

- CSI report

Observation 3: The following power saving techniques can be triggered by WUS.

- Power saving technique associated with C-DRX (WUS)

- Cross-slot scheduling

- Triggering RS transmission

- CSI report

Triggering by BWP switch

In some views, some power saving techniques can be triggered by BWP switch. More specifically, the following power saving techniques can be configured as properties of a BWP.

- Indication of CORESET/search space/candidate of subsequent PDCCH decoding

- PDCCH monitoring periodicity

- MIMO layer adaptation/number of Antenna adaptation

In our view, BWP switching signaling can be used to trigger these power saving techniques. It is also arguable that the number of the semi-statically configured BWPs (3 for now) may be insufficient, since we only considered BWP for bandwidth or numerology adaptation when we specified the maximum number of the semi-statically configured BWPs in R15.

Observation 4: The following power saving techniques can be triggered by BWP switch.

- Indication of CORESET/search space/candidate of subsequent PDCCH decoding

- PDCCH monitoring periodicity

- MIMO layer adaptation/number of Antenna adaptation

Triggering by MAC CE

Some power saving techniques can be triggered by MAC CE in current spec, which can be listed as follows.

- Single vs. multi-cell operation

- PDCCH skipping

For single vs. multi-cell operation, current MAC CE can switch UE between single cell and multi-cell operation through SCell activation/deactivation signaling. Because UE should perform not only PDCCH monitoring but also CSI measurement on activated SCells, it may not be feasible to use DCI to switch UE between single cell and multi-cell.

Observation 5: The following power saving techniques can be triggered by MAC CE.

- Single vs. multi-cell operation

Triggering by GTS signaling

For PDCCH skipping, current MAC CE can stop the DRX timers in the Active Time, e.g. on-duration timer through MAC CE command of DRX. In some views, it is too aggressive to change the above MAC CE to DCI. However, it was common understanding that power saving techniques to reduce PDCCH monitoring on activated SCells can be specified in R16 with possible L1 signaling (do not violate the general principle of single DRX configuration per MAC entity), as shown below.

Note: The following objective may be considered in RAN#85 pending progress on SCell dormant behaviour and will be re-evaluated in RAN#85 - Study and specify (if agreed) power saving techniques to reduce PDCCH monitoring on activated SCells for CA and DC (EN-DC in particular). - Solutions (e.g. power saving signal/channel, enhancing Rel-15 DRX procedure) are to be discussed - Enhancements to Rel-15 DRX procedure can be studied further if they do not violate the general principle of single DRX configuration per MAC entity

If the reduction of PDCCH monitoring on activated SCells is introduced, we do not see the reason not to introduce the reduction of PDCCH monitoring on PCell. In our view, the reduction of PDCCH monitoring on PCell can be jointly discussed with the reduction of PDCCH monitoring on SCells.

In RAN2 discussion, some companies have concern on GTS signaling may “override” DRX configuration [5]. We agree that GTS signaling shall not override DRX configuration. So, if PDCCH skipping is triggered by GTS signaling, PDCCH skipping should not have impact on RAN2 spec, which is similar to cross-slot scheduling currently being defined.

As shown in observation 2, GTS signaling is related to the various “power saving” operations. Similar to the triggering of cross-slot scheduling, GTS signaling can be implemented by enhancements of the existing scheduling DCI format. Hence, GTS signaling should be flexible enough to support PDCCH skipping, or extendable to support PDCCH skipping in the future. Furthermore, in our previous contribution [6], PDCCH skipping can be triggered together with cross-slot scheduling to further reduce UE power consumption, which can reduce the triggering signaling overhead.

Observation 6: The following power saving techniques can be triggered by GTS signaling.

- Cross-slot scheduling

- PDCCH skipping

  - The reduction of PDCCH monitoring on PCell can be jointly discussed with the reduction of PDCCH monitoring on SCells.

  - No impact on RAN2 spec.

  - Signaling can be jointly designed with cross-slot scheduling

Wakeup signaling

DCI format

It was agreed in RAN1#97 [4] that WUS is PDCCH with a new DCI format.

Agreements: For power saving signal/channel configured outside Active Time, introduce a new DCI format for a UE, where the UE is configured to monitor the DCI format, with the power saving information for the UE in the DCI configurable by RRC - FFS whether the DCI is in UESS or CSS or both - FFS detailed configuration of the power saving information - FFS the new DCI format - Note: the same DCI may carry power saving information for one or more UEs

There were some remaining issues about the new DCI format, e.g. UESS or CSS, detailed configuration of information.

UESS or CSS

As per agreement in RAN1#97, RNTI, search space set and DCI field size of WUS can be semi-statically configured per UE.

For RNTI flexibility, it has been agreed in RAN1#97 [4] that the new RNTI can be UE specifically configured.

Agreements: A new RNTI (e.g., PS-RNTI) is introduced for the PDCCH-based power saving signal/channel decoding at least outside Active Time, UE-specifically configured - FFS how to use the PS-RNTI for scrambling of the PDCCH-based power saving signal/channel

For flexibility of search space set, it has been agreed in RAN1#97 [4] that search space set can be UE specifically configured.

Agreements: Support UE-specific configuration of the search space set(s) dedicated to the PDCCH-based power saving signal/channel for UE to monitor outside Active Time - Following the principle of Rel-15 search space configuration - FFS: the corresponding UE behaviour in monitoring the power saving signal/channel outside Active Time - FFS whether UE can further monitor the search space set(s) inside Active Time

For flexibility of DCI field size, it has been agreed in RAN1#96bis [3] that DCI field size can be UE specifically configured.

Agreements: The DCI format(s) of the PDCCH-based power saving signal/channel is designed to provide: - Indication of one or more power saving associated operations. - Supporting configurability (e.g., 0, 1, 2, …bits, etc.), if needed, for one or more information fields in the DCI at least for one DCI format

Because the new DCI format for WUS is sufficiently flexible, the PDCCH of WUS can contain one or multiple UE, which is up to gNB implementation. As in R15 NR, such PDCCH is defined under the terminology of CSS, as so-called GC-PDCCH. Therefore, CSS of WUS can be defined.

Proposal 1: For power saving signal/channel for UE to monitor outside the Active Time, a new type of CSS set can be introduced.

Configuration of the power saving information

In our view, the new DCI format should be sufficiently flexible to enable per-UE based power saving technique and easily be extended.

In a flexible way, UE can only parse the field(s) configured to its own. In this way, the field(s) should be semi-statically configured to UE. The position of the field(s) can be semi-statically configured to UE. Moreover, the field(s) can be UE-group basis or per-UE basis. But it may be transparent to UE whether a given field is UE-group basis or per-UE basis.

Proposal 2: For power saving signal/channel for UE to monitor outside the Active Time, the position of the field(s) per UE can be semi-statically configured to UE.

Details of DCI contents

It was agreed in RAN1#97 [4] that the potential information can be contained in WUS, which is shown as follows.

Agreements: Outside Active Time, the PDCCH-based power saving signal/channel is configured for triggering UE to or not to monitor the subsequent ON duration(s) - FFS a single vs. multiple durations, particularlly checking consistency with RAN2 agreements Further study in the new DCI how to potentially indicate at least the following techniques (subject to further WID update): - Indicating UE to use the aperiodic RS - Aperiodic CSI-RS - Aperiodic SRS - TRS - Triggering aperiodic CSI report - Cross-slot scheduling - Rel-15 DCI-based BWP switching The power saving techniques can be explicitly included in the DCI contents or implicitly indicated by other techniques (e.g., BWP switching).

In our view, the fields should contain wakeup bit(s), by which UE can know whether to wake up or not. Moreover, the additional RS information can be included in the field(s) too. As described in Appendix Section A.2 and A.4, the additional RS can potentially shorten the gap between the WUS and the beginning of DRX on-duration. The additional RS can the aperiodic CSI-RS which is supported in R15, and the aperiodic CSI-RS is usually triggered together with the aperiodic CSI report. Hence, the periodic CSI report can be also supported in WUS.

For content of “Rel-15 DCI-based BWP switching”, in our view, Rel-15 BWP switching is designed for scheduling DCI with the effective time and fall back timer. But WUS is not scheduling DCI. Hence, whether Rel-15 BWP switching is a content of WUS is an FFS point.

Proposal 3: For power saving signal/channel for UE to monitor outside the Active Time, the following content can be considered in the PDCCH-based power saving signal/channel:

- Bit(s) triggering UE to monitor the subsequent ON duration(s);

- Indicating UE to use the aperiodic RS;

- Triggering aperiodic CSI report;

Monitor occasion of WUS

It was agreed in RAN1#97 [4] that monitoring occasion(s) of WUS is occasion(s) with an offset before the DRX ON.

Agreements: The monitoring occasion(s) of the power saving signal/channel outside the Active Time is “indicated” to the UE by the gNB with an offset before the DRX ON - “Indicated” implies the explicit signalling by higher layer signalling or implicit through the CORESET/search space - FFS: The value and the range of offset

In our view, PDCCH monitoring occasion(s) can be configured by search space set, but configurations in search space set may not flexible enough to cope with DRX configuration, and WUS should be aligned with DRX configuration. Therefore, it is better monitoring occasion(s) of WUS can be explicitly signaled by higher layer with parameters independent of search space set, i.e. the offset can be explicitly signaled by higher layer with parameters independent of search space set. On the other hand, monitoring occasion(s) within a slot can be configured by search space set. In addition, if the offset is explicitly configured independent of search space set, potentially the ID of search space set may not be consumed by WUS.

Proposal 4: The offset between the starting time of the monitoring occasion(s) of the power saving signal/channel outside the Active Time and the DRX ON can be explicitly indicated to the UE by the gNB through RRC signaling independent of search space set.

Handling of miss detection of WUS

In GC-PDCCH based WUS there is at least one explicit bit to indicate UE whether to wake up, and this indication can be interpreted as either “to” or “not to”. The interpretation of the bit indication is not critical issue to us.

The possible critical issue may be the handling of miss detection of WUS, i.e. UE behaviour if miss detection of WUS happens. In our view, as a safety mode, UE should operate like being indicated to wake up, if WUS is not detected successfully.

Proposal 5: If UE does not detect power saving signal/channel for UE to monitor outside the Active Time, UE should operate like being indicated to wake up.

Go-to-sleep signaling

It was agreed in RAN1#97 [4] that GTS signaling is supported by the enhancement of existing scheduling DCI formats with additional field(s) and/or repurposing the existing field(s), which is shown as follows.

Agreements: The indication of at least one power saving technique(s) is supported at least by the enhancement of existing scheduling DCI formats with additional field(s), if any, and/or repurposing the existing field(s), if identified, when UE is in the Active Time. - It applies to UE-specific search space. - It is FFS for the common search space. - The at least one power saving technique(s) includes at least “Cross-slot scheduling” - FFS: Which existing DCI formats includes the power saving information - Whether power saving information is not included in the fallback DCI(s) (e.g., DCI format 0_0, DCI format 1_0) - Use of non-scheduling DCI formats. - It is FFS which field(s) is used to be repurposed for the indication of the power saving technique if the repurpose of existing field(s) is used. - FFS: New DCI format with size aligned with existing DCI format

Details of DCI contents

As mentioned in Section 2, in our view, GTS signaling is used to trigger UE to adapt to the various power saving techniques. To reduce overhead of DCI, the joint signaling of RRC and DCI can be considered. For instance, UE can be RRC configured with 4 types of power saving operations, and each type contains a combination of power saving techniques w.r.t. time/frequency/spatial domain, and then UE can be trigged by DCI to use one of 4 types of power saving operations in a time interval.

For power saving technique associated with C-DRX, it is general view that GTS signaling can be the triggering of stopping the timers in DRX configurations. But in some views, it may “override” DRX configuration or lead to cross-layer operation (from PHY layer to MAC layer) [5], which is not implementation friendly. Therefore, it can be an FFS point.

Proposal 6: The following power saving techniques are indicated at least by the enhancement of existing scheduling DCI formats with additional field(s), if any, and/or repurposing the existing field(s), if identified, when UE is in the Active Time.

- Cross-slot scheduling

- PDCCH skipping

  - The reduction of PDCCH monitoring on PCell can be jointly discussed with the reduction of PDCCH monitoring on SCells.

  - No impact on RAN2 spec.

  - Signaling can be jointly designed with cross-slot scheduling

Conclusion

PDCCH-based power saving signal/channel outside the Active Time:

Proposal 1: For power saving signal/channel for UE to monitor outside the Active Time, a new type of CSS set can be introduced.

Proposal 2: For power saving signal/channel for UE to monitor outside the Active Time, the position of the field(s) per UE can be semi-statically configured to UE.

Proposal 3: For power saving signal/channel for UE to monitor outside the Active Time, the following content can be considered in the PDCCH-based power saving signal/channel:

- Bit(s) triggering UE to monitor the subsequent ON duration(s);

- Indicating UE to use the aperiodic RS;

- Triggering aperiodic CSI report;

Proposal 4: The offset between the starting time of the monitoring occasion(s) of the power saving signal/channel outside the Active Time and the DRX ON can be explicitly indicated to the UE by the gNB through RRC signaling independent of search space set.

Proposal 5: If UE does not detect power saving signal/channel for UE to monitor outside the Active Time, UE should operate like being indicated to wake up.

PDCCH-based power saving signal/channel in Active Time:

Proposal 6: The following power saving techniques are indicated at least by the enhancement of existing scheduling DCI formats with additional field(s), if any, and/or repurposing the existing field(s), if identified, when UE is in the Active Time.

- Cross-slot scheduling

- PDCCH skipping

  - The reduction of PDCCH monitoring on PCell can be jointly discussed with the reduction of PDCCH monitoring on SCells.

  - No impact on RAN2 spec.

  - Signaling can be jointly designed with cross-slot scheduling

Reference

- RP-190727, “New WID: UE Power Saving in NR”, CATT, RAN#83, Mar. 18th – 21st, 2019.

- RP-191607, “New WID: UE Power Saving in NR”, CATT, RAN#84, Jun. 3rd – 6th, 2019.

- 3GPP RAN1, “Chairman’s notes of RAN1 96bis”, April 8th – 12th, 2019.

- 3GPP RAN1, “Chairman’s notes of RAN1 97”, May 13th – 17th, 2019.

- R2-1908072, “Report on [105bis#27][NR/Power Saving] – PDCCH skipping”, CATT, RAN2#106, May 13th – 17th, 2019.

- R1-1906372, “Discussion on cross-slot scheduling for UE power saving”, Spreadtrum, RAN1#97, April 13th – 17th, 2019.

- R1-1902734, “Remaining issues on evaluation methodology for UE power saving”, Spreadtrum, RAN1#96, Feb. 25th – Mar. 1st, 2019.

- Appendix

- A.1  Signaling reliability captured in 36.300

The different characteristics of MAC and RRC control are summarized in the table below.

Table B.1-1: Summary of the difference between MAC and RRC control

|  | MAC control |  | RRC control |
| --- | --- | --- | --- |
| Control entity | MAC |  | RRC |
| Signalling | PDCCH | MAC control PDU | RRC message |
| Signalling reliability | ~ 10-2 (no retransmission) | ~ 10-3 (after HARQ) | ~ 10-6 (after ARQ) |
| Control delay | Very short | Short | Longer |
| Extensibility | None or very limited | Limited | High |
| Security | No integrity protection No ciphering | No integrity protection No ciphering | Integrity protection Ciphering |

The main difference between MAC and RRC control lies in the signalling reliability. Due to the signalling reliability, signalling involving state transitions and radio bearer configurations should be performed by RRC. Basically, all signalling performed by RRC in UTRA should also be performed by RRC also for E-UTRA.

- A.2  Preparation period

- A.2.1  “Group-common” preparation period

There could be a misalignment between UE-specific DRX configuration and “group-common” RS configuration within preparation period. In general, gNB may configure a group of UEs to share a common RS configuration to reduce network overhead. More specifically, the group of UEs may be served with the same beam or TRP due to their similar locations, and they process the same CSI-RS for channel tracking, for CSI acquisition and for beam tracking within preparation period. This issue has been discussed in the standardization of R15 aperiodic TRS.

Observation A1: Preparation period may have to be “group-common” to reduce network overhead, but DRX configuration is better to be UE-specific.

- A.2.2  Re-sync with the existing RS

It was identified in RAN1#95 that “RS resources can be considered to assist UE in performing RRM/CSI measurement and channel time/frequency and/or beam tracking”. Recall that this issue had been discussed in TRS topic in R15 NR, it was concluded in RAN1#90bis that it is left to gNB implementation. In R15, UE may mainly rely on SSB or P-TRS for T/F tracking, and UE may have to re-sync before DRX on-duration.

Conclusion: Periodic SS burst and periodic TRS burst can be arranged such that the UE can utilize one SS burst and one TRS burst for AGC adjustment and T/F tracking before DRX ON No further specification impact may be necessary

When the last occasion of SSB or periodic TRS before DRX on-duration is far away from the beginning of DRX on-duration, UE should stay in micro-sleep state for a long time, which is power consumed.

Observation A2: UE should stay in micro-sleep state after re-sync, which is power consumed.

It can be observed that UE power consumption in re-sync procedure mainly depends on the slots number in micro-sleep state. One straightforward way to reduce micro-sleep time after re-sync is to plan SSB or P-TRS close to the beginning of DRX on-duration. However, SSB is cell specific and P-TRS is group common, while DRX configuration is UE specific. For instance, {periodicity, offset} of DRX cycle of UE 1 is {40ms, 10ms}, and {periodicity, offset} of DRX cycle of UE 2 is {40ms, 20ms}, and SSB periodicity is 20ms, in this case, SSB is always close to DRX on-duration of UE 1, and far away from DRX on-duration of UE 2.

If P-TRS is applied in example mention above, {periodicity, offset} of P-TRS of UE 1 can be {40ms, 5ms}, and {periodicity, offset} of P-TRS of UE 2 can be {40ms, 15ms}, and thereby P-TRS can be equally close to DRX on-duration of UE 1 and UE 2. However, per-UE configuration of P-TRS will cause large network overhead. This issue will be worse for multi beam deployment, since P-TRS may be different for different beam.

Observation A3: Re-sync with the existing periodic RS, such as SSB and TRS, may cause additional UE power consumption or large network overhead.

- A.2.3  Additional RS

It was identified in RAN1 Ad Hoc 1901 that additional RS is beneficial to assist UE in performing e.g. fine synchronization, channel/beam tracking, and/or CSI/RRM measurements.

Agreements: - Capture in the TR: New section 5.2.3 “Additional RS used for UE power saving” in TR Additional RS is the RS provided by gNB to assist UE in performing e.g.,fine synchronization, channel/beam tracking, and/or CSI/RRM measurements in addition to the existing RS in Rel-15. The additional RS was proposed for study for power saving schemes for e.g, UE adaptation to the DRX operation, BWP switching, fast SCell activation, reducing PDCCH monitoring, and/or RRM measurements. RS design is assumed to reuse Rel-15 waveform. Power saving signal could be used to meet the purpose of additional RS.

Additional RS can be triggered within preparation period in some cases as an optimization on top of periodic RS.

- A.3  Miss detection rate of WUS

We perform a link level simulation for different DCI size. The simulation assumptions are shown in the following table.

Table A1: Assumptions of the link level simulation

| Parameters | Value |
| --- | --- |
| DCI size (excluding 24bits CRC) | 30, 12, 3 bits |
| Number of symbols for CORESET | 1 |
| Subcarrier spacing | 30KHz |
| Aggregation level | 4 |
| Transmission type | Non-interleaved |
| REG bundling size | 6 |
| Channel estimation | Realistic |
| Channel model | TDL-B (delay spread 100ns) |
| Doppler spread | 5Hz |
| Number of BS antennas | 2Tx |
| Number of UE antennas | 4Rx |

As the simulation results, the required SNR for different DCI size is shown in the following table.

Table A2: The required SNR for different DCI size

|  | 30 bits | 12 bits | 3 bits (padding to 12 bits) |
| --- | --- | --- | --- |
| The required SNR | -4.5 dB | -6.5 dB | -7 dB |

It should be noted that PDCCH always contains 24 bits CRC which cause large number of total bits and then limit the performance. It can be observed that under the above simulation assumptions AL 4 can meet the required 0.1% miss detection rate with 12bits DCI size at SNR equal to -6.5dB.

- A.3  Power saving gain for WUS

In our previous contribution [7], the simple numerical evaluations with deterministic traffic model is exercised. It can be observed that power saving gain mainly is related to three points:

- The reduction in the reference energy overhead for transition, in the case power saving signal indicates NO further activity, i.e. P1 in [7]. The power saving gain is enlarged as P1 is enlarged.

- The time gap between transition and the beginning of DRX ON duration, in the case power saving signal indicates further activity, i.e. T2 in [7]. The power saving gain is enlarged as T2 is shortened.

- Length of DRX on-duration

The definition of P1/T2 can be shown in the following figure.

(1) Power saving scheme with optimization, in the case WUS indicates NO further activity

(2) Power saving scheme with optimization, in the case WUS indicates further activity

Figure 1: Illustration of P1, T2

For P1, if PDCCH-based WUS is detected by a low-complexity receiver, e.g. narrow-band receiver, PDCCH-based WUS may not need the full transition power, e.g. 100 units for transition from light sleep to active time. There are some methods to enable a low-complexity receiver from system perspective, e.g. narrow-band CORESET configured by gNB to reduce CCE complexity.

For T2, if PDCCH-based WUS can trigger an additional RS, the time gap can be shortened. But due to background activities and periodic activities, e.g. RRM measurement and channel tracking, UE may need to wake up periodically regardless of indication of WUS. Therefore, optimization of T2 may not be practical in realistic.

For length of DRX on-duration, it is not related to design of PDCCH-based WUS.

Therefore, PDCCH-based WUS may be designed to be detected with a low-complexity receive and to trigger an additional RS.

Observation A4: The reduction in the reference energy overhead for transition and the time gap between transition and the beginning of DRX on-duration directly impact the power saving gain. The power saving gain can be increased by the additional RS.