3GPP TSG RAN WG1 #98							       											R1-1908978

Prague, CZ, August 26th – 30th, 2019

Source:	Panasonic

Title: 	MIMO layer adaptation for NR power saving

Agenda Item:	7.2.9.3

Document for:	Discussion

# Introduction

In the RAN plenary meeting #84, the work item of NR power saving was updated [1]. More particularly, the standardization work on MIMO layer adaptation was approved in Section 4.1, which is quoted as below:

- Specify the power saving techniques of UE adaptation to the maximum number of MIMO layers [RAN1, RAN2, RAN4]

  - Specify configuration of a different MIMO layer configuration of the initial/default BWP compared with other BWPs of a Serving Cell.  [RAN2, RAN4]

    - Discuss whether to also extend this to define per-BWP MIMO layer configuration [RAN1, RAN2]

  - Evaluate if switching and interruption times for UE dynamic adaptation to the maximum number of MIMO layers are needed and which case assuming a relationship between the number of RF ports and the MIMO layer configuration [RAN4]

NOTE: Switching on/off the RF is part of the evaluation

In this contribution, the views on power saving techniques of the antenna and MIMO layer domain adaptation are provided. The essential factors to provide the potential power saving gain and corresponding procedure are also discussed.

# Discussion on performance gain from MIMO layer adaptation

From the observation in [2] in Section 5.1.3, the simulation results show some power saving gain for the dynamic and semi-static antenna adaptation by limiting the maximum layer number but with some performance degradation on UPT and latency. However, some other sources show also some negative power saving gain and large latency under some simulation assumptions.

To save UE power consumption, the motivation for reducing maximum layer number is to reduce the number of the transceiver chain. For downlink, switching off some of the receiver chains can save power/energy from amplifier, ADC, buffering and processing in the baseband. These are all receiving antenna specific. As for per layer power consumption reduction, reducing layer number can also alleviate  baseband processing pressure due to potential less TBS in a code word and even less code word, e.g. from 2 to 1.

However, as mentioned in the TR 38.840, the power saving gain largely depends on the simulation assumption, more specifically, the traffic characteristic and channel condition. Below are a few rough cases to facilitate the analysis on what circumstance and use cases can achieve potential power saving gain.

Table 1. Analysis on the potential impact given to antenna domain adaptation power saving gain by different traffic characteristics and channel condition assumptions

| Cases | Traffic arrival rate | Packet size | Channel condition | Pros and Cons comparison if maximum layer and antenna reduction is used |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Spectrum efficiency loss | UPT loss | Active time extension | Power saving gain |
| Case 1 | Low | Small | good | Yes, perhaps marginal | Possible but marginal | Possible but marginal | Possible |
| Case 2 | Low | Small | Not good | Yes | Possible | Possible | Not clear |
| Case 3 | Low | Large | good | Yes | Yes | Yes | Not clear |
| Case 4 | Low | Large | Not good | Yes | Yes | Yes | Not clear |
| Case 5 | High | Small | good | Possible | Possible | Possible | Not clear |
| Case 6 | High | Small | Not good | Yes | Yes | Yes | Not clear |
| Case 7 | High | Large | good | Yes | Yes | Yes | No |
| Case 8 | High | Large | Not good | Yes | Yes | Yes | No |

Ideally, the most effective power saving strategy is to make UE finish all the transmission/reception as soon as possible and then go to sleep to acquire more sleeping time. Hence even though reducing antenna number can save power per slot, the potential active time extension could cause even more power consumption.

In any case, if the channel condition is not good, reducing antenna number will bring coverage performance loss and power saving gain is not clear. More receiving antennas can improve coverage performance for single layer transmission. Then active time can be reduced. Hence, it is not a good option to reduce the antenna number in weak channel condition.

Observation 1: If channel condition is not good, reducing antenna number will impact coverage performance and extend reception/transmission period thus is not beneficial to achieve power saving gain.

Furthermore, it is obvious that when UE deals with large packet size traffic, it is also not suitable to reduce antenna or layer number. This will impact the spectrum efficiency and make the active time longer no matter the channel condition is good or not.

Observation 2: Traffic with large arrival packet size is not suitable for antenna or layer number restriction.

If the traffic arrival rate is low and the packet size is small, it is possible to just use smaller number of receiving/transmitting antennas to achieve power saving gain. In this case, probably there is no need to use more spatial layers for higher spectrum efficiency but the active time can still remain short. This requires good channel condition since reduced antenna number can not handle weak coverage case thus increases the UE active time to finish receiving/transmitting all the data.

Observation 3: Scenarios with low traffic arrival rate, small packet size and good channel condition is potentially suitable to apply MIMO layer and antenna number adaptation to achieve power saving gain.

If the traffic arrival rate is high, it is not easy for the UE to acquire more sleep time to save power, especially for the large packet case. Only if the packet size is sufficiently small and channel condition is sufficiently good, the UE may use less receiving/transmitting antenna without increasing the total active time.

Thus, overall, in our opinion,

Observation 4: The power saving performance of the UE adaptation to the number of MIMO layers or antennas deeply depends on the traffic characteristic and channel condition.

For high frequency band, beamforming and beamforming sweeping can be used. Then layer reduction will play less crucial role on the power saving. In case digital/hybrid beam forming using multiple AD/DA converters, not to apply beam forming by reducing antenna can save some power consumption but increase the active time. Therefore, similar discussion of MIMO layer adaptation would be applied.

# Potential specification impacts

Indication of the layer restriction

Current specification supports MIMO layer restriction by RRC configuration. For DL traffic, if the gNB may predict the traffic characteristics in a certain level, together with the CSI report, the judgement on whether to limit layer or antenna number can be made. Thus current specification framework can still work if the traffic does not change so dynamically.

On the other hand, one may argue the RRC-configured maximum MIMO layer is not adaptable to dynamic traffic arrival and thus the L1 signaling is needed. Note that for Rx chain hardware, frequently switching on and off may also introduce the transition power and interruption latency, which are practical constrains and negative for power saving gain. As discussed in the previous section, only in limited cases or scenarios can power saving gain be achieved. If the traffic and channel condition is not so stable and does not stay long enough in the range friendly to use less antenna (i.e. good channel condition, small packet size and low traffic arrival rate), it may not worth the efforts since the consequent the total power saving gain may not obvious enough to justify the dynamic operation.

Observation 5: The UE adaptation of the number of MIMO layers is beneficial to apply in the scenario that traffic characteristic and channel condition are stable and stay long enough in “power saving friendly zone”.

Therefore,

Proposal 1: RRC configuration of the maximum layer and antenna number could be supported. The current layer restriction in the specification is the starting point.

After UE receives the signaling, below are two possibilities of how the UE interprets the layer restriction:

- Channel condition is not good thus layer number is limited to e.g. 1. Then the UE does not need to calculate in the CSI measurement and report RI larger than 1. Typically, gNB would not schedule the UE with higher rank. But still, UE may use 4 receiving antennas to improve DL performance to overcome inter-cell interference or even intra-cell interference for MU-MIMO. It is noted that there can be also power saving benefits by only applying layer restriction without Rx chain switching off operation. Thus, it would be UE implementation on whether to switch off some of the Rx chain.

- For power saving purpose, no matter the channel condition is good or not, UE keeps the Rx chains active which are aligned with the configured minimum layer number and switches off other ones.

Therefore the question is whether this MIMO layer restriction indication leads to a “hard” or mandatory Rx chain switching off operation from UE behaviour perspective. In the WID, it is also mentioned that RAN4 needs to evaluate which case assuming a relationship between the number of RF ports and the MIMO layer configuration. As analyzed in the previous section, the traffic characteristics and channel condition together determine which operation would be best for power saving. For downlink, gNB knows better about the traffic characteristics while UE is more accurate and up-to-dated with channel condition.

Proposal 2: If new signaling of maximum MIMO layer indication is introduced for power saving, the UE behaviour of the adaptation should be discussed.

UE report to propose the maximum MIMO layer number

UE report of the maximum layer number was proposed by some companies. As mentioned before, the power saving gain depends on traffic characteristic and channel condition. To figure out a best option on the layer number for power saving, UE needs to predict the traffic characteristic. However, for downlink case, the gNB has better position to do this. Furthermore, other than reporting small layer number, it is not clear that how UE decides when to increase the layer or antenna number. A typical case is when large amount of downlink traffic arrives. Similarly, gNB knows earlier and better than UE.

However, if power saving for uplink case is also considered, UE knows better about the traffic characteristics. In this case, it is more justified for UE to report the assistance information. Unlike downlink case, gNB is more accurate in the uplink channel condition. Hence, UE may not be able to calculate how many Tx antennas are appropriate. Instead of directly reporting the antenna or layer number, traffic related assistance information is more helpful for gNB to make decision on the antenna adaptation.

Proposal 3: For downlink case, the UE assistance information in support of the MIMO layer and antenna domain adaptation is not supported. For uplink case, the UE assistance information could be studied, which can be considered in the next release.

# Conclusions

This contribution provide our views on the MIMO layer and antenna number adaptation for NR power saving. The follow conclusions are summarized below:

Observation 1: If channel condition is not good, reducing antenna number will impact coverage performance and extend reception/transmission period thus is not beneficial to achieve power saving gain.

Observation 2: Traffic with large arrival packet size is not suitable for antenna or layer number restriction.

Observation 3: Scenarios with low traffic arrival rate, small packet size and good channel condition is potentially suitable to apply MIMO layer and antenna number adaptation to achieve power saving gain.

Observation 4: The power saving performance of the UE adaptation to the number of MIMO layers or antennas deeply depends on the traffic characteristic and channel condition.

Observation 5: The UE adaptation of the number of MIMO layers is beneficial to apply in the scenario that traffic characteristic and channel condition are stable and stay long enough in “power saving friendly zone”.

Proposal 1: RRC configuration of the maximum layer and antenna number could be supported. The current layer restriction in the specification is the starting point.

Proposal 2: If new signaling of maximum MIMO layer indication is introduced for power saving, the UE behaviour of the adaptation should be discussed.

Proposal 3: For downlink case, the UE assistance information in support of the MIMO layer and antenna domain adaptation is not supported. For uplink case, the UE assistance information could be studied, which can be considered in the next release.

# References

[1] RP-191607, New WID: UE Power Saving in NR, 3GPP TSG RAN Meetings #84

[2] 3GPP TR 38.840 V16.0.0, Study on User Equipment (UE) power saving in NR