3GPP TSG RAN WG1 Meeting #98	R1-1908999

Prague, CZ, 26th – 30th August 2019

Title:	NTN LLS Calibration Assumptions

Source: 	Fraunhofer IIS, Fraunhofer HHI

Type:	Discussion

Document for:	Approval

Agenda Item:	7.2.5.1

Study Item: 	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Introduction

In June 2018, the study item “Study on NR to support non-terrestrial networks” was completed. Potential impacts have been identified (see [1] for the full list) and solutions are currently investigated in the study item “Solutions on NR to support non-terrestrial networks”. From a RAN1 perspective, TUs have been allocated starting from April 2019 to November 2019. The following objectives for RAN 1 have been approved in [2] :

Consolidation of potential impacts as initially identified in TR 38.811 and identification of related solutions if needed [RAN1]:

- Physical layer control procedures (e.g. CSI feedback, power control)

- Uplink Timing advance/RACH procedure including PRACH sequence/format/message

- Making retransmission mechanisms at the physical layer more delay-tolerant as appropriate. This may also include capability to deactivate the HARQ mechanisms.

Performance assessment of NR in selected deployment scenarios (LEO based satellite access, GEO based satellite access) through link level (Radio link) and system level (cell) simulations [RAN1].

This contribution focuses on the issue of NR performance assessment through link level simulations. In order to compare link level simulations (LLS) from different companies, calibration of the simulation may be beneficial. In this contribution we propose a set of parameters to be considered for LLS calibration.

# NTN LLS Calibration Assumptions

In the last RAN1#97 meeting in Reno it has been agreed that calibration for system level simulations (SLS) is mandatory. This triggers the question, if calibration of LLS is also needed.

Proposal 1: RAN1 shall decide whether LLS calibration is needed or not.

It has also been agreed in the last meeting, that the following LLS simulations need to be performed: DL synchronisation, PRACH performance, and data transmission performance evaluation. Therefore, three tables have been provided, which specify the current set of parameters for these simulations. Based on the given tables in [1] with the LLS parameters for data transmission performance evaluation and impairments due to satellite payload and satellite movement, a first table for LLS calibration assumptions is given below.

Table X.13: LLS assumptions for calibration

|  | S-band | Ka-band |
| --- | --- | --- |
| Carrier Frequency | DL: 2 GHz UL: 2 GHz | DL: 20 GHz UL: 30 GHz |
| Subcarrier Spacing(s) | 15 kHz | 120 kHz |
| Bandwidth | 30 MHz, FFS | 400 MHz, FFS |
| Modulation order Coding rate | FFS: specify MCS from the tables in 38.214 for DL and UL | FFS: : specify MCS from the tables in 38.214 for DL and UL |
| Intended data allocation | 16 RBs: FFS | 50 RBs: FFS |
|  | • Note: Error-free PDCCH decoding is assumed. • First 2 OFDM symbols for PDCCH, and following 12 OFDM symbols for data channel • One-port DMRS (12 REs) |  |
| Total number of PRBs | FFS: relevant in DL if payload HPA non-linearity is enabled to model ICI from other allocations, as proposed in R1-1907369 |  |
| Channel Model | For GEO: Baseline TDL model in TR38.811, with delay scaling considering the mean delay spread and mean K-factor for suburban LOS elevation angle of 10 degrees. For LEO: Baseline TDL model in TR38.811, with delay scaling considering the mean delay spread and mean K-factor for suburban LOS elevation angle of 30 degrees. |  |
| DL RS | DMRS and PT-RS |  |
| Antenna Configuration at the TRP (satellite) | DL: 1 Tx UL: 1 Rx | DL: 1 Tx UL: 1 Rx |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter (1, 1, 2) with directional antenna element (HPBW=65deg, directivity 8 dBi) |
| UE speed | 3 km/h | 0 km/h, 1000 km/h |
| UE elevation angle | For GEO: 10deg, For LEO: 30deg |  |
| Frequency Offset | Residual frequency error after DL synchronisation: FFS |  |
| Frequency drift | FFS: Max Doppler rate Satellite alt. = 1200 km :0.09 ppm/s Satellite alt. = 600 km : 0.27 ppm/s |  |
| Frequency tracking | FFS |  |
| Phase noise model | FFS: Table 6 from R1-1907481 (ESA) | FFS: Table 7 from R1-1907481 (ESA) |
| PA nonlinearity model | FFS: [mandatory for throughput evaluation, needs to be captures for performance evaluation as well] Tables 8 and 9 from R1-1907481 (ESA) |  |
| Channel estimation | Realistic estimation, DMRS-based |  |
| Phase tracking | FFS: PT-RS based if phase noise model is enabled |  |
| Receiver type | FFS: MRC |  |
| Metrics | BLER vs SNR, - for the specified MCS - FFS: for specified input back-offs (IBOs) when PA nonlinearity model is enabled |  |

Proposal 2: RAN1 shall use the LLS assumptions for calibration given in Table X.13.

Proposal 3: RAN1 shall decide whether specific calibration for DL synchronisation performance and PRACH performance are needed.

# Conclusion

In this paper, the following observations and proposals have been made:

Proposal 1: RAN1 shall decide whether LLS calibration is needed or not.

Proposal 2: RAN1 shall use the LLS assumptions for calibration given in Table X.13.

Proposal 3: RAN1 shall decide whether specific calibration for DL synchronisation performance and PRACH performance are needed.

# References

[1] Chairman's Notes RAN1#97